14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

This content is from the eCFR and is authoritative but unofficial.

14 CFRPart91(Jan. 7, 2026)

Title 14 —Aeronautics and Space

Chapter I—Federal AviationAdministration, Department ofTransportation Subchapter F—Air  TrafficandGeneral Operating Rules

Part 91 General Operating and Flight Rules

Special FederalAviation RegulationNo. 50-2

Special Flight Rules in the Vicinity of the Grand Canyon National Park, AZ

Special FederalAviation RegulationNo. 60

Air Traffic Control System Emergency Operation

Special FederalAviation RegulationNo. 97

Special Operating Rules for the Conduct of Instrument Flight Rules (IFR) Area Navigation (RNAV) Operations using Global Positioning Systems (GPS) in Alaska

Special FederalAviation RegulationNo. 104

Prohibition Against Certain Flights by Syrian Air Carriers to the United States

14 CFRPart91(Jan.7,2026)(enhanceddisplay)

page 1of243

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFRPart91(Jan. 7, 2026)

Subpart A General

§ 91.1   Applicability.

§ 91.3    Responsibility and authority of the pilot in command.

§ 91.5    Pilot in command of aircraft requiring more than one required pilot. § 91.7    Civil aircraft airworthiness.

§ 91.9   Civil aircraft flight manual, marking, and placard requirements. § 91.11  Prohibition on interference with crewmembers.

§ 91.13 Careless or reckless operation. § 91.15 Dropping objects.

§ 91.17 Alcohol or drugs.

§ 91.19 Carriage of narcotic drugs, marihuana, and depressant or stimulant drugs or substances.

§ 91.21 Portable electronic devices.

§ 91.23 Truth-in-leasing clause requirement in leases and conditional sales contracts.

§ 91.25  Aviation Safety Reporting Program: Prohibition against use of reports for enforcement purposes.

§§ 91.27-91.99 [Reserved] Subpart B  Flight Rules

General

§ 91.101 Applicability.

§ 91.103 Preflight action.

§ 91.105 Flight crewmembers at stations.

§ 91.107 Use of safety belts, shoulder harnesses, and child restraint systems. § 91.108 Use of supplemental restraint systems.

§ 91.109  Flight instruction; Simulated instrument flight and certain flight tests. § 91.111 Operating near other aircraft.

§ 91.113 Right-of-way rules: Except water operations. § 91.115 Right-of-way rules: Water operations.

§ 91.117 Aircraft speed.

§ 91.119 Minimum safe altitudes: General. § 91.121 Altimeter settings.

§ 91.123 Compliance with ATC clearances and instructions. § 91.125 ATC light signals.

§ 91.126 Operating on or in the vicinity of an airport in Class G airspace. § 91.127 Operating on or in the vicinity of an airport in Class E airspace. § 91.129 Operations in Class D airspace.

§ 91.130 Operations in Class C airspace. § 91.131 Operations in Class B airspace. § 91.133 Restricted and prohibited areas.

14 CFRPart91(Jan.7,2026)(enhanceddisplay)

page 2of243

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFRPart91(Jan. 7, 2026)

§ 91.135 Operations in Class A airspace.

§ 91.137  Temporary flight restrictions in the vicinity of disaster/hazard areas.

§ 91.138 Temporary flight restrictions in national disaster areas in the State of Hawaii. § 91.139 Emergency air traffic rules.

§ 91.141  Flight restrictions in the proximity of the Presidential and other parties. § 91.143  Flight limitation in the proximity of space flight operations.

§ 91.144  Temporary restriction on flight operations during abnormally high barometric pressure conditions.

§ 91.145  Management of aircraft operations in the vicinity of aerial demonstrations and major sporting events.

§ 91.146 Passenger-carrying flights for the benefit of a charitable, nonprofit, or community event.

§ 91.147 Passenger-carrying flights for compensation or hire. §§ 91.148-91.149 [Reserved]

Visual Flight Rules

§ 91.151 Fuel requirements for flight in VFR conditions. § 91.153 VFR flight plan: Information required.

§ 91.155 Basic VFR weather minimums.

§ 91.157 Special VFR weather minimums.

§ 91.159 VFR cruising altitude or flight level.

§ 91.161 Special awareness training required for pilots flying under visual flight rules within a 60-nautical mile radius of the Washington, DC VOR/DME.

§§ 91.162-91.165 [Reserved] Instrument Flight Rules

§ 91.167 Fuel requirements for flight in IFR conditions. § 91.169 IFR flight plan: Information required.

§ 91.171 VOR equipment check for IFR operations. § 91.173 ATC clearance and flight plan required.

§ 91.175 Takeoff and landing under IFR.

§ 91.176 Straight-in landing operations below DA/DH or MDA using an enhanced flight vision system (EFVS) under IFR.

§ 91.177 Minimum altitudes for IFR operations. § 91.179 IFR cruising altitude or flight level.

§ 91.180 Operations within airspace designated as Reduced Vertical Separation Minimum airspace.

§ 91.181 Course to be flown.

§ 91.183 IFR communications.

§ 91.185 IFR operations: Two-way radio communications failure.

§ 91.187 Operation under IFR in controlled airspace: Malfunction reports.

14 CFRPart91(Jan.7,2026)(enhanceddisplay)

page 3 of243

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFRPart91(Jan. 7, 2026)

§ 91.189 Category II and III operations: General operating rules. § 91.191 Category II and Category III manual.

§ 91.193 Certificate of authorization for certain Category II operations. §§ 91.195-91.199 [Reserved]

Subpart C  Equipment, Instrument, and Certificate Requirements § 91.201 [Reserved]

§ 91.203  Civil aircraft: Certifications required.

§ 91.205  Powered civil aircraft with standard U.S. airworthiness certificates: Instrument and equipment requirements.

§ 91.207  Emergency locator transmitters. § 91.209  Aircraft lights.

§ 91.211 Supplemental oxygen.

§ 91.213 Inoperative instruments and equipment.

§ 91.215 ATC transponder and altitude reporting equipment and use.

§ 91.217 Data correspondence between automatically reported pressure altitude data and the pilot's altitude reference.

§ 91.219 Altitude alerting system or device: Turbojet-powered civil airplanes. § 91.221  Traffic alert and collision avoidance system equipment and use.

§ 91.223 Terrain awareness and warning system.

§ 91.225 Automatic Dependent Surveillance-Broadcast (ADS-B) Out equipment and use.

§ 91.227 Automatic Dependent Surveillance-Broadcast (ADS-B) Out equipment performance requirements.

§§ 91.228-91.299 [Reserved] Subpart D  Special Flight Operations

§ 91.301 [Reserved]

§ 91.303  Aerobatic flight. § 91.305 Flight test areas.

§ 91.307 Parachutes and parachuting.

§ 91.309 Towing: Gliders and unpowered ultralight vehicles. § 91.311 Towing: Other than under § 91.309.

§ 91.313  Restricted category civil aircraft: Operating limitations. § 91.315  Limited category civil aircraft: Operating limitations.

§ 91.317  Provisionally certificated civil aircraft: Operating limitations.

§ 91.319  Aircraft having experimental certificates: Operating limitations. § 91.321 Carriage of candidates in elections.

§ 91.323  Increased maximum certificated weights for certain airplanes operated in Alaska. § 91.325  Primary category aircraft: Operating limitations.

§ 91.326  Exception to operating certain aircraft for the purposes of flight training, flightcrew member checking, or flightcrew member testing.

14 CFRPart91(Jan.7,2026)(enhanceddisplay)

page 4of243

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFRPart91(Jan. 7, 2026)

§ 91.327  Aircraft issued a special airworthiness certificate in the light-sport category: Operating limitations.

§§ 91.328-91.330 [Reserved] § 91.331 xxx

§§ 91.332-91.399 [Reserved]

Subpart E  Maintenance, Preventive Maintenance, and Alterations § 91.401 Applicability.

§ 91.403 General.

§ 91.405 Maintenance required.

§ 91.407 Operation after maintenance, preventive maintenance, rebuilding, or alteration. § 91.409  Inspections.

§ 91.410 [Reserved]

§ 91.411   Altimeter system and altitude reporting equipment tests and inspections. § 91.413 ATC transponder tests and inspections.

§ 91.415  Changes to aircraft inspection programs. § 91.417 Maintenance records.

§ 91.419 Transfer of maintenance records.

§ 91.421 Rebuilt engine maintenance records. §§ 91.423-91.499 [Reserved]

Subpart F  Large and Turbine-Powered Multiengine Airplanes and Fractional Ownership Program Aircraft

§ 91.501 Applicability.

§ 91.503  Flying equipment and operating information.

§ 91.505  Familiarity with operating limitations and emergency equipment. § 91.507 Equipment requirements: Over-the-top or night VFR operations.

§ 91.509 Survival equipment for overwater operations.

§ 91.511  Communication and navigation equipment for overwater operations. § 91.513 Emergency equipment.

§ 91.515 Flight altitude rules.

§ 91.517  Passenger information. § 91.519 Passenger briefing.

§ 91.521 Shoulder harness. § 91.523 Carry-on baggage. § 91.525 Carriage of cargo.

§ 91.527 Operating in icing conditions. § 91.529 Flight engineer requirements.

§ 91.531 Second in command requirements. § 91.533  Flight attendant requirements.

§ 91.535 Stowage of food, beverage, and passenger service equipment during aircraft

14 CFRPart91(Jan.7,2026)(enhanceddisplay)

page 5of243

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFRPart91(Jan. 7, 2026)

movement on the surface, takeoff, and landing. §§ 91.536-91.599 [Reserved]

Subpart G Additional Equipment and Operating Requirements for Large and Transport Category Aircraft

§ 91.601 Applicability.

§ 91.603 Aural speed warning device.

§ 91.605  Transport category civil airplane weight limitations.

§ 91.607 Emergency exits for airplanes carrying passengers for hire. § 91.609 Flight data recorders and cockpit voice recorders.

§ 91.611  Authorization for ferry flight with one engine inoperative. § 91.613  Materials for compartment interiors.

§§ 91.615-91.699 [Reserved]

Subpart H Foreign Aircraft Operations and Operations of U.S.-Registered Civil Aircraft Outside of the United States; and Rules Governing Persons on Board Such Aircraft

§ 91.701 Applicability.

§ 91.702 Persons on board.

§ 91.703 Operations of civil aircraft of U.S. registry outside of the United States. § 91.705 [Reserved]

§ 91.706 Operations within airspace designed as Reduced Vertical Separation Minimum Airspace.

§ 91.707 Flights between Mexico or Canada and the United States. § 91.709 Operations to Cuba.

§ 91.711 Special rules for foreign civil aircraft.

§ 91.713 Operation of civil aircraft of Cuban registry.

§ 91.715  Special flight authorizations for foreign civil aircraft. §§ 91.717-91.799 [Reserved]

Subpart I    Operating Noise Limits

§ 91.801  Applicability: Relation to part 36.

§ 91.803  Part 125 operators: Designation of applicable regulations. § 91.805 Final compliance: Subsonic airplanes.

§§ 91.807-91.813 [Reserved]

§ 91.815  Agricultural and fire fighting airplanes: Noise operating limitations. § 91.817  Civil aircraft sonic boom.

§ 91.818  Special flight authorization to exceed Mach 1.

§ 91.819  Civil supersonic airplanes that do not comply with part 36. § 91.821  Civil supersonic airplanes: Noise limits.

§§ 91.823-91.849 [Reserved] § 91.851  Definitions.

14 CFRPart91(Jan.7,2026)(enhanceddisplay)

page 6of243

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFRPart91(Jan. 7, 2026)

§ 91.853 Final compliance: Civil subsonic airplanes. § 91.855 Entry and nonaddition rule.

§ 91.857 Stage 2 operations outside of the 48 contiguous United States.

§ 91.858 Special flight authorizations for non-revenue Stage 2 operations. § 91.859 Modification to meet Stage 3, Stage 4, or Stage 5 noise levels.

§ 91.861 Base level.

§ 91.863 Transfers of Stage 2 airplanes with base level.

§ 91.865 Phased compliance for operators with base level. § 91.867  Phased compliance for new entrants.

§ 91.869 Carry-forward compliance.

§ 91.871  Waivers from interim compliance requirements. § 91.873  Waivers from final compliance.

§ 91.875 Annual progress reports.

§ 91.877  Annual reporting of Hawaiian operations. §§ 91.879-91.880 [Reserved]

§ 91.881 Final compliance: Civil subsonic jet airplanes weighing 75,000 pounds or less. § 91.883  Special flight authorizations for jet airplanes weighing 75,000 pounds or less. §§ 91.884-91.899 [Reserved]

Subpart J  Waivers

§ 91.901 [Reserved]

§ 91.903 Policy and procedures.

§ 91.905  List of rules subject to waivers. §§ 91.907-91.999 [Reserved]

Subpart K  Fractional Ownership Operations § 91.1001       Applicability.

§ 91.1002       Compliance date.

§ 91.1003       Management contract between owner and program manager. § 91.1005        Prohibitions and limitations.

§ 91.1007       Flights conducted under part 121 or part 135 of this chapter. Operational Control

§ 91.1009  Clarification of operational control.

§ 91.1011 Operational control responsibilities and delegation. § 91.1013 Operational control briefing and acknowledgment.

Program Management

§ 91.1014 Issuing or denying management specifications. § 91.1015  Management specifications.

§ 91.1017 Amending program manager's management specifications. § 91.1019 Conducting tests and inspections.

§ 91.1021  Internal safety reporting and incident/accident response.

14 CFRPart91(Jan.7,2026)(enhanceddisplay)

page 7of243

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFRPart91(Jan. 7, 2026)

§ 91.1023 Program operating manual requirements. § 91.1025 Program operating manual contents.

§ 91.1027 Recordkeeping.

§ 91.1029 Flight scheduling and locating requirements.

§ 91.1031 Pilot in command or second in command: Designation required. § 91.1033 Operating information required.

§ 91.1035 Passenger awareness.

§ 91.1037 Large transport category airplanes: Turbine engine powered; Limitations; Destination and alternate airports.

§ 91.1039 IFR takeoff, approach and landing minimums. § 91.1041 Aircraft proving and validation tests.

§ 91.1043 [Reserved]

§ 91.1045 Additional equipment requirements.

§ 91.1047 Drug and alcohol misuse education program. § 91.1049 Personnel.

§ 91.1050 Employment of former FAA employees. § 91.1053 Crewmember experience.

§ 91.1055  Pilot operating limitations and pairing requirement.

§ 91.1057 Flight, duty and rest time requirements: All crewmembers.

§ 91.1059  Flight time limitations and rest requirements: One or two pilot crews. § 91.1061 Augmented flight crews.

§ 91.1062 Duty periods and rest requirements: Flight attendants. § 91.1063 Testing and training: Applicability and terms used.

§ 91.1065  Initial and recurrent pilot testing requirements.

§ 91.1067  Initial and recurrent flight attendant crewmember testing requirements. § 91.1069 Flight crew: Instrument proficiency check requirements.

§ 91.1071 Crewmember: Tests and checks, grace provisions, training to accepted standards. § 91.1073 Training program: General.

§ 91.1075 Training program: Special rules.

§ 91.1077 Training program and revision: Initial and final approval. § 91.1079 Training program: Curriculum.

§ 91.1081 Crewmember training requirements. § 91.1083 Crewmember emergency training.

§ 91.1085 Hazardous materials recognition training.

§ 91.1087  Approval of aircraft simulators and other training devices.

§ 91.1089  Qualifications: Check pilots (aircraft) and check pilots (simulator).

§ 91.1091  Qualifications: Flight instructors (aircraft) and flight instructors (simulator).

§ 91.1093  Initial and transition training and checking: Check pilots (aircraft), check pilots (simulator).

14 CFRPart91(Jan.7,2026)(enhanceddisplay)

page 8 of243

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFRPart91(Jan. 7, 2026)

§ 91.1095  Initial and transition training and checking: Flight instructors (aircraft), flight instructors (simulator).

§ 91.1097  Pilot and flight attendant crewmember training programs. § 91.1099 Crewmember initial and recurrent training requirements.

§ 91.1101 Pilots: Initial, transition, and upgrade ground training.

§ 91.1103 Pilots: Initial, transition, upgrade, requalification, and differences flight training. § 91.1105  Flight attendants: Initial and transition ground training.

§ 91.1107 Recurrent training.

§ 91.1109 Aircraft maintenance: Inspection program. § 91.1111  Maintenance training.

§ 91.1113 Maintenance recordkeeping.

§ 91.1115 Inoperable instruments and equipment.

§ 91.1411  Continuous airworthiness maintenance program use by fractional ownership program manager.

§ 91.1413 CAMP: Responsibility for airworthiness. § 91.1415 CAMP: Mechanical reliability reports.

§ 91.1417 CAMP: Mechanical interruption summary report. § 91.1423 CAMP: Maintenance organization.

§ 91.1425 CAMP: Maintenance, preventive maintenance, and alteration programs. § 91.1427 CAMP: Manual requirements.

§ 91.1429 CAMP: Required inspection personnel.

§ 91.1431 CAMP: Continuing analysis and surveillance.

§ 91.1433 CAMP: Maintenance and preventive maintenance training program. § 91.1435 CAMP: Certificate requirements.

§ 91.1437 CAMP: Authority to perform and approve maintenance. § 91.1439 CAMP: Maintenance recording requirements.

§ 91.1441 CAMP: Transfer of maintenance records.

§ 91.1443 CAMP: Airworthiness release or aircraft maintenance log entry. Subpart L   Continued Airworthiness and Safety Improvements

§ 91.1501 Purpose and definition. § 91.1503 [Reserved]

§ 91.1505 Repairs assessment for pressurized fuselages. § 91.1507 Fuel tank system inspection program.

Subpart M Special Federal Aviation Regulations

§ 91.1603 Special Federal Aviation Regulation No. 112—Prohibition Against Certain Flights in the Territory and Airspace of Libya.

§ 91.1605 Special Federal Aviation Regulation No. 77—Prohibition Against Certain Flights in the Baghdad Flight Information Region (FIR) (ORBB).

§ 91.1607[Reserved]

14 CFRPart91(Jan.7,2026)(enhanceddisplay)

page 9 of243

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFRPart91(Jan. 7, 2026)

§ 91.1609 Special Federal Aviation Regulation No. 114—Prohibition Against Certain Flights in the Damascus Flight Information Region (FIR) (OSTT).

§ 91.1611 Special Federal Aviation Regulation No. 115—Prohibition Against Certain Flights in Specified Areas of the Sanaa Flight Information Region (FIR) (OYSC).

§ 91.1613 Special Federal Aviation Regulation No. 107—Prohibition Against Certain Flights in the Territory and Airspace of Somalia.

§ 91.1615 Special Federal Aviation Regulation No. 79—Prohibition Against Certain Flights in the Pyongyang Flight Information Region (FIR) (ZKKP).

§ 91.1617 Special Federal Aviation Regulation No. 117—Prohibition Against Certain Flights in the Tehran Flight Information Region (FIR) (OIIX).

§ 91.1619 Special Federal Aviation Regulation No. 119—Prohibition Against Certain Flights in the Kabul Flight Information Region (FIR) (OAKX).

Subpart N Mitsubishi MU-2B Series Special Training, Experience, and Operating Requirements

§ 91.1701 Applicability.

§ 91.1703 Compliance and eligibility. § 91.1705 Required pilot training.

§ 91.1707 Training program hours.

§ 91.1709 Training program approval. § 91.1711 Aeronautical experience.

§ 91.1713 Instruction, checking, and evaluation.

§ 91.1715 Currency requirements and flight review. § 91.1717 Operating requirements.

§ 91.1719 Credit for prior training.

§ 91.1721 Incorporation by reference.

Appendix Ato Part 91

Category II Operations: Manual, Instruments, Equipment, and Maintenance

Appendixes B—C[Reserved]

Appendix Dto Part 91 Airports/Locations: Special

Operating Restrictions

Appendix E to Part91

Airplane Flight Recorder Specifications

Appendix F toPart 91

Helicopter Flight Recorder Specifications

Appendix Gto Part 91

Operations in Reduced Vertical Separation Minimum (RVSM) 14 CFRPart91(Jan.7,2026)(enhanceddisplay)

page 10 of243

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

Airspace

14 CFRPart91(Jan. 7, 2026)

PART 91—GENERAL OPERATING AND FLIGHT RULES

Authority: 49 U.S.C. 106(f), 40101, 40103, 40105, 40113, 40120, 44101, 44111, 44701, 44704, 44709, 44711, 44712, 44715,44716, 44717, 44722, 44740, 46306, 46315, 46316, 46504, 46506-46507, 47122, 47508,47528-47531, 47534; Pub. L. 112-95, 126 Stat. 11; Pub. L. 114-190, 130 Stat. 615 (49 U.S.C. 44703 note); sec. 828, Pub. L. 118-63, 138 Stat. 1330 (49 U.S.C. 44703 note); articles 12 and 29 of the Convention on International Civil Aviation, 61 Stat. 1180.

Special FederalAviation RegulationNo. 50-2—Special Flight Rules in the Vicinity ofthe Grand Canyon National Park,AZ

Section 1. Applicability. This rule prescribes special operating rules for all persons operating aircraft in the following airspace, designated as the Grand Canyon National Park Special Flight Rules Area:

That airspace extending upward from the surface up to but not including 14,500 feet MSL within an area bounded by a line beginning at lat. 36°09′30″ N., long. 114°03′00″ W.; northeast to lat. 36°14′00″ N., long. 113°09′50″ W.; thence northeast along the boundary ofthe Grand Canyon National Park to lat. 36°24′47″ N., long. 112°52′00″ W.; to lat. 36°30′30″ N., long. 112°36′15″ W. to lat. 36°21′30″ N., long. 112°00′00″ W. to lat. 36°35′30″ N., long. 111°53′10″ W., to lat. 36°53′00″ N., long. 111°36′45″ W. to lat. 36°53′00″ N., long. 111°33′00″ W.; to lat. 36°19′00″ N., long. 111°50′50″ W.; to lat. 36°17′00″ N., long. 111°42′00″ W.; to lat. 35°59′30″ N., long. 111°42′00″ W.; to lat. 35°57′30″ N., long. 112°03′55″ W.; thence counterclockwise via the 5 statute mile radius ofthe Grand Canyon Airport airport reference point (lat. 35°57′09″ N., long. 112°08′47″ W.) to lat. 35°57′30″ N., long. 112°14′00″ W.; to lat. 35°57′30″ N., long. 113°11′00″ W.; to lat. 35°42′30″ N., long. 113°11′00″ W.; to 35°38′30″ N.; long. 113°27′30″ W.; thence counterclockwise via the 5

statute mile radius ofthe Peach Springs VORTAC to lat. 35°41′20″ N., long. 113°36′00″ W.; to lat. 35°55′25″ N., long. 113°49′10″ W.; to lat. 35°57′45″ N., 113°45′20″ W.; thence northwest along the park boundaryto lat. 36°02′20″ N., long. 113°50′15″ W.; to 36°00′10″ N., long. 113°53′45″ W.; thence to the point of beginning.

Section 3. Aircraft operations: general. Except in an emergency, no person may operate an aircraft in the Special Flight Rules, Area under VFR on or after September 22, 1988, or under IFR on or afterApril 6, 1989, unless the operation—

(a)   Is conducted in accordance with the following procedures:

Note: The following procedures do not relieve the pilot from see-and-avoid responsibility or compliance with FAR 91.119.

(1)  Unless necessary to maintain a safe distance from other aircraft or terrain— (i)   Remain clear ofthe areas described in Section 4; and

(ii)   Remain at or above the following altitudes in each sector ofthe canyon:

14 CFRSpecial-Federal-Aviation-Regulation-No.-50-2“Section3.Aircraftoperations:general” (a)(1)(ii)

page 11of

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFRSpecial-Federal-Aviation-Regulation-No.-50-2“” (a)(2)

Eastern section from Lees Ferryto North Canyon and North Canyon to Boundary Ridge: as prescribed in Section 5.

Boundary Ridge to Supai Point (Yumtheska Point): 10,000 feet MSL.

Western section from Diamond Creek to the Grant Wash Cliffs: 8,000 feet MSL.

(2)  Proceed through the four flight corridors describe in Section 4 at the following altitudes unless otherwise authorized in writing by the responsible Flight Standards office:

Northbound

11,500 or

13,500 feet MSL

Southbound

>10,500 or >12,500 feet MSL

(b) (c)

(d) (e)

(f)

Is authorized in writing by the responsible Flight Standards office and is conducted in compliance with the conditions contained in that authorization. Normally authorization will be granted for operation in the areas described in Section 4 or below the altitudes listed in Section 5 only for operations of aircraft necessary for law enforcement, firefighting, emergency medical treatment/ evacuation of persons in the vicinity of the Park; for support of Park maintenance or activities; or for aerial access to and maintenance of other property located within the Special Flight Rules Area. Authorization may be issued on a continuing basis.

(1)   Prior to November 1, 1988, is conducted in accordance with a specific authorization to operate in that airspace incorporated in the operator's part 135 operations specifications in accordance with the provisions of SFAR 50-1, notwithstanding the provisions of Sections 4 and 5; and

(2)  On or after November 1, 1988, is conducted in accordance with a specific authorization to operate in that airspace incorporated in the operated in the operator's operations specifications and approved by the responsible Flight Standards office in accordance with the provisions of SFAR 50-2.

Is a search and rescue mission directed bythe U.S. Air Force Rescue Coordination Center.

Is conducted within 3 nautical miles of Whitmore Airstrip, Pearce Ferry Airstrip, North Rim Airstrip, Cliff Dwellers Airstrip, or Marble Canyon Airstrip at an altitudes less than 3,000 feet above airport elevation, forthe purpose of landing at ortaking off from that facility. Or

Is conducted under an IFR clearance and the pilot is acting in accordance with ATC instructions. An IFR flight plan may not be filed on a route or at an altitude that would require operation in an area described in Section 4.

14 CFRSpecial-Federal-Aviation-Regulation-No.-50-2“” (f)(enhanceddisplay)

page 12of243

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR Special-Federal-Aviation-Regulation-No.-50-2 “Section 4.Flight-freezones”

Section 4. Flight-free zones. Except in an emergency or if otherwise necessary for safety of flight, or unless otherwise authorized by the responsible Flight Standards office for a purpose listed in Section 3(b), no person may operate an aircraft in the Special Flight Rules Area within the following areas:

(a) (b)

(c) (d)

Desert View Flight-Free Zone. Within an area bounded by a line beginning at Lat. 35°59′30″ N., Long. 111°46′20″ W. to 35°59′30″ N., Long. 111°52′45″ W.; to Lat. 36°04′50″ N., Long. 111°52′00″ W.; to Lat. 36°06′00″ N., Long. 111°46′20″ W.; to the point of origin; but not including the airspace at and above 10,500 feet MSL within 1 mile of the western boundary of the zone. The area between the Desert View and Bright Angel Flight-Free Zones is designated the “Zuni Point Corridor.”

Bright Angel Flight-Free Zone. Within an area bounded by a line beginning at Lat. 35°59′30″ N., Long. 111°55′30″ W.; to Lat. 35°59′30″ N., Long. 112°04′00″ W.; thence counterclockwise via the 5 statute mile radius of the Grand Canyon Airport point (Lat. 35°57′09″ N., Long. 112°08′47″ W.) to Lat. 36°01′30″ N., Long. 112°11′00″ W.; to Lat. 36°06′15″ N., Long. 112°12′50″ W.; to Lat. 36°14′40″ N., Long. 112°08′50″ W.; to Lat. 36°14′40″ N., Long. 111°57′30″ W.; to Lat. 36°12′30″ N., Long. 111°53′50″ W.; to the point of origin; but not including the airspace at and above 10,500 feet MSL within 1 mile of the eastern boundary between the southern boundary and Lat. 36°04′50″ N. orthe airspace at and above 10,500 feet MSL within 2 miles ofthe northwest boundary. The area bounded by the Bright Angel and Shinumo Flight-Free Zones is designated the “Dragon Corridor.”

Shinumo Flight-Free Zone. Within an area bounded by a line beginning at Lat. 36°04′00″ N., Long. 112°16′40″ W.; northwest along the park boundary to a point at Lat. 36°12′47″ N., Long. 112°30′53″ W.; to Lat. 36°21′15″ N., Long. 112°20′20″ W.; east along the park boundaryto Lat. 36°21′15″ N., Long. 112°13′55″ W.; to Lat. 36°14′40″ N., Long. 112°11′25″ W.; to the point of origin. The area between the Thunder River/Toroweap and Shinumo Flight Free Zones is designated the “Fossil Canyon Corridor.”

Toroweap/Thunder River Flight-Free Zone. Within an area bounded by a line beginning at Lat. 36°22′45″ N., Long. 112°20′35″ W.; thence northwest along the boundary ofthe Grand Canyon National Parkto Lat. 36°17′48″ N., Long. 113°03′15″ W.; to Lat. 36°15′00″ N., Long. 113°07′10″ W.; to Lat. 36°10′30″ N., Long. 113°07′10″ W.; thence east along the Colorado River to the confluence of Havasu Canyon (Lat. 36°18′40″ N., Long. 112°45′45″ W.;) including that area within a 1.5 nautical mile radius of Toroweap Overlook (Lat. 36°12′45″ N., Long. 113°03′30″ W.); to the point of origin; but not including the following airspace designated as the “Tuckup Corridor”: at or above 10,500 feet MSL within 2 nautical miles either side of a line extending between Lat. 36°24′47″ N., Long. 112°48′50″ W. and Lat. 36°17′10″ N., Long. 112°48′50″ W.; to the point of origin.

Section 5. Minimum flightaltitudes. Except in an emergency or if otherwise necessary for safety of flight, or unless otherwise authorized by the responsible Flight Standards office for a purpose listed in Section 3(b), no person may operate an aircraft in the Special Flight Rules Area at an altitude lowerthan the following:

(a) (b) (c) (d) (e)

Eastern section from Lees Ferryto North Canyon: 5,000 feet MSL. Eastern section from North Canyon to Boundary Ridge: 6,000 feet MSL. Boundary Ridge to Supai (Yumtheska) Point: 7,500 feet MSL.

Supai Point to Diamond Creek: 6,500 feet MSL.

Western section from Diamond Creek to the Grand Wash Cliffs: 5,000 feet MSL.

Section 9. Termination date. Section 1. Applicability, Section 4, Flight-free zones, and Section 5. Minimum flight altitudes, expire on April 19, 2001.

14  CFRSpecial-Federal-Aviation-Regulation-No.-50-2“Section9.Terminationdate”(enhanceddisplay)

page 13 of243

2. 3.

4.

5. 6.

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

Note: [Removed]

14 CFRSpecial-Federal-Aviation-Regulation-No.-601.

[66 FR 1003, Jan. 4, 2001, as amendedat 66 FR 16584, Mar. 26, 2001; 72 FR 9846, Mar. 6, 2007; DocketFAA-2018-0119,Amdt. 91-350, 83 FR 9171, Mar. 5, 2018]

Special FederalAviation RegulationNo. 60—Air Traffic Control System EmergencyOperation

1.  Each person shall, before conducting any operation underthe Federal Aviation Regulations (14 CFR chapter I), be familiar with all available information concerning that operation, including Notices to Airmen issued under § 91.139 and, when activated, the provisions of the National Air Traffic Reduced Complement Operations Plan available for inspection at operating air traffic facilities and Regional air traffic division offices, and the General Aviation Reservation Program. No operator may change the designated airport of intended operation for any flight contained in the October 1, 1990, OAG.

Notwithstanding any provision of the Federal Aviation Regulations to the contrary, no person may operate an aircraft in the Air Traffic Control System:

a. b.

Contrary to any restriction, prohibition, procedure or other action taken by the Director of the Office of Air Traffic Systems Management (Director) pursuant to paragraph 3 of this regulation and

announced in a Notice to Airmen pursuant to § 91.139 of the Federal Aviation Regulations.

When the National Air Traffic Reduced Complement Operations Plan is activated pursuant to paragraph 4 ofthis regulation, except in accordance with the pertinent provisions of the National Air Traffic Reduced Complement Operations Plan.

Prior to or in connection with the implementation of the RCOP, and as conditions warrant, the Director is authorized to:

a. b. c.

Restrict, prohibit, or permit VFR and/or IFR operations at any airport, Class B airspace area, Class C airspace area, or other class of controlled airspace.

Give priority at any airport to flights that are of military necessity, or are medical emergency flights, Presidential flights, and flights transporting critical Government employees.

Implement, at any airport, traffic management procedures, that may include reduction of flight operations. Reduction of flight operations will be accomplished, to the extent practical, on a pro rata basis among and between air carrier, commercial operator, and general aviation operations. Flights cancelled under this SFAR at a high density traffic airport will be considered to have been operated for purposes of part 93 of the Federal Aviation Regulations.

The Director may activate the National Air Traffic Reduced Complement Operations Plan at any time he finds that it is necessary forthe safety and efficiency ofthe National Airspace System. Upon activation of the RCOP and notwithstanding any provision of the FAR to the contrary, the Director is authorized to suspend or modify any airspace designation.

Notice of restrictions, prohibitions, procedures and other actions taken by the Director underthis regulation with respect to the operation ofthe Air Traffic Control system will be announced in Notices to Airmen issued pursuant to § 91.139 ofthe Federal Aviation Regulations.

The Director may delegate his authority under this regulation to the extent he considers necessary forthe safe and efficient operation of the National Air Traffic Control System.

14 CFRSpecial-Federal-Aviation-Regulation-No.-60 6.(enhanceddisplay)

page 14of243

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFRSpecial-Federal-Aviation-Regulation-No.-606.

(Authority: 49 U.S.C. app. 1301(7), 1303, 1344, 1348, 1352 through 1355, 1401, 1421 through 1431, 1471, 1472, 1502, 1510, 1522, and 2121 through 2125; articles 12, 29, 31, and 32(a) of the Convention on International Civil Aviation (61 stat. 1180); 42 U.S.C. 4321 et seq.; E.O. 11514, 35 FR 4247, 3 CFR, 1966-1970 Comp., p. 902; 49 U.S.C. 106(g))

[Docket 26351, 55 FR 40760, Oct. 4, 1990, as amended byAmdt. 91-227, 56 FR 65652, Dec. 17, 1991]

Special FederalAviation RegulationNo. 97—Special Operating Rules forthe Conduct of Instrument Flight Rules (IFR) Area Navigation(RNAV) Operations using Global Positioning Systems (GPS) in Alaska

Those persons identified in Section 1 may conduct IFR en route RNAV operations in the State of Alaska and its airspace on published air traffic routes using TSO C145a/C146a navigation systems as the only means of IFR navigation. Despite contrary provisions of parts 71, 91, 95, 121, 125, and 135 of this chapter, a person may operate aircraft in accordance with this SFAR ifthe following requirements are met.

Section 1. Purpose, use, andlimitations

a.

b. c.

This SFAR permits TSO C145a/C146a GPS (RNAV) systems to be used for IFR en route operations in the United States airspace over and near Alaska (as set forth in paragraph c ofthis section) at Special Minimum En Route Altitudes (MEA) that are outside the operational service volume of ground-based navigation aids, if the aircraft operation also meets the requirements of sections 3 and 4 of this SFAR.

Certificate holders and part 91 operators may operate aircraft under this SFAR provided that they comply with the requirements of this SFAR.

Operations conducted underthis SFAR are limited to United States Airspace within and near the State of Alaska as defined in the following area description:

From 62°00′00.000″ N, Long. 141°00′00.00″ W.; to Lat. 59°47′54.11″ N., Long. 135°28′38.34″ W.; to Lat. 56°00′04.11″ N., Long. 130°00′07.80″ W.; to Lat. 54°43′00.00″ N., Long. 130°37′00.00″ W.; to Lat. 51°24′00.00″ N., Long. 167°49′00.00″ W.; to Lat. 50°08′00.00″ N., Long. 176°34′00.00″ W.; to Lat. 45°42′00.00″ N., Long. −162°55′00.00″ E.; to Lat. 50°05′00.00″ N., Long. −159°00′00.00″ E.; to Lat. 54°00′00.00″ N., Long. −169°00′00.00″ E.; to Lat. 60°00 00.00″ N., Long. −180°00′ 00.00″ E; to Lat. 65°00′00.00″ N., Long. 168°58′23.00″ W.; to Lat. 90°00′00.00″ N., Long. 00°00′0.00″ W.; to Lat. 62°00′00.000″ N, Long. 141°00′00.00″ W.

(d) No person may operate an aircraft under IFR during the en route portion of flight belowthe standard MEA or at the special MEA unless the operation is conducted in accordance with sections 3 and 4 of this SFAR.

Section 2. Definitions and abbreviations

For the purposes ofthis SFAR, the following definitions and abbreviations apply.

Area navigation (RNAV). RNAV is a method of navigation that permits aircraft operations on any desired flight path.

Area navigation (RNAV) route. RNAV route is a published route based on RNAV that can be used by suitably equipped aircraft.

14 CFRSpecial-Federal-Aviation-Regulation-No.-60 6.(enhanceddisplay)

page 15of243

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFRSpecial-Federal-Aviation-Regulation-No.-606.

Certificate holder. A certificate holder means a person holding a certificate issued under part  119orpart 125 of this chapter or holding operations specifications issued under part 129 of this chapter.

Global Navigation Satellite System (GNSS). GNSS is a world-wide position and time determination system that uses satellite ranging signals to determine user location. It encompasses all satellite ranging technologies, including GPS and additional satellites. Components ofthe GNSS include GPS, the Global Orbiting Navigation Satellite System, and WAAS satellites.

Global Positioning System (GPS). GPS is a satellite-based radio navigational, positioning, and time transfer system. The system provides highly accurate position and velocity information and precise time on a continuous global basis to properly equipped users.

Minimum crossing altitude (MCA). The minimum crossing altitude (MCA) applies to the operation of an aircraft proceeding to a higher minimum en route altitude when crossing specified fixes.

Required navigation system. Required navigation system means navigation equipment that meets the performance requirements of TSO C145a/C146a navigation systems certified for IFR en route operations.

Route segment. Route segment is a portion of a route bounded on each end by a fix or NAVAID.

Special MEA. Special MEA refers to the minimum en route altitudes, using required navigation systems, on published routes outside the operational service volume of ground-based navigation aids and are depicted on the published Low Altitude and High Altitude En Route Charts using the color blue and with the suffix “G.” For example, a GPS MEA of 4000 feet MSL would be depicted using the color blue, as 4000G.

Standard MEA. Standard MEA refers to the minimum en route IFR altitude on published routes that uses ground-based navigation aids and are depicted on the published Low Altitude and High Altitude En Route Charts using the color black.

Station referenced. Station referenced refers to radio navigational aids or fixes that are referenced by ground based navigation facilities such as VOR facilities.

Wide Area Augmentation System (WAAS). WAAS is an augmentation to GPS that calculates GPS integrity and correction data on the ground and uses geo-stationary satellites to broadcast GPS integrity and correction data to GPS/WAAS users and to provide ranging signals. It is a safety critical system consisting of a ground network of reference and integrity monitor data processing sites to assess current GPS performance, as well as a space segment that broadcasts that assessment to GNSS users to support en route through precision approach navigation. Users of the system include all aircraft applying the WAAS data and ranging signal.

Section 3. Operational Requirements

To operate an aircraft under this SFAR, the following requirements must be met:

a.   Training and qualification for operations and maintenance personnel on required navigation equipment used under this SFAR.

b.   Use authorized procedures for normal, abnormal, and emergency situations unique to these operations, including degraded navigation capabilities, and satellite system outages.

14 CFRSpecial-Federal-Aviation-Regulation-No.-60 6.(enhanceddisplay)

page 16of243

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR Special-Federal-Aviation-Regulation-No.-1041.

c. d.

e.

For certificate holders, training of flight crewmembers and other personnel authorized to exercise operational control on the use of those procedures specified in paragraph b of this section.

Part 129 operators must have approval from the State of the operator to conduct operations in accordance with this SFAR.

In order to operate under this SFAR, a certificate holder must be authorized in operations specifications.

Section 4. Equipment Requirements

a. b.

The certificate holder must have properly installed, certificated, and functional dual required navigation systems as defined in section 2 ofthis SFAR for the en route operations covered under this SFAR.

When the aircraft is being operated under part 91, the aircraft must be equipped with at least one properly installed, certificated, and functional required navigation system as defined in section 2 of this SFAR for the en route operations covered under this SFAR.

Section 5. Expiration date

This Special Federal Aviation Regulation will remain in effect until rescinded. [Docket FAA-2003-14305, 68 FR 14077, Mar. 21, 2003]

Special FederalAviation RegulationNo. 104—Prohibition AgainstCertain Flights by SyrianAir Carriers totheUnited States

1. 2. 3. 4.

5.

Applicability. This Special Federal Aviation Regulation (SFAR) No. 104 applies to any air carrier owned or controlled by Syria that is engaged in scheduled international air services.

Special flight restrictions. Except as provided in paragraphs 3 and 4 of this SFAR No. 104, no air carrier described in paragraph 1 may take off from or land in the territory of the United States.

Permitted operations. This SFAR does not prohibit overflights ofthe territory ofthe United States by any air carrier described in paragraph 1.

Emergency situations. In an emergency that requires immediate decision and action for the safety of the flight, the pilot in command of an aircraft of any air carrier described in paragraph 1 may deviate from this SFAR to the extent required by that emergency. Each person who deviates from this rule must, within 10 days of the deviation, excluding Saturdays, Sundays, and Federal holidays, submit to the responsible Flight Standards office a complete report ofthe operations or the aircraft involved in the deviation, including a description of the deviation and the reasons therefor.

Duration. This SFAR No. 104 will remain in effect until further notice.

[Docket FAA-2004-17763, 69 FR 31719, June 4, 2004, as amendedbyDocketFAA-2018-0119,Amdt. 91-350, 83 FR 9171, Mar. 5, 2018]

14 CFRSpecial-Federal-Aviation-Regulation-No.-1045.(enhanced display)

page 17of243

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

Subpart A—General

14 CFR91.1

Source:

Docket 18334, 54 FR 34292, Aug. 18, 1989, unless otherwise noted.

§ 91.1Applicability.

(a)   Except as provided in paragraphs (b), (c), (e), and (f) ofthis section and §§ 91.701 and 91.703, this part prescribes rules governing the operation of aircraft within the United States, including the waters within 3 nautical miles of the U.S. coast.

(b)

(c) (d) (e)

(f)

(g)

Each person operating an aircraft in the airspace overlying the waters between 3 and 12 nautical miles from the coast of the United States must comply with §§ 91.1 through 91.21; §§ 91.101 through 91.143; §§ 91.151 through 91.159; §§ 91.167 through 91.193; § 91.203; § 91.205; §§ 91.209 through 91.217; § 91.221, § 91.225; §§ 91.303 through 91.319; §§ 91.323 through 91.327; § 91.605; § 91.609; §§ 91.703 through 91.715; and § 91.903.

This part applies to each person on board an aircraft being operated underthis part, unless otherwise specified.

This part also establishes requirements for operators to take actions to support the continued airworthiness of each aircraft.

This part does not apply to any aircraft or vehicle governed by part 103 of this chapter, or subparts B, C, or D of part 101 of this chapter.

Except as provided in §§ 107.13, 107.27, 107.47, 107.57, and 107.59 of this chapter, this part does not apply to any aircraft governed by part 107 of this chapter.

Additional requirements for powered-lift operations are set forth in part 194 of this chapter.

[Docket 18334, 54 FR 34292, Aug. 18, 1989, as amendedbyAmdt. 91-257, 64 FR 1079, Jan. 7, 1999;Amdt. 91-282, 69 FR 44880, July 27, 2004;Amdt. 91-297, 72 FR 63410, Nov. 8, 2007;Amdt. 91-314, 75 FR 30193, May28, 2010;Docket FAA-2015-0150, Amdt. 91-343, 81 FR 42208, June28, 2016; FAA-2023-1275, Amdt. 91-379, 89 FR 92485, Nov. 21, 2024]

§ 91.3 Responsibility andauthority ofthe pilot in command.

(a)   The pilot in command of an aircraft is directly responsible for, and is the final authority as to, the operation of that aircraft.

(b) (c)

In an in-flight emergency requiring immediate action, the pilot in command may deviate from any rule of this part to the extent required to meet that emergency.

Each pilot in command who deviates from a rule under paragraph (b) of this section shall, upon the request of the Administrator, send a written report ofthat deviation to the Administrator.

(Approved by the Office ofManagement and Budget under control number2120-0005)

§ 91.5 Pilot in commandofaircraftrequiring  morethan one required pilot.

No person may operate an aircraft that is type certificated for more than one required pilot flight crewmember unless the pilot in command meets the requirements of § 61.58 of this chapter.

14 CFR91.5(enhanceddisplay)

page 18 of243

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.7

§ 91.7 Civil aircraftairworthiness.

(a)   No person may operate a civil aircraft unless it is in an airworthy condition.

(b)   The pilot in command of a civil aircraft is responsible for determining whether that aircraft is in condition for safe flight. The pilot in command shall discontinue the flight when unairworthy mechanical, electrical, or structural conditions occur.

§ 91.9 Civil aircraftflight manual, marking, andplacard requirements.

(a)   Except as provided in paragraph (d) ofthis section, no person may operate a civil aircraft without complying with the operating limitations specified in the approved Airplane or Rotorcraft Flight Manual, markings, and placards, or as otherwise prescribed by the certificating authority of the country of registry.

(b)

(c) (d)

No person may operate a U.S.-registered civil aircraft—

(1)  For which an Airplane or Rotorcraft Flight Manual is required by § 21.5 of this chapter unless there is available in the aircraft a current, approved Airplane or Rotorcraft Flight Manual or the manual provided for in § 121.141(b); and

(2)  For which an Airplane or Rotorcraft Flight Manual is not required by § 21.5 of this chapter, unless there is available in the aircraft a current approved Airplane or Rotorcraft Flight Manual, approved manual material, markings, and placards, or any combination thereof.

No person may operate a U.S.-registered civil aircraft unless that aircraft is identified in accordance with part 45 or 48of this chapter.

Any person taking off or landing a helicopter certificated under part 29 of this chapter at a heliport constructed over water may make such momentary flight as is necessary for takeoff or landing through the prohibited range of the limiting height-speed envelope established for the helicopter ifthat  flight through the prohibited range takes place over water on which a safe ditching can be accomplished and if the helicopter is amphibious or is equipped with floats or other emergency flotation gear adequate to accomplish a safe emergency ditching on open water.

[Docket 18334, 54 FR 34292, Aug. 18, 1989, as amendedby; Docket FAA-2022-1355, Amdt. 91-366, 87 FR 75846, Dec. 9, 2022]

§ 91.11 Prohibition on interferencewith crewmembers.

No person may assault, threaten, intimidate, or interfere with a crewmember in the performance of the crewmember's duties aboard an aircraft being operated.

§ 91.13 Careless or reckless operation.

(a)   Aircraft operations forthe purpose ofairnavigation. No person may operate an aircraft in a careless or reckless manner so as to endanger the life or property of another.

(b)

Aircraft operations otherthan forthe purpose ofairnavigation. No person may operate an aircraft, other than for the purpose of air navigation, on any part of the surface of an airport used by aircraft for air commerce (including areas used by those aircraft for receiving or discharging persons or cargo), in a careless or reckless manner so as to endangerthe life or property of another.

14 CFR91.13(b)(enhanceddisplay)

page 19 of243

(c)

(d) (e)

(1) (2)

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.15

§ 91.15 Dropping objects.

No pilot in command of a civil aircraft may allow any object to be dropped from that aircraft in flight that creates a hazard to persons or property. However, this section does not prohibit the dropping of any object if reasonable precautions are taken to avoid injury or damage to persons or property.

§ 91.17 Alcohol ordrugs.

(a)   No person may act or attempt to act as a crewmember of a civil aircraft— (1)   Within 8 hours after the consumption of any alcoholic beverage;

(2)   While under the influence of alcohol;

(3)  While using any drug that affects the person's faculties in any way contrary to safety; or

(4)  While having an alcohol concentration of 0.04 or greater in a blood or breath specimen. Alcohol concentration means grams of alcohol per deciliter of blood or grams of alcohol per 210 liters of breath.

(b)  Except in an emergency, no pilot of a civil aircraft may allow a person who appears to be intoxicated or who demonstrates by manner or physical indications that the individual is under the influence of drugs (except a medical patient under proper care) to be carried in that aircraft.

A crewmember shall do the following:

On request of a law enforcement officer, submit to a test to indicate the alcohol concentration in the blood or breath, when—

(i)

The law enforcement officer is authorized under State or local law to conduct the test or to have the test conducted; and

(ii)   The law enforcement officer is requesting submission to the test to investigate a suspected violation of State or local law governing the same or substantially similar conduct prohibited by paragraph (a)(1), (a)(2), or (a)(4) ofthis section.

Whenever the FAA has a reasonable basis to believe that a person may have violated paragraph (a)(1), (a)(2), or (a)(4) ofthis section, on request of the FAA, that person must furnish to the FAAthe results, or authorize any clinic, hospital, or doctor, or other person to release to the FAA, the results of each test taken within 4 hours after acting or attempting to act as a crewmember that indicates an alcohol concentration in the blood or breath specimen.

Whenever the Administrator has a reasonable basis to believe that a person may have violated paragraph (a)(3) of this section, that person shall, upon request by the Administrator, furnish the Administrator, or authorize any clinic, hospital, doctor, or other person to release to the Administrator, the results of each test taken within 4 hours after acting or attempting to act as a crewmember that indicates the presence of any drugs in the body.

Any test information obtained by the Administrator under paragraph (c) or (d) ofthis section may be evaluated in determining a person's qualifications for any airman certificate or possible violations ofthis chapter and may be used as evidence in any legal proceeding under section 602, 609, or 901 of the Federal Aviation Act of 1958.

[Docket 18334, 54 FR 34292, Aug. 18, 1989, as amendedbyAmdt. 91-291, June 21, 2006] 14 CFR91.17(e) (enhanceddisplay)

page 20 of243

(b) (c)

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

§ 91.19 Carriage ofnarcotic drugs, marihuana, anddepressant orstimulant drugs or substances.

14 CFR 91.19

(a) (b)

Except as provided in paragraph (b) ofthis section, no person may operate a civil aircraft within the United States with knowledge that narcotic drugs, marihuana, and depressant or stimulant drugs or substances as defined in Federal or State statutes are carried in the aircraft.

Paragraph (a) of this section does not apply to any carriage of narcotic drugs, marihuana, and depressant or stimulant drugs or substances authorized by or under any Federal or State statute or by any Federal or State agency.

§ 91.21 Portable electronicdevices.

(a)  Except as provided in paragraph (b) ofthis section, no person may operate, nor may any operator or pilot in command of an aircraft allow the operation of, any portable electronic device on any ofthe following U.S.-registered civil aircraft:

(1)

Aircraft operated by a holder of an air carrier operating certificate or an operating certificate; or

(2)  Any other aircraft while it is operated under IFR. Paragraph (a) of this section does not apply to—

(1)  Portable voice recorders; (2)  Hearing aids;

(3)  Heart pacemakers; (4)  Electric shavers; or

(5)

Any other portable electronic device that the operator of the aircraft has determined will not cause interference with the navigation or communication system of the aircraft on which it is to be used.

In the case of an aircraft operated by a holder of an air carrier operating certificate or an operating certificate, the determination required by paragraph (b)(5) of this section shall be made by that operator of the aircraft on which the particular device is to be used. In the case of other aircraft, the determination may be made by the pilot in command or other operator ofthe aircraft.

§ 91.23 Truth-in-leasing clause requirement inleases and conditional sales contracts.

(a)   Except as provided in paragraph (b) ofthis section, the parties to a lease or contract of conditional sale involving a U.S.-registered large civil aircraft and entered into after January 2, 1973, shall execute a written lease or contract and include therein a written truth-in-leasing clause as a concluding paragraph in large print, immediately preceding the space for the signature of the parties, which contains the following with respect to each such aircraft:

(1)

Identification ofthe Federal Aviation Regulations under which the aircraft has been maintained and inspected during the 12 months preceding the execution ofthe lease or contract of conditional sale, and certification by the parties thereto regarding the aircraft's status of compliance with applicable maintenance and inspection requirements in this part for the operation to be conducted under the lease or contract of conditional sale.

14 CFR91.23(a)(1) (enhanceddisplay)

page 21of243

(b) (c)

(d)

(1) (2)

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.23(a)(2)

(2)

(3)

The name and address (printed or typed) and the signature of the person responsible for operational control ofthe aircraft under the lease or contract of conditional sale, and certification that each person understands that person's responsibilities for compliance with applicable Federal Aviation Regulations.

A statement that an explanation of factors bearing on operational control and pertinent Federal Aviation Regulations can be obtained from the responsible Flight Standards office.

The requirements of paragraph (a) ofthis section do not apply—

To a lease or contract of conditional sale when—

(i)

The partyto whom the aircraft is furnished is a foreign air carrier or certificate holder under part 121, 125, 135, or 141 of this chapter, or

(ii)   The party furnishing the aircraft is a foreign air carrier or a person operating under part 121, 125, and 141 of this chapter, or a person operating under part 135 of this chapter having authority to engage in on-demand operations with large aircraft.

To a contract of conditional sale, when the aircraft involved has not been registered anywhere prior to the execution ofthe contract, except as a new aircraft under a dealer's aircraft registration certificate issued in accordance with § 47.61 of this chapter.

No person may operate a large civil aircraft of U.S. registry that is subject to a lease or contract of conditional sale to which paragraph (a) of this section applies, unless—

(1)

(2) (3)

The lessee or conditional buyer, or the registered owner if the lessee is not a citizen ofthe United States, has mailed a copy ofthe lease or contract that complies with the requirements of paragraph (a) of this section, within 24 hours of its execution, to the Aircraft Registration Branch, Attn: Technical Section, P.O. Box 25724, Oklahoma City, OK 73125;

A copy ofthe lease or contract that complies with the requirements of paragraph (a) of this section is carried in the aircraft. The copy of the lease or contract shall be made available for review upon request by the Administrator, and

The lessee or conditional buyer, or the registered owner if the lessee is not a citizen ofthe United States, has notified by telephone or in person the responsible Flight Standards office. Unless otherwise authorized by that office, the notification shall be given at least 48 hours before takeoff in the case ofthe first flight of that aircraft under that lease or contract and inform the FAA of—

(i)   The location ofthe airport of departure; (ii)   The departure time; and

(iii)  The registration number ofthe aircraft involved.

The copy of the lease or contract furnished to the FAA under paragraph (c) ofthis section is commercial or financial information obtained from a person. It is, therefore, privileged and confidential and will not be made available by the FAA for public inspection or copying under 5 U.S.C. 552(b)(4) unless recorded with the FAA under part 49 of this chapter.

14 CFR91.23(d)(enhanceddisplay)

page 22of243

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.23(e)

(e)

For the purpose of this section, a lease means any agreement by a person to furnish an aircraft to another person for compensation or hire, whether with or without flight crewmembers, other than an agreement for the sale of an aircraft and a contract of conditional sale under section 101 of the Federal Aviation Act of 1958. The person furnishing the aircraft is referred to as the lessor, and the person to whom it is furnished the lessee.

(Approved by the Office ofManagement and Budget under control number2120-0005)

[Docket 18334, 54 FR 34292, Aug. 18, 1989, as amendedbyAmdt. 91-212, 54 FR 39293, Sept. 25, 1989;Amdt. 91-253, 62 FR 13253, Mar. 19, 1997;Amdt. 91-267, 66 FR 21066, Apr. 27, 2001; Docket FAA-2018-0119, Amdt. 91-350, 83 FR 9171, Mar. 5, 2018]

§ 91.25 Aviation Safety ReportingProgram: Prohibitionagainst use ofreports  forenforcement purposes.

The Administrator of the FAA will not use reports submitted to the National Aeronautics and Space Administration under the Aviation Safety Reporting Program (or information derived therefrom) in any enforcement action except information concerning accidents or criminal offenses which are wholly excluded from the Program.

§§ 91.27-91.99 [Reserved]

Subpart B—Flight Rules

Source:

Docket 18334, 54 FR 34294, Aug. 18, 1989, unless otherwise noted.

GENERAL

§ 91.101Applicability.

This subpart prescribes flight rules governing the operation of aircraft within the United States and within 12 nautical miles from the coast of the United States.

§ 91.103 Preflight action.

Each pilot in command shall, before beginning a flight, become familiar with all available information concerning that flight. This information must include—

(a) (b)

For a flight under IFR or a flight not in the vicinity of an airport, weather reports and forecasts, fuel requirements, alternatives available if the planned flight cannot be completed, and any known traffic delays of which the pilot in command has been advised byATC;

For any flight, runway lengths at airports of intended use, and the following takeoff and landing distance information:

(1)   For civil aircraft for which an approved Airplane or Rotorcraft Flight Manual containing takeoff and landing distance data is required, the takeoff and landing distance data contained therein; and

(2)   For civil aircraft other than those specified in paragraph (b)(1) ofthis section, other reliable information appropriate to the aircraft, relating to aircraft performance under expected values of airport elevation and runway slope, aircraft gross weight, and wind and temperature.

14 CFR91.103(b)(2) (enhanceddisplay)

page 23 of243

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.105

§ 91.105 Flightcrewmembers at stations.

(a) During takeoff and landing, and while en route, each required flight crewmember shall—

(1)  Be at the crewmember station unless the absence is necessary to perform duties in connection with the operation ofthe aircraft or in connection with physiological needs; and

(2)

Keep the safety belt fastened while at the crewmember station.

(b)

Each required flight crewmember of a U.S.-registered civil aircraft shall, during takeoff and landing, keep his or her shoulder harness fastened while at his or her assigned duty station. This paragraph does not apply if—

(1) (2)

The seat at the crewmember's station is not equipped with a shoulder harness; or

The crewmember would be unable to perform required duties with the shoulder harness fastened.

[Docket 18334, 54 FR 34294, Aug. 18, 1989, as amendedbyAmdt. 91-231, 57 FR 42671, Sept. 15, 1992]

§ 91.107 Use ofsafetybelts, shoulder harnesses, andchild restraintsystems. (a)  Unless otherwise authorized by the Administrator—

(1)   No pilot may take off a U.S.-registered civil aircraft (except a free balloon that incorporates a basket or gondola, or an airship type certificated before November 2, 1987) unless the pilot in command of that aircraft ensures that each person on board is briefed on how to fasten and unfasten that person's safety belt and, if installed, shoulder harness.

(2) (3)

No pilot may cause to be moved on the surface, take off, or land a U.S.-registered civil aircraft (except a free balloon that incorporates a basket or gondola, or an airship type certificated before November 2, 1987) unless the pilot in command of that aircraft ensures that each person on board has been notified to fasten his or her safety belt and, if installed, his or her shoulder harness.

Except as provided in this paragraph, each person on board a U.S.-registered civil aircraft (except a free balloon that incorporates a basket or gondola or an airship type certificated before November 2, 1987) must occupy an approved seat or berth with a safety belt and, if installed, shoulder harness, properly secured about him or her during movement on the surface, takeoff, and landing. For seaplane and float equipped rotorcraft operations during movement on the surface, the person pushing offthe seaplane or rotorcraft from the dock and the person mooring the seaplane or rotorcraft at the dock are excepted from the preceding seating and safety belt requirements. Notwithstanding the preceding requirements ofthis paragraph, a person may:

(i)  Be held by an adult, except as outlined in § 91.108(j), who is occupying an approved seat or berth, provided that the person being held has not reached his or her second birthday and does not occupy or use any restraining device;

(ii)   Use the floor of the aircraft as a seat, provided that the person is on board forthe purpose of engaging in sport parachuting; or

(iii)   Notwithstanding any other requirement of this chapter, occupy an approved child restraint system furnished bythe operator or one ofthe persons described in paragraph (a)(3)(iii)(A) of this section provided that:

14 CFR91.107(a)(3)(iii)(enhanceddisplay)

page 24of243

(A) (B)

(C)

(1) (2)

(3) (4)

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.107(a)(3)(iii)(A)

The child is accompanied by a parent, guardian, or attendant designated bythe child's parent or guardian to attend to the safety of the child during the flight;

Except as provided in paragraph (a)(3)(iii)(B)(4) of this action, the approved child restraint system bears one or more labels as follows:

Seats manufactured to U.S. standards between January 1, 1981, and February 25, 1985, must bear the label: “This child restraint system conforms to all applicable Federal motor vehicle safety standards”;

Seats manufactured to U.S. standards on or after February 26, 1985, must bear two labels:

(i) (ii)

“This child restraint system conforms to all applicable Federal motor vehicle safety standards”; and

“THIS RESTRAINT IS CERTIFIED FOR USE IN MOTOR VEHICLES AND AIRCRAFT” in red lettering;

Seats that do not qualify under paragraphs (a)(3)(iii)(B)(1) and (a)(3)(iii)(B)(2) ofthis section must bear a label or markings showing:

(ii) (iii)

(iv)

That the seat was manufactured under the standards of the United Nations;

That the seat or child restraint device furnished by the operator was approved by the FAAthrough Type Certificate or Supplemental Type Certificate; or

That the seat or child restraint device furnished by the operator, or one of the persons described in paragraph (a)(3)(iii)(A) ofthis section, was approved by the FAA in accordance with § 21.8(d) of this chapter or Technical Standard Order C-100b or a later version. The child restraint device manufactured by AmSafe, Inc. (CARES, Part No. 4082) and approved by the FAA in accordance with § 21.305(d) (2010 ed.) of this chapter may continue to bear a label or markings showing FAA approval in accordance with § 21.305(d) (2010 ed.) of this chapter.

Except as provided in § 91.107(a)(3)(iii)(B)(3)(iii) and § 91.107(a)(3)(iii)(B)(3)(iv), booster-type child restraint systems (as defined in Federal Motor Vehicle Safety Standard No. 213 (49 CFR 571.213)), vest- and harness-type child restraint systems, and lap held child restraints are not approved for use in aircraft; and

The operator complies with the following requirements:

(1) (2)

(3)

The restraint system must be properly secured to an approved forward-facing seat or berth;

The child must be properly secured in the restraint system and must not exceed the specified weight limit for the restraint system; and

The restraint system must bear the appropriate label(s).

(b)   Unless otherwise stated, this section does not apply to operations conducted under part 121, 125, or 135 of this chapter. Paragraph (a)(3) ofthis section does not apply to persons subject to § 91.105.

14 CFR91.107(b)(enhanced display)

page 25of243

(1) (2)

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.108

[Docket 26142, 57 FR 42671, Sept. 15, 1992, as amendedbyAmdt. 91-250, 61 FR 28421, June 4, 1996;Amdt. 91-289, 70 FR 50906, Aug. 26, 2005;Amdt. 91-292, 71 FR 40009, July 14, 2006;Amdt. 91-317, 75 FR 48857, Aug. 12, 2010;Amdt. 91-332, 79 FR 28812, May 20, 2014; Docket FAA-2023-2250, Amdt. 91-376, 89 FR 67849, Aug. 22, 2024]

§ 91.108 Use ofsupplemental restraint systems.

(a)  Use ofsupplemental restraint systems. Except as provided in this section, no person may conduct an operation in a civil aircraft in which any individual on board is secured with a supplemental restraint system, as defined in § 1.1 of this chapter.

(b)  Doors opened or removedflight operations. Except as provided under paragraph (k) ofthis section:

No person may operate a civil aircraft with the doors opened or removed unless—

(i)

Each individual on board occupies an approved seat or berth with a safety belt and, if installed, shoulder harness, properly secured about the individual or an approved child restraint system properly secured to an approved seat or berth with a safety belt and, if installed, shoulder harness in accordance with § 91.107(a)(3)(iii) or § 135.128(a)(2) of this chapter, during all phases of flight; or

(ii)  Each individual on board—

(A) Occupies an approved seat or berth with a safety belt and, if installed, shoulder harness, properly secured about the individual during movement on the surface, takeoff, and landing; and

(B)  Is secured during the remainder of the flight using a supplemental restraint system in accordance with, and that meets the requirements of, this section.

Prior to releasing an FAA-approved safety belt and, if installed, shoulder harness during an operation with the doors opened or removed, an individual must be properly secured by a supplemental restraint system that is connected to an FAA-approved airframe attachment point. An individual cannot release their safety belt and, if installed, shoulder harness until the pilot in command authorizes them to do so.

(c)   Supplementalrestraint system design requirements. Each supplemental restraint system must:

(1) (2)

(3)

Have a harness that secures around the torso ofthe individual using the supplemental restraint system;

Have a lanyard that connects the harness to an FAA-approved airframe attachment point or points inside the aircraft and that ensures the torso of the individual using the supplemental restraint system remains inside the aircraft at all times;

Not impede egress from the aircraft in an emergency after being released; and

(4)  Have a release mechanism that—

(i) (ii)

Can be quickly operated bythe individual using the supplemental restraint system with minimal difficulty;

Is attached to the front or side of the harness in a location easily accessible to and visible by the individual using the supplemental restraint system;

(iii)  Prevents inadvertent release; and 14 CFR91.108(c)(4)(iii)(enhanceddisplay)

page 26of243

(d) (e)

(f)

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.108(c)(4)(iv)

(iv)  Can be released without the use of a knife to cut the restraint, and without any additional tool or the assistance of any other individual.

Who mayprovide the supplementalrestraintsystem. The supplemental restraint system may be provided by the operator or by the individual using the supplemental restraint system. An operator or individual providing a supplemental restraint system must:

(1) (2)

Confirm with the pilot in command, either verbally or in writing, as determined by the pilot in command, the system's continued serviceability and readiness for its intended purpose; and

Ensure the individual who will occupythe supplemental restraint system complies with the sizing criteria for which the system is rated.

Supplemental restraint system operational requirements. The following are supplemental restraint system operational requirements:

(1)

(2) (3)

A qualified person designated by the operator must—

(i)   Connect the supplemental restraint system to an FAA-approved airframe attachment point or points rated equal to or greater than the weight of the individual using the supplemental restraint system (orthe combined weight ifthere is more than one supplemental restraint system attached to an attachment point);

(ii)   Not connect the supplemental restraint system to any airframe attachment point located in the flightdeck; and

(iii)  Not connect the supplemental restraint system to any safety belt or shoulder harness attachment point(s) unless the attachment point is FAA-approved as described in paragraph (e)(1)(i) ofthis section.

A supplemental restraint system must fit the individual using it based on the sizing criteria for which the supplemental restraint system is rated.

Nothing may attach to the supplemental restraint system that is not relevant to its function as defined under § 1.1 of this chapter.

Pilot in command. The pilot in command—

(1) (2) (3) (4) (5)

Has the overall responsibility to ensure that the supplemental restraint system meets the requirements ofthis section and must not permit an individual to use a supplemental restraint system that does not meet the requirements of this section;

Must receive confirmation from the operator or any individual providing the supplemental restraint system of the system's continued serviceability and readiness for its intended purpose before each takeoff;

May only permit an individual to use a supplemental restraint system provided by the operator orthe pilot in command ifthat individual complies with the sizing criteria for which the supplemental restraint system is rated;

Has final authority regarding whether the supplemental restraint system may be used during flight operations; and

Has final authority to authorize an individual to release the FAA-approved safety belt and, if installed, shoulder harness and remain secured only by the supplemental restraint system.

14 CFR91.108(f)(5)(enhanceddisplay)

page 27of243

(g)

(h) (i)

(j)

(1)

(2) (3) (4)

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.108(g)

Passenger briefing. Before each takeoff, the pilot in command must ensure that each passenger who intends to use a supplemental restraint system has been briefed on:

(1)

(2)

How to use, secure, and release the supplemental restraint system properly. This requirement is not necessary for an individual providing their own supplemental restraint system, but that individual must meet the passenger demonstration requirements in paragraph (h) ofthis section.

Means of direct communication between crewmembers and passengers during normal and emergency operating procedures regarding—

(i) (ii)

(iii)

(iv)

(v)

The use of headset and intercom systems, if installed;

How passengers will be notified of an event requiring action, including emergencies, egress procedures, and other unforeseen circumstances;

How each passenger will be notified when the passenger is permitted to release the FAA- approved safety belt and, if installed, shoulder harness, and move within the aircraft using the supplemental restraint system;

How each passenger will be notified when the passenger must return to their seat and secure the FAA-approved safety belt and, if installed, shoulder harness; and

When and how to notify a crewmember of safety concerns.

Passenger demonstration. After the briefing required by paragraph (g) ofthis section, priorto ground movement, any passenger intending to use a supplemental restraint system must demonstrate to the pilot in command, a crewmember, or other qualified person designated by the operator, the following:

(1) (2)

The ability to use, secure, and release the FAA-approved safety belt and, if installed, shoulder harness, and

The ability to accomplish all actions required for quick release ofthe supplemental restraint system without assistance and with minimal difficulty.

Individuals not permitted to use supplemental restraint systems. The following individuals are not permitted to use a supplemental restraint system, as defined in § 1.1 of this chapter:

Any passenger who cannot demonstrate—

(i) (ii)

That they are able to use, secure, and release the FAA-approved safety belt and, if installed, shoulder harness; or

That they are able to release quickly the supplemental restraint system with no assistance and with minimal difficulty.

Any individual who is less than 15 years of age. Any individual seated in the flightdeck.

Any passenger who occupies or uses an approved child restraint system.

Lap-held child. Notwithstanding any other requirement of this chapter, a child who has not reached their second birthday may not be held by an adult during civil aircraft operations when:

(1) (2)

The adult uses a supplemental restraint system; or The aircraft doors are opened or removed.

14 CFR91.108(j)(2) (enhanceddisplay)

page 28 of243

(b)

(c)

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.108(k)

(k)

Excluded operations. Unless otherwise stated:

(1) (2)

(3)

This section does not apply to operations conducted under part 105 or 133 of this chapter and does not apply to the persons described in § 91.107(a)(3)(ii) of this chapter.

Operators subject to the requirements of paragraph (b)(1) of this section may operate an aircraft with doors opened or removed, notwithstanding any flight crewmembers on board who are subject

to the requirements of §§ 91.105 or 135.171 of this chapter and who need to unfasten their shoulder harnesses in accordance with those sections.

Paragraph (b)(2) ofthis section does not apply to any flight crewmembers subject to §§ 91.105 or 135.171 of this chapter to the extent that the flight crewmembers need to unfasten their shoulder harnesses in accordance with those sections.

[Docket FAA-2023-2250, Amdt. 91-376, 89 FR 67849, Aug. 22, 2024]

§ 91.109 Flight instruction;  Simulatedinstrumentflight  and certain flighttests.

(a)   No person may operate a civil aircraft (except a manned free balloon) that is being used for flight instruction unless that aircraft has fully functioning dual controls. However, instrument flight instruction may be given in an airplane that is equipped with a single, functioning throwover control wheel that controls the elevator and ailerons, in place of fixed, dual controls, when—

(1) (2)

The instructor has determined that the flight can be conducted safely; and

The person manipulating the controls has at least a private pilot certificate with appropriate category and class ratings.

An airplane equipped with a single, functioning throwover control wheel that controls the elevator and ailerons, in place of fixed, dual controls may be used for flight instruction to conduct a flight review required by § 61.56 of this chapter, orto obtain recent flight experience or an instrument proficiency check required by § 61.57 when—

(1) (2)

(3)

(4)

The airplane is equipped with operable rudder pedals at both pilot stations;

The pilot manipulating the controls is qualified to serve and serves as pilot in command during the entire flight;

The instructor is current and qualified to serve as pilot in command of the airplane, meets the requirements of § 61.195(b), and has logged at least 25 hours of pilot-in-command flight time in the make and model of airplane; and

The pilot in command and the instructor have determined the flight can be conducted safely.

No person may operate a civil aircraft in simulated instrument flight unless—

(1)

The other control seat is occupied by a safety pilot who possesses at least:

(i) (ii)

A private pilot certificate with category and class ratings appropriate to the aircraft being flown; or

For purposes of providing training for a solo cross-country endorsement under § 61.93 of this chapter, a flight instructor certificate with an appropriate sport pilot rating and meets the requirements of § 61.412 of this chapter.

14 CFR91.109(c)(1)(ii) (enhanceddisplay)

page 29 of243

(c) (d)

(e)

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.109(c)(2)

(2) (3)

The safety pilot has adequate vision forward and to each side ofthe aircraft, or a competent observer in the aircraft adequately supplements the vision of the safety pilot; and

Except in the case of lighter-than-air aircraft, that aircraft is equipped with fully functioning dual controls. However, simulated instrument flight may be conducted in a single-engine airplane, equipped with a single, functioning, throwover control wheel, in place of fixed, dual controls ofthe elevator and ailerons, when—

(i)   The safety pilot has determined that the flight can be conducted safely; and

(ii)   The person manipulating the controls has at least a private pilot certificate with appropriate category and class ratings.

(d)

No person may operate a civil aircraft that is being used for a flight test for an airline transport pilot certificate or a class or type rating on that certificate, or for a part 121 proficiency flight test, unless the pilot seated at the controls, other than the pilot being checked, is fully qualified to act as pilot in command of the aircraft.

[Docket 18334, 54 FR 34294, Aug. 18, 1989, as amendedbyAmdt. 91-324, 76 FR 54107, Aug. 31, 2011;Amdt. 61-142, 83 FR 30281, June27, 2018]

§ 91.111 Operating near otheraircraft.

(a)   No person may operate an aircraft so close to another aircraft as to create a collision hazard.

(b)   No person may operate an aircraft in formation flight except by arrangement with the pilot in command of each aircraft in the formation.

(c)

No person may operate an aircraft, carrying passengers for hire, in formation flight.

§ 91.113 Right-of-wayrules: Except water operations.

(a)  Inapplicability. This section does not apply to the operation of an aircraft on water.

(b)  General. When weather conditions permit, regardless of whether an operation is conducted under instrument flight rules or visual flight rules, vigilance shall be maintained by each person operating an aircraft so as to see and avoid other aircraft. When a rule of this section gives another aircraft the right-of- way, the pilot shall give way to that aircraft and may not pass over, under, or ahead of it unless well clear.

In distress. An aircraft in distress has the right-of-way over all other air traffic.

Converging. When aircraft of the same category are converging at approximately the same altitude (except head-on, or nearly so), the aircraft to the other's right has the right-of-way. Ifthe aircraft are of different categories—

(1) (2) (3)

(4)

A balloon has the right-of-way over any other category of aircraft; A glider has the right-of-way over powered aircraft.

An airship has the right-of-way over all other powered aircraft, except for an aircraft towing or refueling other aircraft.

An aircraft towing or refueling other aircraft has the right-of-way over all other powered aircraft.

Approaching head-on. When aircraft are approaching each other head-on, or nearly so, each pilot of each aircraft shall alter course to the right.

14 CFR91.113(e)(enhanced display)

page 30 of243

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.113(f)

(f) (g)

Overtaking. Each aircraft that is being overtaken has the right-of-way and each pilot of an overtaking aircraft shall alter course to the right to pass well clear.

Landing. Aircraft, while on final approach to land or while landing, have the right-of-way over other aircraft in flight or operating on the surface, except that they shall not take advantage ofthis rule to force an aircraft off the runway surface which has already landed and is attempting to make way for an aircraft on final approach. When two or more aircraft are approaching an airport for the purpose of landing, the aircraft at the lower altitude has the right-of-way, but it shall not take advantage of this rule to cut in front of another which is on final approach to land or to overtake that aircraft.

[Docket 18334, 54 FR 34294, Aug. 18, 1989, as amendedbyAmdt. 91-282, 69 FR 44880, July27, 2004; FAA-2023-1275, Amdt. 91-379, 89 FR 92485, Nov. 21, 2024;Docket FAA-2023-1377, Amdt. 91-381, 90 FR 35220, July24, 2025]

§ 91.115 Right-of-wayrules: Water operations.

(a) General. Each person operating an aircraft on the water shall, insofar as possible, keep clear of all vessels and avoid impeding their navigation, and shall give way to any vessel or other aircraft that is given the right-of-way by any rule ofthis section.

(b) (c) (d) (e)

Crossing. When aircraft, or an aircraft and a vessel, are on crossing courses, the aircraft or vessel to the other's right has the right-of-way.

Approaching head-on. When aircraft, or an aircraft and a vessel, are approaching head-on, or nearly so, each shall alter its course to the right to keep well clear.

Overtaking. Each aircraft orvessel that is being overtaken has the right-of-way, and the one overtaking shall alter course to keep well clear.

Special circumstances. When aircraft, or an aircraft and a vessel, approach so as to involve risk of collision, each aircraft or vessel shall proceed with careful regard to existing circumstances, including the limitations of the respective craft.

§ 91.117 Aircraft speed.

(a)  Unless otherwise authorized by the Administrator, no person may operate an aircraft below 10,000 feet MSL at an indicated airspeed of more than 250 knots (288 m.p.h.).

(b)

(c) (d)

Unless otherwise authorized or required by ATC, no person may operate an aircraft at or below 2,500 feet above the surface within 4 nautical miles of the primary airport of a Class C or Class D airspace area at an indicated airspeed of more than 200 knots (230 mph.). This paragraph (b) does not apply to any operations within a Class B airspace area. Such operations shall comply with paragraph (a) of this

section.

No person may operate an aircraft in the airspace underlying a Class B airspace area designated for an airport or in a VFR corridor designated through such a Class B airspace area, at an indicated airspeed of more than 200 knots (230 mph).

If the minimum safe airspeed for any particular operation is greater than the maximum speed prescribed in this section, the aircraft may be operated at that minimum speed.

[Docket 18334, 54 FR 34292, Aug. 18, 1989, as amendedbyAmdt. 91-219, 55 FR 34708, Aug. 24, 1990;Amdt. 91-227, 56 FR 65657, Dec. 17, 1991;Amdt. 91-233, 58 FR 43554, Aug. 17, 1993]

14 CFR91.117(d)(enhanceddisplay)

page 31of243

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.119

§ 91.119 Minimum safe altitudes: General.

Except when necessary for takeoff or landing, no person may operate an aircraft below the following altitudes:

(a) (b)

(c)

(d)

Anywhere. An altitude allowing, if a power unit fails, an emergency landing without undue hazard to persons or property on the surface.

Over congested areas. Over any congested area of a city, town, or settlement, or over any open air assembly of persons, an altitude of 1,000 feet above the highest obstacle within a horizontal radius of 2,000 feet of the aircraft.

Over other than congested areas. An altitude of 500 feet above the surface, except over open water or sparsely populated areas. In those cases, the aircraft may not be operated closerthan 500 feet to any person, vessel, vehicle, or structure.

Helicopters, poweredparachutes, and weight-shift-controlaircraft. If the operation is conducted without hazard to persons or property on the surface—

(1) (2)

A helicopter may be operated at less than the minimums prescribed in paragraph (b) or (c) ofthis section, provided each person operating the helicopter complies with any routes or altitudes specifically prescribed for helicopters by the FAA; and

A powered parachute or weight-shift-control aircraft may be operated at less than the minimums prescribed in paragraph (c) ofthis section.

[Docket 18334, 54 FR 34294, Aug. 18, 1989, as amendedbyAmdt. 91-311, 75 FR 5223, Feb. 1, 2010]

§ 91.121Altimetersettings.

(a)  Each person operating an aircraft shall maintain the cruising altitude or flight level of that aircraft, as the case may be, by reference to an altimeterthat is set, when operating—

(1)  Below 18,000 feet MSL, to—

(i)   The current reported altimeter setting of a station along the route and within 100 nautical miles of the aircraft;

(ii)   If there is no station within the area prescribed in paragraph (a)(1)(i) of this section, the current reported altimeter setting of an appropriate available station; or

(iii)   In the case of an aircraft not equipped with a radio, the elevation ofthe departure airport or an appropriate altimeter setting available before departure; or

(2)

At or above 18,000 feet MSL, to 29.92″ Hg.

(b)

The lowest usable flight level is determined by the atmospheric pressure in the area of operation as shown in the following table:

Current altimeter setting 29.92 (or higher)

14 CFR91.121(b)(enhanceddisplay)

Lowest usable flight level

180

page 32of243

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

Current altimeter setting 29.91 through 29.42

29.41 through 28.92 28.91 through 28.42 28.41 through 27.92 27.91 through 27.42 27.41 through 26.92

14 CFR91.121(c)

Lowest usable flight level

185 190 195 200 205 210

(c)

To convert minimum altitude prescribed under §§  91.119and91.177to  the minimum flight level, the pilot shall take the flight level equivalent ofthe minimum altitude in feet and add the appropriate number of feet specified below, according to the current reported altimeter setting:

Current altimeter setting 29.92 (or higher)

29.91 through 29.42 29.41 through 28.92 28.91 through 28.42 28.41 through 27.92 27.91 through 27.42 27.41 through 26.92

Adjustment factor

None 500 1,000 1,500 2,000 2,500 3,000

§ 91.123 Compliance withATC clearances and instructions.

(a) When an ATC clearance has been obtained, no pilot in command may deviate from that clearance unless an amended clearance is obtained, an emergency exists, or the deviation is in response to a traffic alert and collision avoidance system resolution advisory. However, except in Class A airspace, a pilot may cancel an IFR flight plan ifthe operation is being conducted in VFR weather conditions. When a pilot is uncertain of an ATC clearance, that pilot shall immediately request clarification from ATC.

(b) (c)

(d)

Except in an emergency, no person may operate an aircraft contraryto an ATC instruction in an area in which air traffic control is exercised.

Each pilot in command who, in an emergency, or in response to a traffic alert and collision avoidance system resolution advisory, deviates from an ATC clearance or instruction shall notify ATC of that deviation as soon as possible.

Each pilot in command who (though not deviating from a rule of this subpart) is given priority by ATC in an emergency, shall submit a detailed report of that emergency within 48 hours to the manager of that ATC facility, if requested byATC.

14 CFR91.123(d)(enhanceddisplay)

page 33 of243

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.123(e)

(e)

Unless otherwise authorized by ATC, no person operating an aircraft may operate that aircraft according to any clearance or instruction that has been issued to the pilot of another aircraft for radar airtraffic control purposes.

(Approved by the Office ofManagement and Budget under control number2120-0005)

[Docket 18834, 54 FR 34294, Aug. 18, 1989, as amendedbyAmdt. 91-227, 56 FR 65658, Dec. 17, 1991;Amdt. 91-244, 60 FR 50679, Sept. 29, 1995]

§ 91.125 ATC lightsignals.

ATC light signals have the meaning shown in the following table:

Color and type of signal

Steady green Flashing green

Steady red

Flashing red Flashing white

Alternating red and green

Meaning with respectto aircraft on the surface

Cleared fortakeoff Cleared to taxi

Stop

Taxi clear of runway in use Return to starting point on airport

Exercise extreme caution

Meaning with respect to aircraft in flight

Cleared to land.

Return for landing (to be followed by steady green at propertime).

Give wayto other aircraft and continue circling.

Airport unsafe—do not land. Not applicable.

Exercise extreme caution.

§ 91.126 Operatingonor inthevicinity ofan airport in Class G airspace.

(a)  General. Unless otherwise authorized or required, each person operating an aircraft on or in the vicinity of an airport in a Class G airspace area must comply with the requirements of this section.

(b)

(c)

Direction ofturns. When approaching to land at an airport without an operating control tower in Class G airspace—

(1)  Each pilot of a powered fixed-wing aircraft must make all turns to the left unless the airport displays approved light signals or visual markings indicating that turns should be made to the right, in which case the pilot must make all turns to the right; and

(2)  Each pilot of any other powered aircraft must avoid the flow of the aircraft specified in paragraph (b)(1) of this section.

Flap settings. Except when necessary for training or certification, the pilot in command of a civil turbojet- powered aircraft must use, as a final flap setting, the minimum certificated landing flap setting set forth in the approved performance information in the Airplane Flight Manual for the applicable conditions.

14 CFR91.126(c)(enhanced display)

page 34of243

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.126(d)

(d)

However, each pilot in command has the final authority and responsibility for the safe operation of the pilot's airplane, and may use a different flap setting for that airplane ifthe pilot determines that it is necessary in the interest of safety.

Communications with controltowers. Unless otherwise authorized or required by ATC, no person may operate an aircraft to, from, through, or on an airport having an operational control tower unless two-way radio communications are maintained between that aircraft and the control tower. Communications must be established priorto 4 nautical miles from the airport, up to and including 2,500 feet AGL. However, if the aircraft radio fails in flight, the pilot in command may operate that aircraft and land ifweather conditions are at or above basic VFR weather minimums, visual contact with the tower is maintained, and a clearance to land is received. If the aircraft radio fails while in flight under IFR, the pilot must comply with § 91.185.

[Docket 24458, 56 FR 65658, Dec. 17, 1991, as amended byAmdt. 91-239, 59 FR 11693, Mar. 11, 1994;Amdt. 91-282, 69 FR 44880, July 27, 2004; Docket FAA-2023-1377, Amdt. 91-381, 90 FR 35220, July24, 2025]

§ 91.127 Operating onorin the vicinityofanairportin Class E airspace.

(a)  Unless otherwise required by part 93 of this chapter or unless otherwise authorized or required by the ATC facility having jurisdiction over the Class E airspace area, each person operating an aircraft on or in the vicinity of an airport in a Class E airspace area must comply with the requirements of § 91.126.

(b) (c)

Departures. Each pilot of an aircraft must comply with any traffic patterns established for that airport in part 93 of this chapter.

Communications with controltowers. Unless otherwise authorized or required by ATC, no person may operate an aircraft to, from, through, or on an airport having an operational control tower unless two-way radio communications are maintained between that aircraft and the control tower. Communications must be established priorto 4 nautical miles from the airport, up to and including 2,500 feet AGL. However, if the aircraft radio fails in flight, the pilot in command may operate that aircraft and land ifweather conditions are at or above basic VFR weather minimums, visual contact with the tower is maintained, and a clearance to land is received. If the aircraft radio fails while in flight under IFR, the pilot must comply with § 91.185.

[Docket 24458, 56 FR 65658, Dec. 17, 1991, as amended byAmdt. 91-239, 59 FR 11693, Mar. 11, 1994]

§ 91.129 Operations in Class D airspace.

(a) General. Unless otherwise authorized or required by the ATC facility having jurisdiction over the Class D airspace area, each person operating an aircraft in Class D airspace must comply with the applicable provisions of this section. In addition, each person must comply with §§ 91.126 and 91.127. For the purpose of this section, the primary airport is the airport for which the Class D airspace area is designated. A satellite airport is any other airport within the Class D airspace area.

(b) (c)

Deviations. An operator may deviate from any provision ofthis section underthe provisions of an ATC authorization issued by the ATC facility having jurisdiction overthe airspace concerned. ATC may authorize a deviation on a continuing basis or for an individual flight, as appropriate.

Communications. Each person operating an aircraft in Class D airspace must meet the following two-way radio communications requirements:

14 CFR91.129(c)(enhanced display)

page 35of243

(d) (e)

(f)

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR 91.129(c)(1)

(1) (2)

Arrival or through flight. Each person must establish two-way radio communications with the ATC facility (including foreign ATC in the case of foreign airspace designated in the United States) providing air traffic services prior to entering that airspace and thereafter maintain those communications while within that airspace.

Departing flight. Each person—

(i)   From the primary airport or satellite airport with an operating control tower must establish and maintain two-way radio communications with the control tower, and thereafter as instructed by ATC while operating in the Class D airspace area; or

(ii)   From a satellite airport without an operating control tower, must establish and maintain two- way radio communications with the ATC facility having jurisdiction over the Class D airspace area as soon as practicable after departing.

Communications failure. Each person who operates an aircraft in a Class D airspace area must maintain two-way radio communications with the ATC facility having jurisdiction overthat area.

(1) (2)

If the aircraft radio fails in flight under IFR, the pilot must comply with § 91.185 of the part.

If the aircraft radio fails in flight under VFR, the pilot in command may operate that aircraft and land if—

(i)  Weather conditions are at or above basic VFR weather minimums; (ii)   Visual contact with the tower is maintained; and

(iii)  A clearance to land is received.

Minimum altitudes when operating to an airport in Class D airspace.

(1) (2)

(3)

(4)

Unless required by the applicable distance-from-cloud criteria, each pilot operating a large or turbine- powered airplane must enter the traffic pattern at an altitude of at least 1,500 feet above the elevation of the airport and maintain at least 1,500 feet until further descent is required for a safe landing.

Each pilot operating a large or turbine-powered airplane approaching to land on a runway served by an instrument approach procedure with vertical guidance, if the airplane is so equipped, must:

(i)   Operate that airplane at an altitude at or above the glide path between the published final approach fix and the decision altitude (DA), or decision height (DH), as applicable; or

(ii)   If compliance with the applicable distance-from-cloud criteria requires glide path interception closer in, operate that airplane at or above the glide path, between the point of interception of glide path and the DA or the DH.

Each pilot operating an airplane approaching to land on a runway served by a visual approach slope indicator must maintain an altitude at or above the glide path until a lower altitude is necessary for a safe landing.

Paragraphs (e)(2) and (e)(3) of this section do not prohibit normal bracketing maneuvers above or below the glide path that are conducted for the purpose of remaining on the glide path.

Approaches. Except when conducting a circling approach under part 97 of this chapter or unless otherwise required by ATC, each pilot must—

14 CFR91.129(f)(enhanceddisplay)

page 36of243

(g)

(h) (i)

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

(1)   Circle the airport to the left, if operating an airplane; or

(2)   Avoid the flow offixed-wing aircraft, if operating a helicopter.

14 CFR91.129(f)(1)

Departures. No person may operate an aircraft departing from an airport except in compliance with the following:

(1)

Each pilot must comply with any departure procedures established for that airport by the FAA.

(2)   Unless otherwise required by the prescribed departure procedure for that airport or the applicable distance from clouds criteria, each pilot of a turbine-powered airplane and each pilot of a large airplane must climb to an altitude of 1,500 feet above the surface as rapidly as practicable.

Noise abatement. Where a formal runway use program has been established bythe FAA, each pilot of a large or turbine-powered airplane assigned a noise abatement runway by ATC must use that runway. However, consistent with the final authority of the pilot in command concerning the safe operation of the aircraft as prescribed in § 91.3(a), ATC may assign a different runway if requested bythe pilot in the interest of safety.

Takeoff, landing, taxi clearance. No person may, at any airport with an operating control tower, operate an aircraft on a runway or taxiway, or take off or land an aircraft, unless an appropriate clearance is received from ATC.

[Docket 24458, 56 FR 65658, Dec. 17, 1991, as amended byAmdt. 91-234, 58 FR 48793, Sept. 20, 1993;Amdt. 91-296, 72 FR 31678, June 7, 2007; 77 FR 28250, May 14, 2012]

§ 91.130 Operations in Class C airspace.

(a) General. Unless otherwise authorized by ATC, each aircraft operation in Class C airspace must be conducted in compliance with this section and § 91.129. For the purpose ofthis section, the primary airport is the airport for which the Class C airspace area is designated. A satellite airport is any other airport within the Class C airspace area.

(b) (c)

Traffic patterns. No person may take off or land an aircraft at a satellite airport within a Class C airspace area except in compliance with FAA arrival and departure traffic patterns.

Communications. Each person operating an aircraft in Class C airspace must meet the following two-way radio communications requirements:

(1)   Arrival or through flight. Each person must establish two-way radio communications with the ATC facility (including foreign ATC in the case of foreign airspace designated in the United States) providing air traffic services prior to entering that airspace and thereafter maintain those communications while within that airspace.

(2) Departing flight. Each person—

(i)   From the primary airport or satellite airport with an operating control tower must establish and maintain two-way radio communications with the control tower, and thereafter as instructed by ATC while operating in the Class C airspace area; or

(ii)   From a satellite airport without an operating control tower, must establish and maintain two- way radio communications with the ATC facility having jurisdiction over the Class C airspace area as soon as practicable after departing.

14 CFR91.130(c)(2)(ii) (enhanceddisplay)

page 37of243

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.130(d)

(d) (e)

Equipment requirements. Unless otherwise authorized by the ATC having jurisdiction overthe Class C airspace area, no person may operate an aircraft within a Class C airspace area designated for an airport unless that aircraft is equipped with the applicable equipment specified in § 91.215, and after January 1, 2020, § 91.225.

Deviations. An operator may deviate from any provision ofthis section underthe provisions of an ATC authorization issued by the ATC facility having jurisdiction overthe airspace concerned. ATC may authorize a deviation on a continuing basis or for an individual flight, as appropriate.

[Docket 24458, 56 FR 65659, Dec. 17, 1991, as amended byAmdt. 91-232, 58 FR 40736, July 30, 1993;Amdt. 91-239, 59 FR 11693, Mar. 11, 1994;Amdt. 91-314, 75 FR 30193, May28, 2010]

§ 91.131 Operations inClass B airspace.

(a) Operating rules. No person may operate an aircraft within a Class B airspace area except in compliance with § 91.129 and the following rules:

(1) (2)

(3)

The operator must receive an ATC clearance from the ATC facility having jurisdiction for that area before operating an aircraft in that area.

Unless otherwise authorized by ATC, each person operating a large turbine engine-powered airplane to or from a primary airport for which a Class B airspace area is designated must operate at or above the designated floors of the Class B airspace area while within the lateral limits of that area.

Any person conducting pilot training operations at an airport within a Class B airspace area must comply with any procedures established by ATC for such operations in that area.

(b)   Pilotrequirements.

(1)

(2)

No person may take off or land a civil aircraft at an airport within a Class B airspace area or operate a civil aircraft within a Class B airspace area unless—

(i)   The pilot in command holds at least a private pilot certificate;

(ii)   The pilot in command holds a recreational pilot certificate and has met— (A)  The requirements of § 61.101(d) of this chapter; or

(B)  The requirements for a student pilot seeking a recreational pilot certificate in § 61.94 of this chapter;

(iii)  The pilot in command holds a sport pilot certificate and has met— (A)  The requirements of § 61.325 of this chapter; or

(B)  The requirements for a student pilot seeking a recreational pilot certificate in § 61.94 of this chapter; or

(iv)  The aircraft is operated by a student pilot who has met the requirements of § 61.94 or § 61.95 of this chapter, as applicable.

Notwithstanding the provisions of paragraphs (b)(1)(ii), (b)(1)(iii) and (b)(1)(iv) ofthis section, no person may take off or land a civil aircraft at those airports listed in section 4 of appendix D to this part unless the pilot in command holds at least a private pilot certificate.

14 CFR91.131(b)(2) (enhanceddisplay)

page 38 of243

(c) (d)

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.131(c)

Communications andnavigation equipmentrequirements. Unless otherwise authorized by ATC, no person may operate an aircraft within a Class B airspace area unless that aircraft is equipped with—

(1) (2)

For IFR operation. An operable VOR or TACAN receiver or an operable and suitable RNAV system; and

For all operations. An operable two-way radio capable of communications with ATC on appropriate frequencies for that Class B airspace area.

Other equipment requirements. No person may operate an aircraft in a Class B airspace area unless the aircraft is equipped with—

(1) (2)

The applicable operating transponder and automatic altitude reporting equipment specified in § 91.215 (a), except as provided in § 91.215 (e), and

After January 1, 2020, the applicable Automatic Dependent Surveillance-Broadcast Out equipment specified in § 91.225.

[Docket 24458, 56 FR 65658, Dec. 17, 1991, as amended byAmdt. 91-282, 69 FR 44880, July 27, 2004;Amdt. 91-296, 72 FR 31678, June 7, 2007;Amdt. 91-314, 75 FR 30193, May28, 2010]

§ 91.133 Restrictedand prohibited areas.

(a)   No person may operate an aircraft within a restricted area (designated in part 73) contrary to the restrictions imposed, or within a prohibited area, unless that person has the permission of the using or controlling agency, as appropriate.

(b)

Each person conducting, within a restricted area, an aircraft operation (approved by the using agency) that creates the same hazards as the operations for which the restricted area was designated may deviate from the rules of this subpart that are not compatible with the operation of the aircraft.

§ 91.135 Operations in Class Aairspace.

Except as provided in paragraph (d) of this section, each person operating an aircraft in Class A airspace must conduct that operation under instrument flight rules (IFR) and in compliance with the following:

(a) (b)

(c) (d)

Clearance. Operations may be conducted only under an ATC clearance received priorto entering the airspace.

Communications. Unless otherwise authorized by ATC, each aircraft operating in Class A airspace must be equipped with a two-way radio capable of communicating with ATC on a frequency assigned by ATC.

Each pilot must maintain two-way radio communications with ATC while operating in Class A airspace.

Equipment requirements. Unless otherwise authorized by ATC, no person may operate an aircraft within Class A airspace unless that aircraft is equipped with the applicable equipment specified in § 91.215, and after January 1, 2020, § 91.225.

ATC authorizations. An operator may deviate from any provision ofthis section underthe provisions of an ATC authorization issued bythe ATC facility having jurisdiction of the airspace concerned. In the case of an inoperative transponder, ATC may immediately approve an operation within a Class A airspace area allowing flight to continue, if desired, to the airport of ultimate destination, including any intermediate stops, or to proceed to a place where suitable repairs can be made, or both. Requests for deviation from any provision ofthis section must be submitted in writing, at least 4 days before the proposed operation. ATC may authorize a deviation on a continuing basis or for an individual flight.

14 CFR91.135(d) (enhanceddisplay)

page 39 of243

(b) (c)

(d)

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

[Docket 24458, 56 FR 65659, Dec. 17, 1991, as amended byAmdt. 91-314, 75 FR 30193, May28, 2010]

14 CFR91.137

§ 91.137 Temporary flightrestrictions inthe  vicinityofdisaster/hazard areas.

(a)   The Administrator will issue a Notice to Airmen (NOTAM) designating an area within which temporary flight restrictions apply and specifying the hazard or condition requiring their imposition, whenever he determines it is necessary in orderto—

(1)

(2) (3)

Protect persons and property on the surface or in the air from a hazard associated with an incident on the surface;

Provide a safe environment for the operation of disaster relief aircraft; or

Prevent an unsafe congestion of sightseeing and other aircraft above an incident or event which may generate a high degree of public interest.

The Notice to Airmen will specifythe hazard or condition that requires the imposition of temporary flight restrictions.

When a NOTAM has been issued under paragraph (a)(1) ofthis section, no person may operate an aircraft within the designated area unless that aircraft is participating in the hazard relief activities and is being operated under the direction of the official in charge of on scene emergency response activities.

When a NOTAM has been issued under paragraph (a)(2) ofthis section, no person may operate an aircraft within the designated area unless at least one of the following conditions are met:

(1)

(2) (3) (4)

(5)

The aircraft is participating in hazard relief activities and is being operated under the direction of the official in charge of on scene emergency response activities.

The aircraft is carrying law enforcement officials.

The aircraft is operating underthe ATC approved IFR flight plan.

The operation is conducted directly to or from an airport within the area, or is necessitated bythe impracticability of VFR flight above or around the area due to weather, or terrain; notification is given to the Flight Service Station (FSS) or ATC facility specified in the NOTAM to receive advisories concerning disaster relief aircraft operations; and the operation does not hamper or endanger relief activities and is not conducted forthe purpose of observing the disaster.

The aircraft is carrying properly accredited news representatives, and, prior to entering the area, a flight plan is filed with the appropriate FAA or ATC facility specified in the Notice to Airmen and the operation is conducted above the altitude used by the disaster relief aircraft, unless otherwise authorized by the official in charge of on scene emergency response activities.

When a NOTAM has been issued under paragraph (a)(3) ofthis section, no person may operate an aircraft within the designated area unless at least one of the following conditions is met:

(1)

(2) (3)

The operation is conducted directly to or from an airport within the area, or is necessitated bythe impracticability of VFR flight above or around the area due to weather orterrain, and the operation is not conducted forthe purpose of observing the incident or event.

The aircraft is operating under an ATC approved IFR flight plan.

The aircraft is carrying incident or event personnel, or law enforcement officials.

14 CFR91.137(d)(3)(enhanceddisplay)

page 40 of243

(b) (c)

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.137(d)(4)

(4)

The aircraft is carrying properly accredited news representatives and, prior to entering that area, a flight plan is filed with the appropriate FSS or ATC facility specified in the NOTAM.

(e)

Flight plans filed and notifications made with an FSS or ATC facility under this section shall include the following information:

(1) (2) (3) (4) (5)

Aircraft identification, type and color.

Radio communications frequencies to be used.

Proposed times of entry of, and exit from, the designated area. Name of news media or organization and purpose of flight. Any other information requested byATC.

§ 91.138 Temporary flight restrictions in nationaldisaster areas in the State ofHawaii.

(a)  When the Administrator has determined, pursuant to a request and justification provided by the Governor of the State of Hawaii, or the Governor's designee, that an inhabited area within a declared national disaster area in the State of Hawaii is in need of protection for humanitarian reasons, the Administrator will issue a Notice to Airmen (NOTAM) designating an area within which temporary flight restrictions apply. The Administrator will designate the extent and duration ofthe temporary flight restrictions necessary to provide for the protection of persons and property on the surface.

When a NOTAM has been issued in accordance with this section, no person may operate an aircraft within the designated area unless at least one of the following conditions is met:

(1)

(2) (3) (4)

(5)

That person has obtained authorization from the official in charge of associated emergency or disaster relief response activities, and is operating the aircraft under the conditions ofthat authorization.

The aircraft is carrying law enforcement officials.

The aircraft is carrying persons involved in an emergency or a legitimate scientific purpose.

The aircraft is carrying properly accredited newspersons, and that prior to entering the area, a flight plan is filed with the appropriate FAA or ATC facility specified in the NOTAM and the operation is conducted in compliance with the conditions and restrictions established by the official in charge of on-scene emergency response activities.

The aircraft is operating in accordance with an ATC clearance or instruction.

A NOTAM issued under this section is effective for 90 days or until the national disaster area designation is terminated, whichever comes first, unless terminated by notice or extended by the Administrator at the request of the Governor of the State of Hawaii or the Governor's designee.

[Docket 26476, 56 FR 23178, May20, 1991, as amended byAmdt. 91-270, 66 FR 47377, Sept. 11, 2001]

§ 91.139 Emergencyairtraffic rules.

(a)   This section prescribes a process for utilizing Notices to Airmen (NOTAMs) to advise of the issuance and operations under emergency air traffic rules and regulations and designates the official who is authorized to issue NOTAMs on behalf of the Administrator in certain matters under this section.

14 CFR91.139(a) (enhanceddisplay)

page 41of243

(b) (c)

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.139(b)

Whenever the Administrator determines that an emergency condition exists, or will exist, relating to the FAA's ability to operate the airtraffic control system and during which normal flight operations underthis chapter cannot be conducted consistent with the required levels of safety and efficiency—

(1) (2)

The Administrator issues an immediately effective air traffic rule or regulation in response to that emergency condition; and

The Administrator or the Associate Administrator for Air Traffic may utilize the NOTAM system to provide notification ofthe issuance ofthe rule or regulation.

Those NOTAMs communicate information concerning the rules and regulations that govern flight operations, the use of navigation facilities, and designation of that airspace in which the rules and regulations apply.

When a NOTAM has been issued underthis section, no person may operate an aircraft, or other device governed by the regulation concerned, within the designated airspace except in accordance with the authorizations, terms, and conditions prescribed in the regulation covered bythe NOTAM.

§ 91.141Flight restrictions in the proximityofthe  Presidential and other parties.

No person may operate an aircraft over or in the vicinity of any area to be visited or traveled by the President, the Vice President, or other public figures contrary to the restrictions established by the Administrator and published in a Notice to Airmen (NOTAM).

§ 91.143 Flightlimitation  inthe  proximityofspaceflight  operations.

When a Notice to Airmen (NOTAM) is issued in accordance with this section, no person may operate any aircraft of U.S. registry, or pilot any aircraft under the authority of an airman certificate issued by the Federal Aviation Administration, within areas designated in a NOTAM for space flight operation except when authorized by ATC.

[Docket FAA-2004-19246, 69 FR 59753, Oct. 5, 2004]

§ 91.144Temporaryrestriction  onflight operations during  abnormallyhighbarometric pressure conditions.

(a)

(b)

Special flight restrictions. When any information indicates that barometric pressure on the route of flight currently exceeds or will exceed 31 inches of mercury, no person may operate an aircraft or initiate a flight contrary to the requirements established bythe Administrator and published in a Notice to Airmen issued under this section.

Waivers. The Administrator is authorized to waive any restriction issued under paragraph (a) of this section to permit emergency supply, transport, or medical services to be delivered to isolated communities, where the operation can be conducted with an acceptable level of safety.

[Amdt. 91-240, 59 FR 17452, Apr. 12, 1994; 59 FR 37669, July25, 1994]

14 CFR91.144(b) (enhanceddisplay)

page 42of243

(a) (b)

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.145

§ 91.145 Management ofaircraft operations in the vicinityofaerial demonstrations and major sporting events.

The FAA will issue a Notice to Airmen (NOTAM) designating an area of airspace in which a temporary flight restriction applies when it determines that a temporary flight restriction is necessary to protect persons or property on the surface or in the air, to maintain air safety and efficiency, orto prevent the unsafe congestion of aircraft in the vicinity of an aerial demonstration or major sporting event. These demonstrations and events may include:

(1) (2) (3)

United States Naval Flight Demonstration Team (Blue Angels); United States Air Force Air Demonstration Squadron (Thunderbirds); United States Army Parachute Team (Golden Knights);

(4)  Summer/Winter Olympic Games;

(5)

Annual Tournament of Roses Football Game;

(6)  World Cup Soccer;

(7)

Major League Baseball All-Star Game;

(8)  World Series;

(9)

Kodak Albuquerque International Balloon Fiesta;

(10) Sandia Classic Hang Gliding Competition; (11) Indianapolis 500 Mile Race;

(12)

Any other aerial demonstration or sporting event the FAA determines to need a temporary flight restriction in accordance with paragraph (b) of this section.

In deciding whether a temporary flight restriction is necessary for an aerial demonstration or major sporting event not listed in paragraph (a) of this section, the FAA considers the following factors:

(1) (2) (3) (4) (5)

Area where the event will be held.

Effect flight restrictions will have on known aircraft operations. Any existing ATC airspace traffic management restrictions. Estimated duration of the event.

Degree of public interest.

(6)   Number of spectators.

(7) (8) (9)

Provisions for spectator safety.

Number and types of participating aircraft.

Use of mixed high and low performance aircraft.

(10)  Impact on non-participating aircraft. (11)  Weather minimums.

(12) Emergency procedures that will be in effect. 14 CFR91.145(b)(12)(enhanced display)

page 43 of243

(c) (d)

(e)

(f) (g)

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.145(c)

A NOTAM issued under this section will state the name ofthe aerial demonstration or sporting event and specify the effective dates and times, the geographic features or coordinates, and any other restrictions or procedures governing flight operations in the designated airspace.

When a NOTAM has been issued in accordance with this section, no person may operate an aircraft or device, or engage in any activity within the designated airspace area, except in accordance with the authorizations, terms, and conditions of the temporary flight restriction published in the NOTAM, unless otherwise authorized by:

(1) (2)

Air traffic control; or

A Flight Standards Certificate of Waiver orAuthorization issued for the demonstration or event.

For the purpose of this section:

(1)

(2)

Flight restrictedairspace area for an aerialdemonstration— The amount of airspace needed to protect persons and property on the surface or in the air, to maintain air safety and efficiency, orto prevent the unsafe congestion of aircraft will vary depending on the aerial demonstration and the factors listed in paragraph (b) of this section. The restricted airspace area will normally be limited to a 5 nautical mile radius from the center of the demonstration and an altitude 17000 mean sea level (for high performance aircraft) or 13000 feet above the surface (for certain parachute operations), but

will be no greater than the minimum airspace necessary forthe management of aircraft operations in the vicinity ofthe specified area.

Flight restrictedarea fora majorsporting event— The amount of airspace needed to protect persons and property on the surface or in the air, to maintain air safety and efficiency, or to prevent the unsafe congestion of aircraft will vary depending on the size ofthe event and the factors listed in paragraph (b) of this section. The restricted airspace will normally be limited to a 3 nautical mile radius from

the center ofthe event and 2500 feet above the surface but will not be greater than the minimum airspace necessary forthe management of aircraft operations in the vicinity ofthe specified area.

A NOTAM issued under this section will be issued at least 30 days in advance of an aerial demonstration or a major sporting event, unless the FAA finds good cause for a shorter period and explains this in the NOTAM.

When warranted, the FAA Administrator may exclude the following flights from the provisions of this section:

(1)  Essential military. (2)  Medical and rescue.

(3) (4) (5) (6)

Presidential and Vice Presidential. Visiting heads of state.

Law enforcement and security. Public health and welfare.

[Docket FAA-2000-8274, 66 FR 47378, Sept. 11, 2001]

14 CFR91.145(g)(6)(enhanceddisplay)

page 44of243

(a) (b)

(c)

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.146

§ 91.146 Passenger-carrying flights for thebenefit  ofacharitable, nonprofit, or community event.

Definitions. For purposes ofthis section, the following definitions apply:

Charitable event means an event that raises funds for the benefit of a charitable organization recognized by the Department ofthe Treasury whose donors may deduct contributions under section 170 ofthe Internal Revenue Code (26 U.S.C. Section 170).

Community event means an event that raises funds for the benefit of any local or community cause that is not a charitable event or non-profit event.

Non-profit event means an event that raises funds for the benefit of a non-profit organization recognized under State or Federal law, as long as one of the organization's purposes is the promotion of aviation safety.

Passenger-carrying flights in airplanes, powered-lift, or rotorcraft for the benefit of a charitable, nonprofit, or community event identified in paragraph (c) of this section are not subject to the certification requirements of part 119 of this chapter orthe drug and alcohol testing requirements in part 120 of this chapter, provided the following conditions are satisfied and the limitations in paragraphs (c) and (d) of this section are not exceeded:

(1) (2) (3)

(4) (5)

(6) (7)

(8) (9) (10)

(11)

The flight is nonstop and begins and ends at the same airport and is conducted within a 25-statute mile radius of that airport;

The flight is conducted from a public airport that is adequate for the aircraft used, or from another location the FAA approves for the operation;

The aircraft has a maximum of 30 seats, excluding each crewmember seat, and a maximum payload capacity of 7,500 pounds;

The flight is not an aerobatic or a formation flight;

Each aircraft holds a standard airworthiness certificate, is airworthy, and is operated in compliance with the applicable requirements of subpart E of this part;

Each flight is made during dayVFR conditions;

Reimbursement ofthe operator ofthe aircraft is limited to that portion of the passenger payment for the flight that does not exceed the pro rata cost of owning, operating, and maintaining the aircraft for that flight, which may include fuel, oil, airport expenditures, and rental fees;

The beneficiary of the funds raised is not in the business of transportation by air; A private pilot acting as pilot in command has at least 500 hours of flight time;

Each flight is conducted in accordance with the safety provisions of part 136, subpart A of this chapter; and

Flights are not conducted over a national park, unit of a national park, or abutting tribal lands, unless the operator has secured a letter of agreement from the FAA, as specified under subpart B of part 136 of this chapter, and is operating in accordance with that agreement during the flights.

14 CFR91.146(c) (enhanceddisplay)

page 45of243

(b) (c)

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.146(c)(1)

(1) (2)

Passenger-carrying flights or series of flights are limited to a total of four charitable events or non- profit events per year, with no event lasting more than three consecutive days.

Passenger-carrying flights or series of flights are limited to one community event per year, with no event lasting more than three consecutive days.

(d) (e)

Pilots and sponsors of events described in this section are limited to no more than 4 events per calendar year.

At least seven days before the event, each sponsor of an event described in this section must furnish to the responsible Flight Standards office for the area where the event is scheduled:

(1) (2) (3)

A signed letter detailing the name of the sponsor, the purpose ofthe event, the date and time of the event, the location ofthe event, all prior events under this section participated in by the sponsor in the current calendar year;

A photocopy of each pilot in command's pilot certificate, medical certificate, and logbook entries that show the pilot is current in accordance with §§ 61.56 and 61.57 of this chapter and that any private pilot has at least 500 hours of flight time; and

A signed statement from each pilot that lists all prior events under this section in which the pilot has participated during the current calendar year.

[Docket FAA-1998-4521, 72 FR 6910, Feb. 13, 2007, as amendedbyAmdt. 91-308, 74 FR 32804, July 9, 2009; Docket FAA-2018-0119, Amdt. 91-350, 83 FR 9171, Mar. 5, 2018; DocketFAA-2022-1563, Amdt. 91-370, 88 FR 48087, July26, 2023]

§ 91.147Passenger-carrying flights for compensation or hire.

(a)  Definitions. For the purposes of this section, Operator means any person conducting nonstop passenger- carrying flights in an airplane, powered-lift, or rotorcraft for compensation or hire in accordance with §§ 119.1(e)(2), 135.1(a)(5), or 121.1(d) of this chapter that begin and end at the same airport and are conducted within a 25-statute mile radius of that airport.

General requirements. An Operator conducting passenger-carrying flights for compensation or hire must meet the following requirements unless all flights are conducted under § 91.146. The Operator must:

(1) (2)

(3) (4)

Comply with the safety provisions of part 136, subpart A of this chapter.

Register and implement its drug and alcohol testing programs in accordance with part 120 of this chapter.

Comply with the applicable requirements of part 5 of this chapter.

Apply for and receive a Letter of Authorization from the responsible Flight Standards office.

Letter ofAuthorization. Each application for a Letter of Authorization must include the following information:

(1)

(2) (3) (4)

Name of Operator, agent, and any d/b/a (doing-business-as) under which that Operator does business.

Principal business address and mailing address.

Principal place of business (if different from business address). Name of person responsible for management of the business.

14 CFR91.147(c)(4) (enhanceddisplay)

page 46of243

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.147(c)(5)

(5) (6) (7)

Name of person responsible for aircraft maintenance.

Type of aircraft, registration number(s), and make/model/series. Antidrug and Alcohol Misuse Prevention Program registration.

(d)

Compliance. The Operator must comply with the provisions ofthe Letter of Authorization received.

[Docket FAA-2021-0419, Amdt. 91-374, 89 FR 33108, Apr. 26, 2024] §§ 91.148-91.149 [Reserved]

VISUAL FLIGHT RULES

§ 91.151Fuel requirements for flightin VFR conditions.

(a)  No person may begin a flight in an airplane under VFR conditions unless (considering wind and forecast weather conditions) there is enough fuel to fly to the first point of intended landing and, assuming normal cruising speed—

(1) (2)

During the day, to fly after that for at least 30 minutes; or At night, to fly after that for at least 45 minutes.

(b)

No person may begin a flight in a rotorcraft under VFR conditions unless (considering wind and forecast weather conditions) there is enough fuel to fly to the first point of intended landing and, assuming normal cruising speed, to fly afterthat for at least 20 minutes.

§ 91.153VFR flight plan: Information required.

(a)  Information required. Unless otherwise authorized by ATC, each person filing a VFR flight plan shall include in it the following information:

(1) (2)

(3)

(4) (5) (6) (7) (8)

(9)

The aircraft identification number and, if necessary, its radio call sign.

The type of the aircraft or, in the case of a formation flight, the type of each aircraft and the number of aircraft in the formation.

The full name and address ofthe pilot in command or, in the case of a formation flight, the formation commander.

The point and proposed time of departure.

The proposed route, cruising altitude (or flight level), and true airspeed at that altitude. The point offirst intended landing and the estimated elapsed time until over that point. The amount of fuel on board (in hours).

The number of persons in the aircraft, except where that information is otherwise readily available to the FAA.

Any other information the pilot in command or ATC believes is necessary for ATC purposes.

(b)

Cancellation. When a flight plan has been activated, the pilot in command, upon canceling or completing the flight under the flight plan, shall notify an FAA Flight Service Station or ATC facility.

14 CFR91.153(b)(enhanceddisplay)

page 47of243

Flight visibility

Not Applicable 3 statute

miles

3 statute miles

3 statute miles

3 statute miles

5 statute miles

1 statute mile

3 statute miles

1⁄2 statute mile

Distance from clouds

Not Applicable. Clear of Clouds. 500 feet below.

1,000 feet above. 2,000 feet horizontal. 500 feet below.

1,000 feet above. 2,000 feet horizontal.

500 feet below.

1,000 feet above. 2,000 feet horizontal. 1,000 feet below.

1,000 feet above. 1 statute mile horizontal.

Clear of clouds. 500 feet below.

1,000 feet above. 2,000 feet horizontal.

Clear of clouds

page 48 of243

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.155

§ 91.155 Basic VFRweather minimums.

(a)   Except as provided in paragraph (b) ofthis section and § 91.157, no person may operate an aircraft under VFR when the flight visibility is less, or at a distance from clouds that is less, than that prescribed for the corresponding altitude and class of airspace in the following table:

Airspace

Class A Class B Class C

Class D

Class E:

Less than 10,000 feet MSL At or above 10,000 feet MSL

Class G:

1,200 feet or less above the surface (regardless of MSL altitude)

For aircraft otherthan helicopters: Day, except as provided in § 91.155(b)

Night, except as provided in § 91.155(b)

For helicopters: Day

14 CFR91.155(a)(enhanced display)

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

Airspace Night, except as provided in § 91.155(b)

More than 1,200 feet above the surface but less than 10,000 feet MSL

Day Night

More than 1,200 feet above the surface and at or above 10,000 feet MSL

14 CFR91.155(b)

Flight             Distance from visibility                 clouds

1 statute mile

1 statute mile

3 statute miles

5 statute miles

Clear of clouds.

500 feet below.

1,000 feet above. 2,000 feet horizontal. 500 feet below.

1,000 feet above. 2,000 feet horizontal. 1,000 feet below.

1,000 feet above.

1 statute mile horizontal.

(b)

(c) (d)

(e)

Class GAirspace. Notwithstanding the provisions of paragraph (a) ofthis section, the following operations may be conducted in Class G airspace below 1,200 feet above the surface:

(1)  Helicopter. A helicopter may be operated clear of clouds in an airport traffic pattern within 1⁄2 mile of the runway or helipad of intended landing ifthe flight visibility is not less than 1⁄2 statute mile.

(2)  Airplane, poweredparachute, or weight-shift-controlaircraft.  Ifthe visibility is less than 3 statute miles but not less than 1 statute mile during night hours and you are operating in an airport traffic pattern within 1⁄2 mile ofthe runway, you may operate an airplane, powered parachute, or weight-shift-control aircraft clear of clouds.

Except as provided in § 91.157, no person may operate an aircraft beneath the ceiling under VFR within the lateral boundaries of controlled airspace designated to the surface for an airport when the ceiling is less than 1,000 feet.

Except as provided in § 91.157 of this part, no person maytake off or land an aircraft, or enterthe traffic pattern of an airport, under VFR, within the lateral boundaries ofthe surface areas of Class B, Class C, Class D, or Class E airspace designated for an airport—

(1)   Unless ground visibility at that airport is at least 3 statute miles; or

(2)   If ground visibility is not reported at that airport, unless flight visibility during landing or takeoff, or while operating in the traffic pattern is at least 3 statute miles.

For the purpose of this section, an aircraft operating at the base altitude of a Class E airspace area is considered to be within the airspace directly below that area.

14 CFR91.155(e)(enhanceddisplay)

page 49 of243

(b) (c) (d)

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.157

[Docket 24458, 56 FR 65660, Dec. 17, 1991, as amended byAmdt. 91-235, 58 FR 51968, Oct. 5, 1993; Amdt. 91-282, 69 FR 44880, July 27, 2004;Amdt. 91-330, 79 FR 9972, Feb. 21, 2014; Amdt. 91-330A, 79 FR 41125, July 15, 2014]

§ 91.157 SpecialVFRweatherminimums.

(a)  Except as provided in appendix D, section 3, ofthis part, special VFR operations may be conducted under the weather minimums and requirements ofthis section, instead ofthose contained in § 91.155, below 10,000 feet MSL within the airspace contained by the upward extension of the lateral boundaries of the controlled airspace designated to the surface for an airport.

Special VFR operations may only be conducted— (1)  With an ATC clearance;

(2)   Clear of clouds;

(3) (4)

Except for helicopters, when flight visibility is at least 1 statute mile; and

Except for helicopters, between sunrise and sunset (or in Alaska, when the sun is 6 degrees or less below the horizon) unless—

(i)

The person being granted the ATC clearance meets the applicable requirements for instrument flight under part 61 of this chapter; and

(ii)   The aircraft is equipped as required in § 91.205(d).

No person may take off or land an aircraft (other than a helicopter) under special VFR—

(1) (2)

Unless ground visibility is at least 1 statute mile; or

If ground visibility is not reported, unless flight visibility is at least 1 statute mile. For the purposes of this paragraph, the term flight visibility includes the visibility from the cockpit of an aircraft in takeoff position if:

(i) (ii)

The flight is conducted under this part 91; and

The airport at which the aircraft is located is a satellite airport that does not have weather reporting capabilities.

The determination of visibility by a pilot in accordance with paragraph (c)(2) of this section is not an official weather report or an official ground visibility report.

[Amdt. 91-235, 58 FR 51968, Oct. 5, 1993, as amendedbyAmdt. 91-247, 60 FR 66874, Dec. 27, 1995;Amdt. 91-262, 65 FR 16116, Mar. 24, 2000; DocketFAA-2022-1355,Amdt. 91-366, 87 FR 75846, Dec. 9, 2022]

§ 91.159 VFR cruising altitude or flight level.

Except while holding in a holding pattern of 2 minutes or less, or while turning, each person operating an aircraft under VFR in level cruising flight more than 3,000 feet above the surface shall maintain the appropriate altitude or flight level prescribed below, unless otherwise authorized by ATC:

(a)

When operating below 18,000 feet MSL and—

(1)

On a magnetic course of zero degrees through 179 degrees, any odd thousand foot MSL altitude + 500 feet (such as 3,500, 5,500, or 7,500); or

14 CFR91.159(a)(1)(enhanceddisplay)

page 50 of243

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.159(a)(2)

(b)

(2) On a magnetic course of 180 degrees through 359 degrees, any even thousand foot MSL altitude + 500 feet (such as 4,500, 6,500, or 8,500).

When operating above 18,000 feet MSL, maintain the altitude or flight level assigned by ATC.

[Docket 18334, 54 FR 34294, Aug. 18, 1989, as amendedbyAmdt. 91-276, 68 FR 61321, Oct. 27, 2003; 68 FR 70133, Dec. 17, 2003]

§ 91.161 Special awareness training required for pilots flying undervisual flight rules within a 60-nautical mile radius ofthe Washington, DCVOR/DME.

(a)

(b) (c)

(d) (e)

Operations within a 60-nautical mile radius ofthe Washington, DC VOR/DME under visual flightrules (VFR). Except as provided under paragraph (e) ofthis section, no person may serve as a pilot in command or as second in command of an aircraft while flying within a 60-nautical mile radius of the DCA VOR/DME, under VFR, unless that pilot has completed Special Awareness Training and holds a certificate of training completion.

Special Awareness Training. The Special Awareness Training consists of information to educate pilots about the procedures for flying in the Washington, DC area and, more generally, in other types of special use airspace. This free training is available on the FAA's Web site. Upon completion ofthe training, each person will need to print out a copy ofthe certificate  oftraining completion.

Inspection ofcertificate  oftraining completion. Each person who holds a certificate for completing the Special Awareness Training must present it for inspection upon request from:

(1)   An authorized representative ofthe FAA;

(2)   An authorized representative ofthe National Transportation Safety Board; (3)  Any Federal, State, or local law enforcement officer; or

(4)   An authorized representative ofthe Transportation Security Administration.

Emergency declared. The failure to complete the Special Awareness Training course on flying in and around the Washington, DC Metropolitan Area is not a violation ofthis section if an emergency is declared by the pilot, as described under § 91.3(b), or there was a failure oftwo-way radio communications when operating under IFR as described under § 91.185.

Exceptions. The requirements of this section do not apply ifthe flight is being performed in an aircraft of an air ambulance operator certificated to conduct part 135 operations under this chapter, the U.S. Armed Forces, or a law enforcement agency.

[Docket FAA-2006-25250, 73 FR 46803, Aug. 12, 2008] §§ 91.162-91.165 [Reserved]

INSTRUMENT FLIGHT RULES

§ 91.167 Fuel requirements for flight in IFR conditions.

(a)   No person may operate a civil aircraft in IFR conditions unless it carries enough fuel (considering weather reports and forecasts and weather conditions) to—

(1)   Complete the flight to the first airport of intended landing; 14 CFR91.167(a)(1)(enhanceddisplay)

page 51of243

(b)

(c)

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.167(a)(2)

(2) (3)

Except as provided in paragraph (b) ofthis section, fly from that airport to the alternate airport; and

Fly after that for 45 minutes at normal cruising speed or, for helicopters, fly after that for 30 minutes at normal cruising speed.

(b)

Paragraph (a)(2) ofthis section does not apply if:

(1)

(2)

Part 97 of this chapter prescribes a standard instrument approach procedure to, or a special instrument approach procedure has been issued by the Administrator to the operator for, the first airport of intended landing; and

Appropriate weather reports or weather forecasts, or a combination ofthem, indicate the following:

(i) (ii)

For aircraft otherthan helicopters. For at least 1 hour before and for 1 hour afterthe estimated time of arrival, the ceiling will be at least 2,000 feet above the airport elevation and the visibility will be at least 3 statute miles.

For helicopters. At the estimated time of arrival and for 1 hour afterthe estimated time of arrival, the ceiling will be at least 1,000 feet above the airport elevation, or at least 400 feet above the lowest applicable approach minima, whichever is higher, and the visibility will be at least 2 statute miles.

[Docket 98-4390, 65 FR 3546, Jan. 21, 2000]

§ 91.169 IFR flight plan: Information required.

(a)  Information required. Unless otherwise authorized by ATC, each person filing an IFR flight plan must include in it the following information:

(1) (2)

Information required under § 91.153 (a) of this part;

Except as provided in paragraph (b) ofthis section, an alternate airport.

Paragraph (a)(2) ofthis section does not apply if :

(1)

(2)

Part 97 of this chapter prescribes a standard instrument approach procedure to, or a special instrument approach procedure has been issued by the Administrator to the operator for, the first airport of intended landing; and

Appropriate weather reports or weather forecasts, or a combination ofthem, indicate the following:

(i) (ii)

For aircraft otherthan helicopters. For at least 1 hour before and for 1 hour afterthe estimated time of arrival, the ceiling will be at least 2,000 feet above the airport elevation and the visibility will be at least 3 statute miles.

For helicopters. At the estimated time of arrival and for 1 hour afterthe estimated time of arrival, the ceiling will be at least 1,000 feet above the airport elevation, or at least 400 feet above the lowest applicable approach minima, whichever is higher, and the visibility will be at least 2 statute miles.

IFR alternate airport weatherminima. Unless otherwise authorized by the Administrator, no person may include an alternate airport in an IFR flight plan unless appropriate weather reports or weather forecasts, or a combination ofthem, indicate that, at the estimated time of arrival at the alternate airport, the ceiling and visibility at that airport will be at or above the following weather minima:

14 CFR91.169(c)(enhanceddisplay)

page 52of243

(1) (2)

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.169(c)(1)

If an instrument approach procedure has been published in part 97 of this chapter, or a special instrument approach procedure has been issued by the Administrator to the operator, for that airport, the following minima:

(i)

(ii)

For aircraft otherthan helicopters: The alternate airport minima specified in that procedure, or if none are specified the following standard approach minima:

(A)  Fora precision approach procedure. Ceiling 600 feet and visibility 2 statute miles.

(B)  Fora nonprecision approach procedure. Ceiling 800 feet and visibility 2 statute miles.

For helicopters: Ceiling 200 feet above the minimum forthe approach to be flown, and visibility at least 1 statute mile but never less than the minimum visibility for the approach to be flown, and

If no instrument approach procedure has been published in part 97 of this chapter and no special instrument approach procedure has been issued by the Administrator to the operator, for the alternate airport, the ceiling and visibility minima are those allowing descent from the MEA, approach, and landing under basic VFR.

(d)

Cancellation. When a flight plan has been activated, the pilot in command, upon canceling or completing the flight under the flight plan, shall notify an FAA Flight Service Station or ATC facility.

[Docket 18334, 54 FR 34294, Aug. 18, 1989, as amendedbyAmdt. 91-259, 65 FR 3546, Jan. 21, 2000]

§ 91.171VORequipmentcheck for IFR operations.

(a)  No person may operate a civil aircraft under IFR using the VOR system of radio navigation unless the VOR equipment ofthat aircraft—

(1) (2)

Is maintained, checked, and inspected under an approved procedure; or

Has been operationally checked within the preceding 30 days, and was found to be within the limits of the permissible indicated bearing error set forth in paragraph (b) or (c) of this section.

(b)

Except as provided in paragraph (c) of this section, each person conducting a VOR check under paragraph (a)(2) of this section shall—

(1)

(2) (3)

(4)

Use, at the airport of intended departure, an FAA-operated or approved test signal or a test signal radiated by a certificated and appropriately rated radio repair station or, outside the United States, a test signal operated or approved by an appropriate authority to check the VOR equipment (the maximum permissible indicated bearing error is plus or minus 4 degrees); or

Use, at the airport of intended departure, a point on the airport surface designated as a VOR system checkpoint by the Administrator, or, outside the United States, by an appropriate authority (the maximum permissible bearing error is plus or minus 4 degrees);

If neither a test signal nor a designated checkpoint on the surface is available, use an airborne checkpoint designated by the Administrator or, outside the United States, by an appropriate authority (the maximum permissible bearing error is plus or minus 6 degrees); or

If no check signal or point is available, while in flight—

(i)

Select a VOR radial that lies along the centerline of an established VOR airway;

14 CFR91.171(b)(4)(i) (enhanceddisplay)

page 53 of243

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.171(b)(4)(ii)

(ii)

(iii)

Select a prominent ground point along the selected radial preferably more than 20 nautical miles from the VOR ground facility and maneuver the aircraft directly overthe point at a reasonably low altitude; and

Note the VOR bearing indicated by the receiver when over the ground point (the maximum permissible variation between the published radial and the indicated bearing is 6 degrees).

(c) (d)

If dual system VOR (units independent of each other except for the antenna) is installed in the aircraft, the person checking the equipment may check one system against the other in place of the check procedures specified in paragraph (b) ofthis section. Both systems shall be tuned to the same VOR ground facility and note the indicated bearings to that station. The maximum permissible variation between the two indicated bearings is 4 degrees.

Each person making the VOR operational check, as specified in paragraph (b) or (c) of this section, shall enter the date, place, bearing error, and sign the aircraft log or other record. In addition, if a test signal radiated by a repair station, as specified in paragraph (b)(1) ofthis section, is used, an entry must be made in the aircraft log or other record by the repair station certificate holder orthe certificate holder's representative certifying to the bearing transmitted by the repair station forthe check and the date of transmission.

(Approved by the Office ofManagement and Budget under control number2120-0005)

§ 91.173ATC clearance and flight plan required.

No person may operate an aircraft in controlled airspace under IFR unless that person has—

(a) (b)

Filed an IFR flight plan; and

Received an appropriate ATC clearance.

§ 91.175 Takeoffand landing under IFR.

(a)   Instrumentapproaches to civil airports. Unless otherwise authorized by the FAA, when it is necessaryto use an instrument approach to a civil airport, each person operating an aircraft must use a standard instrument approach procedure prescribed in part 97 of this chapter for that airport. This paragraph does not apply to United States military aircraft.

(b) (c)

Authorized DA/DH or MDA. For the purpose ofthis section, when the approach procedure being used provides for and requires the use of a DA/DH or MDA, the authorized DA/DH or MDA is the highest ofthe following:

(1)  The DA/DH or MDA prescribed by the approach procedure. (2)   The DA/DH or MDA prescribed for the pilot in command.

(3)   The DA/DH or MDA appropriate forthe aircraft equipment available and used during the approach.

Operation below DA/DHorMDA. Except as provided in § 91.176 of this chapter, where a DA/DH or MDA is applicable, no pilot may operate an aircraft, except a military aircraft of the United States, below the authorized MDA or continue an approach below the authorized DA/DH unless—

14 CFR91.175(c)(enhanceddisplay)

page 54of243

(d) (e)

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.175(c)(1)

(1)

(2) (3)

The aircraft is continuously in a position from which a descent to a landing on the intended runway can be made at a normal rate of descent using normal maneuvers, and for operations conducted under part 121 or part 135 unless that descent rate will allow touchdown to occur within the touchdown zone of the runway of intended landing;

The flight visibility is not less than the visibility prescribed in the standard instrument approach being used; and

Except for a Category II or Category III approach where any necessary visual reference requirements are specified bythe Administrator, at least one of the following visual references for the intended runway is distinctlyvisible and identifiable to the pilot:

(i)

The approach light system, except that the pilot may not descend below 100 feet above the touchdown zone elevation using the approach lights as a reference unless the red terminating bars or the red side row bars are also distinctly visible and identifiable.

(ii)   The threshold.

(iii)  The threshold markings. (iv)  The threshold lights.

(v)

The runway end identifier lights.

(vi)  The visual glideslope indicator.

(vii)  The touchdown zone or touchdown zone markings. (viii)  The touchdown zone lights.

(ix)  The runway or runway markings. (x)   The runway lights.

Landing. No pilot operating an aircraft, except a military aircraft of the United States, may land that aircraft when—

(1) (2)

For operations conducted under § 91.176 of this part, the requirements of paragraphs (a)(3)(iii) or (b)(3)(iii), as applicable, ofthat  section are not met; or

For all other operations underthis part and parts 121, 125, 129, and 135, the flight visibility is less than the visibility prescribed in the standard instrument approach procedure being used.

Missed approachprocedures. Each pilot operating an aircraft, except a military aircraft ofthe United States, shall immediately execute an appropriate missed approach procedure when either ofthe  following conditions exist:

(1)

Whenever operating an aircraft pursuant to paragraph (c) of this section or § 91.176 of this part, and the requirements of that paragraph or section are not met at either of the following times:

(i) (ii)

When the aircraft is being operated below MDA; or

Upon arrival at the missed approach point, including a DA/DH where a DA/DH is specified and its use is required, and at any time afterthat until touchdown.

14 CFR91.175(e)(1)(ii)(enhanceddisplay)

page 55of243

(f)

(g) (h)

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.175(e)(2)

(2)

Whenever an identifiable part ofthe airport is not distinctly visible to the pilot during a circling maneuver at or above MDA, unless the inability to see an identifiable part of the airport results only from a normal bank of the aircraft during the circling approach.

Civil airport takeoffminimums. This paragraph applies to persons operating an aircraft under part 121, 125, 129, or 135 of this chapter.

(1) (2)

(3) (4)

Unless otherwise authorized by the FAA, no pilot may takeoff from a civil airport under IFR unless the weather conditions at time of takeoff are at or above the weather minimums for IFR takeoff prescribed forthat airport under part 97 of this chapter.

If takeoff weather minimums are not prescribed under part 97 of this chapter for a particular airport, the following weather minimums apply to takeoffs under IFR:

(i)   For aircraft, otherthan helicopters, having two engines or less—1 statute mile visibility. (ii)   For aircraft having more than two engines— 1⁄2 statute mile visibility.

(iii)  For helicopters— 1⁄2 statute mile visibility.

Except as provided in paragraph (f)(4) ofthis section, no pilot may takeoff under IFR from a civil airport having published obstacle departure procedures (ODPs) under part 97 of this chapter for the takeoff runway to be used, unless the pilot uses such ODPs or an alternative procedure or route assigned by air traffic control.

Notwithstanding the requirements of paragraph (f)(3) ofthis section, no pilot may takeoff from an airport under IFR unless:

(i)   For part 121 and part 135 operators, the pilot uses a takeoff obstacle clearance or avoidance procedure that ensures compliance with the applicable airplane performance operating limitations requirements under part 121, subpart I or part 135, subpart I for takeoff at that airport; or

(ii)   For part 129 operators, the pilot uses a takeoff obstacle clearance or avoidance procedure that ensures compliance with the airplane performance operating limitations prescribed by the State of the operator for takeoff at that airport.

Military airports. Unless otherwise prescribed by the Administrator, each person operating a civil aircraft under IFR into or out of a military airport shall comply with the instrument approach procedures and the takeoff and landing minimum prescribed by the military authority having jurisdiction of that airport.

Comparable values ofRVRand ground visibility.

(1)

Except for Category II or Category III minimums, if RVR minimums for takeoff or landing are prescribed in an instrument approach procedure, but RVR is not reported for the runway of intended operation, the RVR minimum shall be converted to ground visibility in accordance with the table in paragraph (h)(2) of this section and shall be the visibility minimum  fortakeoff or landing on that runway.

14 CFR91.175(h)(1)(enhanceddisplay)

page 56of243

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

(2)

RVR (feet) 1,600

2,400 3,200 4,000 4,500 5,000 6,000

14 CFR91.175(h)(2)

Visibility (statute miles)

1⁄4 1⁄2 5⁄8 3⁄4 7⁄8

1 11⁄4

(i)

(j) (k)

Operations on unpublishedroutes and use ofradar in instrument approach procedures. When radar is approved at certain locations for ATC purposes, it may be used not only for surveillance and precision radar approaches, as applicable, but also may be used in conjunction with instrument approach procedures predicated on othertypes of radio navigational aids. Radar vectors may be authorized to provide course guidance through the segments of an approach to the final course or fix. When operating on an unpublished route or while being radar vectored, the pilot, when an approach clearance is received, shall, in addition to complying with § 91.177, maintain the last altitude assigned to that pilot until the aircraft is established on a segment of a published route or instrument approach procedure unless a different altitude is assigned byATC. Afterthe aircraft is so established, published altitudes apply to descent within each succeeding route or approach segment unless a different altitude is assigned by ATC. Upon reaching the final approach course or fix, the pilot may either complete the instrument approach in accordance with a procedure approved for the facility or continue a surveillance or precision radar approach to a landing.

Limitation on procedure turns. In the case of a radar vector to a final approach course or fix, a timed approach from a holding fix, or an approach for which the procedure specifies “No PT,” no pilot may make a procedure turn unless cleared to do so by ATC.

ILS components. The basic components of an ILS are the localizer, glide slope, and outer marker, and, when installed for use with Category II or Category III instrument approach procedures, an inner marker. The following means may be used to substitute for the outer marker: Compass locator; precision approach radar (PAR) or airport surveillance radar (ASR); DME, VOR, or nondirectional beacon fixes authorized in the standard instrument approach procedure; or a suitable RNAV system in conjunction with a fix identified in the standard instrument approach procedure. Applicability of, and substitution for, the inner marker for a Category II or III approach is determined by the appropriate 14 CFR part 97 approach procedure, letter of authorization, or operations specifications issued to an operator.

[Docket 18334, 54 FR 34294, Aug. 18, 1989, as amendedbyAmdt. 91-267, 66 FR 21066, Apr. 27, 2001;Amdt. 91-281, 69 FR 1640, Jan. 9, 2004;Amdt. 91-296, 72 FR 31678, June 7, 2007;Amdt. 91-306, 74 FR 20205, May 1, 2009; Docket FAA-2013-0485, Amdt. 91-345, 81 FR 90172, Dec. 13, 2016;Amdt. 91-345B, 83 FR 10568, Mar. 12, 2018]

14 CFR91.175(k)(enhanced display)

page 57of243

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.176

§ 91.176 Straight-in landing operations belowDA/DH or MDAusing anenhanced flight vision system (EFVS) underIFR.

(a)  EFVS operations to touchdown and rollout. Unless otherwise authorized bythe Administrator to use an MDA as a DA/DH with vertical navigation on an instrument approach procedure, or unless paragraph (d) of this section applies, no person may conduct an EFVS operation in an aircraft, except a military aircraft of the United States, at any airport belowthe authorized DA/DH to touchdown and rollout unless the minimums used for the particular approach procedure being flown include a DA or DH, and the following requirements are met:

(1)  Equipment.

(i)   The aircraft must be equipped with an operable EFVS that meets the applicable airworthiness requirements. The EFVS must:

(A) (B)

(C)

(D) (E) (F)

Have an electronic means to provide a display of the forward external scene topography (the applicable natural or manmade features of a place or region especially in a way to show their relative positions and elevation) through the use of imaging sensors, including but not limited to forward-looking infrared, millimeter wave radiometry, millimeter wave radar, or low-light level image intensification.

Present EFVS sensor imagery, aircraft flight information, and flight symbology on a head up display, or an equivalent display, so that the imagery, information and symbology are clearly visible to the pilot flying in his or her normal position with the line of vision looking forward along the flight path. Aircraft flight information and flight symbology must consist of at least airspeed, vertical speed, aircraft attitude, heading, altitude, height above ground level such as that provided by a radio altimeter or other device capable of providing equivalent performance, command guidance as appropriate for the approach to be flown, path deviation indications, flight path vector, and flight path angle reference cue. Additionally, for aircraft otherthan rotorcraft, the EFVS must display flare prompt or flare guidance.

Present the displayed EFVS sensor imagery, attitude symbology, flight path vector, and flight path angle reference cue, and other cues, which are referenced to the EFVS sensor imagery and external scene topography, so that they are aligned with, and scaled to, the external view.

Display the flight path angle reference cue with a pitch scale. The flight path angle reference cue must be selectable by the pilot to the desired descent angle for the approach and be sufficient to monitor the vertical flight path of the aircraft.

Display the EFVS sensor imagery, aircraft flight information, and flight symbology such that they do not adversely obscure the pilot's outside view or field of view through the cockpit window.

Have display characteristics, dynamics, and cues that are suitable for manual control of the aircraft to touchdown in the touchdown zone of the runway of intended landing and during rollout.

(ii)   When a minimum flightcrew of more than one pilot is required, the aircraft must be equipped with a display that provides the pilot monitoring with EFVS sensor imagery. Any symbology displayed may not adversely obscure the sensor imagery ofthe runway environment.

14 CFR91.176(a)(1)(ii)(enhanced display)

page 58 of243

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

(2)  Operations.

14 CFR91.176(a)(2)

(i) (ii)

(iii) (iv)

(v) (vi)

(vii)

The pilot conducting the EFVS operation may not use circling minimums.

Each required pilot flightcrew member must have adequate knowledge of, and familiarity with, the aircraft, the EFVS, and the procedures to be used.

The aircraft must be equipped with, and the pilot flying must use, an operable EFVS that meets the equipment requirements of paragraph (a)(1) of this section.

When a minimum flightcrew of more than one pilot is required, the pilot monitoring must use the display specified in paragraph (a)(1)(ii) to monitor and assess the safe conduct of the approach, landing, and rollout.

The aircraft must continuously be in a position from which a descent to a landing on the intended runway can be made at a normal rate of descent using normal maneuvers.

The descent rate must allow touchdown to occur within the touchdown zone ofthe runway of intended landing.

Each required pilot flightcrew member must meet the following requirements—

(A)  A person exercising the privileges of a pilot certificate issued under this chapter, any person serving as a required pilot flightcrew member of a U.S.-registered aircraft, or any person serving as a required pilot flightcrew member for a part 121, 125, or 135 operator, must be qualified in accordance with part 61 and, as applicable, the training, testing, and qualification provisions of subpart K of this part, part 121, 125, or 135 of this chapter that apply to the operation; or

(B)  Each person acting as a required pilot flightcrew member for a foreign air carrier subject to part 129, or any person serving as a required pilot flightcrew member of a foreign registered aircraft, must be qualified in accordance with the training requirements ofthe civil aviation authority of the State of the operator for the EFVS operation to be conducted.

(viii)  A person conducting operations under this part must conduct the operation in accordance with a letter of authorization for the use of EFVS unless the operation is conducted in an aircraft that has been issued an experimental certificate under § 21.191 of this chapter for the purpose of research and development or showing compliance with regulations, or the operation is being conducted by a person otherwise authorized to conduct EFVS operations under paragraphs (a)(2)(ix) through (xii) of this section. A person applying to the FAA for a letter of authorization must submit an application in a form and manner prescribed bythe Administrator.

(ix) (x)

(xi)

A person conducting operations under subpart K of this part must conduct the operation in accordance with management specifications authorizing the use of EFVS.

A person conducting operations under part 121, 129, or 135 of this chapter must conduct the operation in accordance with operations specifications authorizing the use of EFVS.

A person conducting operations under part 125 of this chapter must conduct the operation in accordance with operations specifications authorizing the use of EFVS or, for a holder of a part 125 letter of deviation authority, a letter of authorization for the use of EFVS.

14 CFR91.176(a)(2)(xi)(enhanceddisplay)

page 59 of243

(3)

(4)

(i) (ii)

(iii)

(A) (B)

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.176(a)(2)(xii)

(xii)

A person conducting an EFVS operation during an authorized Category II or Category III operation must conduct the operation in accordance with operations specifications, management specifications, or a letter of authorization authorizing EFVS operations during authorized Category II or Category III operations.

Visibility and visual reference requirements. No pilot operating under this section or §§ 121.651, 125.381, or 135.225 of this chapter may continue an approach below the authorized DA/DH and land unless:

The pilot determines that the enhanced flight visibility observed by use of an EFVS is not less than the visibility prescribed in the instrument approach procedure being used.

From the authorized DA/DH to 100 feet above the touchdown zone elevation of the runway of intended landing, any approach light system or both the runwaythreshold and the touchdown zone are distinctly visible and identifiable to the pilot using an EFVS.

The pilot must identifythe runway threshold using at least one of the following visual references—

(1)

The beginning ofthe runway landing surface;

(2)   The threshold lights; or

(3)

The runway end identifier lights.

The pilot must identifythe touchdown zone using at least one of the following visual references—

(1)

The runway touchdown zone landing surface;

(2)   The touchdown zone lights;

(3)

The touchdown zone markings; or

(4)   The runway lights.

At 100 feet above the touchdown zone elevation ofthe runway of intended landing and below that altitude, the enhanced flight visibility using EFVS must be sufficient for one of the following visual references to be distinctly visible and identifiable to the pilot—

(A)  The runway threshold;

(B) (C) (D)

The lights or markings of the threshold;

The runway touchdown zone landing surface; or The lights or markings of the touchdown zone.

Additional requirements. The Administrator may prescribe additional equipment, operational, and visibility and visual reference requirements to account for specific equipment characteristics, operational procedures, or approach characteristics. These requirements will be specified in an operator's operations specifications, management specifications, or letter of authorization authorizing the use of EFVS.

14 CFR91.176(a)(4) (enhanceddisplay)

page 60 of243

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.176(b)

(b)

EFVS operations to 100 feet above the touchdown zone elevation. Except as specified in paragraph (d) of this section, no person may conduct an EFVS operation in an aircraft, except a military aircraft of the United States, at any airport below the authorized DA/DH or MDA to 100 feet above the touchdown zone elevation unless the following requirements are met:

(1)  Equipment.

(i)

The aircraft must be equipped with an operable EFVS that meets the applicable airworthiness requirements.

(ii)   The EFVS must meet the requirements of paragraph (a)(1)(i)(A) through (F) ofthis section, but need not present flare prompt, flare guidance, or height above ground level.

(2)  Operations.

(i)

The pilot conducting the EFVS operation may not use circling minimums.

(ii)   Each required pilot flightcrew member must have adequate knowledge of, and familiarity with, the aircraft, the EFVS, and the procedures to be used.

(iii)  The aircraft must be equipped with, and the pilot flying must use, an operable EFVS that meets the equipment requirements of paragraph (b)(1) ofthis section.

(iv)  The aircraft must continuously be in a position from which a descent to a landing on the intended runway can be made at a normal rate of descent using normal maneuvers.

(v)   For operations conducted under part 121 or part 135 of this chapter, the descent rate must allow touchdown to occur within the touchdown zone ofthe runway of intended landing.

(vi)  Each required pilot flightcrew member must meet the following requirements—

(A)  A person exercising the privileges of a pilot certificate issued under this chapter, any person serving as a required pilot flightcrew member of a U.S.-registered aircraft, or any person serving as a required pilot flightcrew member for a part 121, 125, or 135 operator, must be qualified in accordance with part 61 and, as applicable, the training, testing, and qualification provisions of subpart K of this part, part 121, 125, or 135 of this chapter that apply to the operation; or

(B)

Each person acting as a required pilot flightcrew member for a foreign air carrier subject to part 129, or any person serving as a required pilot flightcrew member of a foreign registered aircraft, must be qualified in accordance with the training requirements ofthe civil aviation authority of the State of the operator for the EFVS operation to be conducted.

(vii)  A person conducting operations under subpart K of this part must conduct the operation in accordance with management specifications authorizing the use of EFVS.

(viii)  A person conducting operations under part 121, 129, or 135 of this chapter must conduct the operation in accordance with operations specifications authorizing the use of EFVS.

(ix)  A person conducting operations under part 125 of this chapter must conduct the operation in accordance with operations specifications authorizing the use of EFVS or, for a holder of a part 125 letter of deviation authority, a letter of authorization for the use of EFVS.

14 CFR91.176(b)(2)(ix) (enhanceddisplay)

page 61of243

(i) (ii)

(iii)

(A) (B)

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.176(b)(2)(x)

(x)

A person conducting an EFVS operation during an authorized Category II or Category III operation must conduct the operation in accordance with operations specifications, management specifications, or a letter of authorization authorizing EFVS operations during authorized Category II or Category III operations.

(3)   Visibilityand VisualReference Requirements. No pilot operating under this section or § 121.651, § 125.381, or § 135.225 of this chapter may continue an approach below the authorized MDA or continue an approach below the authorized DA/DH and land unless:

The pilot determines that the enhanced flight visibility observed by use of an EFVS is not less than the visibility prescribed in the instrument approach procedure being used.

From the authorized MDA or DA/DH to 100 feet above the touchdown zone elevation ofthe runway of intended landing, any approach light system or both the runway threshold and the touchdown zone are distinctly visible and identifiable to the pilot using an EFVS.

The pilot must identifythe runway threshold using at least one of the following visual references-

(1)

The beginning ofthe runway landing surface;

(2)   The threshold lights; or

(3)   The runway end identifier lights.

The pilot must identifythe touchdown zone using at least one of the following visual references—

(1)   The runway touchdown zone landing surface; (2)   The touchdown zone lights;

(3)   The touchdown zone markings; or (4)   The runway lights.

At 100 feet above the touchdown zone elevation ofthe runway of intended landing and below that altitude, the flight visibility must be sufficient for one ofthe following visual references to be distinctly visible and identifiable to the pilot without reliance on the EFVS—

(A)  The runway threshold;

(B) (C) (D)

The lights or markings of the threshold;

The runway touchdown zone landing surface; or The lights or markings of the touchdown zone.

(4)  Compliance Date. Beginning on March 13, 2018, a person conducting an EFVS operation to 100 feet above the touchdown zone elevation must comply with the requirements of paragraph (b) of this section.

(c)  Public aircraft certification and training requirements. A public aircraft operator, other than the U.S. military, may conduct an EFVS operation under paragraph (a) or (b) of this section only if:

(1)   The aircraft meets all ofthe civil certification and airworthiness requirements of paragraph (a)(1) or (b)(1) of this section, as applicable to the EFVS operation to be conducted; and

14 CFR91.176(c)(1) (enhanceddisplay)

page 62of243

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.176(c)(2)

(d)

(2)   The pilot flightcrew member, or any other person who manipulates the controls of an aircraft during an EFVS operation, meets the training, recent flight experience and refreshertraining requirements of § 61.66 of this chapter applicable to EFVS operations.

Exception for ExperimentalAircraft. The requirement to use an EFVS that meets the applicable airworthiness requirements specified in paragraphs (a)(1)(i), (a)(2)(iii), (b)(1)(i), and (b)(2)(iii) of this section does not apply to operations conducted in an aircraft issued an experimental certificate under § 21.191 of this chapter for the purpose of research and development or showing compliance with regulations, provided the Administrator has determined that the operations can be conducted safely in accordance with operating limitations issued for that purpose.

[Docket FAA-2013-0485, Amdt. 91-345, 81 FR 90172, Dec. 13, 2016; 82 FR 2193, Jan. 9, 2017]

§ 91.177 Minimum altitudes forIFRoperations.

(a)  Operation ofaircraftat  minimum altitudes. Except when necessary for takeoff or landing, or unless otherwise authorized by the FAA, no person may operate an aircraft under IFR below—

(1)   The applicable minimum altitudes prescribed in parts 95 and 97 of this chapter. However, if both a MEA and a MOCA are prescribed for a particular route or route segment, a person may operate an aircraft below the MEA down to, but not below, the MOCA, provided the applicable navigation signals are available. For aircraft using VOR for navigation, this applies only when the aircraft is within 22 nautical miles of that VOR (based on the reasonable estimate by the pilot operating the aircraft of that distance); or

(2)   If no applicable minimum altitude is prescribed in parts 95 and 97 of this chapter, then—

(i)

In the case of operations over an area designated as a mountainous area in part 95 of this chapter, an altitude of 2,000 feet above the highest obstacle within a horizontal distance of 4 nautical miles from the course to be flown; or

(b)

(ii)   In any other case, an altitude of 1,000 feet above the highest obstacle within a horizontal distance of 4 nautical miles from the course to be flown.

Climb. Climb to a higher minimum IFR altitude shall begin immediately after passing the point beyond which that minimum altitude applies, except that when ground obstructions intervene, the point beyond which that higher minimum altitude applies shall be crossed at or above the applicable MCA.

[Docket 18334, 54 FR 34294, Aug. 18, 1989, as amendedbyAmdt. 91-296, 72 FR 31678, June 7, 2007; Amdt. 91-315, 75 FR 30690, June 2, 2010]

§ 91.179 IFR cruising altitude or flight level.

Unless otherwise authorized byATC, the following rules apply—

(a)

In controlledairspace. Each person operating an aircraft under IFR in level cruising flight in controlled airspace shall maintain the altitude or flight level assigned that aircraft by ATC. However, if the ATC clearance assigns “VFR conditions on-top,” that person shall maintain an altitude or flight level as prescribed by § 91.159.

14 CFR91.179(a)(enhanceddisplay)

page 63 of243

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.179(b)

(b)

In uncontrolled airspace. Except while in a holding pattern of 2 minutes or less or while turning, each person operating an aircraft under IFR in level cruising flight in uncontrolled airspace shall maintain an appropriate altitude as follows:

(1) When operating below 18,000 feet MSL and—

(i) (ii)

On a magnetic course of zero degrees through 179 degrees, any odd thousand foot MSL altitude (such as 3,000, 5,000, or 7,000); or

On a magnetic course of 180 degrees through 359 degrees, any even thousand foot MSL altitude (such as 2,000, 4,000, or 6,000).

(2) When operating at or above 18,000 feet MSL but below flight level 290, and—

(i) (ii)

On a magnetic course of zero degrees through 179 degrees, any odd flight level (such as 190, 210, or 230); or

On a magnetic course of 180 degrees through 359 degrees, any even flight level (such as 180, 200, or 220).

(3) When operating at flight level 290 and above in non-RVSM airspace, and—

(i) (ii)

On a magnetic course of zero degrees through 179 degrees, any flight level, at 4,000-foot intervals, beginning at and including flight level 290 (such as flight level 290, 330, or 370); or

On a magnetic course of 180 degrees through 359 degrees, any flight level, at 4,000-foot intervals, beginning at and including flight level 310 (such as flight level 310, 350, or 390).

(4) When operating at flight level 290 and above in airspace designated as Reduced Vertical Separation Minimum (RVSM) airspace and—

(i) (ii)

On a magnetic course of zero degrees through 179 degrees, any odd flight level, at 2,000-foot intervals beginning at and including flight level 290 (such as flight level 290, 310, 330, 350, 370, 390, 410); or

On a magnetic course of 180 degrees through 359 degrees, any even flight level, at 2000-foot intervals beginning at and including flight level 300 (such as 300, 320, 340, 360, 380, 400).

[Docket 18334, 54 FR 34294, Aug. 18, 1989, as amendedbyAmdt. 91-276, 68 FR 61321, Oct. 27, 2003; 68 FR 70133, Dec. 17, 2003; Amdt. 91-296, 72 FR 31679, June 7, 2007]

§ 91.180 Operations within airspace designated as Reduced Vertical SeparationMinimum airspace.

(a)

(b)

Except as provided in paragraph (b) ofthis section, no person may operate a civil aircraft in airspace designated as Reduced Vertical Separation Minimum (RVSM) airspace unless:

(1)   The operator and the operator's aircraft comply with the minimum standards of appendix G of this part; and

(2)   The operator is authorized by the Administrator or the country of registry to conduct such operations. The Administrator may authorize a deviation from the requirements of this section.

[Amdt. 91-276, 68 FR 70133, Dec. 17, 2003] 14 CFR91.180(b) (enhanceddisplay)

page 64of243

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.181

§ 91.181 Course tobeflown.

Unless otherwise authorized byATC, no person may operate an aircraft within controlled airspace under IFR except as follows:

(a) (b)

On an ATS route, along the centerline of that airway.

On any other route, along the direct course between the navigational aids or fixes defining that route. However, this section does not prohibit maneuvering the aircraft to pass well clear of other air traffic or the maneuvering ofthe aircraft in VFR conditions to clearthe intended flight path both before and during climb or descent.

[Docket 18334, 54 FR 34294, Aug. 18, 1989, as amendedbyAmdt. 91-296, 72 FR 31679, June 7, 2007]

§ 91.183 IFR communications.

Unless otherwise authorized byATC, the pilot in command of each aircraft operated under IFR in controlled airspace must ensure that a continuous watch is maintained on the appropriate frequency and must report the following as soon as possible—

(a)

(b) (c)

The time and altitude of passing each designated reporting point, orthe reporting points specified by ATC, except that while the aircraft is under radar control, only the passing of those reporting points specifically requested by ATC need be reported;

Any unforecast weather conditions encountered; and Any other information relating to the safety of flight.

[Docket 18334, 54 FR 34294, Aug. 18, 1989, as amendedbyAmdt. 91-296, 72 FR 31679, June 7, 2007]

§ 91.185IFRoperations: Two-way radio communications failure.

(a) General. Unless otherwise authorized by ATC, each pilot who has two-way radio communications failure when operating under IFR shall comply with the rules of this section.

(b) (c)

VFR conditions. Ifthe failure occurs in VFR conditions, or if VFR conditions are encountered after the failure, each pilot shall continue the flight under VFR and land as soon as practicable.

IFR conditions. Ifthe failure occurs in IFR conditions, or if paragraph (b) ofthis section cannot be complied with, each pilot shall continue the flight according to the following:

(1)  Route.

(i) (ii)

By the route assigned in the last ATC clearance received;

If being radar vectored, bythe direct route from the point of radio failure to the fix, route, or airway specified in the vector clearance;

(iii)  In the absence of an assigned route, by the route that ATC has advised may be expected in a further clearance; or

(iv) In the absence of an assigned route or a route that ATC has advised may be expected in a further clearance, bythe route filed in the flight plan.

14 CFR91.185(c)(1)(iv)(enhanceddisplay)

page 65of243

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.185(c)(2)

(2)

Altitude. At the highest of the following altitudes or flight levels forthe route segment being flown:

(i) (ii)

The altitude or flight level assigned in the last ATC clearance received;

The minimum altitude (converted, if appropriate, to minimum flight level as prescribed in § 91.121(c)) for IFR operations; or

(iii) The altitude or flight level ATC has advised may be expected in a further clearance. (3) Leave clearance limit.

(i) (ii)

When the clearance limit is a fix from which an approach begins, commence descent or

descent and approach as close as possible to the expect-further-clearance time if one has been received, or if one has not been received, as close as possible to the estimated time of arrival

as calculated from the filed or amended (with ATC) estimated time en route.

If the clearance limit is not a fix from which an approach begins, leave the clearance limit at the expect-further-clearance time if one has been received, or if none has been received, upon arrival over the clearance limit, and proceed to a fix from which an approach begins and commence descent or descent and approach as close as possible to the estimated time of arrival as calculated from the filed or amended (with ATC) estimated time en route.

[Docket 18334, 54 FR 34294, Aug. 18, 1989; Amdt. 91-211, 54 FR 41211, Oct. 5, 1989]

§ 91.187Operationunder IFR in controlled airspace: Malfunction reports.

(a)  The pilot in command of each aircraft operated in controlled airspace under IFR shall report as soon as practical to ATC any malfunctions of navigational, approach, or communication equipment occurring in flight.

(b)

In each report required by paragraph (a) of this section, the pilot in command shall include the— (1)   Aircraft identification;

(2)  Equipment affected;

(3) (4)

Degree to which the capability ofthe pilot to operate under IFR in the ATC system is impaired; and Nature and extent of assistance desired from ATC.

§ 91.189 Category II and III operations: General operating rules.

(a)  No person may operate a civil aircraft in a Category II or III operation unless—

(1)   The flight crew ofthe aircraft consists of a pilot in command and a second in command who hold the appropriate authorizations and ratings prescribed in § 61.3 of this chapter;

(2) (3)

Each flight crewmember has adequate knowledge of, and familiarity with, the aircraft and the procedures to be used; and

The instrument panel in front ofthe pilot who is controlling the aircraft has appropriate instrumentation for the type of flight control guidance system that is being used.

(b)

Unless otherwise authorized by the Administrator, no person may operate a civil aircraft in a Category II or Category III operation unless each ground component required for that operation and the related airborne equipment is installed and operating.

14 CFR91.189(b)(enhanceddisplay)

page 66of243

(c) (d)

(e)

(f) (g)

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.189(c)

Authorized DA/DH. For the purpose of this section, when the approach procedure being used provides for and requires the use of a DA/DH, the authorized DA/DH is the highest of the following:

(1)  The DA/DH prescribed by the approach procedure. (2)   The DA/DH prescribed for the pilot in command. (3)   The DA/DH for which the aircraft is equipped.

Except as provided in § 91.176 of this part or unless otherwise authorized by the Administrator, no pilot operating an aircraft in a Category II or Category III approach that provides and requires the use of a DA/ DH may continue the approach below the authorized decision height unless the following conditions are met:

(1)

(2)

The aircraft is in a position from which a descent to a landing on the intended runway can be made at a normal rate of descent using normal maneuvers, and where that descent rate will allow touchdown to occur within the touchdown zone ofthe runway of intended landing.

At least one of the following visual references forthe intended runway is distinctly visible and identifiable to the pilot:

(i)

The approach light system, except that the pilot may not descend below 100 feet above the touchdown zone elevation using the approach lights as a reference unless the red terminating bars or the red side row bars are also distinctly visible and identifiable.

(ii)   The threshold.

(iii)  The threshold markings. (iv)  The threshold lights.

(v) (vi)

The touchdown zone or touchdown zone markings. The touchdown zone lights.

Except as provided in § 91.176 of this part or unless otherwise authorized by the Administrator, each pilot operating an aircraft shall immediately execute an appropriate missed approach whenever, prior to touchdown, the requirements of paragraph (d) ofthis section are not met.

No person operating an aircraft using a Category III approach without decision height may land that aircraft except in accordance with the provisions ofthe letter of authorization issued bythe Administrator.

Paragraphs (a) through (f) of this section do not applyto operations conducted by certificate holders operating under part 121, 125, 129, or 135 of this chapter, or holders of management specifications issued in accordance with subpart K of this part. Holders of operations specifications or management specifications may operate a civil aircraft in a Category II or Category III operation only in accordance with their operations specifications or management specifications, as applicable.

[Docket 18334, 54 FR 34294, Aug. 18, 1989, as amendedbyAmdt. 91-280, 68 FR 54560, Sept. 17, 2003;Amdt. 91-296, 72 FR 31679, June 7, 2007; Docket FAA-2013-0485, Amdt. 91-345, 81 FR 90175, Dec. 13, 2016]

§ 91.191 CategoryII and Category III manual.

(a)  Except as provided in paragraph (c) of this section, after August 4, 1997, no person may operate a U.S.- registered civil aircraft in a Category II or a Category III operation unless—

14 CFR91.191(a)(enhanceddisplay)

page 67of243

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.191(a)(1)

(1) (2) (3)

There is available in the aircraft a current and approved Category II or Category III manual, as appropriate, for that aircraft;

The operation is conducted in accordance with the procedures, instructions, and limitations in the appropriate manual; and

The instruments and equipment listed in the manual that are required for a particular Category II or Category III operation have been inspected and maintained in accordance with the maintenance program contained in the manual.

(b) (c)

Each operator must keep a current copy of each approved manual at its principal base of operations and must make each manual available for inspection upon request by the Administrator.

This section does not apply to operations conducted by a certificate holder operating under part 121 or part 135 of this chapter or a holder of management specifications issued in accordance with subpart K of this part.

[Docket 26933, 61 FR 34560, July2, 1996, as amendedbyAmdt. 91-280, 68 FR 54560, Sept. 17, 2003]

§ 91.193 Certificate ofauthorizationfor certain CategoryII operations.

The Administrator may issue a certificate of authorization authorizing deviations from the requirements of §§ 91.189, 91.191, and 91.205(f) for the operation of small aircraft identified as Category A aircraft in § 97.3 of this chapter in Category II operations ifthe Administrator finds that the proposed operation can be safely conducted under the terms of the certificate. Such authorization does not permit operation of the aircraft carrying persons or property for compensation or hire.

§§ 91.195-91.199 [Reserved]

Subpart C—Equipment, Instrument, andCertificate Requirements

Source:

Docket 18334, 54 FR 34304, Aug. 18, 1989, unless otherwise noted.

§ 91.201 [Reserved]

§ 91.203 Civil aircraft: Certifications required.

(a)   Except as provided in § 91.715, no person may operate a civil aircraft unless it has within it the following:

(1)   An appropriate and current airworthiness certificate. Each U.S. airworthiness certificate used to comply with this subparagraph (except a special flight permit, a copy of the applicable operations specifications issued under § 21.197(c) of this chapter, appropriate sections of the air carrier manual required by parts 121 and 135 of this chapter containing that portion of the operations specifications issued under § 21.197(c), or an authorization under § 91.611) must have on it the registration number assigned to the aircraft under part 47 or 48 of this chapter. However, the airworthiness certificate need not have on it an assigned special identification number before 10 days after that number is first affixed to the aircraft. A revised airworthiness certificate having on it an assigned special identification number, that has been affixed to an aircraft, may only be obtained upon application to the responsible Flight Standards office.

14 CFR91.203(a)(1) (enhanceddisplay)

page 68 of243

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.203(a)(2)

(2)

An effective U.S. registration certificate issued to its owner or, for operation within the United States, the second copy of the Aircraft registration Application as provided for in § 47.31(c), a Certificate of Aircraft registration as provided in part 48, or a registration certification issued under the laws of a foreign country.

(b) (c) (d)

No person may operate a civil aircraft unless the airworthiness certificate required by paragraph (a) ofthis section or a special flight authorization issued under § 91.715 is displayed at the cabin or cockpit entrance so that it is legible to passengers or crew.

No person may operate an aircraft with a fuel tank installed within the passenger compartment or a baggage compartment unless the installation was accomplished pursuant to part 43 of this chapter, and a copy of FAA Form 337 authorizing that installation is on board the aircraft.

No person may operate a civil airplane (domestic or foreign) into or out of an airport in the United States unless it complies with the fuel venting and exhaust emissions requirements of part 34 of this chapter.

[Docket 18334, 54 FR 34292, Aug. 18, 1989, as amendedbyAmdt. 91-218, 55 FR 32861, Aug. 10, 1990;Amdt. 91-318, 75 FR 41983, July20, 2010;Amdt. 91-338, 80 FR 78648, Dec. 16, 2015;Docket FAA-2018-0119, Amdt. 91-350, 83 FR 9171, Mar. 5, 2018; Docket FAA-2022-1355, Amdt. 91-366, 87 FR 75846, Dec. 9, 2022]

§ 91.205 Powered civilaircraft  withstandard U.S. airworthiness certificates: Instrument and equipment requirements.

(a) (b)

General. Except as provided in paragraphs (c)(3) and (e) ofthis section, no person may operate a powered civil aircraft with a standard U.S. airworthiness certificate in any operation described in paragraphs (b) through (f) ofthis section unless that aircraft contains the instruments and equipment specified in those paragraphs (or FAA-approved equivalents) for that type of operation, and those instruments and items of equipment are in operable condition.

Visual-flight rules (day). For VFR flight during the day, the following instruments and equipment are required:

(1)   Airspeed indicator. (2)   Altimeter.

(3)   Magnetic direction indicator. (4)  Tachometer for each engine.

(5)

Oil pressure gauge for each engine using pressure system.

(6)  Temperature gauge for each liquid-cooled engine.

(7) (8) (9)

Oil temperature gauge for each air-cooled engine. Manifold pressure gauge for each altitude engine. Fuel gauge indicating the quantity of fuel in each tank.

(10)  Landing gear position indicator, if the aircraft has a retractable landing gear.

14 CFR91.205(b)(10)(enhanceddisplay)

page 69 of243

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.205(b)(11)

(11) (12)

(13) (14)

(15)

For small civil airplanes certificated after March 11, 1996, in accordance with part 23 of this chapter, an approved aviation red or aviation white anticollision light system. In the event offailure of any light of the anticollision light system, operation of the aircraft may continue to a location where repairs or replacement can be made.

If the aircraft is operated for hire over water and beyond power-off gliding distance from shore, approved flotation gear readily available to each occupant and, unless the aircraft is operating under part 121 of this subchapter, at least one pyrotechnic signaling device. As used in this section, “shore” means that area ofthe land adjacent to the water which is above the high water mark and excludes land areas which are intermittently under water.

An approved safety belt with an approved metal-to-metal latching device, or other approved restraint system for each occupant 2 years of age or older.

For small civil airplanes manufactured after July 18, 1978, an approved shoulder harness or restraint system for each front seat. For small civil airplanes manufactured after December 12, 1986, an approved shoulder harness or restraint system for all seats. Shoulder harnesses installed at flightcrew stations must permit the flightcrew member, when seated and with the safety belt and shoulder harness fastened, to perform all functions necessary for flight operations. For purposes of this paragraph—

(i)   The date of manufacture of an airplane is the date the inspection acceptance records reflect that the airplane is complete and meets the FAA-approved type design data; and

(ii)   A front seat is a seat located at a flightcrew member station or any seat located alongside such a seat.

An emergency locator transmitter, if required by § 91.207.

(16) [Reserved]

(17)

For rotorcraft manufactured after September 16, 1992, a shoulder harness for each seat that meets the requirements of § 27.2 or § 29.2 of this chapter in effect on September 16, 1991.

(c)

Visual flightrules (night). For VFR flight at night, the following instruments and equipment are required:

(1)

Instruments and equipment specified in paragraph (b) of this section.

(2)   Approved position lights.

(3)

(4) (5) (6)

An approved aviation red or aviation white anticollision light system on all U.S.-registered civil aircraft. Anticollision light systems initially installed after August 11, 1971, on aircraft for which a type certificate was issued or applied for before August 11, 1971, must at least meet the anticollision light standards of part 23, 25, 27, or 29 of this chapter, as applicable, that were in effect

on August 10, 1971, except that the color may be either aviation red or aviation white. In the event of failure of any light of the anticollision light system, operations with the aircraft may be continued to a stop where repairs or replacement can be made.

If the aircraft is operated for hire, one electric landing light.

An adequate source of electrical energy for all installed electrical and radio equipment.

One spare set of fuses, orthree spare fuses of each kind required, that are accessible to the pilot in flight.

14 CFR91.205(c)(6)(enhanceddisplay)

page 70 of243

(d)

(e)

(f)

(g) (h)

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.205(d)

Instrument flightrules. For IFR flight, the following instruments and equipment are required:

(1)

(2) (3)

Instruments and equipment specified in paragraph (b) of this section, and, for night flight, instruments and equipment specified in paragraph (c) of this section.

Two-way radio communication and navigation equipment suitable for the route to be flown. Gyroscopic rate-of-turn indicator, except on the following aircraft:

(i)   Airplanes with a third attitude instrument system usable through flight attitudes of 360 degrees of pitch and roll and installed in accordance with the instrument requirements prescribed in § 121.305(j) of this chapter; and

(ii)   Rotorcraft with a third attitude instrument system usable through flight attitudes of ±80 degrees of pitch and ±120 degrees of roll and installed in accordance with § 29.1303(g) of this chapter.

(4)   Slip-skid indicator.

(5) (6) (7) (8) (9)

Sensitive altimeter adjustable for barometric pressure.

A clock displaying hours, minutes, and seconds with a sweep-second pointer or digital presentation. Generator or alternator of adequate capacity.

Gyroscopic pitch and bank indicator (artificial horizon). Gyroscopic direction indicator (directional gyro or equivalent).

Flight atand above 24,000 feet MSL (FL 240). If VOR navigation equipment is required under paragraph (d)(2) of this section, no person may operate a U.S.-registered civil aircraft within the 50 states and the District of Columbia at or above FL 240 unless that aircraft is equipped with approved DME or a suitable RNAV system. When the DME or RNAV system required by this paragraph fails at and above FL 240, the pilot in command of the aircraft must notify ATC immediately, and then may continue operations at and above FL 240 to the next airport of intended landing where repairs or replacement ofthe equipment can be made.

Category II operations. The requirements for Category II operations are the instruments and equipment specified in—

(1) (2)

Paragraph (d) ofthis section; and Appendix Ato this part.

Category III operations. The instruments and equipment required for Category III operations are specified in paragraph (d) of this section.

Night vision goggle operations. For night vision goggle operations, the following instruments and equipment must be installed in the aircraft, functioning in a normal manner, and approved for use by the FAA:

(1)

Instruments and equipment specified in paragraph (b) of this section, instruments and equipment specified in paragraph (c) of this section;

(2)   Night vision goggles;

(3) (4)

Interior and exterior aircraft lighting system required for night vision goggle operations; Two-way radio communications system;

14 CFR91.205(h)(4) (enhanceddisplay)

page 71of243

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

(5)   Gyroscopic pitch and bank indicator (artificial horizon);

14 CFR91.205(h)(5)

(6)

Generator or alternator of adequate capacity forthe required instruments and equipment; and

(i)

(7)  Radar altimeter.

Exclusions. Paragraphs (f) and (g) of this section do not apply to operations conducted by a holder of a certificate issued under part 121 or part 135 of this chapter.

[Docket 18334, 54 FR 34292, Aug. 18, 1989, as amendedbyAmdt. 91-220, 55 FR 43310, Oct. 26, 1990;Amdt. 91-223, 56 FR 41052, Aug. 16, 1991;Amdt. 91-231, 57 FR 42672, Sept. 15, 1992;Amdt. 91-248, 61 FR 5171, Feb. 9, 1996;Amdt. 91-251, 61 FR 34560, July 2, 1996;Amdt. 91-285, 69 FR 77599, Dec. 27, 2004;Amdt. 91-296, 72 FR 31679, June 7, 2007;Amdt. 91-309, 74 FR 42563, Aug. 21, 2009; Docket FAA-2015-1621, Amdt. 91-346, 81 FR 96700, Dec. 30, 2016; FAA-2023-1275, Amdt. 91-379, 89 FR 92486, Nov. 21, 2024]

§ 91.207 Emergencylocator transmitters.

(a)  Except as provided in paragraphs (e) and (f) of this section, no person may operate a U.S.-registered civil airplane unless—

(1)

(2)

There is attached to the airplane an approved automatic type emergency locator transmitter that is in operable condition for the following operations, except that after June 21, 1995, an emergency locator transmitter that meets the requirements of TSO-C91 may not be used for new installations:

(i)   Those operations governed by the supplemental air carrier and commercial operator rules of parts 121 and 125;

(ii)   Charter flights governed by the domestic and flag air carrier rules of part 121 of this chapter; and

(iii)  Operations governed by part 135 of this chapter; or

For operations other than those specified in paragraph (a)(1) of this section, there must be attached to the airplane an approved personal type or an approved automatic type emergency locator transmitter that is in operable condition, except that after June 21, 1995, an emergency locator transmitter that meets the requirements of TSO-C91 may not be used for new installations.

(b)

(c)

Each emergency locator transmitter required by paragraph (a) of this section must be attached to the airplane in such a manner that the probability of damage to the transmitter in the event of crash impact is minimized. Fixed and deployable automatic type transmitters must be attached to the airplane as far aft as practicable.

Batteries used in the emergency locator transmitters required by paragraphs (a) and (b) of this section must be replaced (or recharged, ifthe batteries are rechargeable)—

(1) (2)

When the transmitter has been in use for more than 1 cumulative hour; or

When 50 percent oftheir useful life (or, for rechargeable batteries, 50 percent oftheir useful life of charge) has expired, as established by the transmitter manufacturer under its approval.

The new expiration date for replacing (or recharging) the battery must be legibly marked on the outside of the transmitter and entered in the aircraft maintenance record. Paragraph (c)(2) ofthis section does not apply to batteries (such as water-activated batteries) that are essentially unaffected during probable storage intervals.

14 CFR91.207(c)(2)(enhanceddisplay)

page 72of243

(d) (e) (f)

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.207(d)

Each emergency locator transmitter required by paragraph (a) of this section must be inspected within 12 calendar months afterthe last inspection for—

(1)   Proper installation; (2)   Battery corrosion;

(3) (4)

Operation of the controls and crash sensor; and

The presence of a sufficient signal radiated from its antenna.

Notwithstanding paragraph (a) of this section, a person may—

(1) (2)

Ferry a newly acquired airplane from the place where possession of it was taken to a place where the emergency locator transmitter is to be installed; and

Ferry an airplane with an inoperative emergency locator transmitter from a place where repairs or replacements cannot be made to a place where they can be made.

No person other than required crewmembers may be carried aboard an airplane being ferried under paragraph (e) ofthis section.

Paragraph (a) of this section does not apply to—

(1) (2) (3)

(4) (5)

(6)

(7) (8)

(9) (10)

(11)

Before January 1, 2004, turbojet-powered aircraft;

Aircraft while engaged in scheduled flights by scheduled air carriers;

Aircraft while engaged in training operations conducted entirely within a 50-nautical mile radius of the airport from which such local flight operations began;

Aircraft while engaged in flight operations incident to design and testing;

New aircraft while engaged in flight operations incident to their manufacture, preparation, and delivery;

Aircraft while engaged in flight operations incident to the aerial application of chemicals and other substances for agricultural purposes;

Aircraft certificated by the Administrator for research and development purposes;

Aircraft while used for showing compliance with regulations, crew training, exhibition, air racing, or market surveys;

Aircraft equipped to carry not more than one person.

An aircraft during any period for which the transmitter has been temporarily removed for inspection, repair, modification, or replacement, subject to the following:

(i)   No person may operate the aircraft unless the aircraft records contain an entry which includes the date of initial removal, the make, model, serial number, and reason for removing the transmitter, and a placard located in view of the pilot to show “ELT not installed.”

(ii)  No person may operate the aircraft more than 90 days after the ELT is initially removed from the aircraft; and

On and after January 1, 2004, aircraft with a maximum payload capacity of more than 18,000 pounds when used in air transportation.

14 CFR91.207(f)(11)(enhanceddisplay)

page 73 of243

(a) (b)

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.209

[Docket 18334, 54 FR 34304, Aug. 18, 1989, as amendedbyAmdt. 91-242, 59 FR 32057, June21, 1994; 59 FR 34578, July 6, 1994; Amdt. 91-265, 65 FR 81319, Dec. 22, 2000; 66 FR 16316, Mar. 23, 2001]

§ 91.209 Aircraftlights. No person may:

During the period from sunset to sunrise (or, in Alaska, during the period a prominent unlighted object cannot be seen from a distance of 3 statute miles or the sun is more than 6 degrees below the horizon)—

(1) (2)

Operate an aircraft unless it has lighted position lights;

Park or move an aircraft in, or in dangerous proximity to, a night flight operations area of an airport unless the aircraft—

(i)   Is clearly illuminated;

(ii)   Has lighted position lights; or

(iii)  is in an area that is marked by obstruction lights;

(3)  Anchor an aircraft unless the aircraft— (i)   Has lighted anchor lights; or

(ii)  Is in an area where anchor lights are not required on vessels; or

Operate an aircraft that is equipped with an anticollision light system, unless it has lighted anticollision lights. However, the anticollision lights need not be lighted when the pilot-in-command determines that, because of operating conditions, it would be in the interest of safety to turn the lights off.

[Docket 27806, 61 FR 5171, Feb. 9, 1996]

§ 91.211 Supplementaloxygen.

(a) General. No person may operate a civil aircraft of U.S. registry—

(1)  At cabin pressure altitudes above 12,500 feet (MSL) up to and including 14,000 feet (MSL) unless the required minimum flight crew is provided with and uses supplemental oxygen for that part of the flight at those altitudes that is of more than 30 minutes duration;

(2) (3)

At cabin pressure altitudes above 14,000 feet (MSL) unless the required minimum flight crew is provided with and uses supplemental oxygen during the entire flight time at those altitudes; and

At cabin pressure altitudes above 15,000 feet (MSL) unless each occupant of the aircraft is provided with supplemental oxygen.

(b)  Pressurized cabin aircraft.

(1)

No person may operate a civil aircraft of U.S. registry with a pressurized cabin—

(i)

At flight altitudes above flight level 250 unless at least a 10-minute supply of supplemental oxygen, in addition to any oxygen required to satisfy paragraph (a) ofthis section, is available for each occupant of the aircraft for use in the event that a descent is necessitated by loss of cabin pressurization; and

14 CFR91.211(b)(1)(i)(enhanceddisplay)

page 74of243

(1) (2)

(3)

(4) (5)

(b)

(c)

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.211(b)(1)(ii)

(ii)

At flight altitudes above flight level 350 unless one pilot at the controls of the airplane is wearing and using an oxygen mask that is secured and sealed and that either supplies oxygen at all times or automatically supplies oxygen wheneverthe cabin pressure altitude of the airplane exceeds 14,000 feet (MSL), except that the one pilot need not wear and use an oxygen mask while at or below flight level 410 if there are two pilots at the controls and each pilot has a quick-donning type of oxygen maskthat can be placed on the face with one hand from the ready position within 5 seconds, supplying oxygen and properly secured and sealed.

(2)

Notwithstanding paragraph (b)(1)(ii) of this section, if for any reason at any time it is necessary for one pilot to leave the controls of the aircraft when operating at flight altitudes above flight level 350, the remaining pilot at the controls shall put on and use an oxygen mask until the other pilot has returned to that crewmember's station.

§ 91.213 Inoperative instruments and equipment.

(a)   Except as provided in paragraph (d) ofthis section, no person may take off an aircraft with inoperative instruments or equipment installed unless the following conditions are met:

An approved Minimum Equipment List exists for that aircraft.

The aircraft has within it a letter of authorization, issued by the responsible Flight Standards office, authorizing operation of the aircraft underthe Minimum Equipment List. The letter of authorization may be obtained by written request of the airworthiness certificate holder. The Minimum Equipment List and the letter of authorization constitute a supplemental type certificate for the aircraft.

The approved Minimum Equipment List must—

(i)

Be prepared in accordance with the limitations specified in paragraph (b) of this section; and

(ii)   Provide for the operation of the aircraft with the instruments and equipment in an inoperable condition.

The aircraft records available to the pilot must include an entry describing the inoperable instruments and equipment.

The aircraft is operated under all applicable conditions and limitations contained in the Minimum Equipment List and the letter authorizing the use ofthe list.

The following instruments and equipment may not be included in a Minimum Equipment List:

(1)

(2)

(3)

Instruments and equipment that are either specifically or otherwise required by the airworthiness requirements under which the aircraft is type certificated and which are essential for safe operations under all operating conditions.

Instruments and equipment required by an airworthiness directive to be in operable condition unless the airworthiness directive provides otherwise.

Instruments and equipment required for specific operations by this part.

A person authorized to use an approved Minimum Equipment List issued for a specific aircraft under subpart K of this part, part 121, 125, or 135 of this chapter must use that Minimum Equipment List to comply with the requirements in this section.

14 CFR91.213(c)(enhanceddisplay)

page 75of243

(d)

(e)

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.213(d)

Except for operations conducted in accordance with paragraph (a) or (c) of this section, a person may takeoff an aircraft in operations conducted under this part with inoperative instruments and equipment without an approved Minimum Equipment List provided—

(1)  The flight operation is conducted in a—

(i)

(ii)

Rotorcraft, non-turbine-powered airplane, glider, lighter-than-air aircraft, powered parachute, or weight-shift-control aircraft, for which a master minimum equipment list has not been developed; or

Small rotorcraft, nonturbine-powered small airplane, glider, or lighter-than-air aircraft for which a Master Minimum Equipment List has been developed; and

(2)  The inoperative instruments and equipment are not—

(i) (ii)

(iii)

Part of the VFR-daytype certification instruments and equipment prescribed in the applicable airworthiness regulations under which the aircraft was type certificated;

Indicated as required on the aircraft's equipment list, or on the Kinds of Operations Equipment List for the kind of flight operation being conducted;

Required by § 91.205 or any other rule ofthis part forthe specific kind of flight operation being conducted; or

(iv)  Required to be operational by an airworthiness directive; and (3)  The inoperative instruments and equipment are—

(i) (ii)

Removed from the aircraft, the cockpit control placarded, and the maintenance recorded in accordance with § 43.9 of this chapter; or

Deactivated and placarded “Inoperative.” If deactivation ofthe inoperative instrument or equipment involves maintenance, it must be accomplished and recorded in accordance with part 43 of this chapter; and

(4)

A determination is made by a pilot, who is certificated and appropriately rated under part 61 of this chapter, or by a person, who is certificated and appropriately rated to perform maintenance on the aircraft, that the inoperative instrument or equipment does not constitute a hazard to the aircraft.

An aircraft with inoperative instruments or equipment as provided in paragraph (d) of this section is considered to be in a properly altered condition acceptable to the Administrator.

Notwithstanding any other provision of this section, an aircraft with inoperable instruments or equipment may be operated under a special flight permit issued in accordance with §§ 21.197 and 21.199 of this chapter.

[Docket 18334, 54 FR 34304, Aug. 18, 1989, as amendedbyAmdt. 91-280, 68 FR 54560, Sept. 17, 2003;Amdt. 91-282, 69 FR 44880, July27, 2004; DocketFAA-2018-0119,Amdt. 91-350, 83 FR 9171, Mar. 5, 2018]

§ 91.215 ATCtransponder and altitude reporting equipment and use.

(a)   Allairspace: U.S.-registeredcivil aircraft. For operations not conducted under part 121 or 135 of this chapter, ATC transponder equipment installed must meet the performance and environmental requirements of any class of TSO-C74b (Mode A) or any class ofTSO-C74c (Mode A with altitude reporting capability) as appropriate, or the appropriate class of TSO-C112 (Mode S).

14 CFR91.215(a)(enhanceddisplay)

page 76of243

(b)

(c) (d)

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.215(b)

All airspace. Unless otherwise authorized or directed byATC, and except as provided in paragraph (e)(1) of this section, no person may operate an aircraft in the airspace described in paragraphs (b)(1) through (5) of this section, unless that aircraft is equipped with an operable coded radar beacon transponder having either Mode A 4096 code capability, replying to Mode A interrogations with the code specified by ATC, or a Mode S capability, replying to Mode A interrogations with the code specified by ATC and Mode S interrogations in accordance with the applicable provisions specified in TSO-C112, and that aircraft is equipped with automatic pressure altitude reporting equipment having a Mode C capabilitythat automatically replies to Mode C interrogations by transmitting pressure altitude information in 100-foot increments. The requirements ofthis paragraph (b) applyto—

(1) (2)

(3)

(4) (5)

All aircraft. In Class A, Class B, and Class C airspace areas;

All aircraft. In all airspace within 30 nautical miles of an airport listed in appendix D, section 1 of this part from the surface upward to 10,000 feet MSL;

Notwithstanding paragraph (b)(2) ofthis section, any aircraft which was not originally certificated with an engine-driven electrical system or which has not subsequently been certified with such a system installed, balloon or glider may conduct operations in the airspace within 30 nautical miles of an airport listed in appendix D, section 1 ofthis part provided such operations are conducted—

(i)  Outside any Class A, Class B, or Class C airspace area; and

(ii)   Below the altitude of the ceiling of a Class B or Class C airspace area designated for an airport or 10,000 feet MSL, whichever is lower; and

All aircraft in all airspace above the ceiling and within the lateral boundaries of a Class B or Class C airspace area designated for an airport upward to 10,000 feet MSL; and

All aircraft except any aircraft which was not originally certificated with an engine-driven electrical system or which has not subsequently been certified with such a system installed, balloon, or glider—

(i)   In all airspace of the 48 contiguous states and the District of Columbia at and above 10,000 feet MSL, excluding the airspace at and below 2,500 feet above the surface; and

(ii)   In the airspace from the surface to 10,000 feet MSL within a 10-nautical-mile radius of any airport listed in appendix D, section 2 ofthis part, excluding the airspace below 1,200 feet outside ofthe lateral boundaries ofthe surface area of the airspace designated for that airport.

Transponder-on operation. Except as provided in paragraph (e)(2) of this section, while in the airspace as specified in paragraph (b) ofthis section or in all controlled airspace, each person operating an aircraft equipped with an operable ATC transponder maintained in accordance with § 91.413 shall operate the transponder, including Mode C equipment if installed, and shall reply on the appropriate code or as assigned by ATC, unless otherwise directed by ATC when transmitting would jeopardize the safe execution of airtraffic control functions.

ATC authorizeddeviations. Requests for ATC authorized deviations must be made to the ATC facility having jurisdiction over the concerned airspace within the time periods specified as follows:

(1)

For operation of an aircraft with an operating transponder but without operating automatic pressure altitude reporting equipment having a Mode C capability, the request may be made at anytime.

14 CFR91.215(d)(1) (enhanceddisplay)

page 77of243

(a) (b)

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.215(d)(2)

(2)

(3)

For operation of an aircraft with an inoperative transponder to the airport of ultimate destination, including any intermediate stops, or to proceed to a place where suitable repairs can be made or both, the request may be made at any time.

For operation of an aircraft that is not equipped with a transponder, the request must be made at least one hour before the proposed operation.

(e)  Unmanned aircraft.

(1) (2)

The requirements of paragraph (b) ofthis section do not apply to a person operating an unmanned aircraft under this part unless the operation is conducted under a flight plan and the person operating the unmanned aircraft maintains two-way communication with ATC.

No person may operate an unmanned aircraft under this part with a transponder on unless:

(i)   The operation is conducted under a flight plan and the person operating the unmanned aircraft maintains two-way communication with ATC; or

(ii)   The use of a transponder is otherwise authorized bythe Administrator.

(Approved by the Office ofManagement and Budget under control number2120-0005)

[Docket 18334, 54 FR 34304, Aug. 18, 1989, as amendedbyAmdt. 91-221, 56 FR 469, Jan. 4, 1991; Amdt. 91-227, 56 FR 65660, Dec. 17, 1991; Amdt. 91-227, 7 FR 328, Jan. 3, 1992;Amdt. 91-229, 57 FR 34618, Aug. 5, 1992;Amdt. 91-267, 66 FR 21066, Apr. 27, 2001; Amdt. 91-355, 84 FR 34287, July 18, 2019;Amdt. 91-361, 86 FR 4512, Jan. 15, 2021; Docket FAA-2023-1836, Amdt. 91-371, 88 FR 71476, Oct. 17, 2023]

§ 91.217 Data correspondence betweenautomatically reported pressure altitude data andthe pilot's altitude reference.

No person may operate any automatic pressure altitude reporting equipment associated with a radar beacon transponder—

(1) (2)

(3)

When deactivation of that equipment is directed by ATC;

Unless, as installed, that equipment was tested and calibrated to transmit altitude data corresponding within 125 feet (on a 95 percent probability basis) of the indicated or calibrated datum of the altimeter normally used to maintain flight altitude, with that altimeter referenced to 29.92 inches of mercury for altitudes from sea level to the maximum operating altitude ofthe aircraft; or

Unless the altimeters and digitizers in that equipment meet the standards of TSO-C10b and TSO-C88, respectively.

No person may operate any automatic pressure altitude reporting equipment associated with a radar beacon transponder or with ADS-B Out equipment unless the pressure altitude reported for ADS-B Out and Mode C/S is derived from the same source for aircraft equipped with both a transponder and ADS-B Out.

[Docket 18334, 54 FR 34304, Aug. 18, 1989, as amendedbyAmdt. 91-314, 75 FR 30193, May28, 2010]

14 CFR91.217(b)(enhanceddisplay)

page 78 of243

(b)

(c) (d)

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.219

§ 91.219Altitude alerting system ordevice: Turbojet-poweredcivil airplanes.

(a)  Except as provided in paragraph (d) ofthis section, no person may operate a turbojet-powered U.S.- registered civil airplane unless that airplane is equipped with an approved altitude alerting system or device that is in operable condition and meets the requirements of paragraph (b) of this section.

Each altitude alerting system or device required by paragraph (a) ofthis section must be able to— (1)  Alert the pilot—

(i)   Upon approaching a preselected altitude in either ascent or descent, by a sequence of both aural and visual signals in sufficient time to establish level flight at that preselected altitude; or

(ii)   Upon approaching a preselected altitude in either ascent or descent, by a sequence ofvisual signals in sufficient time to establish level flight at that preselected altitude, and when deviating above and below that preselected altitude, by an aural signal;

(2) (3)

(4) (5)

Provide the required signals from sea level to the highest operating altitude approved for the airplane in which it is installed;

Preselect altitudes in increments that are commensurate with the altitudes at which the aircraft is operated;

Be tested without special equipment to determine proper operation of the alerting signals; and

Accept necessary barometric pressure settings if the system or device operates on barometric pressure. However, for operation below 3,000 feet AGL, the system or device need only provide one signal, either visual or aural, to comply with this paragraph. A radio altimeter may be included to provide the signal ifthe operator has an approved procedure for its use to determine DA/DH or MDA, as appropriate.

Each operator to which this section applies must establish and assign procedures for the use of the altitude alerting system or device and each flight crewmember must comply with those procedures assigned to him.

Paragraph (a) of this section does not apply to any operation of an airplane that has an experimental certificate orto the operation of any airplane for the following purposes:

(1) (2)

(3)

(4) (5)

(6)

Ferrying a newly acquired airplane from the place where possession of it was taken to a place where the altitude alerting system or device is to be installed.

Continuing a flight as originally planned, ifthe altitude alerting system or device becomes inoperative after the airplane has taken off; however, the flight may not depart from a place where repair or replacement can be made.

Ferrying an airplane with any inoperative altitude alerting system or device from a place where repairs or replacements cannot be made to a place where it can be made.

Conducting an airworthiness flight test of the airplane.

Ferrying an airplane to a place outside the United States for the purpose of registering it in a foreign country.

Conducting a sales demonstration ofthe operation of the airplane.

14 CFR91.219(d)(6) (enhanceddisplay)

page 79 of243

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.219(d)(7)

(7)   Training foreign flight crews in the operation ofthe airplane before ferrying it to a place outside the United States for the purpose of registering it in a foreign country.

[Docket 18334, 54 FR 34304, Aug. 18, 1989, as amendedbyAmdt. 91-296, 72 FR 31679, June 7, 2007]

§ 91.221Traffic alert andcollision avoidance system equipment anduse.

(a)   Allairspace: U.S.-registeredcivil aircraft. Any traffic alert and collision avoidance system installed in a U.S.-registered civil aircraft must be approved bythe Administrator.

(b)

Traffic alert and collision avoidance system, operation required. Each person operating an aircraft equipped with an operable traffic alert and collision avoidance system shall have that system on and operating.

§ 91.223 Terrain awareness and warning system.

(a)  Airplanes manufactured afterMarch 29, 2002. Except as provided in paragraph (d) of this section, no person may operate a turbine-powered U.S.-registered airplane configured with six or more passenger seats, excluding any pilot seat, unless that airplane is equipped with an approved terrain awareness and warning system that as a minimum meets the requirements for Class B equipment in Technical Standard Order (TSO)-C151.

(b)

Airplanes manufactured on or before March 29, 2002. Except as provided in paragraph (d) of this section, no person may operate a turbine-powered U.S.-registered airplane configured with six or more passenger seats, excluding any pilot seat, after March 29, 2005, unless that airplane is equipped with an approved terrain awareness and warning system that as a minimum meets the requirements for Class B equipment in Technical Standard Order (TSO)-C151.

(Approved by the Office ofManagement and Budget under control number2120-0631)

(c) (d)

Airplane FlightManual. The Airplane Flight Manual shall contain appropriate procedures for— (1)   The use ofthe terrain awareness and warning system; and

(2)   Proper flight crew reaction in response to the terrain awareness and warning system audio and visual warnings.

Exceptions. Paragraphs (a) and (b) of this section do not apply to—

(1)   Parachuting operations when conducted entirely within a 50 nautical mile radius of the airport from which such local flight operations began.

(2)   Firefighting operations.

(3)   Flight operations when incident to the aerial application of chemicals and other substances.

[Docket 29312, 65 FR 16755, Mar. 29, 2000]

§ 91.225Automatic Dependent Surveillance-Broadcast (ADS-B) Out equipment and use.

(a)  After January 1, 2020, unless otherwise authorized by ATC, no person may operate an aircraft in Class A airspace unless the aircraft has equipment installed that—

(1)   Meets the performance requirements in-

14 CFR91.225(a)(1) (enhanceddisplay)

page 80 of243

(b)

(c) (d)

(e)

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.225(a)(1)(i)

(2)

(i)  TSO-C166b and Section 2 of RTCA DO-260B (as referenced in TSO-C166b); or

(ii)  TSO-C166c and Section 2 of RTCA DO-260C as modified by DO-260C—Change 1 (as referenced in TSO-C166c); and

Meets the requirements of § 91.227.

After January 1, 2020, except as prohibited in paragraph (h)(2) ofthis section or unless otherwise authorized by ATC, no person may operate an aircraft below 18,000 feet MSL and in airspace described in paragraph (d) of this section unless the aircraft has equipment installed that—

(1)

(2)

Meets the performance requirements in—

(i)  TSO-C166b and Section 2 of RTCA DO-260B (as referenced in TSO-C166b);

(ii)  TSO-C166c and Section 2 of RTCA DO-260C as modified by DO-260C—Change 1 (as referenced in TSO-C166c);

(iii) TSO-C154c and Section 2 of RTCA DO-282B (as referenced in TSO-C154c); or (iv) TSO-C154d and Section 2 of RTCA DO-282C (as referenced in TSO-C154d); Meets the requirements of § 91.227.

Operators with equipment installed with an approved deviation under § 21.618 of this chapter also are in compliance with this section.

After January 1, 2020, except as prohibited in paragraph (h)(2) ofthis section or unless otherwise authorized by ATC, no person may operate an aircraft in the following airspace unless the aircraft has equipment installed that meets the requirements in paragraph (b) of this section:

(1) (2)

(3) (4)

(5)

Class B and Class C airspace areas;

Except as provided for in paragraph (e) of this section, within 30 nautical miles of an airport listed in appendix D, section 1 to this part from the surface upward to 10,000 feet MSL;

Above the ceiling and within the lateral boundaries of a Class B or Class C airspace area designated for an airport upward to 10,000 feet MSL;

Except as provided in paragraph (e) of this section, Class E airspace within the 48 contiguous states and the District of Columbia at and above 10,000 feet MSL, excluding the airspace at and below 2,500 feet above the surface; and

Class E airspace at and above 3,000 feet MSL over the Gulf of Mexico from the coastline of the United States out to 12 nautical miles.

The requirements of paragraph (b) ofthis section do not apply to any aircraft that was not originally certificated with an engine-driven electrical system, or that has not subsequently been certified with such a system installed, including balloons and gliders. These aircraft may conduct operations without ADS-B Out in the airspace specified in paragraph (d)(4) of this section. These aircraft may also conduct operations in the airspace specified in paragraph (d)(2) of this section ifthose operations are conducted—

(1) (2)

Outside any Class B or Class C airspace area; and

Below the altitude of the ceiling of a Class B or Class C airspace area designated for an airport, or 10,000 feet MSL, whichever is lower.

14 CFR91.225(e)(2) (enhanceddisplay)

page 81of243

(f) (g)

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.225(f)

Except as prohibited in paragraph (h)(2) of this section, each person operating an aircraft equipped with ADS-B Out must operate this equipment in the transmit mode at all times unless—

(1)

(2)

Otherwise authorized by the FAA when the aircraft is performing a sensitive government mission for national defense, homeland security, intelligence or law enforcement purposes and transmitting would compromise the operations security ofthe mission or pose a safety risk to the aircraft, crew, or people and property in the air or on the ground; or

Otherwise directed byATC when transmitting would jeopardize the safe execution of air traffic control functions.

Requests for ATC authorized deviations from the requirements of this section must be made to the ATC facility having jurisdiction over the concerned airspace within the time periods specified as follows:

(1)

(2)

For operation of an aircraft with an inoperative ADS-B Out, to the airport of ultimate destination, including any intermediate stops, or to proceed to a place where suitable repairs can be made or both, the request may be made at any time.

For operation of an aircraft that is not equipped with ADS-B Out, the request must be made at least 1 hour before the proposed operation.

(h)   For unmanned aircraft:

(1)

(2)

No person may operate an unmanned aircraft under a flight plan and in two way communication with ATC unless:

(i)   That aircraft has equipment installed that meets the performance requirements in TSO-C166b (including Section 2 of RTCA DO-260B, as referenced in TSO-C166b), TSO-C166c (including Section 2 of RTCA DO-260C as modified by DO-260C—Change 1, as referenced in TSO-C166c), TSO-C154c (including Section 2 of RTCA DO-282B, as referenced in TSO-C154c), or TSO-C154d (including Section 2 of RTCA DO-282C, as referenced in TSO-C154d); and

(ii)   The equipment meets the requirements of § 91.227.

No person may operate an unmanned aircraft under this part with Automatic Dependent Surveillance-Broadcast Out equipment in transmit mode unless:

(i)   The operation is conducted under a flight plan and the person operating that unmanned aircraft maintains two-way communication with ATC; or

(ii)  The use of ADS-B Out is otherwise authorized by the Administrator.

(i)

The standards required in this section are incorporated by reference with the approval of the Director of the Office ofthe Federal Register under 5 U.S.C. 552(a) and 1 CFR part 51. This incorporation by reference (IBR) material is available for inspection at the FAA and the National Archives and Records Administration (NARA). Contact the FAA at: Office of Rulemaking (ARM-1), 800 Independence Avenue SW, Washington, DC 20590 (telephone 202-267-9677). For information on the availability of this material at NARA, visit https://www.archives.gov/federal-register/cfr/ibr-locations.html or email fr.inspection@nara.gov. This material is also available from the following sources in this paragraph (i).

(1)

U.S. Department of Transportation, Subsequent Distribution Office, DOT Warehouse M30, Ardmore East Business Center, 3341 Q 75th Avenue, Landover, MD 20785; telephone (301) 322-5377; website: www.faa.gov/aircraft/air_cert/design_approvals/tso/ (select the link “Search Technical Standard Orders”).

14 CFR91.225(i)(1)(enhanceddisplay)

page 82of243

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.225(i)(1)(i)

(i) (ii)

(iii) (iv)

TSO-C166b, Extended Squitter Automatic Dependent Surveillance-Broadcast (ADS-B) and Traffic Information Service-Broadcast (TIS-B) Equipment Operating on the Radio Frequency of 1090 Megahertz (MHz), December 2, 2009.

TSO-C166c, Extended Squitter Automatic Dependent Surveillance-Broadcast (ADS-B) and Traffic Information Service-Broadcast (TIS-B) Equipment Operating on the Radio Frequency of 1090 Megahertz (MHz), March 10, 2023.

TSO-C154c, Universal Access Transceiver (UAT) Automatic Dependent Surveillance-Broadcast (ADS-B) Equipment Operating on the Frequency of 978 MHz, December 2, 2009.

TSO-C154d, Universal Access Transceiver (UAT) Automatic Dependent Surveillance-Broadcast (ADS-B) Equipment Operating on the Radio Frequency of 978 Megahertz (MHz), March 10, 2023.

(2)

RTCA, Inc., 1150 18th St. NW, Suite 910, Washington, DC 20036; telephone (202) 833-9339; website: www.rtca.org/products.

(i) (ii)

(iii) (iv)

(v)

RTCA DO-260B, Minimum Operational Performance Standards for 1090 MHz Extended Squitter Automatic Dependent Surveillance-Broadcast (ADS-B) and Traffic Information Services- Broadcast (TIS-B), Section 2, Equipment Performance Requirements and Test Procedures, December 2, 2009.

RTCA DO-260C, Minimum Operational Performance Standards for 1090 MHz Extended Squitter Automatic Dependent Surveillance-Broadcast (ADS-B) and Traffic Information Services- Broadcast (TIS-B), Section 2, Equipment Performance Requirements and Test Procedures, December 17, 2020.

RTCA DO-260C, Minimum Operational Performance Standards for 1090 MHz Extended Squitter Automatic Dependent Surveillance—Broadcast (ADS-B) and Traffic Information Services—Broadcast (TIS-B), Change 1, January 25, 2022.

RTCA DO-282B, Minimum Operational Performance Standards for Universal Access Transceiver (UAT) Automatic Dependent Surveillance-Broadcast (ADS-B), Section 2, Equipment

Performance Requirements and Test Procedures, December 2, 2009.

RTCA DO-282C, Minimum Operational Performance Standards (MOPS) for Universal Access Transceiver (UAT) Automatic Dependent Surveillance-Broadcast (ADS-B), Section 2, Equipment Performance Requirements and Test Procedures, June 23, 2022.

[Docket FAA-2007-29305, 75 FR 30193, May28, 2010;Amdt. 91-314-A, 75 FR 37712, June 30, 2010, as amendedbyAmdt. 91-316, 75 FR 37712, June 30, 2010;Amdt. 91-336, 80 FR 6900, Feb. 9, 2015;Amdt. 91-336A, 80 FR 11537, Mar. 4, 2015;Amdt. 91-355, 84 FR 34287, July 18, 2019; Amdt. 91-361, 86 FR 4513, Jan. 15, 2021; DocketFAA-2023-1836, Amdt. 91-371, 88 FR 71476, Oct. 17, 2023; Amdt. 91-371B, 89 FR 33224, Apr. 29, 2024]

§ 91.227Automatic Dependent Surveillance-Broadcast (ADS-B) Out equipment performance requirements.

(a)  Definitions. For the purposes of this section:

ADS-B Out is a function of an aircraft's onboard avionics that periodically broadcasts the aircraft's state vector (3-dimensional position and 3-dimensional velocity) and other required information as described in this section.

14 CFR91.227(a)“ADS-BOut”(enhanceddisplay)

page 83 of243

(b) (c)

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.227(a)“NavigationAccuracyCategoryforPosition”

Navigation Accuracy Category forPosition (NACP) specifies the accuracy of a reported aircraft's position. Navigation Accuracy Category for Velocity (NACV) specifies the accuracy of a reported aircraft's velocity.

Navigation Integrity Category (NIC) specifies an integrity containment radius around an aircraft's reported position.

Position Source refers to the equipment installed onboard an aircraft used to process and provide aircraft position (for example, latitude, longitude, and velocity) information.

Source IntegrityLevel (SIL) indicates the probability ofthe reported horizontal position exceeding the containment radius defined by the NIC on a per sample or per hour basis.

System Design Assurance (SDA) indicates the probability of an aircraft malfunction causing false or misleading information to be transmitted.

Total latency is the total time between when the position is measured and when the position is transmitted by the aircraft.

Uncompensated latency is the time for which the aircraft does not compensate for latency. 1090 MHzES and UATBroadcast Links and Power Requirements —

(1)

(2)

Aircraft operating in Class A airspace must have equipment installed that meets the antenna and power output requirements of Class A1S, A1, A2, A3, B1S, or B1 equipment as defined in TSO-C166b and Section 2 of RTCA DO-260B (as referenced in TSO-C166b), or TSO-C166c and Section 2 of RTCA DO-260C as modified by DO-260C—Change 1 (as referenced in TSO-C166c).

Aircraft operating in airspace designated for ADS-B Out, but outside of Class A airspace, must have equipment installed that meets the antenna and output power requirements of either:

(i)

Class A1S, A1, A2, A3, B1S, or B1 as defined in TSO-C166b and Section 2 of RTCA DO-260B (as referenced in TSO-C166b) or TSO-C166c and Section 2 of RTCA DO-260C as modified by DO-260C—Change 1 (as referenced in TSO-C166c); or

(ii)  Class A1S, A1H, A2, A3, B1S, or B1 equipment as defined in TSO-C154c and Section 2 of RTCA DO-282B (as referenced in TSO-C154c), or TSO-C154d and Section 2 of RTCA DO-282C (as referenced in TSO-C154d).

ADS-B OutPerformance Requirements forNACP, NACV, NIC, SDA , and SIL—

(1)  For aircraft broadcasting ADS-B Out as required under § 91.225 (a) and (b)— (i)  The aircraft's NACP must be less than 0.05 nautical miles;

(ii)  The aircraft's NACV must be less than 10 meters per second; (iii)  The aircraft's NIC must be less than 0.2 nautical miles;

(iv) The aircraft's SDA must be less than or equal to 10−5 per flight hour; and

(v)  The aircraft's SIL must be less than or equal to 10−7 per flight hour or per sample. (2) Changes in NACP, NACV, SDA, and SIL must be broadcast within 10 seconds.

(3)  Changes in NIC must be broadcast within 12 seconds.

14 CFR91.227(c)(3)(enhanceddisplay)

page 84of243

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.227(d)

(d)

Minimum BroadcastMessage ElementSet forADS-B Out. Each aircraft must broadcast the following information, as defined in TSO-C166b (including Section 2 of RTCA DO-260B, as referenced in TSO- C166b), TSO-C166c (including Section 2 of RTCA DO-260C as modified by DO-260C—Change 1, as referenced in TSO-C166c), TSO-C154c (including Section 2 of RTCA DO-282B, as referenced in TSO- C154c), or TSO-C154d (including Section 2 of RTCA DO-282C, as referenced in TSO-C154d). The pilot must enter information for message elements listed in paragraphs (d)(7) through (10) ofthis section during the appropriate phase of flight.

(1) (2) (3) (4) (5)

(6)

(7) (8)

(9)

(10) (11)

The length and width of the aircraft;

An indication of the aircraft's latitude and longitude;

An indication of the aircraft's barometric pressure altitude; An indication of the aircraft's velocity;

An indication if a collision avoidance system is installed and operating in a mode that can generate resolution advisory alerts;

If an operable collision avoidance system is installed, an indication if a resolution advisory is in effect;

An indication of the Mode A transponder code specified by ATC;

An indication of the aircraft identification that is submitted on the flight plan or used for communicating with ATC, except when the pilot has not filed a flight plan, has not requested ATC services, and is using a TSO-C154c or TSO-C154d self-assigned temporary 24-bit address;

An indication if the flightcrew has identified an emergency, radio communication failure, or unlawful interference;

An indication of the aircraft's “IDENT” to ATC;

An indication of the aircraft assigned ICAO 24-bit address, except when the pilot has not filed a flight plan, has not requested ATC services, and is using a TSO-C154c or TSO-C154d self-assigned temporary 24-bit address;

(12)  An indication of the aircraft's emitter category;

(13)

An indication of whether an ADS-B In capability is available;

(14)  An indication of the aircraft's geometric altitude;

(15) An indication of the Navigation Accuracy Category for Position (NACP); (16) An indication of the Navigation Accuracy Category for Velocity (NACV); (17) An indication of the Navigation Integrity Category (NIC);

(18) An indication of the System Design Assurance (SDA); and (19) An indication of the Source Integrity Level (SIL).

(e) ADS-B LatencyRequirements —

(1)   The aircraft must transmit its geometric position no later than 2.0 seconds from the time of measurement ofthe position to the time of transmission.

14 CFR91.227(e)(1)(enhanceddisplay)

page 85of243

(1)

(2)

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.227(e)(2)

(2)

(3) (4)

Within the 2.0 total latency allocation, a maximum of 0.6 seconds can be uncompensated latency. The aircraft must compensate for any latency above 0.6 seconds up to the maximum 2.0 seconds total by extrapolating the geometric position to the time of message transmission.

The aircraft must transmit its position and velocity at least once per second while airborne or while moving on the airport surface.

The aircraft must transmit its position at least once every 5 seconds while stationary on the airport surface.

(f) (g)

Equipment with an approved deviation. Operators with equipment installed with an approved deviation under § 21.618 of this chapter also are in compliance with this section.

Incorporation by reference. The standards required in this section are incorporated by reference with the approval of the Director of the Office ofthe Federal Register under 5 U.S.C. 552(a) and 1 CFR part 51. This incorporation by reference (IBR) material is available for inspection at the FAA and the National Archives and Records Administration (NARA). Contact the FAA at: Office of Rulemaking (ARM-1), 800 Independence Avenue SW, Washington, DC 20590 (telephone 202-267-9677). For information on the availability of this material at NARA, visit www.archives.gov/federal-register/cfr/ibr-locations.html or email fr.inspection@nara.gov. This material is also available from the following sources indicated in this paragraph (g).

U.S. Department of Transportation, Subsequent Distribution Office, DOT Warehouse M30, Ardmore East Business Center, 3341 Q 75th Avenue, Landover, MD 20785; telephone (301) 322-5377; website: www.faa.gov/aircraft/air_cert/design_approvals/tso/ (select the link “Search Technical Standard Orders”).

(i)

TSO-C166b, Extended Squitter Automatic Dependent Surveillance-Broadcast (ADS-B) and Traffic Information Service-Broadcast (TIS-B) Equipment Operating on the Radio Frequency of 1090 Megahertz (MHz), December 2, 2009.

(ii)  TSO-C166c, Extended Squitter Automatic Dependent Surveillance-Broadcast (ADS-B) and Traffic Information Service-Broadcast (TIS-B) Equipment Operating on the Radio Frequency of 1090 Megahertz (MHz), March 10, 2023.

(iii) TSO-C154c, Universal Access Transceiver (UAT) Automatic Dependent Surveillance-Broadcast (ADS-B) Equipment Operating on the Frequency of 978 MHz, December 2, 2009.

(iv) TSO-C154d, Universal Access Transceiver (UAT) Automatic Dependent Surveillance-Broadcast (ADS-B) Equipment Operating on the Radio Frequency of 978 Megahertz (MHz), March 10, 2023.

RTCA, Inc., 1150 18th St. NW, Suite 910, Washington, DC 20036; telephone (202) 833-9339; website: www.rtca.org/products.

(i)

RTCA DO-260B, Minimum Operational Performance Standards for 1090 MHz Extended Squitter Automatic Dependent Surveillance-Broadcast (ADS-B) and Traffic Information Services- Broadcast (TIS-B), Section 2, Equipment Performance Requirements and Test Procedures, December 2, 2009.

14 CFR91.227(g)(2)(i)(enhanceddisplay)

page 86of243

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.227(g)(2)(ii)

(ii)

RTCA DO-260C, Minimum Operational Performance Standards for 1090 MHz Extended Squitter Automatic Dependent Surveillance-Broadcast (ADS-B) and Traffic Information Services- Broadcast (TIS-B), Section 2, Equipment Performance Requirements and Test Procedures, December 17, 2020.

(iii) RTCA DO-260C, Minimum Operational Performance Standards for 1090 MHz Extended Squitter Automatic Dependent Surveillance-Broadcast (ADS-B) and Traffic Information Services- Broadcast (TIS-B), Change 1, January 25, 2022.

(iv) RTCA DO-282B, Minimum Operational Performance Standards for Universal Access Transceiver (UAT) Automatic Dependent Surveillance-Broadcast (ADS-B), Section 2, Equipment

Performance Requirements and Test Procedures, December 2, 2009.

(v) RTCA DO-282C, Minimum Operational Performance Standards (MOPS) for Universal Access Transceiver (UAT) Automatic Dependent Surveillance-Broadcast (ADS-B), Section 2, Equipment Performance Requirements and Test Procedures, June 23, 2022.

[Docket FAA-2007-29305, 75 FR 30194, May28, 2010;Amdt. 91-314-A, 75 FR 37712, June 30, 2010, as amendedbyAmdt. 91-316, 75 FR 37712, June 30, 2010; Docket FAA-2023-1836, Amdt. 91-371, 88 FR 71477, Oct. 17, 2023]

§§ 91.228-91.299 [Reserved]

Subpart D—Special Flight Operations

Source: Docket 18334, 54 FR 34308, Aug. 18, 1989, unless otherwise noted.

§ 91.301 [Reserved]

§ 91.303 Aerobatic flight.

No person may operate an aircraft in aerobatic flight—

(a) Over any congested area of a city, town, or settlement; (b)  Over an open air assembly of persons;

(c)  Within the lateral boundaries of the surface areas of Class B, Class C, Class D, or Class E airspace designated for an airport;

(d)  Within 4 nautical miles of the center line of any Federal airway; (e)  Below an altitude of 1,500 feet above the surface; or

(f)   When flight visibility is less than 3 statute miles.

For the purposes of this section, aerobatic flight means an intentional maneuver involving an abrupt change in an aircraft's attitude, an abnormal attitude, or abnormal acceleration, not necessary for normal flight.

[Docket 18834, 54 FR 34308, Aug. 18, 1989, as amendedbyAmdt. 91-227, 56 FR 65661, Dec. 17, 1991]

14 CFR91.303(f)(enhanceddisplay)

page 87of243

(b) (c)

(d) (e)

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.305

§ 91.305 Flighttestareas.

No person may flight test an aircraft except over open water, or sparsely populated areas, having light air traffic.

§ 91.307 Parachutes and parachuting.

(a)   No pilot of a civil aircraft may allow a parachute that is available for emergency use to be carried in that aircraft unless it is an approved type and has been packed by a certificated and appropriately rated parachute rigger—

(1)

(2)

Within the preceding 180 days, if its canopy, shrouds, and harness are composed exclusively of nylon, rayon, or other similar synthetic fiber or materials that are substantially resistant to damage from mold, mildew, or other fungi and other rotting agents propagated in a moist environment; or

Within the preceding 60 days, if any part ofthe parachute is composed of silk, pongee, or other natural fiber or materials not specified in paragraph (a)(1) ofthis section.

Except in an emergency, no pilot in command may allow, and no person may conduct, a parachute operation from an aircraft within the United States except in accordance with part 105 of this chapter.

Unless each occupant ofthe aircraft is wearing an approved parachute, no pilot of a civil aircraft carrying any person (other than a crewmember) may execute any intentional maneuver that exceeds—

(1) (2)

A bank of 60 degrees relative to the horizon; or

A nose-up or nose-down attitude of 30 degrees relative to the horizon.

Paragraph (c) of this section does not applyto—

(1) (2)

Flight tests for pilot certification or rating; or

Spins and other flight maneuvers required by the regulations for any certificate or rating when given by—

(i)   A certificated flight instructor; or

(ii)   An airline transport pilot instructing in accordance with § 61.67 of this chapter.

For the purposes ofthis section, approvedparachute means—

(1) (2)

A parachute manufactured under a type certificate or a technical standard order (C-23 series); or

A personnel-carrying military parachute identified by an NAF, AAF, or AN drawing number, an AAF order number, or any other military designation or specification number.

[Docket 18334, 54 FR 34308, Aug. 18, 1989, as amendedbyAmdt. 91-255, 62 FR 68137, Dec. 30, 1997;Amdt. 91-268, 66 FR 23553, May 9, 2001;Amdt. 91-305, 73 FR 69530, Nov. 19, 2008]

§ 91.309 Towing: Gliders andunpowered ultralightvehicles.

(a)  No person may operate a civil aircraft towing a glider or unpowered ultralight vehicle unless— (1)   The pilot in command of the towing aircraft is qualified under § 61.69 of this chapter;

(2)   The towing aircraft has:

14 CFR91.309(a)(2)(enhanced display)

page 88 of243

(3)

(4) (5)

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.309(a)(2)(i)

(i) (ii)

(iii)

A standard airworthiness certificate and is equipped with a tow-hitch of a kind, and installed in a manner, that is approved by the Administrator;

A special airworthiness certificate for which a type certificate has been issued, and is equipped with a tow-hitch of a kind, and installed in a manner, that is approved or otherwise authorized by the Administrator; or

A special airworthiness certificate, for which the aircraft has not been previously issued a type certificate, and is equipped with a tow-hitch of a kind that is approved or otherwise acceptable to, and is installed in a manner acceptable to, the Administrator;

The towline used has breaking strength not less than 80 percent ofthe maximum certificated operating weight of the glider or unpowered ultralight vehicle and not more than twice this operating weight. However, the towline used may have a breaking strength more than twice the maximum certificated operating weight of the glider or unpowered ultralight vehicle if—

(i) (ii)

A safety link is installed at the point of attachment of the towline to the glider or unpowered ultralight vehicle with a breaking strength not less than 80 percent of the maximum certificated operating weight of the glider or unpowered ultralight vehicle and not greater than twice this operating weight;

A safety link is installed at the point of attachment of the towline to the towing aircraft with a breaking strength greater, but not more than 25 percent greater, than that of the safety link at the towed glider or unpowered ultralight vehicle end of the towline and not greater than twice the maximum certificated operating weight of the glider or unpowered ultralight vehicle;

Before conducting any towing operation within the lateral boundaries ofthe surface areas of Class B, Class C, Class D, or Class E airspace designated for an airport, or before making each towing flight within such controlled airspace if required by ATC, the pilot in command notifies the control tower. If a control tower does not exist or is not in operation, the pilot in command must notify the FAA flight service station serving that controlled airspace before conducting any towing operations in that airspace; and

The pilots of the towing aircraft and the glider or unpowered ultralight vehicle have agreed upon a general course of action, including takeoff and release signals, airspeeds, and emergency procedures for each pilot.

(b)   No pilot of a civil aircraft may intentionally release a towline, after release of a glider or unpowered ultralight vehicle, in a manner that endangers the life or property of another.

[Docket 18834, 54 FR 34308, Aug. 18, 1989, as amendedbyAmdt. 91-227, 56 FR 65661, Dec. 17, 1991;Amdt. 91-282, 69 FR 44880, July 27, 2004; Docket FAA-2023-1377, Amdt. 91-381, 90 FR 35220, July24, 2025]

§ 91.311 Towing: Otherthanunder § 91.309.

No pilot of a civil aircraft may tow anything with that aircraft (other than under § 91.309) except in accordance with the terms of a certificate of waiver issued bythe Administrator.

§ 91.313 Restricted category civil aircraft: Operating limitations. Link to an amendment published at 90 FR 35220, July 24, 2025.

14 CFR91.313 (enhanceddisplay)

page 89 of243

(a) (b)

(c) (d)

(e) (f)

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.313(a)

No person may operate a restricted category civil aircraft—

(1) (2)

For otherthan the special purpose for which it is certificated; or

In an operation other than one necessaryto accomplish the work activity directly associated with that special purpose.

For the purpose of paragraph (a) of this section, the following operations are considered necessaryto accomplish the work activity directly associated with a special purpose operation:

(1) (2)

(3)

Flights conducted for flight crewmember training in a special purpose operation for which the aircraft is certificated.

Flights conducted to satisfy proficiency check and recent flight experience requirements under part 61 of this chapter provided the flight crewmember holds the appropriate category, class, and type ratings and is employed by the operator to perform the appropriate special purpose operation.

Flights conducted to relocate the aircraft for delivery, repositioning, or maintenance.

No person may operate a restricted category civil aircraft carrying persons or property for compensation or hire. For the purposes ofthis paragraph (c), a special purpose operation involving the carriage of persons or material necessaryto accomplish that operation, such as crop dusting, seeding, spraying, and banner towing (including the carrying of required persons or material to the location of that operation), an operation forthe purpose of providing flight crewmember training in a special purpose operation, and an operation conducted under the authority provided in paragraph (h) of this section are not considered to be the carriage of persons or property for compensation or hire.

No person may be carried on a restricted category civil aircraft unless that person—

(1) (2) (3)

(4)

(5)

Is a flight crewmember;

Is a flight crewmember trainee;

Performs an essential function in connection with a special purpose operation for which the aircraft is certificated;

Is necessary to accomplish the work activity directly associated with that special purpose; or

Is necessary to accomplish an operation under paragraph (h) ofthis section.

Except when operating in accordance with the terms and conditions of a certificate of waiver or special operating limitations issued by the Administrator, no person may operate a restricted category civil aircraft within the United States—

(1) (2) (3)

Over a densely populated area; In a congested airway; or

Near a busy airport where passengertransport operations are conducted.

This section does not apply to nonpassenger-carrying civil rotorcraft external-load operations conducted under part 133 of this chapter.

14 CFR91.313(f)(enhanceddisplay)

page 90 of243

(g) (h)

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR 91.313(g)

No person may operate a small restricted-category civil airplane manufactured after July 18, 1978, unless an approved shoulder harness or restraint system is installed for each front seat. The shoulder harness or restraint system installation at each flightcrew station must permit the flightcrew member, when seated and with the safety belt and shoulder harness fastened or the restraint system engaged, to perform all functions necessary for flight operation. For purposes ofthis paragraph—

(1) (2)

(1)

(2) (3) (4)

The date of manufacture of an airplane is the date the inspection acceptance records reflect that the airplane is complete and meets the FAA-approved type design data; and

A front seat is a seat located at a flight crewmember station or any seat located alongside such a seat.

An operator may apply for deviation authority from the provisions of paragraph (a) of this section to conduct operations forthe following purposes:

(i)   Flight training and the practical test for issuance of a type rating provided—

(A)  The pilot being trained and tested holds at least a commercial pilot certificate with the appropriate category and class ratings for the aircraft type;

(B)  The pilot receiving flight training is employed by the operator to perform a special purpose operation; and

(C)  The flight training is conducted by the operator who employs the pilot to perform a special purpose operation.

(ii)   Flights to designate an examiner or qualify an FAA inspector in the aircraft type and flights necessary to provide continuing oversight and evaluation of an examiner.

The FAA will issue this deviation authority as a letter of deviation authority. The FAA may cancel or amend a letter of deviation authority at any time.

An applicant must submit a request for deviation authority in a form and manner acceptable to the Administrator at least 60 days before the date of intended operations. A request for deviation authority must contain a complete description of the proposed operation and justification that establishes a level of safety equivalent to that provided under the regulations for the deviation requested.

[Docket 18334, 54 FR 34308, Aug. 18, 1989, as amendedbyDocket FAA-2015-1621, Amdt. 91-346, 81 FR 96700, Dec. 30, 2016; Amdt. 60-6, 83 FR 30281, June 27, 2018]

§ 91.315 Limited category civil aircraft: Operating limitations.

Except as provided in § 91.326, no person may operate a limited category civil aircraft carrying persons or property for compensation or hire in operations that:

(a) (b) (c)

Require an air carrier or commercial operator certificate issued under part 119 of this chapter; Are listed in § 119.1(e) of this chapter;

Require management specifications for a fractional ownership program issued in accordance with subpart K of this part; or

14 CFR91.315(c)(enhanced display)

page 91of243

(b) (c) (d)

(e) (f)

(g)

(h) (i)

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.315(d)

(d)

Are conducted under part 129, 133, or 137 of this chapter.

[Docket FAA-2023-1351, Amdt. 91-378, 89 FR 80340, Oct. 2, 2024]

§ 91.317 Provisionallycertificatedcivilaircraft:   Operatinglimitations.

(a)   No person may operate a provisionally certificated civil aircraft unless that person is eligible for a provisional airworthiness certificate under § 21.213 of this chapter.

No person may operate a provisionally certificated civil aircraft outside the United States unless that person has specific authority to do so from the Administrator and each foreign country involved.

Unless otherwise authorized by the Executive Director, Flight Standards Service, no person may operate a provisionally certificated civil aircraft in airtransportation.

Unless otherwise authorized by the Administrator, no person may operate a provisionally certificated civil aircraft except—

(1)

In direct conjunction with the type or supplemental type certification  ofthat aircraft;

(2)   Fortraining flight crews, including simulated air carrier operations;

(3)   Demonstration flight by the manufacturer for prospective purchasers; (4)   Market surveys by the manufacturer;

(5)

Flight checking of instruments, accessories, and equipment that do not affect the basic airworthiness ofthe aircraft; or

(6)  Service testing ofthe aircraft.

Each person operating a provisionally certificated civil aircraft shall operate within the prescribed limitations displayed in the aircraft or set forth in the provisional aircraft flight manual or other appropriate document. However, when operating in direct conjunction with the type or supplemental type certification of the aircraft, that person shall operate under the experimental aircraft limitations of § 21.191 of this chapter and when flight testing, shall operate underthe requirements of § 91.305 of this part.

Each person operating a provisionally certificated civil aircraft shall establish approved procedures for—

(1) (2)

The use and guidance of flight and ground personnel in operating under this section; and

Operating in and out of airports where takeoffs or approaches over populated areas are necessary. No person may operate that aircraft except in compliance with the approved procedures.

Each person operating a provisionally certificated civil aircraft shall ensure that each flight crewmember is properly certificated and has adequate knowledge of, and familiarity with, the aircraft and procedures to be used by that crewmember.

Each person operating a provisionally certificated civil aircraft shall maintain it as required by applicable regulations and as may be specially prescribed by the Administrator.

Whenever the manufacturer, or the Administrator, determines that a change in design, construction, or operation is necessaryto ensure safe operation, no person may operate a provisionally certificated civil aircraft until that change has been made and approved. Section 21.99 of this chapter applies to operations underthis section.

14 CFR91.317(i)(enhanced display)

page 92of243

(a)

(b) (c) (d)

(j) (k)

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.317(j)

Each person operating a provisionally certificated civil aircraft—

(1)

(2)

May carry in that aircraft only persons who have a proper interest in the operations allowed by this section or who are specifically authorized by both the manufacturer and the Administrator; and

Shall advise each person carried that the aircraft is provisionally certificated.

The Administrator may prescribe additional limitations or procedures that the Administrator considers necessary, including limitations on the number of persons who may be carried in the aircraft.

(Approved by the Office ofManagement and Budget under control number2120-0005)

[Docket 18334, 54 FR 34308, Aug. 18, 1989, as amendedbyAmdt. 91-212, 54 FR 39293, Sept. 25, 1989;Docket FAA-2018-0119, Amdt. 91-350, 83 FR 9171, Mar. 5, 2018]

§ 91.319  Aircrafthavingexperimentalcertificates:  Operating limitations. Link to an amendment published at 90 FR 35220, July 24, 2025.

Except as provided in § 91.326, no person may operate an aircraft that has an experimental certificate—

(1) (2)

For otherthan the purpose for which the certificate was issued; or Carrying persons or property for compensation or hire in operations that:

(i)

Require an air carrier or commercial operator certificate issued under part 119 of this chapter;

(ii)   Are listed in § 119.1(e) of this chapter;

(iii)  Require management specifications for a fractional ownership program issued in accordance with subpart K of this part; or

(iv)  Are conducted under part 129, 133, or 137 of this chapter.

No person may operate an aircraft that has an experimental certificate outside of an area assigned by the Administrator until it is shown that—

(1)

(2)

The aircraft is controllable throughout its normal range of speeds and throughout all the maneuvers to be executed; and

The aircraft has no hazardous operating characteristics or design features.

Unless otherwise authorized by the Administrator in special operating limitations, no person may operate an aircraft that has an experimental certificate over a densely populated area or in a congested airway. The Administrator may issue special operating limitations for particular aircraft to permit takeoffs and landings to be conducted over a densely populated area or in a congested airway, in accordance with terms and conditions specified in the authorization in the interest of safety in air commerce.

Each person operating an aircraft that has an experimental certificate shall—

(1) (2) (3)

Advise each person carried of the experimental nature of the aircraft;

Operate under VFR, day only, unless otherwise specifically authorized by the Administrator; and Notify air traffic control of the experimental nature of the aircraft when utilizing airtraffic services.

14 CFR91.319(d)(3) (enhanceddisplay)

page 93 of243

(e)

(f) (g)

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.319(e)

No person may operate a light-sport aircraft that is issued an experimental certificate under § 21.191 of this chapter for compensation or hire, except:

(1)

(2)

A person may operate an aircraft issued an experimental certificate under § 21.191(i)(1) of this chapter to tow a glider that is a light-sport aircraft or unpowered ultralight vehicle in accordance with § 91.309; or

A person may operate a light-sport aircraft issued an experimental certificate under § 21.191 of this chapter to conduct operations authorized under § 91.326.

No person may lease a light-sport aircraft that is issued an experimental certificate under § 21.191 of this chapter, except—

(1) (2)

In accordance with paragraph (e)(1) ofthis section; or

To conduct a solo flight in accordance with a training program included as part of the deviation authority specified under § 91.326(b).

No person may operate an aircraft issued an experimental airworthiness certificate under § 21.191(i)(1) of this chapter to tow a glider that is a light-sport category aircraft or unpowered ultralight vehicle for compensation or hire or to conduct flight training for compensation or hire in an aircraft which that person provides unless within the preceding 100 hours of time in service the aircraft has—

(1)

(2)

Been inspected by a certificated repairman (light-sport) with a maintenance rating, an appropriately rated mechanic, or an appropriately rated repair station in accordance with inspection procedures developed by the aircraft manufacturer or a person acceptable to the FAA; or

Received an inspection forthe issuance of an airworthiness certificate in accordance with part 21 of this chapter.

(h)  [Reserved]

(i) (j)

The Administrator may prescribe additional limitations that the Administrator considers necessary, including limitations on the persons that may be carried in the aircraft.

No person may operate an aircraft that has an experimental certificate under § 61.113(i) of this chapter unless the aircraft is carrying not more than 7 occupants.

(Approved by the Office ofManagement and Budget under control number2120-0005)

[Docket 18334, 54 FR 34308, Aug. 18, 1989, as amendedbyAmdt. 91-282, 69 FR 44881, July27, 2004; DocketFAA-2016-9157, Amdt. 91-347, 82 FR 3167, Jan. 11, 2017; DocketFAA-2024-2580,Amdt. 91-380, 89 FR 90578, Nov. 18, 2024;Docket FAA-2023-1351, Amdt. 91-378, 89 FR 80340, Oct. 2, 2024; Docket FAA-2023-1377, Amdt. 91-381, 90 FR 35220, July24, 2025]

§ 91.321 Carriage ofcandidates in elections.

(a)  As an aircraft operator, you may receive payment for carrying a candidate, agent of a candidate, or person traveling on behalf of a candidate, running for Federal, State, or local election, without having to comply with the rules in parts 121, 125 or 135 of this chapter, under the following conditions:

(1) (2)

Your primary business is not as an air carrier or commercial operator;

You carry the candidate, agent, or person traveling on behalf of a candidate, underthe rules of part 91; and

14 CFR91.321(a)(2)(enhanceddisplay)

page 94of243

(b)

(c) (d)

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.321(a)(3)

(3)

By Federal, state or local law, you are required to receive payment for carrying the candidate, agent, or person traveling on behalf of a candidate. For federal elections, the payment may not exceed the amount required by the Federal Election Commission. For a state or local election, the payment may not exceed the amount required underthe applicable state or local law.

(b)

For the purposes ofthis section, for Federal elections, the terms candidate and election have the same meaning as set forth in the regulations ofthe Federal Election Commission. For State or local elections, the terms candidate and election have the same meaning as provided by the applicable State or local law and those terms relate to candidates for election to public office in State and local government elections.

[Docket FAA-2005-20168, 70 FR 4982, Jan. 31, 2005]

§ 91.323 Increased  maximumcertificatedweights  for certainairplanes operated inAlaska.

(a)   Notwithstanding any other provision of the Federal Aviation Regulations, the Administrator will approve, as provided in this section, an increase in the maximum certificated weight of an airplane type certificated under Aeronautics Bulletin No. 7-A ofthe U.S. Department of Commerce dated January 1, 1931, as amended, or underthe normal category of part 4a of the former Civil Air Regulations (14 CFR part 4a,

1964 ed.) ifthat airplane is operated in the State ofAlaska by—

(1) (2)

A certificate holder conducting operations under part 121 or part 135 of this chapter; or

The U.S. Department of Interior in conducting its game and fish law enforcement activities or its management, fire detection, and fire suppression activities concerning public lands.

The maximum certificated weight approved under this section may not exceed— (1)  12,500 pounds;

(2) (3)

(4)

115 percent of the maximum weight listed in the FAA aircraft specifications;

The weight at which the airplane meets the positive maneuvering load factor n, where n=2.1+(24,000/(W+10,000)) and W=design maximum takeoff weight, except that n need not be more than 3.8; or

The weight at which the airplane meets the climb performance requirements under which it was type certificated.

In determining the maximum certificated weight, the Administrator considers the structural soundness of the airplane and the terrain to be traversed.

The maximum certificated weight determined underthis section is added to the airplane's operation limitations and is identified as the maximum weight authorized for operations within the State of Alaska.

[Docket 18334, 54 FR 34308, Aug. 18, 1989; Amdt. 91-211, 54 FR 41211, Oct. 5, 1989, as amended byAmdt. 91-253, 62 FR 13253, Mar. 19, 1997; DocketFAA-2015-1621,Amdt. 91-346, 81 FR 96700, Dec. 30, 2016]

§ 91.325 Primarycategory aircraft: Operating limitations.

(a)   Unless provided for in this section, no person may operate a primary category aircraft carrying persons or property for compensation or hire in operations that:

(1)

Require an air carrier or commercial operator certificate issued under part 119 of this chapter;

14 CFR91.325(a)(1) (enhanceddisplay)

page 95of243

(b) (c)

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.325(a)(2)

(2) (3)

(4)

Are listed in § 119.1(e) of this chapter;

Require management specifications for a fractional ownership program issued in accordance with subpart K of this part; or

Are conducted under part 129, 133, or 137 of this chapter.

Except as provided in § 91.326(c), no person may operate a primary category aircraft that is maintained by the pilot-owner under an approved special inspection and maintenance program except—

(1)  The pilot-owner; or

(2)

A designee ofthe pilot-owner, provided that the pilot-owner does not receive compensation for the use ofthe aircraft.

A primary category aircraft that is maintained by an appropriately rated mechanic or an authorized certificated repair station in accordance with the applicable provisions of part 43 of this chapter may be used to conduct flight training, checking, and testing for compensation or hire.

[Docket FAA-2023-1351, Amdt. 91-378, 89 FR 80340, Oct. 2, 2024]

§ 91.326 Exceptionto operating certainaircraft for the purposes offlight training, flightcrew member checking, or flightcrewmember testing.

(a) (b)

General. Notwithstanding the prohibitions in §§ 91.315, 91.319(a), and 91.325, a person may conduct flight training, checking, ortesting in a limited category aircraft, experimental aircraft, or primary category aircraft under the provisions ofthis section.

Operations requiring a letter ofdeviation authority. Except as provided in paragraphs (c) and (d) of this section, no person may conduct flight training, checking, or testing in a limited category or experimental aircraft without deviation authority issued under this paragraph (b).

(1) (2)

(3)

No person may operate underthis section without a letter of deviation authority (LODA) issued bythe Administrator.

The FAA may deny an application for a letter of deviation authority if it determines the deviation would not be in the interest of safety or is unnecessary. The FAA may cancel or amend a letter of deviation authority if it determines that the deviation holder has failed to comply with the conditions and limitations or at any time if the Administrator determines that the deviation is no longer necessary or in the interest of safety.

An applicant must submit a request for deviation authority in a form and manner acceptable to the Administrator at least 60 days before the date of intended operations. A request for deviation authority must contain a complete description of the proposed operation that establishes a level of safety equivalent to that provided under the regulations for the deviation requested, including:

(i)   A letter identifying the name and address of the applicant;

(ii)   The name and contact information of the individual with ultimate responsibility for operations authorized under the deviation authority;

(iii)  Specific aircraft make(s), model(s), registration number(s), and serial number(s) to be used;

(iv)  Copies of each aircraft's airworthiness certificate, including the FAA-issued operating limitations, if applicable;

14 CFR91.326(b)(3)(iv) (enhanceddisplay)

page 96of243

(i) (ii)

(iii)

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.326(b)(3)(v)

(v) (vi)

(vii)

Ejection seat information, if applicable;

A detailed training program that demonstrates the proposed activities will meet the intended training objectives;

A description of the applicant's process to determine whether a trainee has a specific need for formation or aerobatic training, or training leading to the issuance of an endorsement, ifthose types of training are being requested; and

(4)

(viii)  Any other information that the Administrator deems necessary to evaluate the application.

The holder of a letter of deviation authority must comply with any conditions and limitations provided in that letter of deviation authority. Unless otherwise authorized by the Administrator, the deviation authority will include the following conditions and limitations:

The operator must use the aircraft-specific flight and ground training program for the training authorized by the letter of deviation authority. Demonstration flights, discovery flights, experience flights, and other flights not related to the training program are not authorized.

As appropriate to the aircraft being flown, all trainees must hold category and class ratings; a type rating, Authorized Experimental Aircraft authorization, or temporary Letter ofAuthorization; and endorsements listed in § 61.31 of this chapter, as appropriate, with the following exceptions:

(A)

(B) (C)

(D)

Persons receiving gyroplane training ortraining leading to the initial issuance of a sport pilot certificate or flight instructor certificate with a sport pilot rating in a low mass, high drag aircraft with an empty weight less than 650 pounds and a VH ≤87 Knots Calibrated Airspeed (KCAS) are not required to hold category or class ratings. For training leading to an endorsement for additional sport pilot privileges, the pilot receiving the training must hold at least a sport pilot certificate with appropriate category and class ratings and endorsements issued under § 61.31 of this chapter, as appropriate.

Persons with a specific need to receive training toward the issuance of an endorsement are not required to hold the § 61.31 of this chapter endorsement sought. Any endorsements being provided must be authorized in the LODA.

Persons receiving jet unusual attitude and upset recovery training, limited category type rating training, or authorized experimental aircraft authorization training, if required for the type of aircraft being flown, are not required to hold the applicable type rating, authorized experimental authorization rating, or a temporary Letter of Authorization prior to the commencement of training.

For ultralight-style training, the person receiving training is not required to meet category and class ratings or § 61.31 of this chapter endorsement requirements. However, if the flight training includes a solo flight segment, this does not relieve the person receiving training from the requirements of part 61, subpart C, of this chapter. This training is limited to a low mass, high drag aircraft with an empty weight less than 650 pounds and a maximum speed in level flight with maximum continuous power less than 87 KCAS.

If the aircraft is equipped with ejection seats and systems, such systems must be rigged, maintained, and inspected in accordance with the manufacturer's recommendations. Before providing training in aircraft equipped with operable ejection systems, whether armed or not armed, all aircraft occupants must complete a course of ejection seat training.

14 CFR91.326(b)(4)(iii)(enhanceddisplay)

page 97of243

(iv) (v)

(vi) (vii)

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.326(b)(4)(iv)

When conducting spin and upset training, the operator must maintain a minimum recovery altitude of 6,000 feet above ground level unless the Administrator authorizes a lower altitude.

A copy ofthe LODA must be carried on board the aircraft during flight training conducted under the LODA.

The LODA holder must keep a record of the training given for a period of 36 calendar months from the completion date of the training. The authorized instructor must sign the trainee's training record certifying that the flight training or ground training was given. The training record must include the following:

(A) (B)

The name and certificate number (if applicable) ofthe trainee; The name, signature, and certificate number ofthe instructor;

(C)  The date trained;

(D)  The training received;

(E)

The trainee's specific need for training, if applicable.

Notwithstanding § 43.1(b) of this chapter or § 91.409(c)(1), all aircraft must:

(A)

(B) (C)

Except forturbine powered or large aircraft, within the preceding 100 hours of time in service, have received an annual, 100-hour, or condition inspection equivalent to the scope and detail of appendix D to part 43 of this chapter and been approved for return to service in accordance with part 43. The 100-hour limitation may be exceeded by not more than 10 hours while enroute to reach a place where the inspection can be done. The excess time used to reach a place where the inspection can be done must be included in computing the next 100 hours oftime in service; or

Except forturbine powered or large aircraft, be inspected in accordance with an FAA- approved inspection program that includes provisions for ensuring continued airworthiness and recording the current status on life-limited parts and in accordance with the manufacturer's instructions.

For turbine-powered or large aircraft, be inspected in accordance with an FAA-approved inspection program that meets the scope and detail of the requirements of § 91.409(e), (f)(4), and (g) for ensuring continued airworthiness and recording time remaining on life- limited parts in accordance with the manufacturer's instructions.

(viii)  Notwithstanding any exception due to the experimental airworthiness certification of the aircraft, LODA holders with experimental aircraft must comply with FAA Airworthiness Directives applicable to any corresponding make or model aircraft holding a different type of airworthiness certificate or applicable to any article installed on the aircraft. The LODA holder must evaluate the aircraft and its articles to determine if compliance with the FAA Airworthiness Directive is necessary for the continued safe operation ofthe aircraft. LODA holders must keep a maintenance record entry of those FAA Airworthiness Directives evaluated. For those FAA Airworthiness Directives for which the LODA holder determined compliance was necessary for the continued safe operation of the aircraft, the record must also include the method of compliance, and ifthe FAA Airworthiness Directive requires recurring action, the time and date when the next action is required.

14 CFR91.326(b)(4)(viii)(enhanceddisplay)

page 98 of243

(c) (d)

(1) (2)

(5) (6)

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.326(b)(5)

Only the following persons may be on board the aircraft during operations conducted underthe deviation authority:

(i)

The authorized instructor, designated examiner, person receiving flight training or being checked or tested, or persons essential forthe safe operation of the aircraft; and

(ii)   Notwithstanding any operating limitation applicable under § 91.9(a) that prohibits the carriage of passengers for compensation or hire, up to two persons enrolled in a LODA training course for the same aircraft may observe the flight training from a forwardmost observer seat with an unobstructed view ofthe flight deck, provided carriage of those persons is not prohibited by any other regulation.

The Administrator may limit the types of training, testing, and checking authorized underthis deviation authority. Training, testing, and checking under this deviation authority must be conducted consistent with the training program submitted for FAA review.

Operations notrequiring a letterofdeviation authority. The following operations may be conducted without a letter of deviation authority.

An authorized instructor, registered owner, lessor, or lessee of an aircraft is not required to obtain a letter of deviation authority from the Administratorto allow, conduct, or receive flight training, checking, ortesting in a limited category aircraft, experimental aircraft, or primary category aircraft if—

(i) (ii)

The authorized instructor is not providing both the training and the aircraft;

No person advertises or broadly offers the aircraft as available for flight training, checking, or testing; and

(iii)  No person receives compensation for the use of the aircraft for any flight during which flight training, checking, or testing was received, otherthan expenses for owning, operating, and maintaining the aircraft. Compensation for the use ofthe aircraft for profit is prohibited.

A person may operate a limited category aircraft, experimental aircraft, or primary category aircraft to conduct flight training, checking, or testing without a letter of deviation authority if no person provides and no person receives compensation for the flight training, checking, or testing, or for the use ofthe aircraft.

Previously issuedletters ofdeviation authority. For deviation authority issued under § 91.319 prior to December 2, 2024, the following requirements apply—

(1) (2)

(3) (4)

The deviation holder may continue to operate under the letter of deviation authority until December 1, 2026;

The deviation holder must continue to comply with the conditions and limitations in the letter of deviation authority when conducting an operation underthe letter of deviation authority in accordance with paragraph (b)(1) ofthis section;

The letter of deviation authority may be cancelled or amended at any time; and The letter of deviation authority terminates on December 1, 2026.

[Docket FAA-2023-1351, Amdt. 91-378, 89 FR 80341, Oct. 2, 2024] 14 CFR91.326(d)(4)(enhanced display)

page 99 of243

(a) (b)

(c)

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.327

§ 91.327Aircraft issueda  specialairworthinesscertificate  inthe light-sport category: Operating limitations.

Link to an amendment published at 90 FR 35221, July 24, 2025.

No person may operate an aircraft that has a special airworthiness certificate in the light-sport category for compensation or hire except—

(1) (2)

To tow a glider or an unpowered ultralight vehicle in accordance with § 91.309 of this chapter; or To conduct flight training, checking, and testing.

No person may operate an aircraft that has a special airworthiness certificate in the light-sport category unless—

(1) (2)

(3) (4)

(5)

(6)

The aircraft is maintained by a certificated repairman (light-sport) with a maintenance rating, an appropriately rated mechanic, or an appropriately rated repair station in accordance with the applicable provisions of part 43 of this chapter and maintenance and inspection procedures developed by the aircraft manufacturer or a person acceptable to the FAA;

A condition inspection is performed once every 12 calendar months by a certificated repairman (light-sport) with a maintenance rating, an appropriately rated mechanic, or an appropriately rated repair station in accordance with inspection procedures developed by the aircraft manufacturer or a person acceptable to the FAA;

The owner or operator complies with all applicable airworthiness directives;

Each repair or alteration to an aircraft meets the applicable and current FAA-accepted or approved consensus standards specified in the statement of compliance submitted to the FAA for the aircraft.

Each major repair or major alteration to an aircraft product produced under a consensus standard is authorized by the manufacturer or a person acceptable to the FAA, and is performed and inspected in accordance with maintenance and inspection procedures developed by the manufacturer or a person acceptable to the FAA; and

The owner or operator complies with the requirements for the recording of major repairs and major alterations performed on type-certificated products in accordance with § 43.9(d) of this chapter, and with the retention requirements in § 91.417.

No person may operate an aircraft issued a special airworthiness certificate in the light-sport category to tow a glider or unpowered ultralight vehicle for compensation or hire or conduct flight training for compensation or hire in an aircraft which that person provides unless within the preceding 100 hours of time in service the aircraft has—

(1) (2)

Been inspected by a certificated repairman (light-sport) with a maintenance rating, an appropriately rated mechanic, or an appropriately rated repair station in accordance with inspection procedures developed by the aircraft manufacturer or maintenance and inspection procedures acceptable to the FAA and been approved for return to service in accordance with part 43 of this chapter; or

Received an inspection forthe issuance of an airworthiness certificate in accordance with part 21 of this chapter.

14 CFR91.327(c)(2)(enhanceddisplay)

page 100 of243

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.327(d)

(d) (e)

(f)

Each person operating an aircraft issued a special airworthiness certificate in the light-sport category must operate the aircraft in accordance with the aircraft's operating instructions, including any provisions for necessary operating equipment specified in the aircraft's equipment list.

Each person operating an aircraft issued a special airworthiness certificate in the light-sport category must advise each person carried ofthe special nature of the aircraft and that the aircraft does not meet the airworthiness requirements for an aircraft issued a standard airworthiness certificate.

The FAA may prescribe additional limitations that it considers necessary.

[Docket FAA-2001-11133, 69 FR 44881, July27, 2004, as amended byDocketFAA-2023-1351;Amdt. 91-378, 89 FR 80342, Oct. 2, 2024; DocketFAA-2023-1377, Amdt. 91-381, 90 FR 35220, July 24, 2025]

§§ 91.328-91.330 [Reserved] § 91.331xxx

Link to an amendment published at 90 FR 35222, July 24, 2025. §§ 91.332-91.399 [Reserved]

Subpart E—Maintenance, Preventive Maintenance, andAlterations

Source:

Docket 18334, 54 FR 34311, Aug. 18, 1989, unless otherwise noted.

§ 91.401Applicability.

(a)  This subpart prescribes rules governing the maintenance, preventive maintenance, and alterations of U.S.- registered civil aircraft operating within or outside of the United States.

(b) (c)

Sections 91.405, 91.409, 91.411, 91.417, and 91.419 of this subpart do not apply to an aircraft maintained in accordance with a continuous airworthiness maintenance program as provided in part 121, 129, or §§ 91.1411or135.411(a)(2)  of this chapter.

Sections 91.405 and 91.409 of this part do not apply to an airplane inspected in accordance with part 125 of this chapter.

[Docket 18334, 54 FR 34311, Aug. 18, 1989, as amendedbyAmdt. 91-267, 66 FR 21066, Apr. 27, 2001;Amdt. 91-280, 68 FR 54560, Sept. 17, 2003]

§ 91.403 General.

(a)   The owner or operator of an aircraft is primarily responsible for maintaining that aircraft in an airworthy condition, including compliance with part 39 of this chapter.

(b)

No person may perform maintenance, preventive maintenance, or alterations on an aircraft other than as prescribed in this subpart and other applicable regulations, including part 43 of this chapter.

14 CFR91.403(b)(enhanced display)

page 101of243

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.403(c)

(c)

(d)

No person may operate an aircraft for which a manufacturer's maintenance manual or instructions for continued airworthiness has been issued that contains an airworthiness limitations section unless the mandatory replacement times, inspection intervals, and related procedures specified in that section or alternative inspection intervals and related procedures set forth in an operations specification approved by the Administrator under part 121 or 135 of this chapter or in accordance with an inspection program approved under § 91.409(e) have been complied with.

A person must not alter an aircraft based on a supplemental type certificate unless the owner or operator of the aircraft is the holder of the supplemental type certificate, or has written permission from the holder.

[Docket 18334, 54 FR 34311, Aug. 18, 1989, as amendedbyAmdt. 91-267, 66 FR 21066, Apr. 27, 2001;Amdt. 91-293, 71 FR 56005, Sept. 26, 2006]

§ 91.405 Maintenance required. Each owner or operator of an aircraft—

(a)

(b) (c) (d)

Shall have that aircraft inspected as prescribed in subpart E of this part and shall between required inspections, except as provided in paragraph (c) ofthis section, have discrepancies repaired as prescribed in part 43 of this chapter;

Shall ensure that maintenance personnel make appropriate entries in the aircraft maintenance records indicating the aircraft has been approved for return to service;

Shall have any inoperative instrument or item of equipment, permitted to be inoperative by § 91.213(d)(2) of this part, repaired, replaced, removed, or inspected at the next required inspection; and

When listed discrepancies include inoperative instruments or equipment, shall ensure that a placard has been installed as required by § 43.11 of this chapter.

§ 91.407 Operation after maintenance, preventive maintenance, rebuilding, oralteration.

(a)  No person may operate any aircraft that has undergone maintenance, preventive maintenance, rebuilding, or alteration unless—

(1)  It has been approved for return to service by a person authorized under § 43.7 of this chapter; and

(2)  The maintenance record entry required by § 43.9 or § 43.11, as applicable, of this chapter has been made.

(b) (c)

No person may carry any person (otherthan crewmembers) in an aircraft that has been maintained, rebuilt, or altered in a manner that may have appreciably changed its flight characteristics or substantially affected its operation in flight until an appropriately rated pilot with at least a private pilot certificate flies the aircraft, makes an operational check of the maintenance performed or alteration made, and logs the flight in the aircraft records.

The aircraft does not have to be flown as required by paragraph (b) of this section if, prior to flight, ground tests, inspection, or both show conclusively that the maintenance, preventive maintenance, rebuilding, or alteration has not appreciably changed the flight characteristics or substantially affected the flight operation of the aircraft.

(Approved by the Office ofManagement and Budget under control number2120-0005) 14 CFR91.407(c)(enhanceddisplay)

page 102of243

(b) (c)

(d)

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.409

§ 91.409 Inspections.

(a)  Except as provided in paragraph (c) of this section, no person may operate an aircraft unless, within the preceding 12 calendar months, it has had—

(1) (2)

An annual inspection in accordance with part 43 of this chapter and has been approved for return to service by a person authorized by § 43.7 of this chapter; or

An inspection for the issuance of an airworthiness certificate in accordance with part 21 of this chapter.

No inspection performed under paragraph (b) of this section may be substituted for any inspection required by this paragraph unless it is performed by a person authorized to perform annual inspections and is entered as an “annual” inspection in the required maintenance records.

Except as provided in paragraph (c) of this section, no person may operate an aircraft carrying any person (other than a crewmember) for hire, and no person may give flight instruction for hire in an aircraft which that person provides, unless within the preceding 100 hours of time in service the aircraft has received an annual or 100-hour inspection and been approved for return to service in accordance with part 43 of this chapter or has received an inspection for the issuance of an airworthiness certificate in accordance with part 21 of this chapter. The 100-hour limitation may be exceeded by not more than 10 hours while en

route to reach a place where the inspection can be done. The excess time used to reach a place where the inspection can be done must be included in computing the next 100 hours of time in service.

Paragraphs (a) and (b) ofthis section do not apply to—

(1) (2)

(3) (4)

An aircraft that carries a special flight permit, a current experimental airworthiness certificate, a special airworthiness certificate in the light-sport category, or provisional airworthiness certificate;

An aircraft inspected in accordance with an approved aircraft inspection program under part 125 or 135 of this chapter and so identified bythe registration number in the operations specifications of the certificate holder having the approved inspection program;

An aircraft subject to the requirements of paragraph (d) or (e) of this section; or

Turbine-powered rotorcraft when the operator elects to inspect that rotorcraft in accordance with paragraph (e) ofthis section.

Progressive inspection. Each registered owner or operator of an aircraft desiring to use a progressive inspection program must submit a written request to the responsible Flight Standards office, and shall provide—

(1) (2)

A certificated mechanic holding an inspection authorization, a certificated airframe repair station, or the manufacturer ofthe aircraft to supervise or conduct the progressive inspection;

A current inspection procedures manual available and readily understandable to pilot and maintenance personnel containing, in detail—

(i)   An explanation of the progressive inspection, including the continuity of inspection responsibility, the making of reports, and the keeping of records and technical reference material;

14 CFR91.409(d)(2)(i)(enhanceddisplay)

page 103 of243

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.409(d)(2)(ii)

(e) (f)

(ii)   An inspection schedule, specifying the intervals in hours or days when routine and detailed inspections will be performed and including instructions for exceeding an inspection interval by not more than 10 hours while en route and for changing an inspection interval because of service experience;

(iii)  Sample routine and detailed inspection forms and instructions for their use; and (iv)  Sample reports and records and instructions for their use;

(3)  Enough housing and equipment for necessary disassembly and proper inspection ofthe aircraft; and (4)   Appropriate current technical information  forthe aircraft.

The frequency and detail ofthe progressive inspection shall provide for the complete inspection ofthe aircraft within each 12 calendar months and be consistent with the manufacturer's recommendations, field service experience, and the kind of operation in which the aircraft is engaged. The progressive inspection schedule must ensure that the aircraft, at all times, will be airworthy and will conform to all applicable FAA aircraft specifications, type certificate data sheets, airworthiness directives, and other approved data. If the progressive inspection is discontinued, the owner or operator shall immediately notify the responsible Flight Standards office, in writing, of the discontinuance. After the discontinuance, the first annual inspection under § 91.409(a)(1) is due within 12 calendar months after the last complete inspection of the aircraft under the progressive inspection. The 100-hour inspection under § 91.409(b) is due within 100 hours after that complete inspection. A complete inspection of the aircraft, for the purpose of determining when the annual and 100-hour inspections are due, requires a detailed inspection ofthe aircraft and all its components in accordance with the progressive inspection. A routine inspection of the aircraft and a detailed inspection of several components is not considered to be a complete inspection.

Large airplanes (to which part 125 is not applicable), turbojet multiengine airplanes, turbopropeller-powered multiengine airplanes, and turbine-powered rotorcraft. No person may operate a large airplane, turbojet multiengine airplane, turbopropeller-powered multiengine airplane, or turbine-powered rotorcraft unless the replacement times for life-limited parts specified in the aircraft specifications, type data sheets, or other documents approved bythe Administrator are complied with and the airplane or turbine-powered rotorcraft, including the airframe, engines, propellers, rotors, appliances, survival equipment, and emergency equipment, is inspected in accordance with an inspection program selected underthe provisions of paragraph (f) ofthis section, except that, the owner or operator of a turbine-powered rotorcraft may elect to use the inspection provisions of § 91.409(a), (b), (c), or (d) in lieu of an inspection option of § 91.409(f).

Selection ofinspection program underparagraph (e) ofthis section. The registered owner or operator of each airplane or turbine-powered rotorcraft described in paragraph (e) of this section must select, identify in the aircraft maintenance records, and use one of the following programs for the inspection of the aircraft:

(1)   A continuous airworthiness inspection program that is part of a continuous airworthiness maintenance program currently in use by a person holding an air carrier operating certificate or an operating certificate issued under part 121 or 135 of this chapter and operating that make and model aircraft under part 121 of this chapter or operating that make and model under part 135 of this chapter and maintaining it under § 135.411(a)(2) of this chapter.

(2)   An approved aircraft inspection program approved under § 135.419 of this chapter and currently in use by a person holding an operating certificate issued under part 135 of this chapter.

14 CFR91.409(f)(2)(enhanceddisplay)

page 104of243

(g) (h)

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.409(f)(3)

(3) (4)

A current inspection program recommended by the manufacturer.

Any other inspection program established by the registered owner or operator ofthat airplane or turbine-powered rotorcraft and approved by the Administrator under paragraph (g) of this section. However, the Administrator may require revision of this inspection program in accordance with the provisions of § 91.415.

Each operator shall include in the selected program the name and address ofthe person responsible for scheduling the inspections required bythe program and make a copy of that program available to the person performing inspections on the aircraft and, upon request, to the Administrator.

Inspection program approvedunderparagraph (e) ofthis section. Each operator of an airplane or turbine- powered rotorcraft desiring to establish or change an approved inspection program under paragraph (f)(4) of this section must submit the program for approval to the responsible Flight Standards office. The program must be in writing and include at least the following information:

(1)

(2)

Instructions and procedures for the conduct of inspections for the particular make and model airplane or turbine-powered rotorcraft, including necessary tests and checks. The instructions and procedures must set forth in detail the parts and areas of the airframe, engines, propellers, rotors, and appliances, including survival and emergency equipment required to be inspected.

A schedule for performing the inspections that must be performed under the program expressed in terms of the time in service, calendartime, number of system operations, or any combination of these.

Changes from one inspection program to another. When an operator changes from one inspection program under paragraph (f) of this section to another, the time in service, calendar times, or cycles of operation accumulated under the previous program must be applied in determining inspection due times under the new program.

(Approved by the Office ofManagement and Budget under control number2120-0005)

[Docket 18334, 54 FR 34311, Aug. 18, 1989; Amdt. 91-211, 54 FR 41211, Oct. 5, 1989;Amdt. 91-267, 66 FR 21066, Apr. 27, 2001; Amdt. 91-282, 69 FR 44882, July27, 2004;Docket FAA-2018-0119, Amdt. 91-350, 83 FR 9171, Mar. 5, 2018; DocketFAA-2023-1377, Amdt. 91-381, 90 FR 35222, July24, 2025]

§ 91.410 [Reserved]

§ 91.411Altimeter system and altitude reporting equipment tests and inspections. (a)  No person may operate an airplane, or helicopter, in controlled airspace under IFR unless—

(1)  Within the preceding 24 calendar months, each static pressure system, each altimeter instrument, and each automatic pressure altitude reporting system has been tested and inspected and found to comply with appendices E and F of part 43 of this chapter;

(2)

Except forthe use of system drain and alternate static pressure valves, following any opening and closing ofthe static pressure system, that system has been tested and inspected and found to comply with paragraph (a), appendix E, of part 43 of this chapter; and

14 CFR91.411(a)(2) (enhanceddisplay)

page 105of243

(b)

(c) (d)

(1) (2)

(3)

(1)

(2) (3)

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.411(a)(3)

(3)

Following installation or maintenance on the automatic pressure altitude reporting system of the ATC transponder where data correspondence error could be introduced, the integrated system has been tested, inspected, and found to comply with paragraph (c), appendix E, of part 43 of this chapter.

The tests required by paragraph (a) of this section must be conducted by—

The manufacturer of the airplane, or helicopter, on which the tests and inspections are to be performed;

A certificated repair station properly equipped to perform those functions and holding—

(i) (ii)

(iii) (iv)

An instrument rating, Class I;

A limited instrument rating appropriate to the make and model of appliance to be tested; A limited rating appropriate to the test to be performed;

An airframe rating appropriate to the airplane, or helicopter, to be tested; or

A certificated mechanic with an airframe rating (static pressure system tests and inspections only).

Altimeter and altitude reporting equipment approved under Technical Standard Orders are considered to be tested and inspected as of the date of their manufacture.

No person may operate an airplane, or helicopter, in controlled airspace under IFR at an altitude above the maximum altitude at which all altimeters and the automatic altitude reporting system ofthat airplane, or helicopter, have been tested.

[Docket 18334, 54 FR 34308, Aug. 18, 1989, as amendedbyAmdt. 91-269, 66 FR 41116, Aug. 6, 2001; 72 FR 7739, Feb. 20, 2007]

§ 91.413 ATC transpondertests and inspections.

(a)  No persons may use an ATC transponder that is specified in 91.215(a), 121.345(c), or § 135.143(c) of this chapter unless, within the preceding 24 calendar months, the ATC transponder has been tested and inspected and found to comply with appendix F of part 43 of this chapter; and

(b)

(c)

Following any installation or maintenance on an ATC transponder where data correspondence error could be introduced, the integrated system has been tested, inspected, and found to comply with paragraph (c), appendix E, of part 43 of this chapter.

The tests and inspections specified in this section must be conducted by—

A certificated repair station properly equipped to perform those functions and holding—

(i) (ii)

(iii)

A radio rating, Class III;

A limited radio rating appropriate to the make and model transponder to be tested; A limited rating appropriate to the test to be performed;

A holder of a continuous airworthiness maintenance program as provided in part 121 or § 135.411(a)(2) of this chapter; or

The manufacturer of the aircraft on which the transponder to be tested is installed, if the transponder was installed by that manufacturer.

14 CFR91.413(c)(3) (enhanceddisplay)

page 106of243

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.415

[Docket 18334, 54 FR 34311, Aug. 18, 1989, as amendedbyAmdt. 91-267, 66 FR 21066, Apr. 27, 2001;Amdt. 91-269, 66 FR 41116, Aug. 6, 2001]

§ 91.415 Changes to aircraftinspectionprograms.

(a)   Wheneverthe Administrator finds that revisions to an approved aircraft inspection program under § 91.409(f)(4) or § 91.1109are necessary for the continued adequacy ofthe program, the owner or operator must, after notification bythe Administrator, make any changes in the program found to be necessary bythe Administrator.

(b) (c) (d)

The owner or operator may petition the Administratorto reconsider the notice to make any changes in a program in accordance with paragraph (a) of this section.

The petition must be filed with the Executive Director, Flight Standards Service within 30 days after the certificate holder or fractional ownership program manager receives the notice.

Except in the case of an emergency requiring immediate action in the interest of safety, the filing of the petition stays the notice pending a decision by the Administrator.

[Docket 18334, 54 FR 34311, Aug. 18, 1989, as amendedbyAmdt. 91-280, 68 FR 54560, Sept. 17, 2003;Docket FAA-2018-0119, Amdt. 91-350, 83 FR 9171, Mar. 5, 2018]

§ 91.417Maintenance records.

(a)   Except for work performed in accordance with §§ 91.411 and 91.413, each registered owner or operator shall keep the following records for the periods specified in paragraph (b) ofthis section:

(1)  Records of the maintenance, preventive maintenance, and alteration and records ofthe 100-hour, annual, progressive, and other required or approved inspections, as appropriate, for each aircraft (including the airframe) and each engine, propeller, rotor, and appliance of an aircraft. The records must include—

(i) (ii)

(iii)

A description (or reference to data acceptable to the Administrator) of the work performed; and The date of completion of the work performed; and

The signature, and certificate number ofthe person approving the aircraft for return to service.

(2)  Records containing the following information:

(i) (ii)

(iii) (iv)

(v)

The total time in service of the airframe, each engine, each propeller, and each rotor.

The current status of life-limited parts of each airframe, engine, propeller, rotor, and appliance.

The time since last overhaul of all items installed on the aircraft which are required to be overhauled on a specified time basis.

The current inspection status of the aircraft, including the time since the last inspection required by the inspection program under which the aircraft and its appliances are maintained.

The current status of applicable airworthiness directives (AD) including, for each, the method of compliance, the AD number and revision date. If the AD involves recurring action, the time and date when the next action is required.

14 CFR91.417(a)(2)(v) (enhanceddisplay)

page 107of243

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.417(a)(2)(vi)

(b)

(c) (d)

(vi)  Copies of the forms prescribed by § 43.9(d) of this chapter for each major alteration to the airframe and currently installed engines, rotors, propellers, and appliances.

The owner or operator shall retain the following records for the periods prescribed:

(1)   The records specified in paragraph (a)(1) ofthis section shall be retained until the work is repeated or superseded by other work or for 1 year after the work is performed.

(2)   The records specified in paragraph (a)(2) ofthis section shall be retained and transferred with the aircraft at the time the aircraft is sold.

(3)   A list of defects furnished to a registered owner or operator under § 43.11 of this chapter shall be retained until the defects are repaired and the aircraft is approved for return to service.

The owner or operator shall make all maintenance records required to be kept by this section available for inspection bythe Administrator or any authorized representative of the National Transportation Safety Board (NTSB). In addition, the owner or operator shall present Form 337 described in paragraph (d) of this section for inspection upon request of any law enforcement officer.

When a fuel tank is installed within the passenger compartment or a baggage compartment pursuant to part 43 of this chapter, a copy of FAA Form 337 shall be kept on board the modified aircraft by the owner or operator.

(Approved by the Office ofManagement and Budget under control number2120-0005)

[Docket 18334, 54 FR 34311, Aug. 18, 1989, as amendedbyAmdt. 91-311, 75 FR 5223, Feb. 1, 2010;Amdt. 91-323, 76 FR 39260, July 6, 2011; Docket FAA-2023-1377, Amdt. 91-381, 90 FR 35222, July24, 2025]

§ 91.419 Transfer ofmaintenance records.

Any owner or operator who sells a U.S.-registered aircraft shall transfer to the purchaser, at the time of sale, the following records of that aircraft, in plain language form or in coded form at the election of the purchaser, if the coded form provides for the preservation and retrieval of information in a manner acceptable to the Administrator:

(a) (b)

The records specified in § 91.417(a)(2).

The records specified in § 91.417(a)(1) which are not included in the records covered by paragraph (a) of this section, except that the purchaser may permit the seller to keep physical custody of such records. However, custody of records by the seller does not relieve the purchaser ofthe responsibility under § 91.417(c) to make the records available for inspection by the Administrator or any authorized representative of the National Transportation Safety Board (NTSB).

§ 91.421 Rebuiltengine maintenance records.

(a)  The owner or operator may use a new maintenance record, without previous operating history, for an aircraft engine rebuilt by the manufacturer or by an agency approved by the manufacturer.

(b)

Each manufacturer or agency that grants zero time to an engine rebuilt by it shall enter in the new record— (1)   A signed statement ofthe date the engine was rebuilt;

(2)  Each change made as required by airworthiness directives; and

14 CFR91.421(b)(2)(enhanceddisplay)

page 108 of243

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.421(b)(3)

(3)

Each change made in compliance with manufacturer's service bulletins, if the entry is specifically requested in that bulletin.

(c)

For the purposes ofthis section, a rebuilt engine is a used engine that has been completely disassembled, inspected, repaired as necessary, reassembled, tested, and approved in the same manner and to the same tolerances and limits as a new engine with either new or used parts. However, all parts used in it must conform to the production drawing tolerances and limits for new parts or be of approved oversized or undersized dimensions for a new engine.

§§ 91.423-91.499 [Reserved]

Subpart F—Large and Turbine-Powered Multiengine Airplanes and Fractional Ownership Program Aircraft

Source:

Docket 18334, 54 FR 34314, Aug. 18, 1989, unless otherwise noted.

§ 91.501 Applicability.

(a)   This subpart prescribes operating rules, in addition to those prescribed in other subparts of this part, governing the operation of large airplanes of U.S. registry, turbojet-powered multiengine civil airplanes of U.S. registry, and fractional ownership program aircraft of U.S. registry that are operating under subpart K of this part in operations not involving common carriage. The operating rules in this subpart do not apply to those aircraft when they are required to be operated under parts 121, 125, 129, 135, and 137 of this chapter. (Section 91.409 prescribes an inspection program for large and for turbine-powered (turbojet and turboprop) multiengine airplanes and turbine-powered rotorcraft of U.S. registry when they are operated under this part or part 129 or 137.)

(b)

Operations that may be conducted under the rules in this subpart instead ofthose in parts 121, 129, 135, and 137 of this chapter when common carriage is not involved, include—

(1)  Ferry or training flights;

(2) (3) (4) (5)

Aerial work operations such as aerial photography or survey, or pipeline patrol, but not including fire fighting operations;

Flights forthe demonstration of an airplane to prospective customers when no charge is made except for those specified in paragraph (d) of this section;

Flights conducted by the operator of an airplane for his personal transportation, or the transportation of his guests when no charge, assessment, or fee is made for the transportation;

Carriage of officials, employees, guests, and property of a company on an airplane operated by that company, or the parent or a subsidiary ofthe company or a subsidiary ofthe parent, when the carriage is within the scope of, and incidental to, the business ofthe company (other than transportation by air) and no charge, assessment or fee is made for the carriage in excess of the cost of owning, operating, and maintaining the airplane, except that no charge of any kind may be made for the carriage of a guest of a company, when the carriage is not within the scope of, and incidental to, the business of that company;

14 CFR91.501(b)(5)(enhanceddisplay)

page 109 of243

(c) (d)

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.501(b)(6)

(6) (7)

(8) (9)

(10)

The carriage of company officials, employees, and guests ofthe company on an airplane operated under a time sharing, interchange, or joint ownership agreement as defined in paragraph (c) ofthis section;

The carriage of property (other than mail) on an airplane operated by a person in the furtherance of a business or employment (other than transportation by air) when the carriage is within the scope of, and incidental to, that business or employment and no charge, assessment, or fee is made for the carriage otherthan those specified in paragraph (d) ofthis section;

The carriage on an airplane of an athletic team, sports group, choral group, or similar group having a common purpose or objective when there is no charge, assessment, or fee of any kind made by any person forthat carriage; and

The carriage of persons on an airplane operated by a person in the furtherance of a business other than transportation by air for the purpose of selling them land, goods, or property, including franchises or distributorships, when the carriage is within the scope of, and incidental to, that business and no charge, assessment, or fee is made for that carriage.

Any operation identified in paragraphs (b)(1) through (b)(9) of this section when conducted— (i)   By a fractional ownership program manager, or

(ii)   By a fractional owner in a fractional ownership program aircraft operated under subpart K of

this part, except that a flight under ajoint ownership arrangement under paragraph (b)(6) ofthis section may not be conducted. For a flight under an interchange agreement under paragraph (b)(6) of this section, the exchange of equal time for the operation must be properly accounted for as part of the total hours associated with the fractional owner's share of ownership.

As used in this section—

(1) (2)

(3)

A time sharing agreement means an arrangement whereby a person leases his airplane with flight crew to another person, and no charge is made forthe flights conducted under that arrangement other than those specified in paragraph (d) of this section;

An interchange agreement means an arrangement whereby a person leases his airplane to another person in exchange for equal time, when needed, on the other person's airplane, and no charge, assessment, or fee is made, except that a charge may be made not to exceed the difference between the cost of owning, operating, and maintaining the two airplanes;

A joint ownership agreement means an arrangement whereby one of the registered joint owners of an airplane employs and furnishes the flight crew forthat airplane and each of the registered joint owners pays a share ofthe charge specified in the agreement.

The following may be charged, as expenses of a specific flight, for transportation as authorized by paragraphs (b) (3) and (7) and (c)(1) ofthis section:

(1) (2) (3) (4) (5)

Fuel, oil, lubricants, and other additives.

Travel expenses of the crew, including food, lodging, and ground transportation. Hangar and tie-down costs away from the aircraft's base of operation. Insurance obtained for the specific flight.

Landing fees, airport taxes, and similar assessments.

14 CFR91.501(d)(5)(enhanceddisplay)

page 110 of243

(b)

(c)

(d)

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.501(d)(6)

(6) (7)

Customs, foreign permit, and similar fees directly related to the flight. In flight food and beverages.

(8)   Passenger ground transportation.

(9) (10)

Flight planning and weather contract services.

An additional charge equal to 100 percent of the expenses listed in paragraph (d)(1) ofthis section.

[Docket 18334, 54 FR 34314, Aug. 18, 1989, as amendedbyAmdt. 91-280, 68 FR 54560, Sept. 17, 2003]

§ 91.503 Flyingequipmentand  operatinginformation.

(a)   The pilot in command of an airplane shall ensure that the following flying equipment and aeronautical charts and data, in current and appropriate form, are accessible for each flight at the pilot station of the airplane:

(1) (2)

A flashlight having at least two size “D” cells, or the equivalent, that is in good working order. A cockpit checklist containing the procedures required by paragraph (b) of this section.

(3)   Pertinent aeronautical charts.

(4)

(5)

For IFR, VFR over-the-top, or night operations, each pertinent navigational en route, terminal area, and approach and letdown chart.

In the case of multiengine airplanes, one-engine inoperative climb performance data.

Each cockpit checklist must contain the following procedures and shall be used by the flight crewmembers when operating the airplane:

(1)   Before starting engines. (2)   Before takeoff.

(3)  Cruise.

(4)  Before landing. (5)   After landing.

(6)  Stopping engines. (7)  Emergencies.

Each emergency cockpit checklist procedure required by paragraph (b)(7) ofthis section must contain the following procedures, as appropriate:

(1) (2)

Emergency operation of fuel, hydraulic, electrical, and mechanical systems. Emergency operation of instruments and controls.

(3)  Engine inoperative procedures.

(4)

Any other procedures necessary for safety.

The equipment, charts, and data prescribed in this section shall be used by the pilot in command and other members of the flight crew, when pertinent.

14 CFR91.503(d) (enhanceddisplay)

page 111of243

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.505

§ 91.505 Familiaritywith  operatinglimitations and emergency equipment.

(a)  Each pilot in command of an airplane shall, before beginning a flight, become familiar with the Airplane Flight Manual forthat airplane, if one is required, and with any placards, listings, instrument markings, or any combination thereof, containing each operating limitation prescribed for that airplane by the Administrator, including those specified in § 91.9(b).

(b)

Each required member of the crew shall, before beginning a flight, become familiar with the emergency equipment installed on the airplane to which that crewmember is assigned and with the procedures to be followed for the use ofthat  equipment in an emergency situation.

§ 91.507Equipment requirements: Over-the-topor night VFR operations.

No person may operate an airplane over-the-top or at night under VFR unless that airplane is equipped with the instruments and equipment required for IFR operations under § 91.205(d) and one electric landing light for night operations. Each required instrument and item of equipment must be in operable condition.

§ 91.509 Survivalequipmentfor  overwateroperations.

(a)   No person may take off an airplane for a flight over water more than 50 nautical miles from the nearest shore unless that airplane is equipped with a life preserver or an approved flotation means for each occupant ofthe airplane.

(b)

(c)

(d)

(e) (f)

Except as provided in paragraph (c) of this section, no person may take off an airplane for flight over water more than 30 minutes flying time or 100 nautical miles from the nearest shore, whichever is less, unless it has on board the following survival equipment:

(1)  A life preserver, equipped with an approved survivor locator light, for each occupant of the airplane.

(2)  Enough liferafts (each equipped with an approved survival locator light) of a rated capacity and buoyancy to accommodate the occupants ofthe airplane.

(3)   At least one pyrotechnic signaling device for each liferaft.

(4)  One self-buoyant, water-resistant, portable emergency radio signaling device that is capable of transmission on the appropriate emergency frequency or frequencies and not dependent upon the airplane power supply.

(5)   A lifeline stored in accordance with § 25.1411(g) of this chapter.

A fractional ownership program manager under subpart K of this part may apply for a deviation from paragraphs (b)(2) through (5) of this section for a particular overwater operation or the Administrator may amend the management specifications to require the carriage of all or any specific items ofthe equipment listed in paragraphs (b)(2) through (5) ofthis section.

The required life rafts, life preservers, and signaling devices must be installed in conspicuously marked locations and easily accessible in the event of a ditching without appreciable time for preparatory procedures.

A survival kit, appropriately equipped for the route to be flown, must be attached to each required life raft.

As used in this section, the term shore means that area of the land adjacent to the waterthat is above the high water mark and excludes land areas that are intermittently under water.

14 CFR91.509(f)(enhanceddisplay)

page 112of243

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

[Docket 18334, 54 FR 34314, Aug. 18, 1989, as amendedbyAmdt. 91-280, 68 FR 54561, Sept. 17, 2003]

14 CFR91.511

§ 91.511 Communication and navigation equipment for overwater operations.

(a)  Except as provided in paragraphs (c), (d), and (f) of this section, no person operating under this subpart may take off an airplane for a flight over water more than 30 minutes flying time or 100 nautical miles from the nearest shore unless it has at least the following operable equipment:

(1)

(2)

Radio communication equipment appropriate to the facilities to be used and able to transmit to, and receive from, at least one communication facility from any place along the route:

(i)   Two transmitters. (ii)   Two microphones.

(iii)  Two headsets or one headset and one speaker. (iv)  Two independent receivers.

Appropriate electronic navigational equipment consisting of at least two independent electronic navigation units capable of providing the pilot with the information necessary to navigate the airplane within the airspace assigned by air traffic control. However, a receiverthat can receive both communications and required navigational signals may be used in place of a separate communications receiver and a separate navigational signal receiver or unit.

(b) (c)

(d)

(e) (f)

For the purposes of paragraphs (a)(1)(iv) and (a)(2) ofthis section, a receiver or electronic navigation unit is independent if the function of any part of it does not depend on the functioning of any part of another receiver or electronic navigation unit.

Notwithstanding the provisions of paragraph (a) ofthis section, a person may operate an airplane on which no passengers are carried from a place where repairs or replacement cannot be made to a place where they can be made, if not more than one of each of the dual items of radio communication and navigational equipment specified in paragraphs (a)(1) (i) through (iv) and (a)(2) of this section malfunctions or becomes inoperative.

Notwithstanding the provisions of paragraph (a) ofthis section, when both VHF and HF communications equipment are required for the route and the airplane has two VHF transmitters and two VHF receivers for communications, only one HF transmitter and one HF receiver is required for communications.

As used in this section, the term shore means that area of the land adjacent to the water which is above the high-water mark and excludes land areas which are intermittently under water.

Notwithstanding the requirements in paragraph (a)(2) of this section, a person may operate in the Gulf of Mexico, the Caribbean Sea, and the Atlantic Ocean west of a line which extends from 44°47′00″ N / 67°00′00″ W to 39°00′00″ N / 67°00′00″ W to 38°30′00″ N / 60°00′00″ W south along the 60°00′00″ W longitude line to the point where the line intersects with the northern coast of South America, when:

(1) (2)

A single long-range navigation system is installed, operational, and appropriate for the route; and

Flight conditions and the aircraft's capabilities are such that no more than a 30-minute gap in two- way radio very high frequency communications is expected to exist.

[Docket 18334, 54 FR 34314, Aug. 18, 1989, as amendedbyAmdt. 91-249, 61 FR 7190, Feb. 26, 1996;Amdt. 91-296, 72 FR 31679, June 7, 2007; DocketFAA-2022-1355, Amdt. 91-366, 87 FR 75846, Dec. 9, 2022]

14 CFR91.511(f)(2)(enhanceddisplay)

page 113 of243

(b) (c)

(d) (e) (f)

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.513

§ 91.513 Emergency equipment.

(a)   No person may operate an airplane unless it is equipped with the emergency equipment listed in this section.

Each item of equipment—

(1)

(2) (3) (4)

Must be inspected in accordance with § 91.409 to ensure its continued serviceability and immediate readiness for its intended purposes;

Must be readily accessible to the crew;

Must clearly indicate its method of operation; and

When carried in a compartment or container, must have that compartment or container marked as to contents and date of last inspection.

Hand fire extinguishers must be provided for use in crew, passenger, and cargo compartments in accordance with the following:

(1) (2) (3)

(4)

The type and quantity of extinguishing agent must be suitable for the kinds of fires likely to occur in the compartment where the extinguisher is intended to be used.

At least one hand fire extinguisher must be provided and located on or near the flight deck in a place that is readily accessible to the flight crew.

At least one hand fire extinguisher must be conveniently located in the passenger compartment of each airplane accommodating more than six but less than 31 passengers, and at least two hand fire extinguishers must be conveniently located in the passenger compartment of each airplane accommodating more than 30 passengers.

Hand fire extinguishers must be installed and secured in such a manner that they will not interfere with the safe operation ofthe airplane or adversely affect the safety of the crew and passengers. They must be readily accessible and, unless the locations ofthe fire extinguishers are obvious, their stowage provisions must be properly identified.

First aid kits for treatment of injuries likely to occur in flight or in minor accidents must be provided. Each airplane accommodating more than 19 passengers must be equipped with a crash axe.

Each passenger-carrying airplane must have a portable battery-powered megaphone or megaphones readily accessible to the crewmembers assigned to direct emergency evacuation, installed as follows:

(1) (2)

One megaphone on each airplane with a seating capacity of more than 60 but less than 100 passengers, at the most rearward location in the passenger cabin where it would be readily accessible to a normal flight attendant seat. However, the Administrator may grant a deviation from the requirements of this subparagraph ifthe Administrator finds that a different location would be more useful for evacuation of persons during an emergency.

On each airplane with a seating capacity of 100 or more passengers, one megaphone installed at the forward end and one installed at the most rearward location where it would be readily accessible to

a normal flight attendant seat.

14 CFR91.513(f)(2)(enhanceddisplay)

page 114of243

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.515

§ 91.515 Flightaltitude rules.

(a)  Notwithstanding § 91.119, and except as provided in paragraph (b) of this section, no person may operate an airplane under VFR at less than—

(1)

(2)

One thousand feet above the surface, or 1,000 feet from any mountain, hill, or other obstruction to flight, for day operations; and

The altitudes prescribed in § 91.177, for night operations.

(b)

This section does not apply— (1)  During takeoff or landing;

(2) (3)

When a different altitude is authorized by a waiver to this section under subpart J of this part; or

When a flight is conducted under the special VFR weather minimums of § 91.157 with an appropriate clearance from ATC.

§ 91.517 Passenger information.

(a)   Except as provided in paragraph (b) ofthis section, no person may operate an airplane carrying passengers unless it is equipped with signs that are visible to passengers and flight attendants to notify them when smoking is prohibited and when safety belts must be fastened.

(1) (2)

The signs that notify when safety belts must be fastened must be so constructed that the crew can turn them on and off.

The signs that prohibit smoking and signs that notify when safety belts must be fastened must be illuminated during airplane movement on the surface, for each takeoff, for each landing, and when otherwise considered to be necessary by the pilot in command.

(b)

(c) (d) (e)

The pilot in command of an airplane that is not required, in accordance with applicable aircraft and equipment requirements of this chapter, to be equipped as provided in paragraph (a) ofthis section shall ensure that the passengers are notified orally each time that it is necessary to fasten their safety belts and when smoking is prohibited.

If passenger information signs are installed, no passenger or crewmember may smoke while any “no smoking” sign is lighted nor may any passenger or crewmember smoke in any lavatory.

Each passenger required by § 91.107(a)(3) to occupy a seat or berth shall fasten his or her safety belt about him or her and keep it fastened while any “fasten seat belt” sign is lighted.

Each passenger shall comply with instructions given him or her by crewmembers regarding compliance with paragraphs (b), (c), and (d) of this section.

[Docket 26142, 57 FR 42672, Sept. 15, 1992, as amendedby Docket FAA-2024-2052;Amdt. 91-377, 89 FR 68100, Aug. 23, 2024]

§ 91.519 Passengerbriefing.

(a)   Before each takeoff the pilot in command of an airplane carrying passengers shall ensure that all passengers have been orally briefed on—

14 CFR91.519(a)(enhanceddisplay)

page 115of243

(b)

(c) (d)

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.519(a)(1)

(1) (2)

(3) (4) (5)

(6)

Smoking. Each passenger shall be briefed on when, where, and under what conditions smoking is prohibited. This briefing shall include a statement, as appropriate, that the Federal Aviation Regulations require passenger compliance with lighted passenger information signs and no smoking placards, prohibit smoking in lavatories, and require compliance with crewmember instructions with regard to these items;

Use ofsafetybelts and shoulderharnesses. Each passenger shall be briefed on when, where, and under what conditions it is necessaryto have his or her safety belt and, if installed, his or her shoulder harness fastened about him or her. This briefing shall include a statement, as appropriate, that Federal Aviation Regulations require passenger compliance with the lighted passenger sign and/or crewmember instructions with regard to these items;

Location and means for opening the passenger entry door and emergency exits; Location of survival equipment;

Ditching procedures and the use of flotation equipment required under § 91.509 for a flight over water; and

The normal and emergency use of oxygen equipment installed on the airplane.

The oral briefing required by paragraph (a) of this section shall be given by the pilot in command or a member of the crew, but need not be given when the pilot in command determines that the passengers are familiar with the contents of the briefing. It may be supplemented by printed cards for the use of each passenger containing—

(1) (2)

A diagram of, and methods of operating, the emergency exits; and Other instructions necessary for use of emergency equipment.

Each card used under paragraph (b) must be carried in convenient locations on the airplane for the use of each passenger and must contain information that is pertinent only to the type and model airplane on which it is used.

For operations under subpart K of this part, the passenger briefing requirements of § 91.1035 apply, instead of the requirements of paragraphs (a) through (c) ofthis section.

[Docket 18334, 54 FR 34314, Aug. 18, 1989, as amendedbyAmdt. 91-231, 57 FR 42672, Sept. 15, 1992;Amdt. 91-280, 68 FR 54561, Sept. 17, 2003]

§ 91.521 Shoulder harness.

(a)   No person may operate a transport category airplane that was type certificated after January 1, 1958, unless it is equipped at each seat at a flight deck station with a combined safety belt and shoulder harness that meets the applicable requirements specified in § 25.785 of this chapter, except that—

(1) (2)

Shoulder harnesses and combined safety belt and shoulder harnesses that were approved and installed before March 6, 1980, may continue to be used; and

Safety belt and shoulder harness restraint systems may be designed to the inertia load factors established underthe certification basis of the airplane.

14 CFR91.521(a)(2)(enhanceddisplay)

page 116of243

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.521(b)

(b)

No person may operate a transport category airplane unless it is equipped at each required flight attendant seat in the passenger compartment with a combined safety belt and shoulder harness that meets the applicable requirements specified in § 25.785 of this chapter, except that—

(1)  Shoulder harnesses and combined safety belt and shoulder harnesses that were approved and installed before March 6, 1980, may continue to be used; and

(2)   Safety belt and shoulder harness restraint systems may be designed to the inertia load factors established underthe certification basis of the airplane.

§ 91.523 Carry-on baggage.

No pilot in command of an airplane having a seating capacity of more than 19 passengers may permit a passenger to stow baggage aboard that airplane except—

(a) (b)

In a suitable baggage or cargo storage compartment, or as provided in § 91.525; or

Under a passenger seat in such a way that it will not slide forward under crash impacts severe enough to induce the ultimate inertia forces specified in § 25.561(b)(3) of this chapter, orthe requirements ofthe regulations under which the airplane was type certificated. Restraining devices must also limit sideward motion of under-seat baggage and be designed to withstand crash impacts severe enough to induce sideward forces specified in § 25.561(b)(3) of this chapter.

§ 91.525 Carriage ofcargo.

(a)  No pilot in command may permit cargo to be carried in any airplane unless—

(1)  It is carried in an approved cargo rack, bin, or compartment installed in the airplane; (2)  It is secured by means approved by the Administrator; or

(3)   It is carried in accordance with each of the following:

(i)   It is properly secured by a safety belt or othertiedown having enough strength to eliminate the possibility of shifting under all normally anticipated flight and ground conditions.

(ii)   It is packaged or covered to avoid possible injuryto passengers.

(iii)   It does not impose any load on seats or on the floor structure that exceeds the load limitation for those components.

(iv)   It is not located in a position that restricts the access to or use of any required emergency or regular exit, orthe use ofthe aisle between the crew and the passenger compartment.

(v)  It is not carried directly above seated passengers.

(b)  When cargo is carried in cargo compartments that are designed to require the physical entry of a crewmember to extinguish any fire that may occur during flight, the cargo must be loaded so as to allow a crewmember to effectively reach all parts ofthe compartment with the contents of a hand fire extinguisher.

14 CFR91.525(b) (enhanceddisplay)

page 117of243

(b)

(c) (d)

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.527

§ 91.527Operating in icingconditions.

(a)  No pilot may take off an airplane that has frost, ice, or snow adhering to any propeller, windshield, stabilizing or control surface; to a powerplant installation; or to an airspeed, altimeter, rate of climb, or flight attitude instrument system or wing, except that takeoffs may be made with frost under the wing in the area ofthe fuel tanks if authorized by the FAA.

No pilot may fly under IFR into known or forecast light or moderate icing conditions, or under VFR into known light or moderate icing conditions, unless—

(1)

(2)

The aircraft has functioning deicing or anti-icing equipment protecting each rotor blade, propeller, windshield, wing, stabilizing or control surface, and each airspeed, altimeter, rate of climb, or flight attitude instrument system;

The airplane has ice protection provisions that meet section 34 of Special Federal Aviation Regulation No. 23; or

(3)   The airplane meets transport category airplane type certification provisions, including the requirements for certification for flight in icing conditions.

Except for an airplane that has ice protection provisions that meet the requirements in section 34 of Special Federal Aviation Regulation No. 23, orthose for transport category airplane type certification, no pilot may fly an airplane into known or forecast severe icing conditions.

If current weather reports and briefing information relied upon by the pilot in command indicate that the forecast icing conditions that would otherwise prohibit the flight will not be encountered during the flight because of changed weather conditions since the forecast, the restrictions in paragraphs (b) and (c) of this section based on forecast conditions do not apply.

[Docket 18334, 54 FR 34314, Aug. 18, 1989, as amendedbyAmdt. 91-310, 74 FR 62696, Dec. 1, 2009]

§ 91.529 Flight engineer requirements.

(a)   No person may operate the following airplanes without a flight crewmember holding a current flight engineer certificate:

(1) (2)

An airplane for which a type certificate was issued before January 2, 1964, having a maximum certificated takeoff weight of more than 80,000 pounds.

An airplane type certificated after January 1, 1964, for which a flight engineer is required bythe type certification requirements.

(b)

No person may serve as a required flight engineer on an airplane unless, within the preceding 6 calendar months, that person has had at least 50 hours offlight  time as a flight engineer on that type airplane or has been checked by the Administrator on that type airplane and is found to be familiar and competent with all essential current information and operating procedures.

§ 91.531 Second incommand requirements.

(a)   Except as provided in paragraph (b) ofthis section, no person may operate the following airplanes without a pilot designated as second in command:

(1)   Any airplane that is type certificated for more than one required pilot.

14 CFR91.531(a)(1)(enhanceddisplay)

page 118 of243

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

(2)  Any large airplane.

(3)   Any commuter category airplane.

14 CFR91.531(a)(2)

(b)

(c)

A person may operate the following airplanes without a pilot designated as second in command: (1)   Any airplane certificated for operation with one pilot.

(2)   A large airplane or turbojet-powered multiengine airplane that holds a special airworthiness certificate, if:

(i)   The airplane was originally designed with only one pilot station; or

(ii)   The airplane was originally designed with more than one pilot station, but single pilot operations were permitted by the airplane flight manual or were otherwise permitted by a branch of the United States Armed Forces or the armed forces of a foreign contracting State to the Convention on International Civil Aviation.

No person may designate a pilot to serve as second in command, nor may any pilot serve as second in command, of an airplane required under this section to have two pilots unless that pilot meets the qualifications for second in command prescribed in § 61.55 of this chapter.

[Docket FAA-2016-6142, Amdt. 91-351, 83 FR 30282, June 27, 2018]

§ 91.533 Flight attendant requirements.

(a)   No person may operate an airplane unless at least the following number of flight attendants are on board the airplane:

(1)  For airplanes having more than 19 but less than 51 passengers on board, one flight attendant. (2)  For airplanes having more than 50 but less than 101 passengers on board, two flight attendants.

(3)   For airplanes having more than 100 passengers on board, two flight attendants plus one additional flight attendant for each unit (or part of a unit) of 50 passengers above 100.

(b)

No person may serve as a flight attendant on an airplane when required by paragraph (a) of this section unless that person has demonstrated to the pilot in command familiarity with the necessary functions to be performed in an emergency or a situation requiring emergency evacuation and is capable of using the emergency equipment installed on that airplane.

§ 91.535 Stowage offood, beverage, andpassenger service equipmentduring aircraft movement onthe surface, takeoff, and landing.

(a) (b) (c) (d)

No operator may move an aircraft on the surface, take off, or land when any food, beverage, or tableware furnished by the operator is located at any passenger seat.

No operator may move an aircraft on the surface, take off, or land unless each food and beverage tray and seat back tray table is secured in its stowed position.

No operator may permit an aircraft to move on the surface, take off, or land unless each passenger serving cart is secured in its stowed position.

No operator may permit an aircraft to move on the surface, take off, or land unless each movie screen that extends into the aisle is stowed.

14 CFR91.535(d) (enhanceddisplay)

page 119 of243

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.535(e)

(e)

Each passenger shall comply with instructions given by a crewmember with regard to compliance with this section.

[Docket 26142, 57 FR 42672, Sept. 15, 1992] §§ 91.536-91.599 [Reserved]

Subpart G—Additional Equipment and OperatingRequirements for Large and Transport Category Aircraft

Source:

Docket 18334, 54 FR 34318, Aug. 18, 1989, unless otherwise noted.

§ 91.601Applicability.

This subpart applies to operation of large and transport category U.S.-registered civil aircraft.

§ 91.603 Aural speedwarning device.

No person may operate a transport category airplane in air commerce unless that airplane is equipped with an aural speed warning device that complies with § 25.1303(c)(1).

§ 91.605  Transportcategorycivilairplaneweight  limitations.

(a)   No person may take off anytransport category airplane (other than a turbine-engine-powered airplane certificated after September 30, 1958) unless—

(1) (2) (3) (4)

The takeoff weight does not exceed the authorized maximum takeoffweight  for the elevation ofthe airport of takeoff;

The elevation of the airport of takeoff is within the altitude range for which maximum  takeoffweights have been determined;

Normal consumption of fuel and oil in flight to the airport of intended landing will leave a weight on arrival not in excess ofthe authorized maximum landing weight for the elevation of that airport; and

The elevations ofthe airport of intended landing and of all specified alternate airports are within the altitude range for which the maximum landing weights have been determined.

(b)

No person may operate a turbine-engine-powered transport category airplane certificated after September 30, 1958, contrary to the Airplane Flight Manual, or take off that airplane unless—

(1) (2)

The takeoff weight does not exceed the takeoff weight specified in the Airplane Flight Manual for the elevation of the airport and for the ambient temperature existing at the time of takeoff;

Normal consumption of fuel and oil in flight to the airport of intended landing and to the alternate airports will leave a weight on arrival not in excess of the landing weight specified in the Airplane Flight Manual forthe elevation of each ofthe airports involved and forthe ambient temperatures expected at the time of landing;

14 CFR91.605(b)(2) (enhanceddisplay)

page 120 of243

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.605(b)(3)

(3)

(4)

The takeoff weight does not exceed the weight shown in the Airplane Flight Manual to correspond with the minimum distances required for takeoff, considering the elevation ofthe airport, the runway to be used, the effective runway gradient, the ambient temperature and wind component at the time of takeoff, and, if operating limitations exist for the minimum distances required for takeoff from wet runways, the runway surface condition (dry or wet). Wet runway distances associated with grooved or porous friction course runways, if provided in the Airplane Flight Manual, may be used only for runways that are grooved or treated with a porous friction course (PFC) overlay, and that the

operator determines are designed, constructed, and maintained in a manner acceptable to the Administrator.

Where the takeoff distance includes a clearway, the clearway distance is not greaterthan one-half of—

(i)   The takeoff run, in the case of airplanes certificated after September 30, 1958, and before August 30, 1959; or

(ii)   The runway length, in the case of airplanes certificated after August 29, 1959.

(c)

No person may take off a turbine-engine-powered transport category airplane certificated after August 29, 1959, unless, in addition to the requirements of paragraph (b) of this section—

(1) (2) (3)

The accelerate-stop distance is no greater than the length ofthe runway plus the length ofthe stopway (if present); and

The takeoff distance is no greater than the length ofthe runway plus the length of the clearway (if present); and

The takeoff run is no greater than the length of the runway.

[Docket 18334, 54 FR 34318, Aug. 18, 1989, as amendedbyAmdt. 91-256, 63 FR 8321, Feb. 18, 1998]

§ 91.607 Emergencyexits for airplanes carryingpassengers forhire.

(a)  Notwithstanding any other provision of this chapter, no person may operate a large airplane (type certificated underthe Civil Air Regulations effective before April 9, 1957) in passenger-carrying operations for hire, with more than the number of occupants—

(1) (2)

Allowed under Civil Air Regulations § 4b.362 (a), (b), and (c) as in effect on December 20, 1951; or

Approved under Special Civil Air Regulations SR-387, SR-389, SR-389A, or SR-389B, or under this section as in effect.

14 CFR91.607(a)(2)(enhanceddisplay)

page 121of243

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.607(b)

However, an airplane type listed in the following table may be operated with up to the listed number of occupants (including crewmembers) and the corresponding number of exits (including emergency exits and doors) approved for the emergency exit of passengers or with an occupant-exit configuration approved under paragraph (b) or (c) ofthis section.

Airplane            Maximum number of occupants type                   including all crewmembers

B-307 B-377 C-46 CV-240

CV-340 and CV-440 DC-3

DC-3 (Super) DC-4

DC-6 DC-6B L-18

L-049, L-649, L-749

L-1049 series M-202 M-404 Viscount 700 series

61 96 67

53 53

35 39 86 87 112 17 87

96

53 53 53

Corresponding number ofexits authorized for passenger use

4 9 4 6 6

4 5 5 7

11 3 7

9

6 7 7

(b)

Occupants in addition to those authorized under paragraph (a) of this section may be carried as follows:

(1)  For each additional floor-level exit at least 24 inches wide by 48 inches high, with an unobstructed 20-inch-wide access aisleway between the exit and the main passenger aisle, 12 additional occupants.

(2)   For each additional window exit located over a wing that meets the requirements ofthe airworthiness standards under which the airplane was type certificated or that is large enough to inscribe an ellipse 19 × 26 inches, eight additional occupants.

(3)   For each additional window exit that is not located over a wing but that otherwise complies with paragraph (b)(2) of this section, five additional occupants.

(4)   For each airplane having a ratio (as computed from the table in paragraph (a) of this section) of maximum number of occupants to number of exits greater than 14:1, and for each airplane that does not have at least one full-size, door-type exit in the side ofthe fuselage in the rear part ofthe cabin, the first additional exit must be a floor-level exit that complies with paragraph (b)(1) ofthis

14 CFR91.607(b)(4)(enhanceddisplay)

page 122of243

(c) (d)

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.607(c)

section and must be located in the rear part ofthe cabin on the opposite side ofthe fuselage from the main entrance door. However, no person may operate an airplane under this section carrying more than 115 occupants unless there is such an exit on each side ofthe fuselage in the rear part of the cabin.

No person may eliminate any approved exit except in accordance with the following:

(1) (2)

(3) (4)

The previously authorized maximum number of occupants must be reduced by the same number of additional occupants authorized for that exit under this section.

Exits must be eliminated in accordance with the following priority schedule: First, non-over-wing window exits; second, over-wing window exits; third, floor-level exits located in the forward part of the cabin; and fourth, floor-level exits located in the rear ofthe cabin.

At least one exit must be retained on each side of the fuselage regardless of the number of occupants.

No person may remove any exit that would result in a ratio of maximum number of occupants to approved exits greaterthan 14:1.

This section does not relieve any person operating under part 121 of this chapter from complying with § 121.291.

§ 91.609 Flight datarecorders and cockpitvoice recorders.

(a)   No holder of an air carrier operating certificate or an operating certificate may conduct any operation under this part with an aircraft listed in the holder's operations specifications or current list of aircraft used in airtransportation unless that aircraft complies with any applicable flight recorder and cockpit voice recorder requirements ofthe part under which its certificate is issued except that the operator may—

(1) (2) (3) (4)

Ferry an aircraft with an inoperative flight recorder or cockpit voice recorder from a place where repair or replacement cannot be made to a place where they can be made;

Continue a flight as originally planned, if the flight recorder or cockpit voice recorder becomes inoperative afterthe aircraft has taken off;

Conduct an airworthiness flight test during which the flight recorder or cockpit voice recorder is turned offto test it orto test any communications or electrical equipment installed in the aircraft; or

Ferry a newly acquired aircraft from the place where possession of it is taken to a place where the flight recorder or cockpit voice recorder is to be installed.

(b)

Notwithstanding paragraphs (c) and (e) of this section, an operator otherthan the holder of an air carrier or a commercial operator certificate may—

(1) (2) (3)

Ferry an aircraft with an inoperative flight recorder or cockpit voice recorder from a place where repair or replacement cannot be made to a place where they can be made;

Continue a flight as originally planned if the flight recorder or cockpit voice recorder becomes inoperative afterthe aircraft has taken off;

Conduct an airworthiness flight test during which the flight recorder or cockpit voice recorder is turned offto test it orto test any communications or electrical equipment installed in the aircraft;

14 CFR91.609(b)(3)(enhanceddisplay)

page 123 of243

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.609(b)(4)

(4)

Ferry a newly acquired aircraft from a place where possession of it was taken to a place where the flight recorder or cockpit voice recorder is to be installed; or

(5)  Operate an aircraft:

(i)   For not more than 15 days while the flight recorder and/or cockpit voice recorder is inoperative and/or removed for repair provided that the aircraft maintenance records contain an entry that indicates the date of failure, and a placard is located in view of the pilot to show that the flight recorder or cockpit voice recorder is inoperative.

(ii)  For not more than an additional 15 days, provided that the requirements in paragraph (b)(5)(i) are met and that a certificated pilot, or a certificated person authorized to return an aircraft to service under § 43.7 of this chapter, certifies in the aircraft maintenance records that additional time is required to complete repairs or obtain a replacement unit.

(c)

(1)

(2) (3)

No person may operate a U.S. civil registered, multiengine, turbine-powered airplane or rotorcraft having a passenger seating configuration, excluding any pilot seats of 10 or more that has been manufactured after October 11, 1991, unless it is equipped with one or more approved flight recorders that utilize a digital method of recording and storing data and a method of readily retrieving that data from the storage medium, that are capable of recording the data specified in appendix E to this part, for an airplane, or appendix F to this part, for a rotorcraft, of this part within the range, accuracy, and recording interval specified, and that are capable of retaining no less than 8 hours of aircraft operation.

All airplanes subject to paragraph (c)(1) of this section that are manufactured before April 7, 2010, by April 7, 2012, must meet the requirements of § 23.1459(a)(7) or § 25.1459(a)(8) of this chapter, as applicable.

All airplanes and rotorcraft subject to paragraph (c)(1) ofthis section that are manufactured on or after April 7, 2010, must meet the flight data recorder requirements of § 23.1459, § 25.1459, § 27.1459, or § 29.1459 of this chapter, as applicable, and retain at least the last 25 hours of recorded information using a recorder that meets the standards of TSO-C124a, or later revision.

(d) (e)

Whenever a flight recorder, required bythis section, is installed, it must be operated continuously from the instant the airplane begins the takeoff roll or the rotorcraft begins lift-off until the airplane has completed the landing roll orthe rotorcraft has landed at its destination.

Unless otherwise authorized by the Administrator, after October 11, 1991, no person may operate a U.S. civil registered multiengine, turbine-powered airplane or rotorcraft having a passenger seating configuration of six passengers or more and for which two pilots are required by type certification or operating rule unless it is equipped with an approved cockpit voice recorder that:

(1) (2)

Is installed in compliance with § 23.1457(a)(1) and (2), (b), (c), (d)(1)(i), (2) and (3), (e), (f), and (g); § 25.1457(a)(1) and (2), (b), (c), (d)(1)(i), (2) and (3), (e), (f), and (g); § 27.1457(a)(1) and (2), (b), (c), (d)(1)(i), (2) and (3), (e), (f), and (g); or § 29.1457(a)(1) and (2), (b), (c), (d)(1)(i), (2) and (3), (e), (f), and (g) of this chapter, as applicable; and

Is operated continuously from the use of the checklist before the flight to completion of the final checklist at the end ofthe flight.

14 CFR91.609(e)(2) (enhanceddisplay)

page 124of243

(f) (g)

(h) (i)

(j) (k)

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.609(f)

In complying with this section, an approved cockpit voice recorder having an erasure feature may be used, so that at any time during the operation of the recorder, information recorded more than 15 minutes earlier may be erased or otherwise obliterated.

In the event of an accident or occurrence requiring immediate notification to the National Transportation Safety Board under 49 CFR part 830 of its regulations that results in the termination ofthe flight, any operator who has installed approved flight recorders and approved cockpit voice recorders shall keep the recorded information for at least 60 days or, if requested by the Administrator orthe Board, for a longer period. Information obtained from the record is used to assist in determining the cause of accidents or occurrences in connection with the investigation under 49 CFR part 830. The Administrator does not use the cockpit voice recorder record in any civil penalty or certificate action.

All airplanes required by this section to have a cockpit voice recorder and a flight data recorder, that are manufactured before April 7, 2010, must by April 7, 2012, have a cockpit voice recorder that also—

(1) (2)

Meets the requirements of § 23.1457(d)(6) or § 25.1457(d)(6) of this chapter, as applicable; and

If transport category, meets the requirements of § 25.1457(a)(3), (a)(4), and (a)(5) of this chapter.

All airplanes or rotorcraft required by this section to have a cockpit voice recorder and flight data recorder, that are manufactured on or after April 7, 2010, must have a cockpit voice recorder installed that also—

(1)

(2) (3)

Is installed in accordance with the requirements of § 23.1457 (except for paragraphs (a)(6) and (d)(5)); § 25.1457 (except for paragraphs (a)(6) and (d)(5)); § 27.1457 (except for paragraphs (a)(6) and (d)(5)); or § 29.1457 (except for paragraphs (a)(6) and (d)(5)) ofthis chapter, as applicable; and

Retains at least the last 2 hours of recorded information using a recorder that meets the standards of TSO-C123a, or later revision.

For all airplanes or rotorcraft manufactured on or after April 6, 2012, also meets the requirements of § 23.1457(a)(6) and (d)(5); § 25.1457(a)(6) and (d)(5); § 27.1457(a)(6) and (d)(5); or § 29.1457(a)(6) and (d)(5) of this chapter, as applicable.

All airplanes or rotorcraft required by this section to have a cockpit voice recorder and a flight data recorder, that install datalink communication equipment on or after April 6, 2012, must record all datalink messages as required by the certification rule applicable to the aircraft.

An aircraft operated under this part under deviation authority from part 125 of this chapter must comply with all of the applicable flight data recorder requirements of part 125 applicable to the aircraft, notwithstanding such deviation authority.

[Docket 18334, 54 FR 34318, Aug. 18, 1989, as amendedbyAmdt. 91-226, 56 FR 51621, Oct. 11, 1991;Amdt. 91-228, 57 FR 19353, May 5, 1992;Amdt. 91-300, 73 FR 12564, Mar. 7, 2008;Amdt. 91-304, 73 FR 73178, Dec. 2, 2008; Amdt. 91-300, 74 FR 32800, July 9, 2009; Amdt. 91-313, 75 FR 17045, Apr. 5, 2010; Docket FAA-2022-1355, Amdt. 91-366, 87 FR 75846, Dec. 9, 2022]

§ 91.611Authorizationforferry  flightwith oneengine inoperative.

(a)  General. The holder of an air carrier operating certificate or an operating certificate issued under part 125 may conduct a ferry flight of a four-engine airplane or a turbine-engine-powered airplane equipped with three engines, with one engine inoperative, to a base for the purpose of repairing that engine subject to the following:

14 CFR91.611(a)(enhanceddisplay)

page 125of243

(1) (2)

(3)

(4)

(5) (6)

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.611(a)(1)

The airplane model has been test flown and found satisfactory for safe flight in accordance with paragraph (b) or (c) ofthis section, as appropriate. However, each operator who before November 19, 1966, has shown that a model of airplane with an engine inoperative is satisfactory for safe flight by a test flight conducted in accordance with performance data contained in the applicable Airplane Flight Manual under paragraph (a)(2) of this section need not repeat the test flight for that model.

The approved Airplane Flight Manual contains the following performance data and the flight is conducted in accordance with that data:

(i)   Maximum weight.

(ii) (iii) (iv)

Center of gravity limits.

Configuration ofthe inoperative propeller (if applicable).

Runway length for takeoff (including temperature accountability).

(v)   Altitude range.

(vi)  Certificate limitations.

(vii)  Ranges of operational limits. (viii)  Performance information.

(ix)  Operating procedures.

The operator has FAA approved procedures for the safe operation of the airplane, including specific requirements for—

(i) (ii)

(iii)

(iv)

Limiting the operating weight on any ferry flight to the minimum necessary for the flight plus the necessary reserve fuel load;

A limitation that takeoffs must be made from dry runways unless, based on a showing of actual operating takeofftechniques on wet runways with one engine inoperative, takeoffs with full controllability from wet runways have been approved for the specific model aircraft and included in the Airplane Flight Manual:

Operations from airports where the runways may require a takeoff or approach over populated areas; and

Inspection procedures for determining the operating condition of the operative engines.

No person may take off an airplane under this section if—

(i) (ii)

The initial climb is over thickly populated areas; or

Weather conditions at the takeoff or destination airport are less than those required for VFR flight.

Persons otherthan required flight crewmembers shall not be carried during the flight.

No person may use a flight crewmember for flight under this section unless that crewmember is thoroughly familiar with the operating procedures for one-engine inoperative ferry flight contained in the certificate holder's manual and the limitations and performance information in the Airplane Flight Manual.

14 CFR91.611(a)(6) (enhanceddisplay)

page 126of243

(b) (c)

(1) (2)

(3) (4)

(5)

(1) (2)

(3)

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.611(b)

Flight tests: reciprocating-engine-poweredairplanes. The airplane performance of a reciprocating-engine- powered airplane with one engine inoperative must be determined by flight test as follows:

A speed not less than 1.3 VS1 must be chosen at which the airplane may be controlled satisfactorily in a climb with the critical engine inoperative (with its propeller removed or in a configuration desired by the operator and with all other engines operating at the maximum power determined in paragraph (b)(3) of this section.

The distance required to accelerate to the speed listed in paragraph (b)(1) of this section and to climb to 50 feet must be determined with—

(i)   The landing gear extended;

(ii) (iii)

The critical engine inoperative and its propeller removed or in a configuration desired by the operator; and

The other engines operating at not more than maximum power established under paragraph (b)(3) of this section.

The takeoff, flight and landing procedures, such as the approximate trim settings, method of power application, maximum power, and speed must be established.

The performance must be determined at a maximum weight not greater than the weight that allows a rate of climb of at least 400 feet per minute in the en route configuration set forth in § 25.67(d) of this chapter in effect on January 31, 1977, at an altitude of 5,000 feet.

The performance must be determined using temperature accountability for the takeoff field length, computed in accordance with § 25.61 of this chapter in effect on January 31, 1977.

Flight tests: Turbine-engine-powered airplanes. The airplane performance of a turbine-engine-powered airplane with one engine inoperative must be determined by flight tests, including at least three takeoff tests, in accordance with the following:

Takeoff speeds VR and V2, not less than the corresponding speeds under which the airplane was type certificated under § 25.107 of this chapter, must be chosen at which the airplane may be controlled satisfactorily with the critical engine inoperative (with its propeller removed or in a configuration desired by the operator, if applicable) and with all other engines operating at not more than the

power selected for type certification as set forth in § 25.101 of this chapter.

The minimum takeoff field length must be the horizontal distance required to accelerate and climb to the 35-foot height at V2 speed (including any additional speed increment obtained in the tests) multiplied by 115 percent and determined with—

(i)   The landing gear extended;

(ii) (iii)

The critical engine inoperative and its propeller removed or in a configuration desired by the operator (if applicable); and

The other engine operating at not more than the power selected for type certification as set forth in § 25.101 of this chapter.

The takeoff, flight, and landing procedures such as the approximate trim setting, method of power application, maximum power, and speed must be established. The airplane must be satisfactorily controllable during the entire takeoff run when operated according to these procedures.

14 CFR91.611(c)(3)(enhanced display)

page 127of243

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.611(c)(4)

(4)

(5)

(6)

The performance must be determined at a maximum weight not greater than the weight determined under § 25.121(c) of this chapter but with—

(i)   The actual steady gradient of the final takeoff climb requirement not less than 1.2 percent at the end ofthe takeoff path with two critical engines inoperative; and

(ii)   The climb speed not less than the two-engine inoperative trim speed forthe actual steady gradient of the final takeoff climb prescribed by paragraph (c)(4)(i) of this section.

The airplane must be satisfactorily controllable in a climb with two critical engines inoperative. Climb performance may be shown by calculations based on, and equal in accuracyto, the results of testing.

The performance must be determined using temperature accountability for takeoff distance and final takeoff climb computed in accordance with § 25.101 of this chapter.

For the purpose of paragraphs (c)(4) and (5) ofthis section, two critical engines means two adjacent engines on one side of an airplane with four engines, and the center engine and one outboard engine on an airplane with three engines.

§ 91.613 Materialsfor compartment interiors.

(a)   No person may operate an airplane that conforms to an amended or supplemental type certificate issued in accordance with SFAR No. 41 for a maximum certificated takeoff weight in excess of 12,500 pounds unless within 1 year after issuance ofthe initial airworthiness certificate under that SFAR the airplane meets the compartment interior requirements set forth in § 25.853 (a), (b), (b-1), (b-2), and (b-3) of this chapter in effect on September 26, 1978.

(b)

Thermal/acoustic insulation materials. For transport category airplanes type certificated after January 1, 1958:

(1)

(2)

For airplanes manufactured before September 2, 2005, when thermal/acoustic insulation is installed in the fuselage as replacements after September 2, 2005, the insulation must meet the flame propagation requirements of § 25.856 of this chapter, effective September 2, 2003, if it is:

(i)   Of a blanket construction or (ii)   Installed around air ducting.

For airplanes manufactured after September 2, 2005, thermal/acoustic insulation materials installed in the fuselage must meet the flame propagation requirements of § 25.856 of this chapter, effective September 2, 2003.

[Docket 18334, 54 FR 34318, Aug. 18, 1989, as amendedbyAmdt. 91-279, 68 FR 45083, July31, 2003; Amdt. 91-290, 70 FR 77752, Dec. 30, 2005]

§§ 91.615-91.699 [Reserved]

Subpart H—ForeignAircraft Operations andOperations ofU.S.-Registered Civil Aircraft Outside ofthe United States; andRules GoverningPersons on Board SuchAircraft

Source:

Docket 18334, 54 FR 34320, Aug. 18, 1989, unless otherwise noted.

14 CFR91.615-91.699(enhanceddisplay)

page 128 of243

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.701

§ 91.701 Applicability.

(a)   This subpart applies to the operations of civil aircraft of U.S. registry outside ofthe United States and the operations of foreign civil aircraft within the United States.

(b)

Section 91.702 ofthis subpart also applies to each person on board an aircraft operated as follows:

(1) (2)

A U.S. registered civil aircraft operated outside the United States; Any aircraft operated outside the United States—

(i)   That has its next scheduled destination or last place of departure in the United States if the aircraft next lands in the United States; or

(ii)   If the aircraft lands in the United States with the individual still on the aircraft regardless of whether it was a scheduled or otherwise planned landing site.

[Docket FAA-1998-4954, 64 FR 1079, Jan. 7, 1999]

§ 91.702 Persons onboard.

Section 91.11 ofthis part (Prohibitions on interference with crewmembers) applies to each person on board an aircraft.

[Docket FAA-1998-4954, 64 FR 1079, Jan. 7, 1999]

§ 91.703 Operations ofcivil aircraft ofU.S. registry outside ofthe United States. (a)  Each person operating a civil aircraft of U.S. registry outside ofthe United States shall—

(1)  When overthe high seas, comply with Annex 2 (Rules of the Air) to the Convention on International Civil Aviation and with §§ 91.117(c), 91.127, 91.129, and 91.131;

(2) (3)

(4) (5)

When within a foreign country, comply with the regulations relating to the flight and maneuver of aircraft there in force;

Except for §§ 91.117(a), 91.307(b), 91.309, 91.323, and 91.711, comply with this part so far as it is not inconsistent with applicable regulations ofthe foreign country where the aircraft is operated or Annex 2 ofthe Convention on International Civil Aviation; and

When operating within airspace designated as Reduced Vertical Separation Minimum (RVSM) airspace, comply with § 91.706.

For aircraft subject to ICAO Annex 16, carry on board the aircraft documents that summarize the noise operating characteristics and certifications of the aircraft that demonstrate compliance with this part and part 36 of this chapter.

(b)

Annex 2 to the Convention on International Civil Aviation, Rules of the Air, Tenth Edition—July 2005, with Amendments through Amendment 45, applicable November 10, 2016, is incorporated by reference into this section with the approval of the Director ofthe Federal Register under 5 U.S.C. 552(a) and 1 CFR part 51. To enforce any edition other than that specified in this section, the FAA must publish a document in the FEDERAL REGISTER and the material must be available to the public. All approved material is available

14 CFR91.703(b)(enhanceddisplay)

page 129 of243

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.705

for inspection at U.S. Department of Transportation, Docket Operations, West Building Ground Floor, Room W12-140, 1200 New Jersey Avenue SE., Washington, DC 20590 and is available from the International Civil Aviation Organization (ICAO), Marketing and Customer Relations Unit, 999 Robert Bourassa Boulevard, Montreal, Quebec H3C 5H7, Canada; http://store1.icao.int/; or by contacting the ICAO Marketing and Customer Relations Unit bytelephone at 514-954-8022 or by email at sales@icao.int. For questions about ICAO Annex 2, contact the FAA's Office of International Affairs at (202) 267-1000. It is also available for inspection at the National Archives and Records Administration (NARA). For information on the availability of this material at NARA, call 202-741-6030, or go to http://www.archives.gov/ federal_register/code_of_federal_regulations/ibr_locations.html.

[Docket 18834, 54 FR 34320, Aug. 18, 1989, as amendedbyAmdt. 91-227, 56 FR 65661, Dec. 17, 1991;Amdt. 91-254, 62 FR 17487, Apr. 9, 1997; 69 FR 18803, Apr. 9, 2004;Amdt. 91-299, 73 FR 10143, Feb. 26, 2008;Amdt. 91-312, 75 FR 9333, Mar. 2, 2010; Docket FAA-2016-9154, Amdt. 91-348, 82 FR 39664, Aug. 22, 2017]

§ 91.705 [Reserved]

§ 91.706 Operationswithin airspace designedas ReducedVertical Separation Minimum Airspace.

(a)

(b)

Except as provided in paragraph (b) ofthis section, no person may operate a civil aircraft of U.S. registry in airspace designated as Reduced Vertical Separation Minimum (RVSM) airspace unless:

(1)  The operator and the operator's aircraft comply with the requirements of appendix G of this part; and (2)   The operator is authorized by the Administrator to conduct such operations.

The Administrator may authorize a deviation from the requirements of this section in accordance with Section 5 of appendix G to this part.

[Docket 28870, 62 FR 17487, Apr. 9, 1997]

§ 91.707Flightsbetween Mexico or Canadaandthe United States.

Unless otherwise authorized byATC, no person may operate a civil aircraft between Mexico or Canada and the United States without filing an IFR or VFR flight plan, as appropriate.

§ 91.709 Operations to Cuba.

No person may operate a civil aircraft from the United States to Cuba unless—

(a) (b)

Departure is from an international airport of entry designated in § 6.13 of the Air Commerce Regulations of the Bureau of Customs (19 CFR 6.13); and

In the case of departure from any ofthe 48 contiguous States orthe District of Columbia, the pilot in command of the aircraft has filed—

(1)  A DVFR or IFR flight plan as prescribed in § 99.11 or § 99.13 of this chapter; and

(2)   A written statement, within 1 hour before departure, with the Office of Immigration and Naturalization Service at the airport of departure, containing—

(i)   All information in the flight plan;

14 CFR91.709(b)(2)(i)(enhanced display)

page 130 of243

(b) (c)

(d) (e)

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

(ii)   The name of each occupant of the aircraft; (iii)  The number of occupants ofthe aircraft; and

(iv)  A description of the cargo, if any.

14 CFR 91.709(b)(2)(ii)

This section does not apply to the operation of aircraft by a scheduled air carrier over routes authorized in operations specifications issued by the Administrator.

(Approved by the Office ofManagement and Budget under control number2120-0005)

§ 91.711 Special rules for foreign civil aircraft.

(a)  General. In addition to the other applicable regulations of this part, each person operating a foreign civil aircraft within the United States shall comply with this section.

VFR. No person may conduct VFR operations which require two-way radio communications under this part unless at least one crewmember of that aircraft is able to conduct two-way radio communications in the English language and is on duty during that operation.

IFR. No person may operate a foreign civil aircraft under IFR unless— (1)  That aircraft is equipped with—

(i)   Radio equipment allowing two-way radio communication with ATC when it is operated in controlled airspace; and

(ii)   Navigation equipment suitable for the route to be flown. (2)  Each person piloting the aircraft—

(i)

Holds a current United States instrument rating or is authorized by his foreign airman certificate to pilot under IFR; and

(3)

(ii)  Is thoroughly familiar with the United States en route, holding, and letdown procedures; and

At least one crewmember of that aircraft is able to conduct two-way radiotelephone communications in the English language and that crewmember is on duty while the aircraft is approaching, operating within, or leaving the United States.

Over water. Each person operating a foreign civil aircraft over water off the shores of the United States shall give flight notification or file a flight plan in accordance with the Supplementary Procedures for the ICAO region concerned.

Flight atand above FL 240. IfVOR navigation equipment is required under paragraph (c)(1)(ii) ofthis section, no person may operate a foreign civil aircraft within the 50 States and the District of Columbia at or above FL 240, unless the aircraft is equipped with approved DME or a suitable RNAV system. When the DME or RNAV system required by this paragraph fails at and above FL 240, the pilot in command of the aircraft must notify ATC immediately and may then continue operations at and above FL 240 to the next airport of intended landing where repairs or replacement ofthe equipment can be made. A foreign civil aircraft may be operated within the 50 States and the District of Columbia at or above FL 240 without DME or an RNAV system when operated for the following purposes, and ATC is notified before each takeoff:

(1)

Ferry flights to and from a place in the United States where repairs or alterations are to be made.

14 CFR91.711(e)(1)(enhanceddisplay)

page 131of243

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

(2)  Ferry flights to a new country of registry.

(3)  Flight of a new aircraft of U.S. manufacture for the purpose of— (i)   Flight testing the aircraft;

(ii)   Training foreign flight crews in the operation ofthe aircraft; or

(iii)  Ferrying the aircraft for export delivery outside the United States.

14 CFR91.711(e)(2)

(4)  Ferry, demonstration, and test flight of an aircraft brought to the United States for the purpose of demonstration or testing the whole or any part thereof.

[Docket 18834, 54 FR 34320, Aug. 18, 1989, as amendedbyAmdt. 91-227, 56 FR 65661, Dec. 17, 1991;Amdt. 91-296, 72 FR 31679, June 7, 2007]

§ 91.713 Operation ofcivil aircraft ofCuban registry.

No person may operate a civil aircraft of Cuban registry except in controlled airspace and in accordance with air traffic clearance or air traffic control instructions that may require use of specific airways or routes and landings at specific airports.

§ 91.715 Special flight authorizations for foreign civil aircraft.

(a)  Foreign civil aircraft may be operated without airworthiness certificates required under § 91.203 if a special flight authorization for that operation is issued under this section. Application for a special flight authorization must be made to the appropriate Flight Standards Division Manager, or Aircraft Certification Service Division Director. However, in the case of an aircraft to be operated in the U.S. forthe purpose of demonstration at an airshow, the application may be made to the appropriate Flight Standards Division Manager or Aircraft Certification Service Division Director responsible for the airshow location.

(b) (c)

The Administrator may issue a special flight authorization for a foreign civil aircraft subject to any conditions and limitations that the Administrator considers necessary for safe operation in the U.S. airspace.

No person may operate a foreign civil aircraft under a special flight authorization unless that operation also complies with part 375 of the Special Regulations of the Department ofTransportation (14 CFR part 375).

(Approved by the Office ofManagement and Budget under control number2120-0005)

[Docket 18334, 54 FR 34320, Aug. 18, 1989, as amendedbyAmdt. 91-212, 54 FR 39293, Sept. 25, 1989;Docket FAA-2018-0119, Amdt. 91-350, 83 FR 9171, Mar. 5, 2018]

§§ 91.717-91.799 [Reserved]

Subpart I—OperatingNoise Limits

Source:

Docket 18334, 54 FR 34321, Aug. 18, 1989, unless otherwise noted.

14 CFR91.717-91.799(enhanceddisplay)

page 132of243

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.801

§ 91.801Applicability: Relationto part 36.

(a)   This subpart prescribes operating noise limits and related requirements that apply, as follows, to the operation of civil aircraft in the United States.

(1)

(2) (3)

Sections 91.803, 91.805, 91.807, 91.809, and 91.811 apply to civil subsonicjet (turbojet) airplanes with maximum weights of more than 75,000 pounds and—

(i)   If U.S. registered, that have standard airworthiness certificates; or

(ii)  If foreign registered, that would be required by this chapter to have a U.S. standard airworthiness certificate in order to conduct the operations intended for the airplane were it registered in the United States. Those sections apply to operations to or from airports in the United States under this part and parts 121, 125, 129, and 135 of this chapter.

Section 91.813 applies to U.S. operators of civil subsonic jet (turbojet) airplanes covered bythis subpart. This section applies to operators operating to or from airports in the United States under this part and parts 121, 125, and 135, but not to those operating under part 129 of this chapter.

Sections 91.803, 91.819, and 91.821 apply to U.S.-registered civil supersonic airplanes having standard airworthiness certificates and to foreign-registered civil supersonic airplanes that, if registered in the United States, would be required by this chapter to have U.S. standard airworthiness certificates in order to conduct the operations intended for the airplane. Those sections applyto operations underthis part and under parts 121, 125, 129, and 135 of this chapter.

(b)

(c) (d)

Unless otherwise specified, as used in this subpart “part 36” refers to 14 CFR part 36, including the noise levels under appendix C of that part, notwithstanding the provisions of that part excepting certain airplanes from the specified noise requirements. For purposes of this subpart, the various stages of noise levels, the terms used to describe airplanes with respect to those levels, and the terms “subsonic airplane” and “supersonic airplane” have the meanings specified under part 36 of this chapter. For purposes ofthis subpart, for subsonic airplanes operated in foreign air commerce in the United States, the Administrator may accept compliance with the noise requirements under annex 16 of the International Civil Aviation Organization when those requirements have been shown to be substantially compatible with, and achieve results equivalent to those achievable under, part 36 for that airplane. Determinations made under these provisions are subject to the limitations of § 36.5 of this chapter as if those noise levels were part 36

noise levels.

Sections 91.851 through 91.877 ofthis subpart prescribe operating noise limits and related requirements that applyto any civil subsonic jet (turbojet) airplane (for which an airworthiness certificate other than an experimental certificate has been issued by the Administrator) with a maximum certificated takeoff weight of more than 75,000 pounds operating to or from an airport in the 48 contiguous United States and

the District of Columbia under this part, parts 121, 125, 129, or 135 of this chapter on and after September 25, 1991.

Section 91.877 prescribes reporting requirements that apply to any civil subsonic jet (turbojet) airplane with a maximum weight of more than 75,000 pounds operated by an air carrier or foreign air carrier between the contiguous United States and the State of Hawaii, between the State of Hawaii and any point outside ofthe 48 contiguous United States, or between the islands of Hawaii in turnaround service, under part 121 or 129 of this chapter on or after November 5, 1990.

14 CFR91.801(d)(enhanceddisplay)

page 133 of243

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.801(e)

(e)

Sections 91.881 through 91.883 ofthis subpart prescribe operating noise limits and related requirements that applyto any civil subsonic jet airplane with a maximum takeoffweight of 75,000 pounds or less and for which an airworthiness certificate (other than an experimental certificate) has been issued, operating to or from an airport in the contiguous United States under this part, part 121, 125, 129, or 135 of this chapter on and after December 31, 2015.

[Docket 18334, 54 FR 34321, Aug. 18, 1989; Amdt. 91-211, 54 FR 41211, Oct. 5, 1989, as amended byAmdt. 91-225, 56 FR 48658, Sept. 25, 1991;Amdt. 91-252, 61 FR 66185, Dec. 16, 1996;Amdt. 91-275, 67 FR 45237, July8, 2002;Amdt. 91-276, 67 FR 46571, July 15, 2002;Amdt. 91-328, 78 FR 39583, July 2, 2013]

§ 91.803 Part 125operators: Designation ofapplicable regulations.

For airplanes covered bythis subpart and operated under part 125 of this chapter, the following regulations apply as specified:

(a) (b)

(c) (d)

For each airplane operation to which requirements prescribed under this subpart applied before November 29, 1980, those requirements of this subpart continue to apply.

For each subsonic airplane operation to which requirements prescribed under this subpart did not apply before November 29, 1980, because the airplane was not operated in the United States under this part or part 121, 129, or 135 of this chapter, the requirements prescribed under § 91.805 of this subpart apply.

For each supersonic airplane operation to which requirements prescribed under this subpart did not apply before November 29, 1980, because the airplane was not operated in the United States under this part or part 121, 129, or 135 of this chapter, the requirements of §§ 91.819 and 91.821 of this subpart apply.

For each airplane required to operate under part 125 for which a deviation under that part is approved to operate, in whole or in part, under this part or part 121, 129, or 135 of this chapter, notwithstanding the approval, the requirements prescribed under paragraphs (a), (b), and (c) of this section continue to apply.

[Docket 18334, 54 FR 34321, Aug. 18, 1989, as amendedbyAmdt. 91-276, 67 FR 46571, July 15, 2002]

§ 91.805 Final compliance: Subsonicairplanes.

Except as provided in §§ 91.809 and 91.811, on and after January 1, 1985, no person may operate to or from an airport in the United States any subsonic airplane covered by this subpart unless that airplane has been shown to comply with Stage 2 or Stage 3 noise levels under part 36 of this chapter.

§§ 91.807-91.813 [Reserved]

§ 91.815Agricultural and fire fighting airplanes: Noise operating limitations.

(a)   This section applies to propeller-driven, small airplanes having standard airworthiness certificates that are designed for “agricultural aircraft operations” (as defined in § 137.3 of this chapter, as effective on January 1, 1966) or for dispensing fire fighting materials.

(b)

If the Airplane Flight Manual, or other approved manual material information, markings, or placards forthe airplane indicate that the airplane has not been shown to comply with the noise limits under part 36 of

this chapter, no person may operate that airplane, except—

14 CFR91.815(b)(enhanceddisplay)

page 134of243

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.815(b)(1)

(1) (2) (3)

To the extent necessary to accomplish the work activity directly associated with the purpose for which it is designed;

To provide flight crewmember training in the special purpose operation for which the airplane is designed; and

To conduct “nondispensing aerial work operations” in accordance with the requirements under § 137.29(c) of this chapter.

§ 91.817 Civil aircraftsonic boom.

(a)  No person may operate a civil aircraft in the United States at a true flight Mach number greater than 1 except in compliance with conditions and limitations in an authorization to exceed Mach 1 issued to the operator in accordance with § 91.818.

(b)

In addition, no person may operate a civil aircraft for which the maximum operating limit speed MM0 exceeds a Mach number of 1, to or from an airport in the United States, unless—

(1) (2)

Information available to the flight crew includes flight limitations that ensure that flights entering or leaving the United States will not cause a sonic boom to reach the surface within the United States; and

The operator complies with the flight limitations prescribed in paragraph (b)(1) of this section or complies with conditions and limitations in an authorization to exceed Mach 1 issued in accordance with § 91.818.

(Approved by the Office ofManagement and Budget under control number2120-0005) [Docket 18334, 54 FR 34321, Aug. 18, 1989, as amendedbyAmdt. 91-362, 86 FR 3792, Jan. 15, 2021]

§ 91.818 Special flight authorization to exceed Mach1.

For all civil aircraft, any operation that exceeds Mach 1 may be conducted only in accordance with a special flight authorization issued to an operator in accordance with the requirements ofthis section.

(a)

Application. Application for a special flight authorization to exceed Mach 1 must be made to the FAA Office of Environment and Energy for consideration by the Administrator. Each application must include:

(1) (2) (3) (4) (5)

(6) (7)

The name of the operator;

The number and model(s) of the aircraft to be operated; The number of proposed flights;

The date range during which the flight(s) would be conducted;

The time of day the flight(s) would be conducted. Proposed night operations may require further justification for their necessity;

A description of the flight area requested by the applicant, including any environmental information required to be submitted pursuant to paragraph (c) of this section;

All conditions and limitations on the flight(s) that will ensure that no measurable sonic boom overpressure will reach the surface outside of the proposed flight area; and

14 CFR91.818(a)(7)(enhanceddisplay)

page 135of243

(8) (9)

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.818(a)(8)

The reason(s) that operation at a speed greater than Mach 1 is necessary. A special flight authorization to exceed Mach 1 may be granted only for operations that are intended to:

(i) (ii)

(iii) (iv)

(v)

Show compliance with airworthiness requirements; Determine the sonic boom characteristics of an aircraft;

Establish a means of reducing or eliminating the effects of sonic boom, including flight profiles and special features of an aircraft;

Demonstrate the conditions and limitations under which speeds in excess of Mach 1 will not cause a measurable sonic boom overpressure to reach the surface; or

Measure the noise characteristics of an aircraft to demonstrate compliance with noise requirements imposed under this chapter, or to determine the limits for operation in accordance with § 91.817(b).

For any purpose listed in paragraph (a)(8) ofthis section, each applicant must indicate why its intended operation cannot be safely or properly accomplished over the ocean at a distance ensuring that no sonic boom overpressure reaches any land surface in the United States.

(b)

Operation outside a testarea. An applicant may apply for an authorization to conduct flights outside a test area under certain conditions and limitations upon a conservative showing that:

(1) (2)

(3)

Flight(s) within a test area have been conducted in accordance with an authorization issued for the purpose specified in paragraph (a)(8)(iv) of this section;

The results ofthe flight test(s) required by paragraph (b)(1) ofthis section demonstrate that a speed in excess of Mach 1 does not cause a measurable sonic boom overpressure to reach the surface; and

The conditions and limitations determined bythe test(s) represent all foreseeable operating conditions and are effective on all flights conducted under an authorization.

(c)  Environmental findings.

(1) (2)

No special flight authorization will be granted ifthe Administrator finds that such action is necessary to protect or enhance the environment.

The Administrator is required to considerthe potential environmental impacts resulting from the issuance of an authorization for a particular flight area pursuant to the National Environmental Policy Act of 1969 (NEPA) (42 U.S.C 4321 et seq.), all applicable regulations implementing NEPA, and related Executive orders and guidance. Accordingly, each applicant must provide information that sufficiently describes the potential environmental impact of any flight in excess of Mach 1, including the effect of a sonic boom reaching the surface in the proposed flight area, to enable the FAA to determine whether such impacts are significant within the meaning of NEPA.

(d)

Issuance. An authorization to operate a civil aircraft in excess of Mach 1 may be issued only after an applicant has submitted the information described in this section and the Administrator has taken the required action regarding the environmental findings described in paragraph (c) ofthis section.

(e)  Duration.

(1)

An authorization to exceed Mach 1 will be granted forthe time the Administrator determines necessary to conduct the flights for the described purposes.

14 CFR91.818(e)(1)(enhanced display)

page 136of243

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.818(e)(2)

(2) (3)

(4)

(5)

An authorization to exceed Mach 1 is effective until it expires or is surrendered.

An authorization to exceed Mach 1 may be terminated, suspended, or amended by the Administrator at any time the Administrator finds that such action is necessary to protect the environment.

The holder of an authorization to exceed Mach 1 may request reconsideration of a termination, amendment, or suspension issued under paragraph (e)(3) ofthis section within 30 days of notice of the action. Failure to request reconsideration and provide information whythe Administrator's action is not appropriate will result in permanent termination of the authorization.

Findings made by and actions taken by the Administrator under this section do not affect any certificate issued under chapter 447 of Title 49 of the United States Code.

[Docket FAA-2019-0451, Amdt. 91-362, 86 FR 3792, Jan. 15, 2021]

§ 91.819 Civilsupersonic airplanes that do not comply with part 36.

(a)   Applicability. This section applies to civil supersonic airplanes that have not been shown to complywith the Stage 2 noise limits of part 36 in effect on October 13, 1977, using applicable trade-off provisions, and that are operated in the United States, after July 31, 1978.

(b)

Airport use. Except in an emergency, the following apply to each person who operates a civil supersonic airplane to or from an airport in the United States:

(1)

(2)

Regardless of whether a type design change approval is applied for under part 21 of this chapter, no person may land or take off an airplane covered by this section for which the type design is changed, after July 31, 1978, in a manner constituting an “acoustical change” under § 21.93 unless the acoustical change requirements of part 36 are complied with.

No flight may be scheduled, or otherwise planned, fortakeoff or landing after 10 p.m. and before 7 a.m. local time.

§ 91.821 Civil supersonic airplanes: Noise limits.

Except for Concorde airplanes having flight time before January 1, 1980, no person may operate in the United States, a civil supersonic airplane that does not comply with Stage 2 noise limits of part 36 in effect on October 13, 1977, using applicable trade-off provisions.

§§ 91.823-91.849 [Reserved] § 91.851 Definitions.

For the purposes of §§ 91.851 through

91.877 of this subpart:

Chapter 4 noise level means a noise level at or below the maximum noise level prescribed in Chapter 4, Paragraph 4.4, Maximum Noise Levels, ofthe International Civil Aviation Organization (ICAO) Annex 16, Volume I, Amendment 7, effective March 21, 2002. The Director of the Federal Register in accordance with 5 U.S.C. 552(a) and 1 CFR part 51 approved the incorporation by reference of this document, which can be obtained from the International Civil Aviation Organization (ICAO), Document Sales Unit, 999 University Street, Montreal, Quebec H3C 5H7, Canada. Also, you may obtain documents on the Internet at http://www.ICAO.int/eshop/index.cfm. Copies may be reviewed at the U.S. Department of Transportation, Docket Operations, West Building Ground Floor, Room W12-140, 1200 New Jersey Avenue, SE.,

14 CFR91.851“Chapter4noiselevel”(enhanceddisplay)

page 137of243

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.851“ContiguousUnited States”

Washington, DC 20590 or at the National Archives and Records Administration (NARA). For information on the availability of this material at NARA, call 202-741-6030, or go to: http://www.archives.gov/ federal_register/code_of_federal_regulations/ibr_locations.html.

Contiguous United States means the area encompassed by the 48 contiguous United States and the District of Columbia.

Fleet

means those civil subsonic jet (turbojet) airplanes with a maximum certificated weight of more than 75,000 pounds that are listed on an operator's operations specifications as eligible for operation in the contiguous United States.

Import means a change in ownership of an airplane from a non-U.S. person to a U.S. person when the airplane is brought into the United States for operation.

Operations specifications means an enumeration of airplanes by type, model, series, and serial number operated by the operator or foreign air carrier on a given day, regardless of how or whether such airplanes are formally listed or designated by the operator.

Owner means any person that has indicia of ownership sufficient to register the airplane in the United States pursuant to part 47 of this chapter.

New entrant means an air carrier or foreign air carrier that, on or before November 5, 1990, did not conduct operations under part 121 or 129 of this chapter using an airplane covered by this subpart to or from any airport in the contiguous United States, but that initiates such operation afterthat date.

Stage 2 noise levels mean the requirements for Stage 2 noise levels as defined in part 36 of this chapter in effect on November 5, 1990.

Stage 3 noise levels mean the requirements for Stage 3 noise levels as defined in part 36 of this chapter in effect on November 5, 1990.

Stage 4 noise level means a noise level at or below the Stage 4 noise limit prescribed in part 36 of this chapter.

Stage 2 airplane means a civil subsonic jet (turbojet) airplane with a maximum certificated weight of 75,000 pounds or more that complies with Stage 2 noise levels as defined in part 36 of this chapter.

Stage 3 airplane means a civil subsonic jet (turbojet) airplane with a maximum certificated weight of 75,000 pounds or more that complies with Stage 3 noise levels as defined in part 36 of this chapter.

Stage 4 airplane means an airplane that has been shown not to exceed the Stage 4 noise limit prescribed in part 36 of this chapter. A Stage 4 airplane complies with all of the noise operating rules ofthis part.

Stage 5 airplane means an airplane that has been shown not to exceed the Stage 5 noise limit prescribed in part 36 of this chapter. A Stage 5 airplane complies with all of the noise operating rules ofthis part.

Stage 5 noise level means a noise level at or below the Stage 5 noise limit prescribed in part 36 of this chapter.

[Docket 26433, 56 FR 48658, Sept. 25, 1991, as amendedbyAmdt. 91-252, 61 FR 66185, Dec. 16, 1996;Amdt. 91-275, 67 FR 45237, July8, 2002;Amdt. 91-288, 70 FR 38749, July5, 2005; 72 FR 68475, Dec. 5, 2007;Docket FAA-2015-3782, Amdt. 91-349, 82 FR 46132, Oct. 4, 2017]

14 CFR91.851“Stage5 noiselevel”(enhanceddisplay)

page 138 of243

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.853

§ 91.853 Final compliance: Civil subsonicairplanes.

Except as provided in § 91.873, after December 31, 1999, no person shall operate to or from any airport in the contiguous United States any airplane subject to § 91.801(c), unless that airplane has been shown to comply with Stage 3, Stage 4, or Stage 5 noise levels.

[Docket FAA-2015-3782, Amdt. 91-349, 82 FR 46132, Oct. 4, 2017]

§ 91.855 Entryandnonaddition  rule.

No person may operate any airplane subject to § 91.801(c) of this subpart to or from an airport in the contiguous United States unless one or more of the following apply:

(a) (b)

(c)

(d) (e)

(f)

(g) (h)

The airplane complies with Stage 3, Stage 4, or Stage 5 noise levels.

The airplane complies with Stage 2 noise levels and was owned by a U.S. person on and since November 5, 1990. Stage 2 airplanes that meet these criteria and are leased to foreign airlines are also subject to the return provisions of paragraph (e) of this section.

The airplane complies with Stage 2 noise levels, is owned by a non-U.S. person, and is the subject of a binding lease to a U.S. person effective before and on September 25, 1991. Any such airplane may be operated for the term of the lease in effect on that date, and any extensions thereof provided for in that lease.

The airplane complies with Stage 2 noise levels and is operated by a foreign air carrier.

The airplane complies with Stage 2 noise levels and is operated by a foreign operator other than forthe purpose of foreign air commerce.

The airplane complies with Stage 2 noise levels and— (1) On November 5, 1990, was owned by:

(i)  A corporation, trust, or partnership organized under the laws of the United States or any State (including individual States, territories, possessions, and the District of Columbia);

(ii)   An individual who is a citizen ofthe United States; or

(iii)  An entity owned or controlled by a corporation, trust, partnership, or individual described in paragraph (f)(1) (i) or (ii) ofthis section; and

(2)  Enters into the United States not later than 6 months after the expiration of a lease agreement (including any extensions thereof) between an owner described in paragraph (f)(1) of this section and a foreign airline.

The airplane complies with Stage 2 noise levels and was purchased by the importer under a written contract executed before November 5, 1990.

Any Stage 2 airplane described in this section is eligible for operation in the contiguous United States only as provided under § 91.865 or 91.867.

[Docket 26433, 56 FR 48658, Sept. 25, 1991; 56 FR 51167, Oct. 10, 1991, as amendedbyAmdt. 91-288, 70 FR 38750, July5, 2005; Docket FAA-2015-3782, Amdt. 91-349, 82 FR 46132, Oct. 4, 2017]

14 CFR91.855(h)(enhanceddisplay)

page 139 of243

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.857

§ 91.857 Stage 2 operations outside ofthe 48 contiguous United States.

An operator of a Stage 2 airplane that is operating only between points outside the contiguous United States on or after November 5, 1990, must include in its operations specifications a statement that such airplane may not be used to provide air transportation to or from any airport in the contiguous United States.

[Docket FAA-2002-12771, 67 FR 46571, July 15, 2002]

§ 91.858 Special flightauthorizations fornon-revenue Stage 2 operations.

(a)  After December 31, 1999, any operator of a Stage 2 airplane over 75,000 pounds may operate that airplane in nonrevenue service in the contiguous United States only forthe following purposes:

(1) Sell, lease, or scrap the airplane;

(2) Obtain modifications to meet Stage 3, Stage 4, or Stage 5 noise levels. (3)  Obtain scheduled heavy maintenance or significant modifications;

(4)  Deliver the airplane to a lessee or return it to a lessor; (5)  Park or store the airplane; and

(6)   Prepare the airplane for any of the purposes listed in paragraph (a)(1) thru (a)(5) ofthis section.

(b)

An operator of a Stage 2 airplane that needs to operate in the contiguous United States for any of the purposes listed above may apply to FAA's Office of Environment and Energy for a special flight authorization. The applicant must file in advance. Applications are due 30 days in advance of the planned flight and must provide the information necessary for the FAAto determine that the planned flight is within the limits prescribed in the law.

[Docket FAA-2002-12771, 67 FR 46571, July 15, 2002, as amended byDocketFAA-2015-3782, Amdt. 91-349, 82 FR 46132, Oct. 4, 2017]

§ 91.859 Modificationto meet Stage 3, Stage 4, or Stage 5 noise levels.

For an airplane subject to § 91.801(c) of this subpart and otherwise prohibited from operation to or from an airport in the contiguous United States by § 91.855, any person may apply for a special flight authorization for that airplane to operate in the contiguous United States forthe purpose of obtaining modifications to meet Stage 3, Stage 4, or Stage 5 noise levels.

[Docket FAA-2015-3782, Amdt. 91-349, 82 FR 46132, Oct. 4, 2017]

§ 91.861 Base level.

(a) U.S. Operators. The base level of a U.S. operator is equal to the number of owned or leased Stage 2 airplanes subject to § 91.801(c) of this subpart that were listed on that operator's operations specifications for operations to or from airports in the contiguous United States on any one day selected by the operator during the period January 1, 1990, through July 1, 1991, plus or minus adjustments made pursuant to paragraphs (a) (1) and (2).

(1)  The base level of a U.S. operator shall be increased by a number equal to the total of the following—

14 CFR91.861(a)(1)(enhanceddisplay)

page 140 of243

(b) (c)

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.861(a)(1)(i)

(i)

(ii) (iii)

The number of Stage 2 airplanes returned to service in the United States pursuant to § 91.855(f);

The number of Stage 2 airplanes purchased pursuant to § 91.855(g); and

Any U.S. operator base level acquired with a Stage 2 airplane transferred from another person under § 91.863.

(2)

The base level of a U.S. operator shall be decreased by the amount of U.S. operator base level transferred with the corresponding number of Stage 2 airplanes to another person under § 91.863.

Foreign air carriers. The base level of a foreign air carrier is equal to the number of owned or leased Stage 2 airplanes that were listed on that carrier's U.S. operations specifications on any one day during the period January 1, 1990, through July 1, 1991, plus or minus any adjustments to the base levels made pursuant to paragraphs (b) (1) and (2).

(1) (2)

The base level of a foreign air carrier shall be increased by the amount of foreign air carrier base level acquired with a Stage 2 airplane from another person under § 91.863.

The base level of a foreign air carrier shall be decreased bythe amount of foreign air carrier base level transferred with a Stage 2 airplane to another person under § 91.863.

New entrants do not have a base level.

[Docket 26433, 56 FR 48659, Sept. 25, 1991; 56 FR 51167, Oct. 10, 1991]

§ 91.863 Transfers ofStage 2 airplaneswith baselevel.

(a)  Stage 2 airplanes may be transferred with or without the corresponding amount of base level. Base level may not be transferred without the corresponding number of Stage 2 airplanes.

(b) (c)

No portion of a U.S. operator's base level established under § 91.861(a) may be used for operations by a foreign air carrier. No portion of a foreign air carrier's base level established under § 91.861(b) may be used for operations by a U.S. operator.

Whenever a transfer of Stage 2 airplanes with base level occurs, the transferring and acquiring parties shall, within 10 days, jointly submit written notification of the transfer to the FAA, Office of Environment and Energy. Such notification shall state:

(1) (2)

(3) (4) (5)

The names ofthe transferring and acquiring parties;

The name, address, and telephone number ofthe individual responsible for submitting the notification on behalf ofthe transferring and acquiring parties;

The total number of Stage 2 airplanes transferred, listed by airplane type, model, series, and serial number;

The corresponding amount of base level transferred and whether it is U.S. operator or foreign air carrier base level; and

The effective date ofthe transaction.

14 CFR91.863(c)(5)(enhanced display)

page 141of243

(a) (b)

(c) (d)

(e)

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.863(d)

(d)

If, taken as a whole, a transaction or series oftransactions made pursuant to this section does not produce an increase or decrease in the number of Stage 2 airplanes for eitherthe acquiring or transferring operator, such transaction or series of transactions may not be used to establish compliance with the requirements of § 91.865.

[Docket 26433, 56 FR 48659, Sept. 25, 1991]

§ 91.865 Phased compliance foroperators withbase level.

Except as provided in paragraph (a) of this section, each operator that operates an airplane under part 91, 121, 125, 129, or 135 of this chapter, regardless of the national registry ofthe airplane, shall comply with paragraph (b) or (d) of this section at each interim compliance date with regard to its subsonic airplane fleet covered by § 91.801(c) of this subpart.

This section does not apply to new entrants covered by § 91.867 or to foreign operators not engaged in foreign air commerce.

Each operator that chooses to comply with this paragraph pursuant to any interim compliance requirement shall reduce the number of Stage 2 airplanes it operates that are eligible for operation in the contiguous United States to a maximum of:

(1) (2) (3)

After December 31, 1994, 75 percent of the base level held by the operator; After December 31, 1996, 50 percent of the base level held by the operator; After December 31, 1998, 25 percent of the base level held by the operator.

Except as provided under § 91.871, the number of Stage 2 airplanes that must be reduced at each compliance date contained in paragraph (b) of this section shall be determined by reference to the amount of base level held bythe operator on that compliance date, as calculated under § 91.861.

Each operator that chooses to comply with this paragraph pursuant to any interim compliance requirement shall operate a fleet that consists of:

(1) (2)

(3)

After December 31, 1994, not less than 55 percent Stage 3 airplanes; After December 31, 1996, not less than 65 percent Stage 3 airplanes; After December 31, 1998, not less than 75 percent Stage 3 airplanes.

Calculations resulting in fractions may be rounded to permit the continued operation of the next whole number of Stage 2 airplanes.

[Docket 26433, 56 FR 48659, Sept. 25, 1991]

§ 91.867Phased compliance for newentrants. (a)  New entrant U.S. air carriers.

(1)  A new entrant initiating operations under part 121 of this chapter on or before December 31, 1994, may initiate service without regard to the percentage of its fleet composed of Stage 3 airplanes.

(2)

After December 31, 1994, at least 25 percent of the fleet of a new entrant must comply with Stage 3 noise levels.

14 CFR91.867(a)(2)(enhanceddisplay)

page 142of243

(b) (c)

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.867(a)(3)

(3) (4)

After December 31, 1996, at least 50 percent of the fleet of a new entrant must comply with Stage 3 noise levels.

After December 31, 1998, at least 75 percent of the fleet of a new entrant must comply with Stage 3 noise levels.

New entrant foreign air carriers.

(1) (2) (3) (4)

A new entrant foreign air carrier initiating part 129 operations on or before December 31, 1994, may initiate service without regard to the percentage of its fleet composed of Stage 3 airplanes.

After December 31, 1994, at least 25 percent of the fleet on U.S. operations specifications of a new entrant foreign air carrier must comply with Stage 3 noise levels.

After December 31, 1996, at least 50 percent of the fleet on U.S. operations specifications of a new entrant foreign air carrier must comply with Stage 3 noise levels.

After December 31, 1998, at least 75 percent of the fleet on U.S. operations specifications of a new entrant foreign air carrier must comply with Stage 3 noise levels.

Calculations resulting in fractions may be rounded to permit the continued operation of the next whole number of Stage 2 airplanes.

[Docket 26433, 56 FR 48659, Sept. 25, 1991, as amendedbyAmdt. 91-252, 61 FR 66185, Dec. 16, 1996]

§ 91.869 Carry-forward compliance.

(a)   Any operator that exceeds the requirements of paragraph (b) of § 91.865 of this part on or before December 31, 1994, or on or before December 31, 1996, may claim a credit that may be applied at a subsequent interim compliance date.

(b)

Any operator that eliminates or modifies more Stage 2 airplanes pursuant to § 91.865(b) than required as of December 31, 1994, or December 31, 1996, may count the number of additional Stage 2 airplanes reduced as a credit toward—

(1) (2)

The number of Stage 2 airplanes it would otherwise be required to reduce following a subsequent interim compliance date specified in § 91.865(b); or

The number of Stage 3 airplanes it would otherwise be required to operate in its fleet following a subsequent interim compliance date to meet the percentage requirements specified in § 91.865(d).

[Docket 26433, 56 FR 48659, Sept. 25, 1991; 56 FR 65783, Dec. 18, 1991]

§ 91.871Waivers from  interimcompliancerequirements.

(a)   Any U.S. operator or foreign air carrier subject to the requirements of § 91.865 or 91.867 of this subpart may request a waiver from any individual compliance requirement.

(b)

Applications must be filed with the Secretary of Transportation at least 120 days priorto the compliance date from which the waiver is requested.

14 CFR91.871(b)(enhanced display)

page 143 of243

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.871(c)

(c)

(d)

(e) (f)

Applicants must show that a grant ofwaiver would be in the public interest, and must include in its application its plans and activities for modifying its fleet, including evidence of good faith efforts to comply with the requirements of § 91.865 or § 91.867. The application should contain all information the applicant considers relevant, including, as appropriate, the following:

(1)   The applicant's balance sheet and cash flow positions; (2)   The composition of the applicant's current fleet; and

(3)   The applicant's delivery position with respect to new airplanes or noise-abatement equipment.

Waivers will be granted only upon a showing bythe applicant that compliance with the requirements of § 91.865 or 91.867 at a particular interim compliance date is financially onerous, physically impossible, or technologically infeasible, or that it would have an adverse effect on competition or on service to small communities.

The conditions of any waiver granted under this section shall be determined by the circumstances presented in the application, but in no case may the term extend beyond the next interim compliance date.

A summary of any request for a waiver under this section will be published in the FEDERAL REGISTER, and public comment will be invited. Unless the Secretary finds that circumstances require otherwise, the public comment period will be at least 14 days.

[Docket 26433, 56 FR 48660, Sept. 25, 1991]

§ 91.873 Waivers from final compliance.

(a)  A U.S. air carrier or a foreign air carrier may apply for a waiver from the prohibition contained in § 91.853 of this part for its remaining Stage 2 airplanes, provided that, by July 1, 1999, at least 85 percent of the airplanes used by the carrier to provide service to or from an airport in the contiguous United States will comply with the Stage 3 noise levels.

(b)

(c) (d)

(e) (f)

An application for the waiver described in paragraph (a) ofthis section must be filed with the Secretary of Transportation no later than January 1, 1999, or, in the case of a foreign air carrier, no later than April 20, 2000. Such application must include a plan with firm orders for replacing or modifying all airplanes to comply with Stage 3 noise levels at the earliest practicable time.

To be eligible to apply for the waiver under this section, a new entrant U.S. air carrier must initiate service no later than January 1, 1999, and must comply fully with all provisions of this section.

The Secretary may grant a waiver under this section if the Secretary finds that granting such waiver is in the public interest. In making such a finding, the Secretary shall include consideration ofthe effect of granting such waiver on competition in the air carrier industry and the effect on small community air service, and any other information submitted by the applicant that the Secretary considers relevant.

The term of any waiver granted under this section shall be determined bythe circumstances presented in the application, but in no case will the waiver permit the operation of any Stage 2 airplane covered by this subchapter in the contiguous United States after December 31, 2003.

A summary of any request for a waiver under this section will be published in the FEDERAL REGISTER, and public comment will be invited. Unless the secretary finds that circumstances require otherwise, the public comment period will be at least 14 days.

14 CFR91.873(f) (enhanceddisplay)

page 144of243

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

[Docket 26433, 56 FR 48660, Sept. 25, 1991; 56 FR 51167 Oct. 10, 1991; Amdt. 91-276, 67 FR 46571, July 15, 2002]

14 CFR91.875

§ 91.875Annual progress reports.

(a)  Each operator subject to § 91.865 or § 91.867 of this chapter shall submit an annual report to the FAA, Office of Environment and Energy, on the progress it has made toward complying with the requirements of that section. Such reports shall be submitted no later than 45 days after the end of a calendar year. All progress reports must provide the information through the end of the calendar year, be certified by the operator as true and complete (under penalty of 18 U.S.C. 1001), and include the following information:

(1) (2)

(3)

The name and address of the operator;

The name, title, and telephone number ofthe person designated by the operator to be responsible for ensuring the accuracy of the information in the report;

The operator's progress during the reporting period toward compliance with the requirements of § 91.853, § 91.865 or § 91.867. For airplanes on U.S. operations specifications, each operator shall identify the airplanes by type, model, series, and serial number.

(i) (ii)

(iii) (iv)

Each Stage 2 airplane added or removed from operation or U.S. operations specifications (grouped separately by those airplanes acquired with and without base level);

Each Stage 2 airplane modified to Stage 3 noise levels (identifying the manufacturer and model of noise abatement retrofit equipment;

Each Stage 3 airplane on U.S. operations specifications as of the last day ofthe reporting period; and

For each Stage 2 airplane transferred or acquired, the name and address ofthe recipient or transferor; and, if base level was transferred, the person to or from whom base level was transferred or acquired pursuant to Section 91.863 along with the effective date of each base level transaction, and the type of base level transferred or acquired.

(b)

Each operator subject to § 91.865 or § 91.867 of this chapter shall submit an initial progress report covering the period from January 1, 1990, through December 31, 1991, and provide:

(1)

For each operator subject to § 91.865:

(i) (ii)

The date used to establish its base level pursuant to § 91.861(a); and

A list of those Stage 2 airplanes (by type, model, series and serial number) in its base level, including adjustments made pursuant to § 91.861 after the date its base level was established.

(2)  For each U.S. operator:

(i) (ii)

A plan to meet the compliance schedules in § 91.865 or § 91.867 and the final compliance date of § 91.853, including the schedule for delivery of replacement Stage 3 airplanes or the installation of noise abatement retrofit equipment; and

A separate list (by type, model, series, and serial number) ofthose airplanes included in the operator's base level, pursuant to § 91.861(a)(1) (i) and (ii), underthe categories “returned” or “purchased,” along with the date each was added to its operations specifications.

14 CFR91.875(b)(2)(ii)(enhanceddisplay)

page 145of243

(b)

(c)

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.875(c)

(c)

(d) (e)

(f)

Each operator subject to § 91.865 or § 91.867 of this chapter shall submit subsequent annual progress reports covering the calendar year preceding the report and including any changes in the information provided in paragraphs (a) and (b) of this section; including the use of any carry-forward credits pursuant to § 91.869.

An operator may request, in any report, that specific planning data be considered proprietary.

If an operator's actions during any reporting period cause it to achieve compliance with § 91.853, the report should include a statement to that effect. Further progress reports are not required unless there is any change in the information reported pursuant to paragraph (a) of this section.

For each U.S. operator subject to § 91.865, progress reports submitted for calendar years 1994, 1996, and 1998, shall also state how the operator achieved compliance with the requirements of that section, i.e.—

(1) (2)

By reducing the number of Stage 2 airplanes in its fleet to no more than the maximum permitted percentage of its base level under § 91.865(b), or

By operating a fleet that consists of at least the minimum required percentage of Stage 3 airplanes under § 91.865(d).

(Approved by the Office ofManagement and Budget under control number2120-0553)

[Docket 26433, 56 FR 48660, Sept. 25, 1991; 56 FR 51168, Oct. 10, 1991, as amendedby57FR 5977, Feb. 19, 1992]

§ 91.877 Annual reporting  ofHawaiianoperations.

(a)  Each air carrier or foreign air carrier subject to § 91.865 or § 91.867 of this part that conducts operations between the contiguous United States and the State of Hawaii, between the State of Hawaii and any point outside ofthe contiguous United States, or between the islands of Hawaii in turnaround service, on or since November 5, 1990, shall include in its annual report the information described in paragraph (c) of this section.

Each air carrier or foreign air carrier not subject to § 91.865 or § 91.867 of this part that conducts operations between the contiguous U.S. and the State of Hawaii, between the State of Hawaii and any point outside of the contiguous United States, or between the islands of Hawaii in turnaround service, on or since November 5, 1990, shall submit an annual report to the FAA, Office of Environment and Energy, on its compliance with the Hawaiian operations provisions of 49 U.S.C. 47528. Such reports shall be submitted no laterthan 45 days after the end of a calendar year. All progress reports must provide the information through the end of the calendar year, be certified by the operator as true and complete (under penalty of 18 U.S.C. 1001), and include the following information—

(1)  The name and address of the air carrier or foreign air carrier;

(2)

The name, title, and telephone number ofthe person designated by the air carrier or foreign air carrier to be responsible for ensuring the accuracy ofthe information in the report; and

(3)   The information specified in paragraph (c) of this section.

The following information must be included in reports filed pursuant to this section—

(1)

For operations conducted between the contiguous United States and the State of Hawaii—

(i)  The number of Stage 2 airplanes used to conduct such operations as of November 5, 1990;

14 CFR91.877(c)(1)(i)(enhanced display)

page 146of243

(2) (3)

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR 91.877(c)(1)(ii)

(ii)   Any change to that number during the calendaryear being reported, including the date of such change;

For air carriers that conduct inter-island turnaround service in the State of Hawaii—

(i)

The number of Stage 2 airplanes used to conduct such operations as of November 5, 1990;

(ii)   Any change to that number during the calendaryear being reported, including the date of such change;

(iii)  For an air carrier that provided inter-island trunaround service within the state of Hawaii on November 5, 1990, the number reported under paragraph (c)(2)(i) ofthis section may include all Stage 2 airplanes with a maximum certificated takeoff weight of more than 75,000 pounds that were owned or leased by the air carrier on November 5, 1990, regardless of whether such airplanes were operated by that air carrier or foreign air carrier on that date.

For operations conducted between the State of Hawaii and a point outside the contiguous United States—

(i)

The number of Stage 2 airplanes used to conduct such operations as of November 5, 1990; and

(d)

(ii)   Any change to that number during the calendaryear being reported, including the date of such change.

Reports or amended reports for years predating this regulation are required to be filed concurrently with the next annual report.

[Docket 28213, 61 FR 66185, Dec. 16, 1996]

§§ 91.879-91.880 [Reserved]

§ 91.881Final compliance: Civilsubsonicjet airplanesweighing 75,000 pounds or less.

Except as provided in § 91.883, after December 31, 2015, a person may not operate to or from an airport in the contiguous United States a civil subsonic jet airplane subject to § 91.801(e) of this subpart that weighs less than 75,000 pounds unless that airplane has been shown to comply with Stage 3, Stage 4, or Stage 5 noise levels.

[Docket FAA-2015-3782, Amdt. 91-349, 82 FR 46132, Oct. 4, 2017]

§ 91.883 Special flightauthorizations  forjetairplanesweighing 75,000 pounds orless.

(a)  After December 31, 2015, an operator of a jet airplane weighing 75,000 pounds or less that does not comply with Stage 3 noise levels may, when granted a special flight authorization by the FAA, operate that airplane in the contiguous United States only for one of the following purposes:

(1)

To sell, lease, or use the airplane outside the 48 contiguous States;

(2)  To scrap the airplane;

(3) (4)

To obtain modifications to the airplane to meet Stage 3, Stage 4, or Stage 5 noise levels.

To perform scheduled heavy maintenance or significant modifications on the airplane at a maintenance facility located in the contiguous 48 States;

14 CFR91.883(a)(4)(enhanced display)

page 147of243

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.883(a)(5)

(b)

(5)   To deliverthe airplane to an operator leasing the airplane from the owner or return the airplane to the lessor;

(6)   To prepare, park, or store the airplane in anticipation of any ofthe activities described in paragraphs (a)(1) through (a)(5) ofthis section;

(7)   To provide transport of persons and goods in the relief of an emergency situation; or

(8)   To divert the airplane to an alternative airport in the 48 contiguous States on account ofweather, mechanical, fuel, air traffic control, or other safety reasons while conducting a flight in order to perform any ofthe activities described in paragraphs (a)(1) through (a)(7) ofthis section.

An operator of an affected airplane may apply for a special flight authorization for one of the purposes listed in paragraph (a) of this section by filing an application with the FAA's Office of Environment and Energy. Except for emergency relief authorizations sought under paragraph (a)(7) of this section, applications must be filed at least 30 days in advance ofthe planned flight. All applications must provide the information necessary for the FAA to determine that the planned flight is within the limits prescribed in the law.

[Docket FAA-2013-0503, 78 FR 39583, July2, 2013, as amendedbyDocket FAA-2015-3782, Amdt. 91-349, 82 FR 46132, Oct. 4, 2017]

§§ 91.884-91.899 [Reserved]

Subpart J—Waivers § 91.901 [Reserved]

§ 91.903 Policy andprocedures.

(a)   The Administrator may issue a certificate of waiver authorizing the operation of aircraft in deviation from any rule listed in this subpart or any rule listed in this subpart as modified by subpart C of part 194 of this chapter if the Administratorfinds that the proposed operation can be safely conducted under the terms of that certificate of waiver.

(b)

(c)

An application for a certificate of waiver under this part is made on a form and in a manner prescribed by the Administrator and may be submitted to any FAA office.

A certificate of waiver is effective as specified in that certificate of waiver.

[Docket 18334, 54 FR 34325, Aug. 18, 1989, as amendedbyFAA-2023-1275,Amdt. 91-379, 89 FR 92486, Nov. 21, 2024]

§ 91.905 List ofrules subjectto waivers. Sec.

91.107 Use of safety belts.

91.111 Operating near other aircraft.

91.113 Right-of-way rules: Except water operations.

14 CFR91.905(enhanceddisplay)

page 148 of243

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

91.115 Right-of-way rules: Water operations. 91.117 Aircraft speed.

91.119 Minimum safe altitudes: General. 91.121 Altimeter settings.

91.123 Compliance with ATC clearances and instructions. 91.125 ATC light signals.

91.126 Operating on or in the vicinity of an airport in Class G airspace. 91.127 Operating on or in the vicinity of an airport in Class E airspace. 91.129 Operations in Class D airspace.

91.130 Operations in Class C airspace. 91.131 Operations in Class B airspace. 91.133 Restricted and prohibited areas. 91.135 Operations in Class A airspace. 91.137 Temporary flight restrictions.

91.141 Flight restrictions in the proximity ofthe Presidential and other parties. 91.143 Flight limitation in the proximity of space flight operations.

91.153 VFR flight plan: Information required. 91.155 Basic VFR weather minimums 91.157 Special VFR weather minimums. 91.159 VFR cruising altitude or flight level. 91.169 IFR flight plan: Information required.

91.173 ATC clearance and flight plan required. 91.175 Takeoff and landing under IFR.

14 CFR91.905(enhanceddisplay)

14 CFR91.905

page 149 of243

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.907-91.999

91.176 Operations below DA/DH or MDA using an enhanced flight vision system (EFVS) under IFR. 91.177 Minimum altitudes for IFR operations.

91.179 IFR cruising altitude or flight level. 91.181 Course to be flown.

91.183 IFR radio communications.

91.185 IFR operations: Two-way radio communications failure.

91.187 Operation under IFR in controlled airspace: Malfunction reports. 91.209 Aircraft lights.

91.303 Aerobatic flights. 91.305 Flight test areas.

91.311 Towing: Other than under § 91.309.

91.313(e) Restricted category civil aircraft: Operating limitations. 91.515 Flight altitude rules.

91.707 Flights between Mexico or Canada and the United States. 91.713 Operation of civil aircraft of Cuban registry.

[Docket 18334, 54 FR 34325, Aug. 18, 1989, as amendedbyAmdt. 91-227, 56 FR 65661, Dec. 17, 1991; Docket FAA-2013-0485, Amdt. 91-345, 81 FR 90175, Dec. 13, 2016; DocketFAA-2016-9154, Amdt. 91-348, 82 FR 39664, Aug. 22, 2017]

§§ 91.907-91.999 [Reserved]

Subpart K—FractionalOwnership Operations

Source:

Docket FAA-2001-10047, 68 FR 54561, Sept. 17, 2003, unless otherwise noted.

§ 91.1001 Applicability.

(a)   This subpart prescribes rules, in addition to those prescribed in other subparts of this part, that apply to fractional owners and fractional ownership program managers governing—

(1)   The provision of program management services in a fractional ownership program;

(2)   The operation of a fractional ownership program aircraft in a fractional ownership program; and

14 CFR91.1001(a)(2)(enhanceddisplay)

page 150 of243

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.1001(a)(3)

(3)

The operation of a program aircraft included in a fractional ownership program managed by an affiliate of the manager of the program to which the owner belongs.

(b)

As used in this part—

(1)

(2) (3)

(4) (5)

(6)

Affiliate ofa program manager means a managerthat, directly, or indirectly, through one or more intermediaries, controls, is controlled by, or is under common control with, another program manager. The holding of at least forty percent (40 percent) of the equity and forty percent (40 percent) ofthe voting power of an entity will be presumed to constitute control for purposes of determining an affiliation underthis subpart.

A dry-lease aircraft exchange means an arrangement, documented by the written program agreements, under which the program aircraft are available, on an as needed basis without crew, to each fractional owner.

A fractional owneror owner means an individual or entitythat possesses a minimum fractional ownership interest in a program aircraft and that has entered into the applicable program agreements; provided, however, that in the case of the flight operations described in paragraph (b)(6)(ii) ofthis section, and solely for purposes of requirements pertaining to those flight operations, the fractional owner operating the aircraft will be deemed to be a fractional owner in the program managed by the affiliate.

A fractional ownership interest means the ownership of an interest or holding of a multi-year leasehold interest and/or a multi-year leasehold interest that is convertible into an ownership interest in a program aircraft.

A fractional ownership program orprogram means any system of aircraft ownership and exchange that consists of all of the following elements:

(i)   The provision for fractional ownership program management services by a single fractional ownership program manager on behalf ofthe fractional owners.

(ii)   Two or more airworthy aircraft.

(iii)  One or more fractional owners per program aircraft, with at least one program aircraft having more than one owner.

(iv)  Possession of at least a minimum fractional ownership interest in one or more program aircraft by each fractional owner.

(v)   A dry-lease aircraft exchange arrangement among all ofthe fractional owners.

(vi)  Multi-year program agreements covering the fractional ownership, fractional ownership program management services, and dry-lease aircraft exchange aspects of the program.

A fractional ownership program aircraft or program aircraft means:

(i)   An aircraft in which a fractional owner has a minimal fractional ownership interest and that has been included in the dry-lease aircraft exchange pursuant to the program agreements, or

(ii)   In the case of a fractional owner from one program operating an aircraft in a different fractional ownership program managed by an affiliate ofthe operating owner's program manager, the aircraft being operated by the fractional owner, so long as the aircraft is:

14 CFR91.1001(b)(6)(ii)(enhanceddisplay)

page 151of243

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.1001(b)(6)(ii)(A)

(7) (8)

(9)

(A)   Included in the fractional ownership program managed bythe affiliate of the operating owner's program manager, and

(B)  Included in the operating owner's program's dry-lease aircraft exchange pursuant to the program agreements ofthe operating owner's program.

(iii)   An aircraft owned in whole or in part by the program managerthat has been included in the dry- lease aircraft exchange and is used to supplement program operations.

A Fractional Ownership Program Flight or Program Flight means a flight under this subpart when one or more passengers or property designated by a fractional owner are on board the aircraft.

Fractional ownership program managementservices orprogram management services mean administrative and aviation support services furnished in accordance with the applicable requirements ofthis subpart or provided bythe program manager on behalf ofthe fractional owners, including, but not limited to, the—

(i)   Establishment and implementation of program safety guidelines;

(ii)   Employment, furnishing, or contracting of pilots and other crewmembers; (iii)  Training and qualification of pilots and other crewmembers and personnel; (iv)  Scheduling and coordination ofthe program aircraft and crews;

(v)   Maintenance of program aircraft;

(vi)   Satisfaction of recordkeeping requirements;

(vii) Development and use of a program operations manual and procedures; and

(viii)  Application for and maintenance of management specifications and other authorizations and approvals.

A fractional ownership program manager orprogram manager means the entitythat offers fractional ownership program management services to fractional owners, and is designated in the multi-year program agreements referenced in paragraph (b)(5)(vi) of this section to fulfill the requirements of this chapter applicable to the manager of the program containing the aircraft being flown. When a fractional owner is operating an aircraft in a fractional ownership program managed by an affiliate of the owner's program manager, the references in this subpart to the flight-related responsibilities of the program manager apply, with respect to that particular flight, to the affiliate ofthe owner's program manager rather than to the owner's program manager.

(10) A minimum fractionalownership interest means—

(i) (ii)

A fractional ownership interest equal to, or greater than, one-sixteenth (1⁄16) of at least one subsonic, fixed-wing or powered-lift program aircraft; or

A fractional ownership interest equal to, or greater than, one-thirty-second (1⁄32) of at least one rotorcraft program aircraft.

(c)

The rules in this subpart that refer to a fractional owner or a fractional ownership program manager also apply to any person who engages in an operation governed by this subpart without the management specifications required by this subpart.

14 CFR91.1001(c)(enhanceddisplay)

page 152of243

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.1002

[Docket FAA-2001-10047, 68 FR 54561, Sept. 17, 2003, as amendedbyDocketFAA-2022-1355,Amdt. 91-366, 87 FR 75846, Dec. 9, 2022]

§ 91.1002 Compliance date.

No person that conducted flights before November 17, 2003 under a program that meets the definition offractional ownership program in § 91.1001 may conduct such flights after February 17, 2005 unless it has obtained management specifications under this subpart.

[Docket FAA-2001-10047, 68 FR 54561, Sept. 17, 2003; 69 FR 74413, Dec. 14, 2004]

§ 91.1003 Managementcontractbetween  owner andprogram manager. Each owner must have a contract with the program manager that—

(a) (b)

(c)

(d)

Requires the program manager to ensure that the program conforms to all applicable requirements ofthis chapter.

Provides the owner the right to inspect and to audit, or have a designee of the owner inspect and audit, the records of the program manager pertaining to the operational safety of the program and those records required to show compliance with the management specifications and all applicable regulations. These records include, but are not limited to, the management specifications, authorizations, approvals,

manuals, log books, and maintenance records maintained bythe program manager.

Designates the program manager as the owner's agent to receive service of notices pertaining to the program that the FAA seeks to provide to owners and authorizes the FAA to send such notices to the program manager in its capacity as the agent ofthe owner for such service.

Acknowledges the FAA's right to contact the owner directly ifthe Administrator determines that direct contact is necessary.

§ 91.1005 Prohibitions andlimitations.

(a)  Except as provided in § 91.321 or § 91.501, no owner may carry persons or property for compensation or hire on a program flight.

(b) (c)

During the term of the multi-year program agreements under which a fractional owner has obtained a minimum fractional ownership interest in a program aircraft, the flight hours used during that term by the owner on program aircraft must not exceed the total hours associated with the fractional owner's share of ownership.

No person may sell or lease an aircraft interest in a fractional ownership program that is smaller than that prescribed in the definition of “minimum fractional ownership interest” in § 91.1001(b)(10) unless flights associated with that interest are operated under part 121 or 135 of this chapter and are conducted by an air carrier or commercial operator certificated under part 119 of this chapter.

§ 91.1007Flights conducted under part 121 orpart 135 of this chapter.

(a)   Except as provided in § 91.501(b), when a nonprogram aircraft is used to substitute for a program flight, the flight must be operated in compliance with part 121 or part 135 of this chapter, as applicable.

14 CFR91.1007(a) (enhanceddisplay)

page 153 of243

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.1007(b)

(b) (c)

A program manager who holds a certificate under part 119 of this chapter may conduct a flight for the use of a fractional owner under part 121 or part 135 of this chapter ifthe aircraft is listed on that certificate holder's operations specifications for part 121 or part 135, as applicable.

The fractional owner must be informed when a flight is being conducted as a program flight or is being conducted under part 121 or part 135 of this chapter.

OPERATIONAL CONTROL

§ 91.1009 Clarificationofoperational control.

(a)   An owner is in operational control of a program flight when the owner—

(1)   Has the rights and is subject to the limitations set forth in §§ 91.1003 through 91.1013;

(2)  Has directed that a program aircraft carry passengers or property designated by that owner; and (3)   The aircraft is carrying those passengers or property.

(b)   An owner is not in operational control of a flight in the following circumstances:

(1)   A program aircraft is used for a flight for administrative purposes such as demonstration, positioning, ferrying, maintenance, or crew training, and no passengers or property designated by such owner are being carried; or

(2)   The aircraft being used for the flight is being operated under part 121 or 135 of this chapter. § 91.1011 Operational control responsibilities and delegation.

(a)  Each owner in operational control of a program flight is ultimately responsible for safe operations and for complying with all applicable requirements ofthis chapter, including those related to airworthiness and operations in connection with the flight. Each owner may delegate some or all of the performance ofthe tasks associated with carrying out this responsibilityto the program manager, and may rely on the program manager for aviation expertise and program management services. When the owner delegates performance of tasks to the program manager or relies on the program manager's expertise, the owner and the program manager arejointly and individually responsible for compliance.

(b)

The management specifications, authorizations, and approvals required bythis subpart are issued to, and in the sole name of, the program manager on behalf ofthe fractional owners collectively. The management specifications, authorizations, and approvals will not be affected by any change in ownership of a program aircraft, as long as the aircraft remains a program aircraft in the identified program.

§ 91.1013 Operational  controlbriefingandacknowledgment.

(a)  Upon the signing of an initial program management services contract, or a renewal or extension of a program management services contract, the program manager must brief the fractional owner on the owner's operational control responsibilities, and the owner must review and sign an acknowledgment of these operational control responsibilities. The acknowledgment must be included with the program management services contract. The acknowledgment must define when a fractional owner is in operational control and the owner's responsibilities and liabilities under the program. These include:

(1)   Responsibility for compliance with the management specifications and all applicable regulations.

14 CFR91.1013(a)(1)(enhanced display)

page 154of243

(b) (c)

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.1013(a)(2)

(2) (3)

Enforcement actions for any noncompliance.

Liability risk in the event of a flight-related occurrence that causes personal injury or property damage.

(b) (c)

The fractional owner's signature on the acknowledgment will serve as the owner's affirmation that the owner has read, understands, and accepts the operational control responsibilities described in the acknowledgment.

Each program manager must ensure that the fractional owner or owner's representatives have access to the acknowledgments for such owner's program aircraft. Each program manager must ensure that the FAA has access to the acknowledgments for all program aircraft.

PROGRAM MANAGEMENT

§ 91.1014 Issuing or denying management specifications.

(a)   A person applying to the Administrator for management specifications under this subpart must submit an application—

(1) (2)

In a form and manner prescribed by the Administrator; and

Containing any information the Administrator requires the applicant to submit.

Management specifications will be issued to the program manager on behalf ofthe fractional owners if, after investigation, the Administrator finds that the applicant:

(1) (2)

Meets the applicable requirements of this subpart; and

Is properly and adequately equipped in accordance with the requirements of this chapter and is able to conduct safe operations under appropriate provisions of part 91 of this chapter and management specifications issued under this subpart.

An application for management specifications will be denied if the Administrator finds that the applicant is not properly or adequately equipped or is not able to conduct safe operations under this part.

§ 91.1015 Managementspecifications.

(a)  Each person conducting operations underthis subpart or furnishing fractional ownership program management services to fractional owners must do so in accordance with management specifications issued bythe Administrator to the fractional ownership program manager under this subpart. Management specifications must include:

(1) (2)

(3) (4)

The current list of all fractional owners and types of aircraft, registration markings and serial numbers;

The authorizations, limitations, and certain procedures under which these operations are to be conducted,

Certain other procedures under which each class and size of aircraft is to be operated;

Authorization for an inspection program approved under § 91.1109, including the type of aircraft, the registration markings and serial numbers of each aircraft to be operated under the program. No person may conduct any program flight using any aircraft not listed.

14 CFR91.1015(a)(4)(enhanceddisplay)

page 155of243

(b) (c) (d) (e)

(f)

(g) (h)

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.1015(a)(5)

(5) (6)

(7) (8) (9)

Time limitations, or standards for determining time limitations, for overhauls, inspections, and checks for airframes, engines, propellers, rotors, appliances, and emergency equipment of aircraft.

The specific location of the program manager's principal base of operations and, if different, the address that will serve as the primary point of contact for correspondence between the FAA and the program manager and the name and mailing address ofthe program manager's agent for service;

Other business names the program manager may use;

Authorization for the method of controlling weight and balance of aircraft;

Any authorized deviation and exemption that applies to the person conducting operations underthis subpart; and

(10)  Any other information the Administrator determines is necessary.

The program manager may keep the current list of all fractional owners required by paragraph (a)(1) of this section at its principal base of operation or other location approved by the Administrator and referenced in its management specifications. Each program manager shall make this list of owners available for inspection by the Administrator.

Management specifications issued underthis subpart are effective unless—

(1) (2)

The management specifications are amended as provided in § 91.1017; or The Administrator suspends or revokes the management specifications.

At least 30 days before it proposes to establish or change the location of its principal base of operations, its main operations base, or its main maintenance base, a program manager must provide written notification to the Flight Standards office that issued the program manager's management specifications.

Each program manager must maintain a complete and separate set of its management specifications at its principal base of operations, or at a place approved by the Administrator, and must make its management specifications available for inspection by the Administrator and the fractional owner(s) to whom the program manager furnishes its services for review and audit.

Each program manager must insert pertinent excerpts of its management specifications, or references thereto, in its program manual and must—

(1) (2)

Clearly identify each such excerpt as a part of its management specifications; and

State that compliance with each management specifications requirement is mandatory.

Each program manager must keep each of its employees and other persons who perform duties material to its operations informed of the provisions of its management specifications that applyto that employee's or person's duties and responsibilities.

A program manager may obtain approval to provide a temporary document verifying a flightcrew member's airman certificate and medical certificate privileges under an approved certificate verification plan set forth in the program manager's management specifications. A document provided by the program manager may be carried as an airman certificate or medical certificate on flights within the United States for up to 72 hours.

[Docket FAA-2001-10047, 68 FR 54561, Sept. 17, 2003, as amendedbyDocketFAA-2018-0119,Amdt. 91-350, 83 FR 9171, Mar. 5, 2018; Amdt. 60-6, 83 FR 30282, June 27, 2018; Docket FAA-2022-1563, Amdt. 91-370, 88 FR 48087, July26, 2023]

14 CFR91.1015(h) (enhanceddisplay)

page 156of243

(b)

(c)

(1) (2)

(3) (4)

(1)

(2)

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.1017

§ 91.1017Amending program manager's management specifications.

(a)   The Administrator may amend any management specifications issued under this subpart if—

(1)   The Administrator determines that safety and the public interest require the amendment of any management specifications; or

(2)

The program manager applies for the amendment of any management specifications, and the Administrator determines that safety and the public interest allows the amendment.

Except as provided in paragraph (e) of this section, when the Administrator initiates an amendment of a program manager's management specifications, the following procedure applies:

The Flight Standards office that issued the program manager's management specifications will notify the program manager in writing of the proposed amendment.

The Flight Standards office that issued the program manager's management specifications will set a reasonable period (but not less than 7 days) within which the program manager may submit written information, views, and arguments on the amendment.

After considering all material presented, the Flight Standards office that issued the program manager's management specifications will notify the program manager of—

(i) (ii)

The adoption of the proposed amendment,

The partial adoption ofthe proposed amendment, or

(iii)  The withdrawal ofthe proposed amendment.

If the Flight Standards office that issued the program manager's management specifications issues an amendment of the management specifications, it becomes effective not less than 30 days after the program manager receives notice of it unless—

(i)

(ii)

The Flight Standards office that issued the program manager's management specifications finds under paragraph (e) ofthis section that there is an emergency requiring immediate action with respect to safety; or

The program manager petitions for reconsideration of the amendment under paragraph (d) of this section.

When the program manager applies for an amendment to its management specifications, the following procedure applies:

The program manager must file an application to amend its management specifications—

(i)

(ii)

At least 90 days before the date proposed by the applicant for the amendment to become effective, unless a shortertime is approved, in cases such as mergers, acquisitions of operational assets that require an additional showing of safety (for example, proving tests or validation tests), and resumption of operations following a suspension of operations as a result of bankruptcy actions.

At least 15 days before the date proposed by the applicant for the amendment to become effective in all other cases.

The application must be submitted to the Flight Standards office that issued the program manager's management specifications in a form and manner prescribed by the Administrator.

14 CFR91.1017(c)(2)(enhanceddisplay)

page 157of243

(d) (e)

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.1017(c)(3)

(3)

(4)

After considering all material presented, the Flight Standards office that issued the program manager's management specifications will notify the program manager of—

(i)   The adoption of the applied for amendment;

(ii)   The partial adoption ofthe applied for amendment; or

(iii)  The denial of the applied for amendment. The program manager may petition for reconsideration of a denial under paragraph (d) ofthis section.

If the Flight Standards office that issued the program manager's management specifications approves the amendment, following coordination with the program manager regarding its implementation, the amendment is effective on the date the Administrator approves it.

When a program manager seeks reconsideration of a decision of the Flight Standards office that issued the program manager's management specifications concerning the amendment of management specifications, the following procedure applies:

(1)

(2) (3)

(4)

The program manager must petition for reconsideration of that decision within 30 days ofthe date that the program manager receives a notice of denial ofthe amendment of its management specifications, or of the date it receives notice of an FAA-initiated amendment of its management specifications, whichever circumstance applies.

The program manager must address its petition to the Executive Director, Flight Standards Service.

A petition for reconsideration, if filed within the 30-day period, suspends the effectiveness of any amendment issued bythe Flight Standards office that issued the program manager's management specifications unless that office has found, under paragraph (e) ofthis section, that an emergency exists requiring immediate action with respect to safety.

If a petition for reconsideration is not filed within 30 days, the procedures of paragraph (c) of this section apply.

If the Flight Standards office that issued the program manager's management specifications finds that an emergency exists requiring immediate action with respect to safety that makes the procedures set out in this section impracticable or contrary to the public interest—

(1) (2)

The Flight Standards office amends the management specifications and makes the amendment effective on the day the program manager receives notice of it; and

In the notice to the program manager, the Flight Standards office will articulate the reasons for its finding that an emergency exists requiring immediate action with respect to safety or that makes it impracticable or contrary to the public interest to stay the effectiveness of the amendment.

[Docket FAA-2001-10047, 68 FR 54561, Sept. 17, 2003, as amendedbyDocketFAA-2018-0119,Amdt. 91-350, 83 FR 9171, Mar. 5, 2018]

§ 91.1019 Conductingtests andinspections.

(a)   At any time or place, the Administrator may conduct an inspection or test, otherthan an en route inspection, to determine whether a program manager underthis subpart is complying with title 49 ofthe United States Code, applicable regulations, and the program manager's management specifications.

(b)

The program manager must—

14 CFR91.1019(b) (enhanceddisplay)

page 158 of243

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.1019(b)(1)

(c)

(d) (e)

(1)   Make available to the Administrator at the program manager's principal base of operations, or at a place approved by the Administrator, the program manager's management specifications; and

(2)   Allow the Administrator to make any test or inspection, other than an en route inspection, to determine compliance respecting any matter stated in paragraph (a) of this section.

Each employee of, or person used by, the program manager who is responsible for maintaining the program manager's records required by or necessaryto demonstrate compliance with this subpart must make those records available to the Administrator.

The Administrator may determine a program manager's continued eligibilityto hold its management specifications on any grounds listed in paragraph (a) of this section, or any other appropriate grounds.

Failure by any program manager to make available to the Administrator upon request, the management specifications, or any required record, document, or report is grounds for suspension of all or any part of the program manager's management specifications.

§ 91.1021Internalsafetyreporting  andincident/accidentresponse.

(a)  Each program manager must establish an internal anonymous safety reporting procedure that fosters an environment of safety without any potential for retribution for filing the report.

(b)

Each program manager must establish procedures to respond to an aviation incident/accident.

§ 91.1023 Program operating manualrequirements.

(a)  Each program manager must prepare and keep current a program operating manual setting forth procedures and policies acceptable to the Administrator. The program manager's management, flight, ground, and maintenance personnel must use this manual to conduct operations underthis subpart. However, the Administrator may authorize a deviation from this paragraph if the Administrator finds that, because of the limited size ofthe operation, part ofthe manual is not necessary for guidance of management, flight, ground, or maintenance personnel.

(b) (c)

(d)

(e)

(f) (g)

Each program manager must maintain at least one copy ofthe manual at its principal base of operations.

No manual may be contrary to any applicable U.S. regulations, foreign regulations applicable to the program flights in foreign countries, orthe program manager's management specifications.

The program manager must make a copy ofthe manual, or appropriate portions ofthe manual (and changes and additions), available to its maintenance and ground operations personnel and must furnish the manual to—

(1)   Its crewmembers; and

(2)  Representatives ofthe Administrator assigned to the program manager.

Each employee ofthe program manager to whom a manual or appropriate portions of it are furnished under paragraph (d)(1) of this section must keep it up-to-date with the changes and additions furnished to them.

The program manager must ensure the appropriate parts ofthe manual are accessible to flight, ground, and maintenance personnel at all times when such personnel are performing their assigned duties.

The information and instructions contained in the manual must be displayed clearly and be retrievable in the English language.

14 CFR91.1023(g) (enhanceddisplay)

page 159 of243

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR 91.1023(h)

(h)

Program managers that are also certificated to operate under part 121 or 135 of this chapter may be authorized to use the operating manual required by those parts to meet the manual requirements of subpart K, provided:

(1)   The policies and procedures are consistent for both operations, or

(2)  When policies and procedures are different, the applicable policies and procedures are identified and used.

[Docket FAA-2001-10047, 68 FR 54561, Sept. 17, 2003, as amendedbyDocketFAA-2022-0912,Amdt. 91-368, 88 FR 34443, May 30, 2023]

§ 91.1025 Program operating manual contents.

Each program operating manual accessed in paper format must display the date of last revision on each page. Each program operating manual accessed in electronic format must displaythe date of last revision in a manner in which a person can immediately ascertain it. Unless otherwise authorized bythe Administrator, the manual must include the following:

(a) (b)

(c) (d)

(e) (f)

(g)

(h) (i)

(j) (k)

Procedures for ensuring compliance with aircraft weight and balance limitations;

Copies of the program manager's management specifications or appropriate extracted information, including area of operations authorized, category and class of aircraft authorized, crew complements, and types of operations authorized;

Procedures for complying with accident notification requirements;

Procedures for ensuring that the pilot in command knows that required airworthiness inspections have been made and that the aircraft has been approved for return to service in compliance with applicable maintenance requirements;

Procedures for reporting and recording mechanical irregularities that come to the attention of the pilot in command before, during, and after completion of a flight;

Procedures to be followed by the pilot in command for determining that mechanical irregularities or defects reported for previous flights have been corrected or that correction of certain mechanical irregularities or defects have been deferred;

Procedures to be followed by the pilot in command to obtain maintenance, preventive maintenance, and servicing of the aircraft at a place where previous arrangements have not been made by the program manager or owner, when the pilot is authorized to so act forthe operator;

Procedures under § 91.213 forthe release of, and continuation of flight if any item of equipment required for the particular type of operation becomes inoperative or unserviceable en route;

Procedures for refueling aircraft, eliminating fuel contamination, protecting from fire (including electrostatic protection), and supervising and protecting passengers during refueling;

Procedures to be followed by the pilot in command in the briefing under § 91.1035.

Procedures for ensuring compliance with emergency procedures, including a list of the functions assigned each category of required crewmembers in connection with an emergency and emergency evacuation duties;

14 CFR91.1025(k)(enhanceddisplay)

page 160 of243

(l) (m)

(n) (o)

(p)

(q) (r)

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.1025(l)

The approved aircraft inspection program, when applicable;

Procedures forthe evacuation of persons who may need the assistance of another person to move expeditiously to an exit if an emergency occurs;

Procedures for performance planning that take into account take off, landing and en route conditions;

An approved Destination Airport Analysis, when required by § 91.1037(c), that includes the following elements, supported by aircraft performance data supplied by the aircraft manufacturer for the appropriate runway conditions—

(1)   Pilot qualifications and experience;

(2)

Aircraft performance data to include normal, abnormal and emergency procedures as supplied by the aircraft manufacturer;

(3)   Airport facilities and topography;

(4)  Runway conditions (including contamination);

(5) (6)

Airport or area weather reporting;

Appropriate additional runway safety margins, if required;

(7)  Airplane inoperative equipment; (8)   Environmental conditions; and

(9)

Other criteria that affect aircraft performance.

A suitable system (which may include a coded or electronic system) that provides for preservation and retrieval of maintenance recordkeeping information required by § 91.1113in a manner acceptable to the Administrator that provides—

(1) (2)

(3)

A description (or reference to date acceptable to the Administrator) of the work performed:

The name of the person performing the work if the work is performed by a person outside the organization of the program manager; and

The name or other positive identification ofthe individual approving the work.

Flight locating and scheduling procedures; and

Other procedures and policy instructions regarding program operations that are issued by the program manager or required bythe Administrator.

[Docket FAA-2001-10047, 68 FR 54561, Sept. 17, 2003, as amendedbyDocketFAA-2022-0912,Amdt. 91-368, 88 FR 34443, May 30, 2023]

§ 91.1027Recordkeeping.

(a)  Each program manager must keep at its principal base of operations or at other places approved bythe Administrator, and must make available for inspection bythe Administrator all of the following:

(1) (2)

The program manager's management specifications.

A current list of the aircraft used or available for use in operations under this subpart, the operations for which each is equipped (for example, RNP5/10, RVSM.).

14 CFR91.1027(a)(2)(enhanceddisplay)

page 161of243

(3)

(4) (5)

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.1027(a)(3)

An individual record of each pilot used in operations under this subpart, including the following information:

(i) (ii)

(iii)

(iv) (v)

(vi) (vii)

The full name of the pilot.

The pilot certificate (bytype and number) and ratings that the pilot holds.

The pilot's aeronautical experience in sufficient detail to determine the pilot's qualifications to pilot aircraft in operations under this subpart.

The pilot's current duties and the date of the pilot's assignment to those duties. The effective date and class of the medical certificate that the pilot holds.

The date and result of each of the initial and recurrent competency tests and proficiency checks required by this subpart and the type of aircraft flown during that test or check.

The pilot's flight time in sufficient detail to determine compliance with the flight time limitations of this subpart.

(viii) The pilot's check pilot authorization, if any.

(ix) (x)

Any action taken concerning the pilot's release from employment for physical or professional disqualification; and

The date ofthe satisfactory completion of initial, transition, upgrade, and differences training and each recurrent training phase required by this subpart.

An individual record for each flight attendant used in operations under this subpart, including the following information:

(i) (ii)

The full name of the flight attendant, and

The date and result oftraining required by § 91.1063, as applicable.

A current list of all fractional owners and associated aircraft. This list or a reference to its location must be included in the management specifications and should be of sufficient detail to determine the minimum fractional ownership interest of each aircraft.

(b)

(c)

Each program manager must keep each record required by paragraph (a)(2) of this section for at least 6 months, and must keep each record required by paragraphs (a)(3) and (a)(4) ofthis section for at least 12 months. When an employee is no longer employed or affiliated with the program manager or fractional owner, each record required by paragraphs (a)(3) and (a)(4) of this section must be retained for at least

12 months.

Each program manager is responsible forthe preparation and accuracy of a load manifest in duplicate containing information concerning the loading ofthe aircraft. The manifest must be prepared before each takeoff and must include—

(1) (2) (3) (4)

The number of passengers;

The total weight ofthe loaded aircraft;

The maximum allowable takeoffweight  for that flight; The center of gravity limits;

14 CFR91.1027(c)(4)(enhanceddisplay)

page 162of243

(b)

(c)

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.1027(c)(5)

(5)

(6)

The center of gravity of the loaded aircraft, except that the actual center of gravity need not be computed ifthe aircraft is loaded according to a loading schedule or other approved method that ensures that the center of gravity of the loaded aircraft is within approved limits. In those cases, an entry must be made on the manifest indicating that the center of gravity is within limits according to a loading schedule or other approved method;

The registration number ofthe aircraft or flight number;

(7)  The origin and destination; and

(8)

Identification of crewmembers and their crew position assignments.

(d) (e)

(f) (g)

The pilot in command of the aircraft for which a load manifest must be prepared must carry a copy of the completed load manifest in the aircraft to its destination. The program manager must keep copies of completed load manifest for at least 30 days at its principal operations base, or at another location used by it and approved by the Administrator.

Each program manager is responsible for providing a written document that states the name ofthe entity having operational control on that flight and the part ofthis chapter under which the flight is operated. The pilot in command of the aircraft must carry a copy ofthe document in the aircraft to its destination. The program manager must keep a copy of the document for at least 30 days at its principal operations base, or at another location used by it and approved by the Administrator.

Records may be kept either in paper or other form acceptable to the Administrator.

Program managers that are also certificated to operate under part 121 or 135 of this chapter may satisfy the recordkeeping requirements of this section and of § 91.1113with records maintained to fulfill equivalent obligations under part 121 or 135 of this chapter.

[Docket FAA-2001-10047, 68 FR 54561, Sept. 17, 2003, as amendedbyDocketFAA-2016-9154,Amdt. 91-348, 82 FR 39664, Aug. 22, 2017]

§ 91.1029 Flight scheduling and locating requirements.

(a)  Each program manager must establish and use an adequate system to schedule and release program aircraft.

Except as provided in paragraph (d) ofthis section, each program manager must have adequate procedures established for locating each flight, for which a flight plan is not filed, that—

(1) (2) (3)

Provide the program manager with at least the information required to be included in a VFR flight plan;

Provide for timely notification of an FAA facility or search and rescue facility, if an aircraft is overdue or missing; and

Provide the program manager with the location, date, and estimated time for reestablishing radio or telephone communications, ifthe flight will operate in an area where communications cannot be maintained.

Flight locating information must be retained at the program manager's principal base of operations, or at other places designated by the program manager in the flight locating procedures, until the completion of the flight.

14 CFR91.1029(c)(enhanced display)

page 163 of243

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.1029(d)

(d)

The flight locating requirements of paragraph (b) of this section do not apply to a flight for which an FAA flight plan has been filed and the flight plan is canceled within 25 nautical miles ofthe destination airport.

§ 91.1031Pilot in commandor second incommand: Designationrequired. (a)  Each program manager must designate a—

(1)   Pilot in command for each program flight; and

(2)   Second in command for each program flight requiring two pilots.

(b)   The pilot in command, as designated bythe program manager, must remain the pilot in command at all times during that flight.

§ 91.1033 Operating information required.

(a)  Each program manager must, for all program operations, provide the following materials, in current and appropriate form, accessible to the pilot at the pilot station, and the pilot must use them—

(1)   A cockpit checklist;

(2)   For multiengine aircraft or for aircraft with retractable landing gear, an emergency cockpit checklist containing the procedures required by paragraph (c) of this section, as appropriate;

(3)   At least one set of pertinent aeronautical charts; and

(4)  For IFR operations, at least one set of pertinent navigational en route, terminal area, and instrument approach procedure charts.

(b)  Each cockpit checklist required by paragraph (a)(1) ofthis section must contain the following procedures: (1)   Before starting engines;

(2)   Before takeoff; (3)  Cruise;

(4)  Before landing; (5)   After landing; and (6)  Stopping engines.

(c)   Each emergency cockpit checklist required by paragraph (a)(2) of this section must contain the following procedures, as appropriate:

(1)  Emergency operation of fuel, hydraulic, electrical, and mechanical systems. (2)  Emergency operation of instruments and controls.

(3)  Engine inoperative procedures.

(4)  Any other emergency procedures necessary for safety. § 91.1035 Passenger awareness.

(a)   Prior to each takeoff, the pilot in command of an aircraft carrying passengers on a program flight must ensure that all passengers have been orally briefed on—

14 CFR91.1035(a)(enhanceddisplay)

page 164of243

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.1035(a)(1)

(1) (2)

(3) (4) (5) (6)

(7) (8)

Smoking: Each passenger must be briefed on when, where, and under what conditions smoking is prohibited. This briefing must include a statement, as appropriate, that the regulations require passenger compliance with lighted passenger information signs and no smoking placards, prohibit smoking in lavatories, and require compliance with crewmember instructions with regard to these items;

Use ofsafetybelts, shoulder harnesses, andchild restraint systems: Each passenger must be briefed on when, where and under what conditions it is necessaryto have his or her safety belt and, if installed, his or her shoulder harness fastened about him or her, and if a child is being transported, the appropriate use of child restraint systems, if available. This briefing must include a statement, as appropriate, that the regulations require passenger compliance with the lighted passenger information sign and/or crewmember instructions with regard to these items;

The placement of seat backs in an upright position before takeoff and landing; Location and means for opening the passenger entry door and emergency exits; Location of survival equipment;

Ditching procedures and the use of flotation equipment required under § 91.509 for a flight over water;

The normal and emergency use of oxygen installed in the aircraft; and Location and operation of fire extinguishers.

(b)

(c) (d) (e)

(f)

Prior to each takeoff, the pilot in command of an aircraft carrying passengers on a program flight must ensure that each person who may need the assistance of another person to move expeditiouslyto an exit if an emergency occurs and that person's attendant, if any, has received a briefing as to the procedures to be followed if an evacuation occurs. This paragraph does not apply to a person who has been given a briefing before a previous leg of that flight in the same aircraft.

Prior to each takeoff, the pilot in command must advise the passengers of the name of the entity in operational control of the flight.

The oral briefings required by paragraphs (a), (b), and (c) of this section must be given by the pilot in command or another crewmember.

The oral briefing required by paragraph (a) of this section may be delivered by means of an approved recording playback device that is audible to each passenger under normal noise levels.

The oral briefing required by paragraph (a) of this section must be supplemented by printed cards that must be carried in the aircraft in locations convenient for the use of each passenger. The cards must—

(1) (2) (3)

Be appropriate forthe aircraft on which they are to be used;

Contain a diagram of, and method of operating, the emergency exits; and

Contain other instructions necessary for the use of emergency equipment on board the aircraft.

14 CFR91.1035(f)(3)(enhanceddisplay)

page 165of243

(a) (b)

(c)

(d)

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.1037

§ 91.1037 Large transport categoryairplanes: Turbine engine powered; Limitations; Destination and alternate airports.

No program manager or any other person may permit a turbine engine powered large transport category airplane on a program flight to take offthat airplane at a weight that (allowing for normal consumption of fuel and oil in flight to the destination or alternate airport) the weight of the airplane on arrival would exceed the landing weight in the Airplane Flight Manual for the elevation of the destination or alternate airport and the ambient temperature expected at the time of landing.

Except as provided in paragraph (c) of this section, no program manager or any other person may permit a turbine engine powered large transport category airplane on a program flight to take off that airplane unless its weight on arrival, allowing for normal consumption of fuel and oil in flight (in accordance with the landing distance in the Airplane Flight Manual for the elevation of the destination airport and the wind conditions expected there at the time of landing), would allow a full stop landing at the intended destination airport within 60 percent ofthe effective length of each runway described below from a point 50 feet above the intersection of the obstruction clearance plane and the runway. For the purpose of determining the allowable landing weight at the destination airport, the following is assumed:

(1)

The airplane is landed on the most favorable runway and in the most favorable direction, in still air.

(2)  The airplane is landed on the most suitable runway considering the probable wind velocity and direction and the ground handling characteristics ofthat airplane, and considering other conditions such as landing aids and terrain.

A program manager or other person flying a turbine engine powered large transport category airplane on a program flight may permit that airplane to take off at a weight in excess of that allowed by paragraph (b) of this section if all of the following conditions exist:

(1)   The operation is conducted in accordance with an approved Destination Airport Analysis in that person's program operating manual that contains the elements listed in § 91.1025(o).

(2)   The airplane's weight on arrival, allowing for normal consumption of fuel and oil in flight (in accordance with the landing distance in the Airplane Flight Manual for the elevation of the destination airport and the wind conditions expected there at the time of landing), would allow a full stop landing at the intended destination airport within 80 percent of the effective length of each runway described below from a point 50 feet above the intersection of the obstruction clearance plane and the runway. For the purpose of determining the allowable landing weight at the destination airport, the following is assumed:

(i)

The airplane is landed on the most favorable runway and in the most favorable direction, in still air.

(ii)   The airplane is landed on the most suitable runway considering the probable wind velocity and direction and the ground handling characteristics ofthat airplane, and considering other conditions such as landing aids and terrain.

(3)   The operation is authorized by management specifications.

No program manager or other person may select an airport as an alternate airport for a turbine engine powered large transport category airplane unless (based on the assumptions in paragraph (b) of this section) that airplane, at the weight expected at the time of arrival, can be brought to a full stop landing within 80 percent of the effective length of the runway from a point 50 feet above the intersection ofthe obstruction clearance plane and the runway.

14 CFR91.1037(d) (enhanceddisplay)

page 166of243

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.1037(e)

(e)

Unless, based on a showing of actual operating landing techniques on wet runways, a shorter landing distance (but never less than that required by paragraph (b) or (c) ofthis section) has been approved for a specific type and model airplane and included in the Airplane Flight Manual, no person maytake off a turbojet airplane when the appropriate weather reports or forecasts, or any combination of them, indicate that the runways at the destination or alternate airport may be wet or slippery at the estimated time of arrival unless the effective runway length at the destination airport is at least 115 percent of the runway length required under paragraph (b) or (c) of this section.

§ 91.1039 IFRtakeoff, approach andlandingminimums.

(a)   No pilot on a program aircraft operating a program flight may begin an instrument approach procedure to an airport unless—

(1)  Either that airport or the alternate airport has a weather reporting facility operated by the U.S. National Weather Service, a source approved by the U.S. National Weather Service, or a source approved by the Administrator; and

(2)   The latest weather report issued by the weather reporting facility includes a current local altimeter setting for the destination airport. If no local altimeter setting is available at the destination airport, the pilot must obtain the current local altimeter setting from a source provided bythe facility designated on the approach chart for the destination airport.

(b) (c)

(d) (e)

For flight planning purposes, ifthe destination airport does not have a weather reporting facility described in paragraph (a)(1) of this section, the pilot must designate as an alternate an airport that has a weather reporting facility meeting that criteria.

The MDA or Decision Altitude and visibility landing minimums prescribed in part 97 of this chapter or in the program manager's management specifications are increased by 100 feet and 1⁄2 mile respectively, but not to exceed the ceiling and visibility minimums for that airport when used as an alternate airport, for each pilot in command of a turbine-powered aircraft who has not served at least 100 hours as pilot in command in that type of aircraft.

No person may take off an aircraft under IFR from an airport where weather conditions are at or above takeoff minimums but are below authorized IFR landing minimums unless there is an alternate airport within one hour's flying time (at normal cruising speed, in still air) of the airport of departure.

Except as provided in § 91.176 of this chapter, each pilot making an IFR takeoff or approach and landing at an airport must comply with applicable instrument approach procedures and takeoff and landing weather minimums prescribed by the authority having jurisdiction over the airport. In addition, no pilot may take off at that airport when the visibility is less than 600 feet, unless otherwise authorized in the program manager's management specifications for EFVS operations.

[Docket FAA-2001-10047, 68 FR 54561, Sept. 17, 2003, as amendedbyDocketFAA-2013-0485,Amdt. 91-345, 81 FR 90175, Dec. 13, 2016]

§ 91.1041Aircraft proving andvalidationtests.

(a)   No program manager may permit the operation of an aircraft, other than a turbojet aircraft, for which two pilots are required by the type certification requirements of this chapter for operations under VFR, if it has not previously proved such an aircraft in operations underthis part in at least 25 hours of proving tests acceptable to the Administrator including—

14 CFR91.1041(a)(enhanceddisplay)

page 167of243

(b)

(c) (d)

(e)

(f) (g)

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.1041(a)(1)

(1) (2)

(3)

Five hours of night time, if night flights are to be authorized;

Five instrument approach procedures under simulated or actual conditions, if IFR flights are to be authorized; and

Entry into a representative number of en route airports as determined by the Administrator.

No program manager may permit the operation of a turbojet airplane if it has not previously proved a turbojet airplane in operations under this part in at least 25 hours of proving tests acceptable to the Administrator including—

(1) (2)

(3)

Five hours of night time, if night flights are to be authorized;

Five instrument approach procedures under simulated or actual conditions, if IFR flights are to be authorized; and

Entry into a representative number of en route airports as determined by the Administrator.

No program manager may carry passengers in an aircraft during proving tests, except those needed to make the tests and those designated by the Administrator to observe the tests. However, pilot flight training may be conducted during the proving tests.

Validation testing is required to determine that a program manager is capable of conducting operations safely and in compliance with applicable regulatory standards. Validation tests are required for the following authorizations:

(1)

The addition of an aircraft for which two pilots are required for operations under VFR or a turbojet airplane, if that aircraft or an aircraft of the same make or similar design has not been previously proved orvalidated in operations under this part.

(2)  Operations outside U.S. airspace. (3)   Class II navigation authorizations.

(4)

Special performance or operational authorizations.

Validation tests must be accomplished by test methods acceptable to the Administrator. Actual flights may not be required when an applicant can demonstrate competence and compliance with appropriate regulations without conducting a flight.

Proving tests and validation tests may be conducted simultaneously when appropriate.

The Administrator may authorize deviations from this section if the Administratorfinds that special circumstances make full compliance with this section unnecessary.

§ 91.1043 [Reserved]

§ 91.1045 Additional equipment requirements.

No person may operate a program aircraft on a program flight unless the aircraft is equipped with the following—

(a)

Airplanes having a passenger-seat configuration of more than 30 seats or a payload capacity of more than 7,500 pounds:

(1)

A cockpit voice recorder as required by § 121.359 of this chapter as applicable to the aircraft specified in that section.

14 CFR91.1045(a)(1) (enhanceddisplay)

page 168 of243

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.1045(a)(2)

(2) (3) (4) (5)

A flight recorder as required by § 121.343 or § 121.344 of this chapter as applicable to the aircraft specified in that section.

A terrain awareness and warning system as required by § 121.354 of this chapter as applicable to the aircraft specified in that section.

A traffic alert and collision avoidance system as required by § 121.356 of this chapter as applicable to the aircraft specified in that section.

Airborne weather radar as required by § 121.357 of this chapter, as applicable to the aircraft specified in that section.

(b)

Airplanes having a passenger-seat configuration of 30 seats or fewer, excluding each crewmember, and a payload capacity of 7,500 pounds or less, and any rotorcraft (as applicable):

(1) (2) (3) (4) (5)

A cockpit voice recorder as required by § 135.151 of this chapter as applicable to the aircraft specified in that section.

A flight recorder as required by § 135.152 of this chapter as applicable to the aircraft specified in that section.

A terrain awareness and warning system as required by § 135.154 of this chapter as applicable to the aircraft specified in that section.

A traffic alert and collision avoidance system as required by § 135.180 of this chapter as applicable to the aircraft specified in that section.

As applicable to the aircraft specified in that section, either:

(i)   Airborne thunderstorm detection equipment as required by § 135.173 of this chapter; or (ii)   Airborne weather radar as required by § 135.175 of this chapter.

§ 91.1047 Drug andalcohol misuse education program.

(a)  Each program manager must provide each direct employee performing flight crewmember, flight attendant, flight instructor, or aircraft maintenance duties with drug and alcohol misuse education.

(b) (c)

No program manager may use any contract employee to perform flight crewmember, flight attendant, flight instructor, or aircraft maintenance duties forthe program manager unless that contract employee has been provided with drug and alcohol misuse education.

Program managers must disclose to their owners and prospective owners the existence of a company drug and alcohol misuse testing program. Ifthe program manager has implemented a companytesting program, the program manager's disclosure must include the following:

(1) (2)

(3)

Information on the substances that theytest for, for example, alcohol and a list of the drugs;

The categories of employees tested, the types of tests, for example, pre-employment, random, reasonable cause/suspicion, post accident, return to duty and follow-up; and

The degree to which the program manager's company testing program is comparable to the federally mandated drug and alcohol testing program required under part 120 of this chapter regarding the information in paragraphs (c)(1) and (c)(2) ofthis section.

14 CFR91.1047(c)(3)(enhanceddisplay)

page 169 of243

(d)

(e) (f)

(b)

(c) (d)

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.1047(d)

If a program aircraft is operated on a program flight into an airport at which no maintenance personnel are available that are subject to the requirements of paragraphs (a) or (b) of this section and emergency maintenance is required, the program manager may use persons not meeting the requirements of paragraphs (a) or (b) of this section to provide such emergency maintenance under both of the following conditions:

(1)

(2)

The program manager must notifythe Drug Abatement Program Division, AAM-800, 800 Independence Avenue, SW., Washington, DC 20591 in writing within 10 days after being provided emergency maintenance in accordance with this paragraph. The program manager must retain copies of all such written notifications  fortwo years.

The aircraft must be reinspected by maintenance personnel who meet the requirements of paragraph (a) or (b) ofthis section when the aircraft is next at an airport where such maintenance personnel are available.

For purposes of this section, emergency maintenance means maintenance that—

(1) (2)

Is not scheduled, and

Is made necessary by an aircraft condition not discovered prior to the departure for that location.

Notwithstanding paragraphs (a) and (b) ofthis section, drug and alcohol misuse education conducted under an FAA-approved drug and alcohol misuse prevention program may be used to satisfy these requirements.

[Docket FAA-2001-10047, 68 FR 54561, Sept. 17, 2003, as amendedbyAmdt. 91-307, 74 FR 22653, May 14, 2009]

§ 91.1049 Personnel.

(a)  Each program manager and each fractional owner must use in program operations on program aircraft flight crews meeting § 91.1053 criteria and qualified under the appropriate regulations. The program manager must provide oversight ofthose crews.

Each program manager must employ (either directly or by contract) an adequate number of pilots per program aircraft. Flight crew staffing must be determined based on the following factors, at a minimum:

(1) (2)

Number of program aircraft.

Program manager flight, duty, and rest time considerations, and in all cases within the limits set forth in §§ 91.1057 through 91.1061.

(3)   Vacations.

(4)   Operational efficiencies. (5)  Training.

(6)

Single pilot operations, if authorized by deviation under paragraph (d) of this section.

Each program manager must publish pilot and flight attendant duty schedules sufficiently in advance to follow the flight, duty, and rest time limits in §§ 91.1057 through 91.1061 in program operations.

Unless otherwise authorized by the Administrator, when any program aircraft is flown in program operations with passengers onboard, the crew must consist of at least two qualified pilots employed or contracted by the program manager or the fractional owner.

14 CFR91.1049(d)(enhanceddisplay)

page 170 of243

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.1049(e)

(e)

The program manager must ensure that trained and qualified scheduling or flight release personnel are on duty to schedule and release program aircraft during all hours that such aircraft are available for program operations.

§ 91.1050 Employment offormerFAA employees.

(a)   Except as specified in paragraph (c) of this section, no fractional owner or fractional ownership program manager may knowingly employ or make a contractual arrangement which permits an individual to act as an agent or representative ofthe fractional owner or fractional ownership program manager in any matter before the Federal Aviation Administration ifthe individual, in the preceding 2 years—

(1)  Served as, or was directly responsible for the oversight of, a Flight Standards Service aviation safety inspector; and

(2)   Had direct responsibility to inspect, or oversee the inspection of, the operations of the fractional owner or fractional ownership program manager.

(b)

(c)

For the purpose of this section, an individual shall be considered to be acting as an agent or representative of a fractional owner or fractional ownership program manager in a matter before the agency if the individual makes any written or oral communication on behalf of the fractional owner or fractional ownership program manager to the agency (or any of its officers or employees) in connection with a particular matter, whether or not involving a specific party and without regard to whether the individual has participated in, or had responsibility for, the particular matter while serving as a Flight Standards Service aviation safety inspector.

The provisions of this section do not prohibit a fractional owner or fractional ownership program manager from knowingly employing or making a contractual arrangement which permits an individual to act as an agent or representative ofthe fractional owner or fractional ownership program manager in any matter before the Federal Aviation Administration  ifthe individual was employed by the fractional owner or fractional ownership program manager before October 21, 2011.

[Docket FAA-2008-1154, 76 FR 52235, Aug. 22, 2011]

§ 91.1053 Crewmember experience.

(a)  No program manager or owner may use any person, nor may any person serve, as a pilot in command or second in command of a program aircraft, or as a flight attendant on a program aircraft, in program operations underthis subpart unless that person has met the applicable requirements of part 61 of this chapter and has the following experience and ratings:

(1)   Total flight time for all pilots:

(i)

Pilot in command—A minimum of 1,500 hours.

(ii)   Second in command—A minimum of 500 hours.

(2)   For multi-engine turbine-powered fixed-wing and powered-lift aircraft, the following FAA certification and ratings requirements:

(i)

Pilot in command—Airline transport pilot and applicable type ratings not limited to VFR only.

(ii)   Second in command—Commercial pilot and instrument ratings.

(iii)  Flight attendant (if required or used)—Appropriately trained personnel. 14 CFR91.1053(a)(2)(iii)(enhanced display)

page 171of243

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.1053(a)(3)

(3)

For all other aircraft, the following FAA certification and rating requirements: (i)   Pilot in command—Commercial pilot and instrument ratings.

(ii)   Second in command—Commercial pilot and instrument ratings.

(iii)  Flight attendant (if required or used)—Appropriately trained personnel.

(b)

The Administrator may authorize deviations from paragraph (a)(1) of this section ifthe Flight Standards office that issued the program manager's management specifications finds that the crewmember has comparable experience, and can effectively perform the functions associated with the position in accordance with the requirements ofthis chapter. Grants of deviation underthis paragraph may be granted after consideration of the size and scope of the operation, the qualifications ofthe intended personnel and the circumstances set forth in § 91.1055(b)(1) through (3). The Administrator may, at any time, terminate any grant of deviation authority issued under this paragraph.

[Docket FAA-2001-10047, 68 FR 54561, Sept. 17, 2003, as amendedbyDocketFAA-2018-0119,Amdt. 91-350, 83 FR 9171, Mar. 5, 2018; FAA-2023-1275, Amdt. 91-379, 89 FR 92486, Nov. 21, 2024]

§ 91.1055 Pilot operating limitations  andpairingrequirement.

(a)   If the second in command of a fixed-wing program aircraft has fewer than 100 hours of flight time as second in command flying in the aircraft make and model and, if a type rating is required, in the type aircraft being flown, and the pilot in command is not an appropriately qualified check pilot, the pilot in command shall make all takeoffs and landings in any of the following situations:

(1)

Landings at the destination airport when a Destination Airport Analysis is required by § 91.1037(c); and

(b)

(2)   In any ofthe following conditions:

(i)   The prevailing visibility forthe airport is at or below 3⁄4 mile.

(ii)  The runway visual range for the runway to be used is at or below 4,000 feet.

(iii)  The runway to be used has water, snow, slush, ice or similar contamination that may adversely affect aircraft performance.

(iv)  The braking action on the runway to be used is reported to be less than “good.” (v)   The crosswind component for the runway to be used is in excess of 15 knots. (vi)  Windshear is reported in the vicinity ofthe airport.

(vii)  Any other condition in which the pilot in command determines it to be prudent to exercise the pilot in command's authority.

No program manager may release a program flight under this subpart unless, for that aircraft make or model and, if a type rating is required, for that type aircraft, either the pilot in command or the second in command has at least 75 hours of flight time, either as pilot in command or second in command. The Administrator may, upon application by the program manager, authorize deviations from the requirements of this paragraph by an appropriate amendment to the management specifications in any ofthe following circumstances:

14 CFR91.1055(b)(enhanceddisplay)

page 172of243

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.1055(b)(1)

(1) (2) (3)

A newly authorized program manager does not employ any pilots who meet the minimum requirements ofthis paragraph.

An existing program manager adds to its fleet a new category and class aircraft not used before in its operation.

An existing program manager establishes a new base to which it assigns pilots who will be required to become qualified on the aircraft operated from that base.

(c)   No person may be assigned in the capacity of pilot in command in a program operation to more than two aircraft types that require a separate type rating.

§ 91.1057 Flight, duty and resttime requirements: Allcrewmembers. (a)  For purposes of this subpart—

Augmented flight crew means at least three pilots.

Calendar day means the period of elapsed time, using Coordinated Universal Time or local time that begins at midnight and ends 24 hours later at the next midnight.

Duty period means the period of elapsed time between reporting for an assignment involving flight time and release from that assignment by the program manager. All time between these two points is part ofthe duty period, even if flight time is interrupted by nonflight-related duties. The time is calculated using either Coordinated Universal Time or local time to reflect the total elapsed time.

Extension offlight time means an increase in the flight time because of circumstances beyond the control of the program manager or flight crewmember (such as adverse weather) that are not known at the time of departure and that prevent the flightcrew from reaching the destination within the planned flight time.

Flight attendant means an individual, other than a flight crewmember, who is assigned by the program manager, in accordance with the required minimum crew complement underthe program manager's management specifications or in addition to that minimum complement, to duty in an aircraft during flight time and whose duties include but are not necessarily limited to cabin-safety-related responsibilities.

Multi-time zone flight means an easterly or westerly flight or multiple flights in one direction in the same duty period that results in a time zone difference of 5 or more hours and is conducted in a geographic area that is south of 60 degrees north latitude and north of 60 degrees south latitude.

Reserve status means that status in which a flight crewmember, by arrangement with the program manager: Holds himself or herself fit to fly to the extent that this is within the control of the flight crewmember; remains within a reasonable response time of the aircraft as agreed between the flight crewmember and the program manager; and maintains a ready means whereby the flight crewmember may be contacted by the program manager. Reserve status is not part of any duty period or rest period.

Rest period means a period oftime required pursuant to this subpart that is free of all responsibility for work or duty prior to the commencement of, or following completion of, a duty period, and during which the flight crewmember or flight attendant cannot be required to receive contact from the program manager. A rest period does not include any time during which the program manager imposes on a flight crewmember or flight attendant any duty or restraint, including any actual work or present responsibility for work should the occasion arise.

14 CFR91.1057(a) “Restperiod”(enhanceddisplay)

page 173 of243

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.1057(a)“Standby”

(b)

(c) (d)

(e)

(f) (g) (h)

(i) (j)

Standby means that portion of a duty period during which a flight crewmember is subject to the control of the program manager and holds himself or herself in a condition of readiness to undertake a flight. Standby is not part of any rest period.

A program manager may assign a crewmember and a crewmember may accept an assignment for flight time only when the applicable requirements ofthis section and §§ 91.1059-91.1062 are met.

No program manager may assign any crewmember to any duty during any required rest period.

Time spent in transportation, not local in character, that a program manager requires of a crewmember and provides to transport the crewmember to an airport at which he or she is to serve on a flight as a crewmember, or from an airport at which he or she was relieved from dutyto return to his or her home station, is not considered part of a rest period.

A flight crewmember may continue a flight assignment if the flight to which he or she is assigned would normally terminate within the flight time limitations, but because of circumstances beyond the control of the program manager or flight crewmember (such as adverse weather conditions), is not at the time of departure expected to reach its destination within the planned flight time. The extension of flight time under this paragraph may not exceed the maximum time limits set forth in § 91.1059.

Each flight assignment must provide for at least 10 consecutive hours of rest during the 24-hour period that precedes the completion time ofthe assignment.

The program manager must provide each crewmember at least 13 rest periods of at least 24 consecutive hours each in each calendar quarter.

A flight crewmember may decline a flight assignment if, in the flight crewmember's determination, to do so would not be consistent with the standard of safe operation required under this subpart, this part, and applicable provisions ofthis title.

Any rest period required by this subpart may occur concurrently with any other rest period.

If authorized by the Administrator, a program manager may use the applicable unscheduled flight time limitations, duty period limitations, and rest requirements of part 121 or part 135 of this chapter instead of the flight time limitations, duty period limitations, and rest requirements of this subpart.

§ 91.1059 Flighttime limitations  andrestrequirements: One or twopilot crews.

(a)   No program manager may assign any flight crewmember, and no flight crewmember may accept an assignment, for flight time as a member of a one- or two-pilot crew if that crewmember's total flight time in all commercial flying will exceed—

(1)  500 hours in any calendar quarter;

(2)   800 hours in any two consecutive calendar quarters; (3)  1,400 hours in any calendar year.

(b)

Except as provided in paragraph (c) of this section, during any 24 consecutive hours the total flight time of the assigned flight, when added to any commercial flying by that flight crewmember, may not exceed—

(1)   8 hours for a flight crew consisting of one pilot; or

(2)   10 hours for a flight crew consisting of two pilots qualified under this subpart for the operation being conducted.

14 CFR91.1059(b)(2) (enhanceddisplay)

page 174of243

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.1059(c)

(c)

No program manager may assign any flight crewmember, and no flight crewmember may accept an assignment, if that crewmember's flight time or duty period will exceed, or rest time will be less than—

(1) Minimum Rest Immediately Before Duty (2) Duty Period

(3) Flight Time For 1 Pilot (4) Flight Time For 2 Pilots

(5) Minimum After Duty Rest

(6) Minimum After Duty Rest Period for Multi-Time Zone Flights

Normal duty

10 Hours Up to 14 Hours

Up to 8 Hours Up to 10 Hours

10 Hours 14 Hours

Extension offlight time

10 Hours.

Up to 14 Hours.

Exceeding 8 Hours up to 9 Hours.

Exceeding 10 Hours up to 12 Hours.

12 Hours. 18 Hours.

§ 91.1061Augmentedflight crews.

(a)   No program manager may assign any flight crewmember, and no flight crewmember may accept an assignment, for flight time as a member of an augmented crew if that crewmember's total flight time in all commercial flying will exceed—

(1) (2) (3)

500 hours in any calendar quarter;

800 hours in any two consecutive calendar quarters; 1,400 hours in any calendar year.

(b)

No program manager may assign any pilot to an augmented crew, unless the program manager ensures:

(1) (2) (3)

(4)

Adequate sleeping facilities are installed on the aircraft for the pilots.

No more than 8 hours of flight deck duty is accrued in any 24 consecutive hours. For a three-pilot crew, the crew must consist of at least the following:

(i)   A pilot in command (PIC) who meets the applicable flight crewmember requirements of this subpart and § 61.57 of this chapter.

(ii)   A PIC qualified pilot who meets the applicable flight crewmember requirements ofthis subpart and § 61.57(c) and (d) of this chapter.

(iii)  A second in command (SIC) who meets the SIC qualifications of this subpart. For flight under IFR, that person must also meet the recent instrument experience requirements of part 61 of this chapter.

For a four-pilot crew, at least three pilots who meet the conditions of paragraph (b)(3) of this section, plus a fourth pilot who meets the SIC qualifications of this subpart. For flight under IFR, that person must also meet the recent instrument experience requirements of part 61 of this chapter.

14 CFR91.1061(b)(4)(enhanced display)

page 175of243

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.1061(c)

(c)

No program manager may assign any flight crewmember, and no flight crewmember may accept an assignment, if that crewmember's flight time or duty period will exceed, or rest time will be less than—

(1) Minimum Rest Immediately Before Duty (2) Duty Period

(3) Flight Time

(4) Minimum After Duty Rest

(5) Minimum After Duty Rest Period for Multi-Time

Zone Flights

3-Pilot crew 10 Hours

Up to 16 Hours Up to 12 Hours 12 Hours

18 hours

4-Pilot crew 10 Hours

Up to 18 Hours Up to 16 Hours 18 Hours

24 hours

§ 91.1062 Dutyperiods and rest requirements: Flight attendants.

(a)   Except as provided in paragraph (b) ofthis section, a program manager may assign a duty period to a flight attendant onlywhen the assignment meets the applicable duty period limitations and rest requirements ofthis paragraph.

(1) (2)

(3)

(4) (5)

(6)

Except as provided in paragraphs (a)(4), (a)(5), and (a)(6) ofthis section, no program manager may assign a flight attendant to a scheduled duty period of more than 14 hours.

Except as provided in paragraph (a)(3) ofthis section, a flight attendant scheduled to a duty period of 14 hours or less as provided under paragraph (a)(1) of this section must be given a scheduled rest period of at least 9 consecutive hours. This rest period must occur between the completion of the scheduled duty period and the commencement of the subsequent duty period.

The rest period required under paragraph (a)(2) ofthis section may be scheduled or reduced to 8 consecutive hours ifthe flight attendant is provided a subsequent rest period of at least 10 consecutive hours; this subsequent rest period must be scheduled to begin no later than 24 hours after the beginning ofthe reduced rest period and must occur between the completion of the scheduled duty period and the commencement of the subsequent duty period.

A program manager may assign a flight attendant to a scheduled duty period of more than 14 hours, but no more than 16 hours, ifthe program manager has assigned to the flight or flights in that duty period at least one flight attendant in addition to the minimum flight attendant complement required for the flight or flights in that duty period under the program manager's management specifications.

A program manager may assign a flight attendant to a scheduled duty period of more than 16 hours, but no more than 18 hours, ifthe program manager has assigned to the flight or flights in that duty period at least two flight attendants in addition to the minimum flight attendant complement required forthe flight or flights in that duty period under the program manager's management specifications.

A program manager may assign a flight attendant to a scheduled duty period of more than 18 hours, but no more than 20 hours, ifthe scheduled duty period includes one or more flights that land or take off outside the 48 contiguous states and the District of Columbia, and if the program manager has assigned to the flight or flights in that duty period at least three flight attendants in addition to the minimum flight attendant complement required forthe flight or flights in that duty period under the program manager's management specifications.

14 CFR91.1062(a)(6)(enhanceddisplay)

page 176of243

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.1062(a)(7)

(7) (8) (9)

Except as provided in paragraph (a)(8) ofthis section, a flight attendant scheduled to a duty period of more than 14 hours but no more than 20 hours, as provided in paragraphs (a)(4), (a)(5), and (a)(6) of this section, must be given a scheduled rest period of at least 12 consecutive hours. This rest period must occur between the completion ofthe scheduled duty period and the commencement of the subsequent duty period.

The rest period required under paragraph (a)(7) ofthis section may be scheduled or reduced to 10 consecutive hours ifthe flight attendant is provided a subsequent rest period of at least 14 consecutive hours; this subsequent rest period must be scheduled to begin no later than 24 hours after the beginning ofthe reduced rest period and must occur between the completion of the scheduled duty period and the commencement of the subsequent duty period.

Notwithstanding paragraphs (a)(4), (a)(5), and (a)(6) ofthis section, if a program manager elects to reduce the rest period to 10 hours as authorized by paragraph (a)(8) of this section, the program manager may not schedule a flight attendant for a duty period of more than 14 hours during the 24-hour period commencing afterthe beginning ofthe reduced rest period.

(b)

Notwithstanding paragraph (a) of this section, a program manager may applythe flight crewmember flight time and duty limitations and rest requirements of this part to flight attendants for all operations conducted under this part provided that the program manager establishes written procedures that—

(1) (2)

(3)

(4)

Apply to all flight attendants used in the program manager's operation;

Include the flight crewmember rest and duty requirements of §§ 91.1057, 91.1059, and 91.1061, as appropriate to the operation being conducted, except that rest facilities on board the aircraft are not required;

Include provisions to add one flight attendant to the minimum flight attendant complement for each flight crewmember who is in excess ofthe minimum number required in the aircraft type certificate data sheet and who is assigned to the aircraft under the provisions of § 91.1061; and

Are approved by the Administrator and described or referenced in the program manager's management specifications.

§ 91.1063 Testing andtraining: Applicability andtermsused. (a)  Sections 91.1065 through 91.1107:

(1)   Prescribe the tests and checks required for pilots and flight attendant approval of check pilots in operations under this subpart;

crewmembers and forthe

(2)

(3) (4)

Prescribe the requirements for establishing and maintaining an approved training program for crewmembers, check pilots and instructors, and other operations personnel employed or used by the program manager in program operations;

Prescribe the requirements for the qualification, approval and use of aircraft simulators and flight training devices in the conduct of an approved training program; and

Permits training center personnel authorized under part 142 of this chapter who meet the requirements of § 91.1075 to conduct training, testing and checking under contract or other arrangements to those persons subject to the requirements of this subpart.

14 CFR91.1063(a)(4)(enhanceddisplay)

page 177of243

(b)

(c) (d)

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.1063(b)

If authorized by the Administrator, a program manager may comply with the applicable training and testing sections of part 121, subparts N and O of this chapter instead of §§ 91.1065 through 91.1107, provided that the following additional limitations and allowances apply to program managers so authorized:

(1)

Operating experience and operations familiarization. Program managers are not required to comply with the operating experience requirements of § 121.434 or the operations familiarization requirements of § 121.435 of this chapter.

(2)  Upgrade training.

(i)   Each program manager must include in upgrade ground training for pilots, instruction in at least the subjects identified in § 121.419(a) of this chapter, as applicable to their assigned duties; and, for pilots serving in crews oftwo or more pilots, instruction and facilitated discussion in the subjects identified in § 121.419(c) of this chapter.

(ii)   Each program manager must include in upgrade flight training for pilots, flight training for the maneuvers and procedures required in § 121.424(a), (c), (e), and (f) of this chapter; and, for pilots serving in crews oftwo or more pilots, the flight training required in § 121.424(b) of this chapter.

(3)

(4)

Initial and recurrentleadership and commandand mentoring training. Program managers are not required to include leadership and command training in §§ 121.409(b)(2)(ii)(B)(6), 121.419(c)(1), 121.424(b) and 121.427(d)(1) of this chapter, and mentoring training in §§ 121.419(c)(2) and 121.427(d)(1) of this chapter in initial and recurrent training for pilots in command who serve in operations that use only one pilot.

One-time leadership and commandand mentoring training. Section 121.429 of this chapter does not apply to program managers conducting operations under this subpart when those operations use only one pilot.

If authorized by the Administrator, a program manager may comply with the applicable training and testing sections of subparts G and H of part 135 of this chapter instead of §§ 91.1065 through 91.1107, except for the operating experience requirements of § 135.244 of this chapter.

For the purposes ofthis subpart, the following terms and definitions apply:

(1) (2) (3) (4)

(5)

Initial training. The training required for crewmembers who have not qualified and served in the same capacity on an aircraft.

Transition training. The training required for crewmembers who have qualified and served in the same capacity on another aircraft.

Upgrade training. The training required for crewmembers who have qualified and served as second in command on a particular aircraft type, before they serve as pilot in command on that aircraft.

Differences training. The training required for crewmembers who have qualified and served on a particular type aircraft, when the Administrator finds differences training is necessary before a crewmember serves in the same capacity on a particular variation of that aircraft.

Recurrent training. The training required for crewmembers to remain adequatelytrained and currently proficient for each aircraft crewmember position, and type of operation in which the crewmember serves.

14 CFR91.1063(d)(5)(enhanced display)

page 178 of243

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.1063(d)(6)

(6) (7)

(8)

In flight. The maneuvers, procedures, or functions that will be conducted in the aircraft.

Training center. An organization governed by the applicable requirements of part 142 of this chapter that conducts training, testing, and checking under contract or other arrangement to program managers subject to the requirements of this subpart.

Requalification training. The training required for crewmembers previouslytrained and qualified, but who have become unqualified because of not having met within the required period any of the following:

(i)

Recurrent crewmembertraining requirements of § 91.1107.

(ii)   Instrument proficiency check requirements of § 91.1069. (iii)  Testing requirements of § 91.1065.

(iv)  Recurrent flight attendant testing requirements of § 91.1067.

[Docket FAA-2001-10047, 68 FR 54561, Sept. 17, 2003, as amendedbyAmdt. 61-144, 85 FR 10920, Feb. 25, 2020; Docket FAA-2019-0360, Amdt. 91-375, 89 FR 51424, June 18, 2024]

§ 91.1065 Initial and recurrentpilot  testingrequirements.

(a)  No program manager or owner may use a pilot, nor may any person serve as a pilot, unless, since the beginning ofthe 12th month before that service, that pilot has passed either a written or oral test (or a combination), given bythe Administrator or an authorized check pilot, on that pilot's knowledge in the following areas—

(1) (2)

(3) (4)

(5) (6)

The appropriate provisions of parts 61 and 91 of this chapter and the management specifications and the operating manual of the program manager;

For each type of aircraft to be flown by the pilot, the aircraft powerplant, major components and systems, major appliances, performance and operating limitations, standard and emergency operating procedures, and the contents of the accepted operating manual or equivalent, as applicable;

For each type of aircraft to be flown by the pilot, the method of determining compliance with weight and balance limitations for takeoff, landing and en route operations;

Navigation and use of air navigation aids appropriate to the operation or pilot authorization, including, when applicable, instrument approach facilities and procedures;

Air traffic control procedures, including IFR procedures when applicable;

Meteorology in general, including the principles of frontal systems, icing, fog, thunderstorms, and windshear, and, if appropriate for the operation of the program manager, high altitude weather;

(7)  Procedures for—

(i)  Recognizing and avoiding severe weather situations;

(ii)

Escaping from severe weather situations, in case of inadvertent encounters, including low- altitude windshear (except that rotorcraft aircraft pilots are not required to be tested on escaping from low-altitude windshear); and

14 CFR91.1065(a)(7)(ii)(enhanced display)

page 179 of243

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR 91.1065(a)(7)(iii)

(b)

(c) (d)

(e) (f) (g)

(iii)  Operating in or near thunderstorms (including best penetration altitudes), turbulent air (including clear air turbulence), icing, hail, and other potentially hazardous meteorological conditions; and

(8)  New equipment, procedures, or techniques, as appropriate.

No program manager or owner may use a pilot, nor may any person serve as a pilot, in any aircraft unless, since the beginning ofthe 12th month before that service, that pilot has passed a competency check given by the Administrator or an authorized check pilot in that class of aircraft, if single-engine aircraft other than turbojet, or that type of aircraft, if rotorcraft, multiengine aircraft, or turbojet airplane, to determine the pilot's competence in practical skills and techniques in that aircraft or class of aircraft. The extent of the competency check will be determined by the Administrator or authorized check pilot conducting the competency check. The competency check may include any of the maneuvers and procedures currently required for the original issuance of the particular pilot certificate required for the operations authorized and appropriate to the category, class and type of aircraft involved. For the purposes ofthis paragraph, type, as to an airplane, means any one of a group of airplanes determined by the Administratorto have a similar means of propulsion, the same manufacturer, and no significantly different handling or flight characteristics. For the purposes of this paragraph, type, as to a rotorcraft, means a basic make and model.

The instrument proficiency check required by § 91.1069 may be substituted for the competency check required by this section for the type of aircraft used in the check.

For the purpose of this subpart, competent performance of a procedure or maneuver by a person to be used as a pilot requires that the pilot be the obvious master ofthe aircraft, with the successful outcome of the maneuver never in doubt.

The Administrator or authorized check pilot certifies the competency of each pilot who passes the knowledge or flight check in the program manager's pilot records.

All or portions of a required competency check may be given in an aircraft simulator or other appropriate training device, if approved bythe Administrator.

If the program manager is authorized to conduct EFVS operations, the competency check in paragraph (b) of this section must include tasks appropriate to the EFVS operations the certificate holder is authorized to conduct.

[Docket FAA-2001-10047, 68 FR 54561, Sept. 17, 2003, as amendedbyDocketFAA-2013-0485,Amdt. 91-345, 81 FR 90175, Dec. 13, 2016]

§ 91.1067 Initial  andrecurrentflight   attendantcrewmembertesting  requirements.

No program manager or owner may use a flight attendant crewmember, nor may any person serve as a flight attendant crewmember unless, since the beginning ofthe 12th month before that service, the program manager has determined by appropriate initial and recurrent testing that the person is knowledgeable and competent in the following areas as appropriate to assigned duties and responsibilities—

(a) (b)

Authority of the pilot in command;

Passenger handling, including procedures to be followed in handling deranged persons or other persons whose conduct might jeopardize safety;

14 CFR91.1067(b)(enhanceddisplay)

page 180 of243

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.1067(c)

(c)

Crewmember assignments, functions, and responsibilities during ditching and evacuation of persons who may need the assistance of another person to move expeditiously to an exit in an emergency;

(d)   Briefing of passengers;

(e) (f) (g) (h)

(i)

Location and operation of portable fire extinguishers and other items of emergency equipment; Proper use of cabin equipment and controls;

Location and operation of passenger oxygen equipment;

Location and operation of all normal and emergency exits, including evacuation slides and escape ropes; and

Seating of persons who may need assistance of another person to move rapidly to an exit in an emergency as prescribed by the program manager's operations manual.

§ 91.1069 Flight crew:  Instrumentproficiencycheckrequirements.

(a)  No program manager or owner may use a pilot, nor may any person serve, as a pilot in command of an aircraft under IFR unless, since the beginning ofthe 6th month before that service, that pilot has passed an instrument proficiency check under this section administered by the Administrator or an authorized check pilot.

(b) (c)

(d)

(e)

No program manager or owner may use a pilot, nor may any person serve, as a second command pilot of an aircraft under IFR unless, since the beginning ofthe 12th month before that service, that pilot has passed an instrument proficiency check under this section administered by the Administrator or an authorized check pilot.

No pilot may use any type of precision instrument approach procedure under IFR unless, since the beginning ofthe 6th month before that use, the pilot satisfactorily demonstrated that type of approach procedure. No pilot may use anytype of nonprecision approach procedure under IFR unless, since the beginning ofthe 6th month before that use, the pilot has satisfactorily demonstrated eitherthat type of approach procedure or any other two different types of nonprecision approach procedures. The instrument approach procedure or procedures must include at least one straight-in approach, one circling approach, and one missed approach. Each type of approach procedure demonstrated must be conducted to published minimums for that procedure.

The instrument proficiency checks required by paragraphs (a) and (b) of this section consists of either an oral or written equipment test (or a combination) and a flight check under simulated or actual IFR conditions. The equipment test includes questions on emergency procedures, engine operation, fuel and lubrication systems, power settings, stall speeds, best engine-out speed, propeller and supercharger operations, and hydraulic, mechanical, and electrical systems, as appropriate. The flight check includes navigation by instruments, recovery from simulated emergencies, and standard instrument approaches involving navigational facilities which that pilot is to be authorized to use.

Each pilot taking the instrument proficiency check must show that standard of competence required by § 91.1065(d).

(1)  The instrument proficiency check must—

(i)

For a pilot in command of an aircraft requiring that the PIC hold an airline transport pilot certificate, include the procedures and maneuvers for an airline transport pilot certificate in the particular type of aircraft, if appropriate; and

14 CFR91.1069(e)(1)(i) (enhanceddisplay)

page 181of243

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.1069(e)(1)(ii)

(2)

(ii)   For a pilot in command of a rotorcraft or a second in command of any aircraft requiring that the SIC hold a commercial pilot certificate include the procedures and maneuvers for a commercial pilot certificate with an instrument rating and, if required, for the appropriate type rating.

The instrument proficiency check must be given by an authorized check pilot or bythe Administrator.

(f) (g)

(h)

(i)

If the pilot is assigned to pilot only one type of aircraft, that pilot must take the instrument proficiency check required by paragraph (a) of this section in that type of aircraft.

If the pilot in command is assigned to pilot more than one type of aircraft, that pilot must take the instrument proficiency check required by paragraph (a) of this section in each type of aircraft to which that pilot is assigned, in rotation, but not more than one flight check during each period described in paragraph (a) of this section.

If the pilot in command is assigned to pilot both single-engine and multiengine aircraft, that pilot must initially take the instrument proficiency check required by paragraph (a) of this section in a multiengine aircraft, and each succeeding check alternately in single-engine and multiengine aircraft, but not more than one flight check during each period described in paragraph (a) ofthis section.

All or portions of a required flight check may be given in an aircraft simulator or other appropriate training device, if approved by the Administrator.

§ 91.1071 Crewmember: Tests and checks, grace provisions,trainingto acceptedstandards.

(a)   If a crewmember who is required to take a test or a flight check under this subpart, completes the test or flight check in the month before or after the month in which it is required, that crewmember is considered to have completed the test or check in the month in which it is required.

(b)

If a pilot being checked under this subpart fails any ofthe required maneuvers, the person giving the check may give additional training to the pilot during the course of the check. In addition to repeating the maneuvers failed, the person giving the check may require the pilot being checked to repeat any other maneuvers that are necessary to determine the pilot's proficiency. Ifthe pilot being checked is unable to demonstrate satisfactory performance to the person conducting the check, the program manager may

not use the pilot, nor may the pilot serve, as a flight crewmember in operations under this subpart until the pilot has satisfactorily completed the check. If a pilot who demonstrates unsatisfactory performance is employed as a pilot for a certificate holder operating under part 121, 125, or 135 of this chapter, he or she must notify that certificate holder of the unsatisfactory performance.

§ 91.1073 Training program: General.

(a)  Each program manager must have a training program and must:

(1)   Establish, obtain the appropriate initial and final approval of, and provide a training program that meets this subpart and that ensures that each crewmember, including each flight attendant if the program manager uses a flight attendant crewmember, flight instructor, check pilot, and each person assigned duties for the carriage and handling of hazardous materials (as defined in 49 CFR 171.8) is adequately trained to perform these assigned duties.

(2) (3)

Provide adequate ground and flight training facilities and properly qualified ground instructors  forthe training required by this subpart.

Provide and keep current for each aircraft type used and, if applicable, the particular variations within the aircraft type, appropriate training material, examinations, forms, instructions, and procedures for use in conducting the training and checks required by this subpart.

14 CFR91.1073(a)(3)(enhanceddisplay)

page 182of243

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.1073(a)(4)

(b) (c)

(d)

(e) (f)

(g)

(4)  Provide enough flight instructors, check pilots, and simulator instructors to conduct required flight training and flight checks, and simulator training courses allowed under this subpart.

Whenever a crewmember who is required to take recurrent training under this subpart completes the training in the month before, or the month after, the month in which that training is required, the crewmember is considered to have completed it in the month in which it was required.

Each instructor, supervisor, or check pilot who is responsible for a particular ground training subject, segment of flight training, course oftraining, flight check, or competence check under this subpart must certify as to the proficiency and knowledge of the crewmember, flight instructor, or check pilot concerned upon completion ofthat training or check. That certification must be made a part of the crewmember's record. When the certification required by this paragraph is made by an entry in a computerized recordkeeping system, the certifying instructor, supervisor, or check pilot, must be identified with that entry. However, the signature of the certifying instructor, supervisor, or check pilot is not required for computerized entries.

Training subjects that applyto more than one aircraft or crewmember position and that have been satisfactorily completed during previous training while employed by the program manager for another aircraft or another crewmember position, need not be repeated during subsequent training other than recurrent training.

Aircraft simulators and other training devices may be used in the program manager's training program if approved by the Administrator.

Each program manager is responsible for establishing safe and efficient crew management practices for all phases of flight in program operations including crew resource management training for all crewmembers used in program operations.

If an aircraft simulator has been approved by the Administrator for use in the program manager's training program, the program manager must ensure that each pilot annually completes at least one flight training session in an approved simulator for at least one program aircraft. The training session may be the flight training portion of any of the pilot training or check requirements ofthis subpart, including the initial, transition, upgrade, requalification, differences, or recurrent training, orthe accomplishment of a competency check or instrument proficiency check. If there is no approved simulator for that aircraft type in operation, then all flight training and checking must be accomplished in the aircraft.

§ 91.1075 Trainingprogram: Special rules.

Other than the program manager, only the following are eligible under this subpart to conduct training, testing, and checking under contract or other arrangement to those persons subject to the requirements ofthis subpart.

(a) (b)

Another program manager operating underthis subpart:

A training center certificated under part 142 of this chapter to conduct training, testing, and checking required by this subpart ifthe training center—

(1)   Holds applicable training specifications issued under part 142 of this chapter;

(2)   Has facilities, training equipment, and courseware meeting the applicable requirements of part 142 of this chapter;

(3)   Has approved curriculums, curriculum segments, and portions of curriculum segments applicable for use in training courses required by this subpart; and

14 CFR91.1075(b)(3)(enhanceddisplay)

page 183 of243

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.1075(b)(4)

(4)

Has sufficient instructors and check pilots qualified under the applicable requirements of §§ 91.1089 through 91.1095 to conduct training, testing, and checking to persons subject to the requirements of this subpart.

(c) (d)

A part 119 certificate holder operating under part 121 or part 135 of this chapter.

As authorized by the Administrator, a training center that is not certificated under part 142 of this chapter.

§ 91.1077Trainingprogram and revision: Initial and final approval.

(a)  To obtain initial and final approval of a training program, or a revision to an approved training program, each program manager must submit to the Administrator—

(1)

(2)

An outline ofthe proposed or revised curriculum, that provides enough information for a preliminary evaluation of the proposed training program or revision; and

Additional relevant information that may be requested by the Administrator.

(b) (c) (d)

If the proposed training program or revision complies with this subpart, the Administrator grants initial approval in writing after which the program manager may conduct the training under that program. The Administrator then evaluates the effectiveness ofthe training program and advises the program manager of deficiencies, if any, that must be corrected.

The Administrator grants final approval of the proposed training program or revision if the program manager shows that the training conducted under the initial approval in paragraph (b) of this section ensures that each person who successfully completes the training is adequately trained to perform that person's assigned duties.

Whenever the Administrator finds that revisions are necessary for the continued adequacy of a training program that has been granted final approval, the program manager must, after notification by the Administrator, make any changes in the program that are found necessary bythe Administrator. Within 30 days after the program manager receives the notice, it may file a petition to reconsider the notice with the Administrator. The filing of a petition to reconsider stays the notice pending a decision by the Administrator. However, if the Administrator finds that there is an emergency that requires immediate action in the interest of safety, the Administrator may, upon a statement of the reasons, require a change effective without stay.

§ 91.1079 Training program: Curriculum.

(a)  Each program manager must prepare and keep current a written training program curriculum for each type of aircraft for each crewmember required for that type aircraft. The curriculum must include ground and flight training required by this subpart.

(b)

Each training program curriculum must include the following:

(1) (2)

(3)

A list of principal ground training subjects, including emergency training subjects, that are provided.

A list of all the training devices, mock-ups, systems trainers, procedures trainers, or other training aids that the program manager will use.

Detailed descriptions or pictorial displays ofthe approved normal, abnormal, and emergency maneuvers, procedures and functions that will be performed during each flight training phase or flight check, indicating those maneuvers, procedures and functions that are to be performed during the inflight portions of flight training and flight checks.

14 CFR91.1079(b)(3)(enhanceddisplay)

page 184of243

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.1081

§ 91.1081 Crewmembertraining requirements.

(a)  Each program manager must include in its training program the following initial and transition ground training as appropriate to the particular assignment of the crewmember:

(1)

(2) (3)

Basic indoctrination ground training for newly hired crewmembers including instruction in at least the—

(i)   Duties and responsibilities of crewmembers as applicable; (ii)   Appropriate provisions of this chapter;

(iii)  Contents of the program manager's management specifications (not required for flight attendants); and

(iv)   Appropriate portions of the program manager's operating manual.

The initial and transition ground training in §§ 91.1101and91.1105, as applicable. Emergency training in § 91.1083.

(b) (c) (d)

(e)

Each training program must provide the initial and transition flight training in § 91.1103, as applicable. Each training program must provide recurrent ground and flight training as provided in § 91.1107.

Upgrade training in §§  91.1101and91.1103for  a particulartype aircraft may be included in the training program for crewmembers who have qualified and served as second in command on that aircraft.

In addition to initial, transition, upgrade and recurrent training, each training program must provide ground and flight training, instruction, and practice necessary to ensure that each crewmember—

(1)

(2)

Remains adequately trained and currently proficient for each aircraft, crewmember position, and type of operation in which the crewmember serves; and

Qualifies in new equipment, facilities, procedures, and techniques, including modifications to aircraft.

§ 91.1083 Crewmemberemergencytraining.

(a)  Each training program must provide emergency training under this section for each aircraft type, model, and configuration, each crewmember, and each kind of operation conducted, as appropriate for each crewmember and the program manager.

(b)

Emergency training must provide the following:

(1) (2)

(3)

Instruction in emergency assignments and procedures, including coordination among crewmembers. Individual instruction in the location, function, and operation of emergency equipment including—

(i)   Equipment used in ditching and evacuation; (ii)  First aid equipment and its proper use; and

(iii)   Portable fire extinguishers, with emphasis on the type of extinguisher to be used on different classes offires.

Instruction in the handling of emergency situations including— (i)   Rapid decompression;

14 CFR91.1083(b)(3)(i)(enhanced display)

page 185of243

(c) (d)

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR 91.1083(b)(3)(ii)

(4)

(ii)   Fire in flight or on the surface and smoke control procedures with emphasis on electrical equipment and related circuit breakers found in cabin areas;

(iii)  Ditching and evacuation;

(iv)  Illness, injury, or other abnormal situations involving passengers or crewmembers; and (v)   Hijacking and other unusual situations.

Review and discussion of previous aircraft accidents and incidents involving actual emergency situations.

Each crewmember must perform at least the following emergency drills, using the proper emergency equipment and procedures, unless the Administrator finds that, for a particular drill, the crewmember can be adequately trained by demonstration:

(1)   Ditching, if applicable. (2)  Emergency evacuation.

(3) (4)

(5) (6)

(7)

Fire extinguishing and smoke control.

Operation and use of emergency exits, including deployment and use of evacuation slides, if applicable.

Use of crew and passenger oxygen.

Removal of life rafts from the aircraft, inflation of the life rafts, use of lifelines, and boarding of passengers and crew, if applicable.

Donning and inflation of life vests and the use of other individual flotation devices, if applicable.

Crewmembers who serve in operations above 25,000 feet must receive instruction in the following: (1)  Respiration.

(2)  Hypoxia.

(3)

Duration of consciousness without supplemental oxygen at altitude.

(4)  Gas expansion.

(5)  Gas bubble formation.

(6)

Physical phenomena and incidents of decompression.

§ 91.1085 Hazardous materials recognition training.

No program manager may use any person to perform, and no person may perform, any assigned duties and responsibilities for the handling or carriage of hazardous materials (as defined in 49 CFR 171.8), unless that person has received training in the recognition of hazardous materials.

§ 91.1087 Approval ofaircraftsimulators  andother training devices.

(a)   Training courses using aircraft simulators and other training devices may be included in the program manager's training program if approved by the Administrator.

14 CFR91.1087(a)(enhanceddisplay)

page 186of243

(b)

(c) (d)

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.1087(b)

Each aircraft simulator and other training device that is used in a training course or in checks required under this subpart must meet the following requirements:

(1)

(2) (3)

It must be specifically approved for— (i)   The program manager; and

(ii)  The particular maneuver, procedure, or crewmember function involved.

It must maintain the performance, functional, and other characteristics that are required for approval. Additionally, for aircraft simulators, it must be—

(i)

Approved for the type aircraft and, if applicable, the particularvariation within type for which the training or check is being conducted; and

(ii)   Modified to conform with any modification to the aircraft being simulated that changes the performance, functional, or other characteristics required for approval.

A particular aircraft simulator or other training device may be used by more than one program manager.

In granting initial and final approval oftraining programs or revisions to them, the Administrator considers the training devices, methods, and procedures listed in the program manager's curriculum under § 91.1079.

§ 91.1089 Qualifications: Checkpilots (aircraft) and checkpilots (simulator). (a)   Forthe purposes ofthis section and § 91.1093:

(1)   A check pilot (aircraft) is a person who is qualified to conduct flight checks in an aircraft, in a flight simulator, or in a flight training device for a particular type aircraft.

(2) (3)

A check pilot (simulator) is a person who is qualified to conduct flight checks, but only in a flight simulator, in a flight training device, or both, for a particular type aircraft.

Check pilots (aircraft) and check pilots (simulator) are those check pilots who perform the functions described in § 91.1073(a)(4) and (c).

(b)

No program manager may use a person, nor may any person serve as a check pilot (aircraft) in a training program established underthis subpart unless, with respect to the aircraft type involved, that person—

(1) (2) (3)

(4) (5)

(6)

Holds the pilot certificates and ratings required to serve as a pilot in command in operations under this subpart;

Has satisfactorily completed the training phases for the aircraft, including recurrent training, that are required to serve as a pilot in command in operations under this subpart;

Has satisfactorily completed the proficiency or competency checks that are required to serve as a pilot in command in operations under this subpart;

Has satisfactorily completed the applicable training requirements of § 91.1093;

Holds at least a Class III medical certificate unless serving as a required crewmember, in which case holds a Class I or Class II medical certificate as appropriate; and

Has been approved by the Administrator for the check pilot duties involved.

14 CFR91.1089(b)(6) (enhanceddisplay)

page 187of243

(c)

(d) (e) (f)

(g)

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.1089(c)

No program manager may use a person, nor may any person serve as a check pilot (simulator) in a training program established under this subpart unless, with respect to the aircraft type involved, that person meets the provisions of paragraph (b) of this section, or—

(1) (2) (3)

(4) (5)

Holds the applicable pilot certificates and ratings, except medical certificate, required to serve as a pilot in command in operations under this subpart;

Has satisfactorily completed the appropriate training phases for the aircraft, including recurrent training, that are required to serve as a pilot in command in operations under this subpart;

Has satisfactorily completed the appropriate proficiency or competency checks that are required to serve as a pilot in command in operations under this subpart;

Has satisfactorily completed the applicable training requirements of § 91.1093; and Has been approved by the Administrator for the check pilot (simulator) duties involved.

Completion ofthe requirements in paragraphs (b)(2), (3), and (4) or (c)(2), (3), and (4) of this section, as applicable, must be entered in the individual's training record maintained bythe program manager.

A check pilot who does not hold an appropriate medical certificate may function as a check pilot (simulator), but may not serve as a flightcrew member in operations underthis subpart.

A check pilot (simulator) must accomplish the following—

(1)

(2)

Fly at least two flight segments as a required crewmember forthe type, class, or category aircraft involved within the 12-month period preceding the performance of any check pilot duty in a flight simulator; or

Before performing any check pilot duty in a flight simulator, satisfactorily complete an approved line- observation program within the period prescribed by that program.

The flight segments or line-observation program required in paragraph (f) of this section are considered to be completed in the month required if completed in the month before or the month after the month in which they are due.

§ 91.1091 Qualifications: Flight instructors (aircraft) andflight instructors (simulator). (a)   Forthe purposes ofthis section and § 91.1095:

(1)   A flight instructor (aircraft) is a person who is qualified to instruct in an aircraft, in a flight simulator, or in a flight training device for a particular type, class, or category aircraft.

(2) (3)

A flight instructor (simulator) is a person who is qualified to instruct in a flight simulator, in a flight training device, or in both, for a particulartype, class, or category aircraft.

Flight instructors (aircraft) and flight instructors (simulator) are those instructors who perform the functions described in § 91.1073(a)(4) and (c).

(b)

No program manager may use a person, nor may any person serve as a flight instructor (aircraft) in a training program established under this subpart unless, with respect to the type, class, or category aircraft involved, that person—

(1)

Holds the pilot certificates and ratings required to serve as a pilot in command in operations under this subpart or part 121 or 135 of this chapter;

14 CFR91.1091(b)(1) (enhanceddisplay)

page 188 of243

(c)

(d) (e)

(f) (g)

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.1091(b)(2)

(2) (3)

(4) (5)

Has satisfactorily completed the training phases for the aircraft, including recurrent training, that are required to serve as a pilot in command in operations under this subpart;

Has satisfactorily completed the proficiency or competency checks that are required to serve as a pilot in command in operations under this subpart;

Has satisfactorily completed the applicable training requirements of § 91.1095; and Holds at least a Class III medical certificate.

No program manager may use a person, nor may any person serve as a flight instructor (simulator) in a training program established under this subpart, unless, with respect to the type, class, or category aircraft involved, that person meets the provisions of paragraph (b) of this section, or—

(1) (2) (3)

(4)

Holds the pilot certificates and ratings, except medical certificate, required to serve as a pilot in command in operations under this subpart or part 121 or 135 of this chapter;

Has satisfactorily completed the appropriate training phases for the aircraft, including recurrent training, that are required to serve as a pilot in command in operations under this subpart;

Has satisfactorily completed the appropriate proficiency or competency checks that are required to serve as a pilot in command in operations under this subpart; and

Has satisfactorily completed the applicable training requirements of § 91.1095.

Completion ofthe requirements in paragraphs (b)(2), (3), and (4) or (c)(2), (3), and (4) of this section, as applicable, must be entered in the individual's training record maintained bythe program manager.

A pilot who does not hold a medical certificate may function as a flight instructor in an aircraft if functioning as a non-required crewmember, but may not serve as a flightcrew member in operations under this subpart.

A flight instructor (simulator) must accomplish the following—

(1)

(2)

Fly at least two flight segments as a required crewmember forthe type, class, or category aircraft involved within the 12-month period preceding the performance of any flight instructor duty in a flight simulator; or

Satisfactorily complete an approved line-observation program within the period prescribed bythat program preceding the performance of any flight instructor duty in a flight simulator.

The flight segments or line-observation program required in paragraph (f) of this section are considered completed in the month required if completed in the month before, or in the month after, the month in which they are due.

[Docket FAA-2001-10047, 68 FR 54561, Sept. 17, 2003, as amendedbyAmdt. 91-322, 76 FR 31823, June2, 2011]

§ 91.1093 Initial and  transitiontrainingandchecking: Check pilots (aircraft), checkpilots (simulator).

(a)

No program manager may use a person nor may any person serve as a check pilot unless—

(1)

That person has satisfactorily completed initial or transition check pilot training; and

14 CFR91.1093(a)(1)(enhanceddisplay)

page 189 of243

(b) (c)

(d) (e)

(f) (g)

(1) (2)

(3) (4)

(5) (6)

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.1093(a)(2)

(2)

Within the preceding 24 months, that person satisfactorily conducts a proficiency or competency check under the observation of an FAA inspector or an aircrew designated examiner employed by the program manager. The observation check may be accomplished in part or in full in an aircraft, in a flight simulator, or in a flight training device.

The observation check required by paragraph (a)(2) of this section is considered to have been completed in the month required if completed in the month before or the month after the month in which it is due.

The initial ground training for check pilots must include the following:

Check pilot duties, functions, and responsibilities.

The applicable provisions ofthe Code of Federal Regulations and the program manager's policies and procedures.

The applicable methods, procedures, and techniques for conducting the required checks. Proper evaluation of student performance including the detection of—

(i) (ii)

Improper and insufficient training; and

Personal characteristics of an applicant that could adversely affect safety.

The corrective action in the case of unsatisfactory checks.

The approved methods, procedures, and limitations for performing the required normal, abnormal, and emergency procedures in the aircraft.

The transition ground training for a check pilot must include the approved methods, procedures, and limitations for performing the required normal, abnormal, and emergency procedures applicable to the aircraft to which the check pilot is in transition.

The initial and transition flight training for a check pilot (aircraft) must include the following—

(1) (2) (3)

(4)

The safety measures for emergency situations that are likely to develop during a check;

The potential results of improper, untimely, or nonexecution of safety measures during a check;

Training and practice in conducting flight checks from the left and right pilot seats in the required normal, abnormal, and emergency procedures to ensure competence to conduct the pilot flight checks required by this subpart; and

The safety measures to be taken from either pilot seat for emergency situations that are likely to develop during checking.

The requirements of paragraph (e) of this section may be accomplished in full or in part in flight, in a flight simulator, or in a flight training device, as appropriate.

The initial and transition flight training for a check pilot (simulator) must include the following:

(1) (2)

Training and practice in conducting flight checks in the required normal, abnormal, and emergency procedures to ensure competence to conduct the flight checks required by this subpart. This training and practice must be accomplished in a flight simulator or in a flight training device.

Training in the operation of flight simulators, flight training devices, or both, to ensure competence to conduct the flight checks required by this subpart.

14 CFR91.1093(g)(2)(enhanceddisplay)

page 190 of243

(a)

(b) (c)

(d) (e)

(1) (2) (3) (4)

(5) (6)

(7)

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.1095

§ 91.1095 Initial   andtransitiontrainingand  checking: Flightinstructors (aircraft), flight instructors (simulator).

No program manager may use a person nor may any person serve as a flight instructor unless—

(1) (2)

That person has satisfactorily completed initial or transition flight instructortraining; and

Within the preceding 24 months, that person satisfactorily conducts instruction under the observation of an FAA inspector, a program manager check pilot, or an aircrew designated examiner employed bythe program manager. The observation check may be accomplished in part or in full in an aircraft, in a flight simulator, or in a flight training device.

The observation check required by paragraph (a)(2) of this section is considered to have been completed in the month required if completed in the month before, or the month after, the month in which it is due.

The initial ground training for flight instructors must include the following:

Flight instructor duties, functions, and responsibilities.

The applicable Code of Federal Regulations and the program manager's policies and procedures. The applicable methods, procedures, and techniques for conducting flight instruction.

Proper evaluation of student performance including the detection of—

(i) (ii)

Improper and insufficient training; and

Personal characteristics of an applicant that could adversely affect safety.

The corrective action in the case of unsatisfactory training progress.

The approved methods, procedures, and limitations for performing the required normal, abnormal, and emergency procedures in the aircraft.

Except for holders of a flight instructor certificate—

(i) (ii)

The fundamental principles of the teaching-learning process; Teaching methods and procedures; and

(iii)  The instructor-student relationship.

The transition ground training for flight instructors must include the approved methods, procedures, and limitations for performing the required normal, abnormal, and emergency procedures applicable to the type, class, or category aircraft to which the flight instructor is in transition.

The initial and transition flight training for flight instructors (aircraft) must include the following—

(1) (2) (3)

(4)

The safety measures for emergency situations that are likely to develop during instruction; The potential results of improper or untimely safety measures during instruction;

Training and practice from the left and right pilot seats in the required normal, abnormal, and emergency maneuvers to ensure competence to conduct the flight instruction required by this subpart; and

The safety measures to be taken from either the left or right pilot seat for emergency situations that are likely to develop during instruction.

14 CFR91.1095(e)(4)(enhanceddisplay)

page 191of243

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.1095(f)

(f)

(g)

The requirements of paragraph (e) of this section may be accomplished in full or in part in flight, in a flight simulator, or in a flight training device, as appropriate.

The initial and transition flight training for a flight instructor (simulator) must include the following:

(1)

(2)

Training and practice in the required normal, abnormal, and emergency procedures to ensure competence to conduct the flight instruction required by this subpart. These maneuvers and procedures must be accomplished in full or in part in a flight simulator or in a flight training device.

Training in the operation of flight simulators, flight training devices, or both, to ensure competence to conduct the flight instruction required by this subpart.

§ 91.1097 Pilotand  flightattendantcrewmember  training programs.

(a)  Each program manager must establish and maintain an approved pilot training program, and each program manager who uses a flight attendant crewmember must establish and maintain an approved flight attendant training program, that is appropriate to the operations to which each pilot and flight attendant is to be assigned, and will ensure that they are adequatelytrained to meet the applicable knowledge and practical testing requirements of §§ 91.1065 through 91.1071.

(b)

(c) (d)

Each program manager required to have a training program by paragraph (a) of this section must include in that program ground and flight training curriculums for—

(1)   Initial training;

(2)   Transition training; (3)  Upgrade training;

(4)   Differences training; (5)  Recurrent training; and (6)   Requalification training.

Each program manager must provide current and appropriate study materials for use by each required pilot and flight attendant.

The program manager must furnish copies ofthe pilot and flight attendant crewmember training program, and all changes and additions, to the assigned representative of the Administrator. If the program manager uses training facilities of other persons, a copy ofthose training programs or appropriate portions used forthose facilities must also be furnished. Curricula that follow FAA published curricula may be cited by reference in the copy ofthe training program furnished to the representative of the Administrator and need not be furnished with the program.

§ 91.1099 Crewmember initial and recurrenttraining  requirements.

No program manager may use a person, nor may any person serve, as a crewmember in operations under this subpart unless that crewmember has completed the appropriate initial or recurrent training phase of the training program appropriate to the type of operation in which the crewmember is to serve since the beginning of the 12th month before that service.

14 CFR91.1099 (enhanceddisplay)

page 192of243

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.1101

§ 91.1101 Pilots: Initial,transition, andupgrade ground training.

Initial, transition, and upgrade ground training for pilots must include instruction in at least the following, as applicable to their duties:

(a)  General subjects—

(1) (2)

(3)

(4) (5) (6) (7) (8)

The program manager's flight locating procedures;

Principles and methods for determining weight and balance, and runway limitations for takeoff and landing;

Enough meteorology to ensure a practical knowledge ofweather phenomena, including the principles of frontal systems, icing, fog, thunderstorms, windshear and, if appropriate, high altitude weather situations;

Air traffic control systems, procedures, and phraseology;

Navigation and the use of navigational aids, including instrument approach procedures; Normal and emergency communication procedures;

Visual cues before and during descent below Decision Altitude or MDA; and Other instructions necessary to ensure the pilot's competence.

(b)

For each aircraft type—

(1)  A general description;

(2)   Performance characteristics; (3)  Engines and propellers;

(4)   Major components;

(5)

(6)

Major aircraft systems (that is, flight controls, electrical, and hydraulic), other systems, as appropriate, principles of normal, abnormal, and emergency operations, appropriate procedures and limitations;

Knowledge and procedures for—

(i) (ii)

(iii) (iv)

Recognizing and avoiding severe weather situations;

Escaping from severe weather situations, in case of inadvertent encounters, including low- altitude windshear (except that rotorcraft pilots are not required to be trained in escaping from low-altitude windshear);

Operating in or near thunderstorms (including best penetration altitudes), turbulent air (including clear air turbulence), inflight icing, hail, and other potentially hazardous meteorological conditions; and

Operating airplanes during ground icing conditions, (that is, anytime conditions are such that frost, ice, or snow may reasonably be expected to adhere to the aircraft), ifthe program manager expects to authorize takeoffs in ground icing conditions, including:

(A)  The use of holdovertimes when using deicing/anti-icing fluids;

14 CFR91.1101(b)(6)(iv)(A)(enhanceddisplay)

page 193 of243

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.1101(b)(6)(iv)(B)

(B)   Airplane deicing/anti-icing procedures, including inspection and check procedures and responsibilities;

(C)  Communications;

(D)   Airplane surface contamination (that is, adherence of frost, ice, or snow) and critical area identification, and knowledge of how contamination adversely affects airplane performance and flight characteristics;

(E)  Types and characteristics of deicing/anti-icing fluids, if used by the program manager; (F)  Cold weather preflight inspection procedures;

(G)  Techniques for recognizing contamination on the airplane; (7)  Operating limitations;

(8)  Fuel consumption and cruise control; (9)   Flight planning;

(10) Each normal and emergency procedure; and

(11) The approved Aircraft Flight Manual or equivalent.

§ 91.1103 Pilots: Initial, transition, upgrade, requalification, and differences flight training.

(a)   Initial, transition, upgrade, requalification, and differences training for pilots must include flight and practice in each ofthe maneuvers and procedures contained in each ofthe curriculums that are a part of the approved training program.

(b)

(c)

The maneuvers and procedures required by paragraph (a) ofthis section must be performed in flight, except to the extent that certain maneuvers and procedures may be performed in an aircraft simulator, or an appropriate training device, as allowed by this subpart.

If the program manager's approved training program includes a course of training using an aircraft simulator or other training device, each pilot must successfully complete—

(1) (2)

Training and practice in the simulator or training device in at least the maneuvers and procedures in this subpart that are capable of being performed in the aircraft simulator or training device; and

A flight check in the aircraft or a check in the simulator or training device to the level of proficiency of a pilot in command or second in command, as applicable, in at least the maneuvers and procedures that are capable of being performed in an aircraft simulator or training device.

§ 91.1105 Flight attendants: Initial and  transitiongroundtraining.

Initial and transition ground training for flight attendants must include instruction in at least the following—

(a)  General subjects—

(1)   The authority of the pilot in command; and

(2)

Passenger handling, including procedures to be followed in handling deranged persons or other persons whose conduct might jeopardize safety.

(b)

For each aircraft type—

14 CFR91.1105(b)(enhanceddisplay)

page 194of243

(b) (c)

(a)

(b)

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.1105(b)(1)

(1) (2)

(3)

A general description of the aircraft emphasizing physical characteristics that may have a bearing on ditching, evacuation, and inflight emergency procedures and on other related duties;

The use of both the public address system and the means of communicating with other flight crewmembers, including emergency means in the case of attempted hijacking or other unusual situations; and

Proper use of electrical galley equipment and the controls for cabin heat and ventilation.

§ 91.1107 Recurrent training.

(a)  Each program manager must ensure that each crewmember receives recurrent training and is adequately trained and currently proficient forthe type aircraft and crewmember position involved.

Recurrent ground training for crewmembers must include at least the following:

(1) (2)

A quiz or other review to determine the crewmember's knowledge ofthe aircraft and crewmember position involved.

Instruction as necessary in the subjects required for initial ground training by this subpart, as appropriate, including low-altitude windshear training and training on operating during ground icing conditions, as prescribed in § 91.1097 and described in § 91.1101, and emergencytraining.

Recurrent flight training for pilots must include, at least, flight training in the maneuvers or procedures in this subpart, except that satisfactory completion of the check required by § 91.1065 within the preceding 12 months may be substituted for recurrent flight training.

§ 91.1109Aircraft maintenance: Inspection program.

Each program manager must establish an aircraft inspection program for each make and model program aircraft and ensure each aircraft is inspected in accordance with that inspection program.

The inspection program must be in writing and include at least the following information:

(1)

(2) (3)

Instructions and procedures for the conduct of inspections for the particular make and model aircraft, including necessary tests and checks. The instructions and procedures must set forth in detail the parts and areas ofthe airframe, engines, propellers, rotors, and appliances, including survival and emergency equipment required to be inspected.

A schedule for performing the inspections that must be accomplished under the inspection program expressed in terms ofthe time in service, calendar time, number of system operations, or any combination thereof.

The name and address of the person responsible for scheduling the inspections required bythe inspection program. A copy ofthe inspection program must be made available to the person performing inspections on the aircraft and, upon request, to the Administrator.

Each person desiring to establish or change an approved inspection program underthis section must submit the inspection program for approval to the Flight Standards office that issued the program manager's management specifications. The inspection program must be derived from one of the following programs:

(1)

An inspection program currently recommended by the manufacturer ofthe aircraft, aircraft engines, propellers, appliances, and survival and emergency equipment;

14 CFR91.1109(b)(1) (enhanceddisplay)

page 195of243

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.1109(b)(2)

(2)

(3) (4) (5)

An inspection program that is part of a continuous airworthiness maintenance program currently in use by a person holding an air carrier or operating certificate issued under part 119 of this chapter and operating that make and model aircraft under part 121 or 135 of this chapter;

An aircraft inspection program approved under § 135.419 of this chapter and currently in use under part 135 of this chapter by a person holding a certificate issued under part 119 of this chapter; or

An airplane inspection program approved under § 125.247 of this chapter and currently in use under part 125 of this chapter.

An inspection program that is part ofthe program manager's continuous airworthiness maintenance program under §§  91.1411through91.1443.

(c)

The Administrator may require revision of the inspection program approved under this section in accordance with the provisions of § 91.415.

[Docket FAA-2001-10047, 68 FR 54561, Sept. 17, 2003, as amendedbyDocketFAA-2018-0119,Amdt. 91-350, 83 FR 9171, Mar. 5, 2018]

§ 91.1111Maintenance training.

The program manager must ensure that all employees who are responsible for maintenance related to program aircraft undergo appropriate initial and annual recurrent training and are competent to perform those duties.

§ 91.1113 Maintenance recordkeeping.

Each fractional ownership program manager must keep (using the system specified in the manual required in § 91.1025) the records specified in § 91.417(a) for the periods specified in § 91.417(b).

§ 91.1115 Inoperable instruments and equipment.

(a)   No person may take off an aircraft with inoperable instruments or equipment installed unless the following conditions are met:

(1) (2)

(3)

(4)

An approved Minimum Equipment List exists for that aircraft.

The program manager has been issued management specifications authorizing operations in accordance with an approved Minimum Equipment List. The flight crew must have direct access at all times prior to flight to all of the information contained in the approved Minimum Equipment List through printed or other means approved by the Administrator in the program manager's management specifications. An approved Minimum Equipment List, as authorized by the management specifications, constitutes an approved change to the type design without requiring recertification.

The approved Minimum Equipment List must:

(i)   Be prepared in accordance with the limitations specified in paragraph (b) of this section.

(ii)   Provide for the operation of the aircraft with certain instruments and equipment in an inoperable condition.

Records identifying the inoperable instruments and equipment and the information required by (a)(3)(ii) of this section must be available to the pilot.

14 CFR91.1115(a)(4) (enhanceddisplay)

page 196of243

(b)

(c) (d)

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.1115(a)(5)

(5)

The aircraft is operated under all applicable conditions and limitations contained in the Minimum Equipment List and the management specifications authorizing use ofthe Minimum Equipment List.

The following instruments and equipment may not be included in the Minimum Equipment List:

(1)

(2)

(3)

Instruments and equipment that are either specifically or otherwise required by the airworthiness requirements under which the aircraft is type certificated and that are essential for safe operations under all operating conditions.

Instruments and equipment required by an airworthiness directive to be in operable condition unless the airworthiness directive provides otherwise.

Instruments and equipment required for specific operations by this part.

Notwithstanding paragraphs (b)(1) and (b)(3) of this section, an aircraft with inoperable instruments or equipment may be operated under a special flight permit under §§ 21.197 and 21.199 of this chapter.

A person authorized to use an approved Minimum Equipment List issued for a specific aircraft under part 121, 125, or 135 of this chapter must use that Minimum Equipment List to comply with this section.

[Docket FAA-2001-10047, 68 FR 54561, Sept. 17, 2003, as amendedbyFAA-2023-1275, Amdt. 91-379, 89 FR 92486, Nov. 21, 2024]

§ 91.1411Continuous airworthiness maintenance program use by fractional ownership program manager.

Fractional ownership program aircraft may be maintained under a continuous airworthiness maintenance program (CAMP) under §§ 91.1413through 91.1443. Any program manager who elects to maintain the program aircraft using a continuous airworthiness maintenance program must comply with §§ 91.1413through 91.1443.

§ 91.1413 CAMP: Responsibilityfor airworthiness.

(a)   For aircraft maintained in accordance with a Continuous Airworthiness Maintenance Program, each program manager is primarily responsible for the following:

(1)

(2) (3)

Maintaining the airworthiness of the program aircraft, including airframes, aircraft engines, propellers, rotors, appliances, and parts.

Maintaining its aircraft in accordance with the requirements of this chapter.

Repairing defects that occur between regularly scheduled maintenance required under part 43 of this chapter.

(b)

Each program managerwho maintains program aircraft under a CAMP must—

(1) (2)

Employ a Director of Maintenance or equivalent position. The Director of Maintenance must be a certificated mechanic with airframe and powerplant ratings who has responsibility for the maintenance program on all program aircraft maintained under a continuous airworthiness maintenance program. This person cannot also act as Chief Inspector.

Employ a Chief Inspector or equivalent position. The Chief Inspector must be a certificated mechanic with airframe and powerplant ratings who has overall responsibility for inspection aspects ofthe CAMP. This person cannot also act as Director of Maintenance.

14 CFR91.1413(b)(2)(enhanceddisplay)

page 197of243

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.1413(b)(3)

(3)

Have the personnel to perform the maintenance of program aircraft, including airframes, aircraft engines, propellers, rotors, appliances, emergency equipment and parts, under its manual and this chapter; or make arrangements with another person forthe performance of maintenance. However, the program manager must ensure that any maintenance, preventive maintenance, or alteration that is performed by another person is performed under the program manager's operating manual and this chapter.

§ 91.1415 CAMP: Mechanical reliability reports.

(a)  Each program managerwho maintains program aircraft under a CAMP must report the occurrence or detection of each failure, malfunction, or defect in an aircraft concerning—

(1) (2) (3) (4)

(5)

(6) (7) (8) (9) (10) (11) (12)

(13)

Fires during flight and whether the related fire-warning system functioned properly; Fires during flight not protected by related fire-warning system;

False fire-warning during flight;

An exhaust system that causes damage during flight to the engine, adjacent structure, equipment, or components;

An aircraft component that causes accumulation or circulation of smoke, vapor, or toxic or noxious fumes in the crew compartment or passenger cabin during flight;

Engine shutdown during flight because of flameout;

Engine shutdown during flight when external damage to the engine or aircraft structure occurs; Engine shutdown during flight because of foreign object ingestion or icing;

Shutdown of more than one engine during flight;

A propeller feathering system or ability of the system to control overspeed during flight;

A fuel or fuel-dumping system that affects fuel flow or causes hazardous leakage during flight;

An unwanted landing gear extension or retraction or opening or closing of landing gear doors during flight;

Brake system components that result in loss of brake actuating force when the aircraft is in motion on the ground;

(14)  Aircraft structure that requires major repair;

(15) (16)

Cracks, permanent deformation, or corrosion of aircraft structures, if more than the maximum acceptable to the manufacturer or the FAA; and

Aircraft components or systems that result in taking emergency actions during flight (except action to shut down an engine).

(b) (c)

For the purpose of this section, during flight means the period from the moment the aircraft leaves the surface of the earth on takeoff until it touches down on landing.

In addition to the reports required by paragraph (a) of this section, each program manager must report any other failure, malfunction, or defect in an aircraft that occurs or is detected at any time if, in the manager's opinion, the failure, malfunction, or defect has endangered or may endanger the safe operation of the aircraft.

14 CFR91.1415(c)(enhanceddisplay)

page 198 of243

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.1415(d)

(d) (e)

(f)

(g) (h)

Each program manager must send each report required bythis section, in writing, covering each 24-hour period beginning at 0900 hours local time of each day and ending at 0900 hours local time on the next day to the Flight Standards office that issued the program manager's management specifications. Each report of occurrences during a 24-hour period must be mailed or transmitted to that office within the next 72 hours. However, a report that is due on Saturday or Sunday may be mailed or transmitted on the following Monday and one that is due on a holiday may be mailed or transmitted on the next workday. For aircraft operated in areas where mail is not collected, reports may be mailed or transmitted within 72 hours afterthe aircraft returns to a point where the mail is collected.

The program manager must transmit the reports required by this section on a form and in a manner prescribed by the Administrator, and must include as much of the following as is available:

(1)   The type and identification number ofthe aircraft. (2)  The name of the program manager.

(3)  The date.

(4)   The nature ofthe failure, malfunction, or defect.

(5)   Identification ofthe part and system involved, including available information pertaining to type designation ofthe major component and time since last overhaul, if known.

(6)  Apparent cause of the failure, malfunction or defect (for example, wear, crack, design deficiency, or personnel error).

(7)   Other pertinent information necessary for more complete identification, determination of seriousness, or corrective action.

A program manager that is also the holder of a type certificate (including a supplemental type certificate), a Parts ManufacturerApproval, or a Technical Standard Order Authorization, or that is the licensee of a type certificate need not report a failure, malfunction, or defect under this section if the failure, malfunction, or defect has been reported by it under § 21.3 of this chapter or under the accident reporting provisions of part 830 of the regulations of the National Transportation Safety Board.

No person may withhold a report required by this section even when not all information required by this section is available.

When the program manager receives additional information, including information from the manufacturer or other agency, concerning a report required bythis section, the program manager must expeditiously submit it as a supplement to the first report and reference the date and place of submission ofthe first report.

[Docket FAA-2001-10047, 68 FR 54561, Sept. 17, 2003, as amendedbyDocketFAA-2018-0119,Amdt. 91-350, 83 FR 9171, Mar. 5, 2018]

§ 91.1417 CAMP: Mechanicalinterruption summaryreport.

Each program manager who maintains program aircraft under a CAMP must mail or deliver, before the end of the 10th day ofthe following month, a summary report of the following occurrences in multiengine aircraft for the preceding month to the Flight Standards office that issued the management specifications:

14 CFR91.1417(enhanceddisplay)

page 199 of243

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.1417(a)

(a) (b)

Each interruption to a flight, unscheduled change of aircraft en route, or unscheduled stop or diversion from a route, caused by known or suspected mechanical difficulties or malfunctions that are not required to be reported under § 91.1415.

The number of propeller featherings in flight, listed bytype of propeller and engine and aircraft on which it was installed. Propeller featherings for training, demonstration, or flight check purposes need not be reported.

[Docket FAA-2001-10047, 68 FR 54561, Sept. 17, 2003, as amendedbyDocketFAA-2018-0119,Amdt. 91-350, 83 FR 9171, Mar. 5, 2018]

§ 91.1423 CAMP: Maintenance organization.

(a)  Each program managerwho maintains program aircraft under a CAMP that has its personnel perform any of its maintenance (other than required inspections), preventive maintenance, or alterations, and each person with whom it arranges forthe performance of that work, must have an organization adequate to perform the work.

(b) (c)

Each program managerwho has personnel perform any inspections required by the program manager's manual under § 91.1427(b) (2) or (3), (in this subpart referred to as required inspections), and each person with whom the program manager arranges forthe performance of that work, must have an organization adequate to perform that work.

Each person performing required inspections in addition to other maintenance, preventive maintenance, or alterations, must organize the performance ofthose functions so as to separate the required inspection functions from the other maintenance, preventive maintenance, or alteration functions. The separation must be below the level of administrative control at which overall responsibility for the required inspection functions and other maintenance, preventive maintenance, or alterations is exercised.

§ 91.1425 CAMP: Maintenance, preventive maintenance, and alteration programs.

Each program manager who maintains program aircraft under a CAMP must have an inspection program and a program covering other maintenance, preventive maintenance, or alterations that ensures that—

(a) (b) (c)

Maintenance, preventive maintenance, or alterations performed by its personnel, or by other persons, are performed under the program manager's manual;

Competent personnel and adequate facilities and equipment are provided for the proper performance of maintenance, preventive maintenance, or alterations; and

Each aircraft released to service is airworthy and has been properly maintained for operation under this part.

§ 91.1427 CAMP: Manualrequirements.

(a)  Each program managerwho maintains program aircraft under a CAMP must put in the operating manual the chart or description ofthe program manager's organization required by § 91.1423 and a list of persons with whom it has arranged for the performance of any of its required inspections, and other maintenance, preventive maintenance, or alterations, including a general description of that work.

14 CFR91.1427(a) (enhanceddisplay)

page 200 of243

(b)

(c) (d)

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.1427(b)

Each program manager must put in the operating manual the programs required by § 91.1425 that must be followed in performing maintenance, preventive maintenance, or alterations ofthat program manager's aircraft, including airframes, aircraft engines, propellers, rotors, appliances, emergency equipment, and parts, and must include at least the following:

(1) (2)

(3) (4) (5)

(6) (7)

(8)

(9)

The method of performing routine and nonroutine maintenance (other than required inspections), preventive maintenance, or alterations.

A designation of the items of maintenance and alteration that must be inspected (required inspections) including at least those that could result in a failure, malfunction, or defect endangering the safe operation of the aircraft, if not performed properly or if improper parts or materials are used.

The method of performing required inspections and a designation by occupational title of personnel authorized to perform each required inspection.

Procedures forthe reinspection of work performed under previous required inspection findings (buy- back procedures).

Procedures, standards, and limits necessary for required inspections and acceptance or rejection of the items required to be inspected and for periodic inspection and calibration of precision tools, measuring devices, and test equipment.

Procedures to ensure that all required inspections are performed.

Instructions to prevent any person who performs any item of work from performing any required inspection of that work.

Instructions and procedures to prevent any decision of an inspector regarding any required inspection from being countermanded by persons other than supervisory personnel of the

inspection unit, or a person at the level of administrative control that has overall responsibility for the management of both the required inspection functions and the other maintenance, preventive maintenance, or alterations functions.

Procedures to ensure that maintenance (including required inspections), preventive maintenance, or alterations that are not completed because of work interruptions are properly completed before the aircraft is released to service.

Each program manager must put in the manual a suitable system (which may include an electronic or coded system) that provides for the retention of the following information—

(1) (2)

(3)

A description (or reference to data acceptable to the Administrator) of the work performed;

The name of the person performing the work if the work is performed by a person outside the organization of the program manager; and

The name or other positive identification ofthe individual approving the work.

For the purposes ofthis part, the program manager must prepare that part of its manual containing maintenance information and instructions, in whole or in part, in a format acceptable to the Administrator, that is retrievable in the English language.

14 CFR91.1427(d)(enhanced display)

page 201of243

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR 91.1429

§ 91.1429 CAMP: Requiredinspectionpersonnel.

(a)  No person who maintains an aircraft under a CAMP may use any person to perform required inspections unless the person performing the inspection is appropriately certificated, properlytrained, qualified, and authorized to do so.

(b)

(c) (d)

No person may allow any person to perform a required inspection unless, at the time the work was performed, the person performing that inspection is under the supervision and control ofthe chief inspector.

No person may perform a required inspection ifthat person performed the item of work required to be inspected.

Each program manager must maintain, or must ensure that each person with whom it arranges to perform required inspections maintains, a current listing of persons who have been trained, qualified, and authorized to conduct required inspections. The persons must be identified by name, occupational title, and the inspections that they are authorized to perform. The program manager (or person with whom it arranges to perform its required inspections) must give written information to each person so authorized, describing the extent ofthat person's responsibilities, authorities, and inspectional limitations. The list must be made available for inspection bythe Administrator upon request.

§ 91.1431 CAMP: Continuing analysis and surveillance.

(a)  Each program managerwho maintains program aircraft under a CAMP must establish and maintain a system for the continuing analysis and surveillance of the performance and effectiveness of its inspection program and the program covering other maintenance, preventive maintenance, and alterations and for the correction of any deficiency in those programs, regardless of whether those programs are carried out by employees ofthe program manager or by another person.

(b) (c)

Whenever the Administrator finds that the programs described in paragraph (a) of this section does not contain adequate procedures and standards to meet this part, the program manager must, after notification bythe Administrator, make changes in those programs requested by the Administrator.

A program manager may petition the Administrator to reconsider the notice to make a change in a program. The petition must be filed with the Executive Director, Flight Standards Service, within 30 days after the program manager receives the notice. Except in the case of an emergency requiring immediate action in the interest of safety, the filing ofthe petition stays the notice pending a decision by the Administrator.

[Docket FAA-2001-10047, 68 FR 54561, Sept. 17, 2003, as amendedbyDocketFAA-2018-0119,Amdt. 91-350, 83 FR 9171, Mar. 5, 2018]

§ 91.1433 CAMP: Maintenance andpreventive  maintenancetrainingprogram.

Each program manager who maintains program aircraft under a CAMP or a person performing maintenance or preventive maintenance functions for it must have a training program to ensure that each person (including inspection personnel) who determines the adequacy of work done is fully informed about procedures and techniques and new equipment in use and is competent to perform that person's duties.

14 CFR91.1433(enhanceddisplay)

page 202of243

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.1435

§ 91.1435 CAMP: Certificate requirements.

(a)  Except for maintenance, preventive maintenance, alterations, and required inspections performed by repair stations located outside the United States certificated under the provisions of part 145 of this chapter, each person who is directly in charge of maintenance, preventive maintenance, or alterations for a CAMP, and each person performing required inspections for a CAMP must hold an appropriate airman certificate.

(b)

For the purpose of this section, a person “directly in charge” is each person assigned to a position in which that person is responsible for the work of a shop or station that performs maintenance, preventive maintenance, alterations, or other functions affecting airworthiness. A person who is directly in charge need not physically observe and direct each worker constantly but must be available for consultation and decision on matters requiring instruction or decision from higher authority than that ofthe person performing the work.

§ 91.1437CAMP:Authorityto performand approve maintenance.

A program manager who maintains program aircraft under a CAMP may employ maintenance personnel, or make arrangements with other persons to perform maintenance and preventive maintenance as provided in its maintenance manual. Unless properly certificated, the program manager may not perform or approve maintenance for return to service.

§ 91.1439 CAMP: Maintenance recordingrequirements.

(a)  Each program managerwho maintains program aircraft under a CAMP must keep (using the system specified in the manual required in § 91.1427) the following records for the periods specified in paragraph (b) of this section:

(1)   All the records necessary to show that all requirements for the issuance of an airworthiness release under § 91.1443 have been met.

(2)  Records containing the following information:

(i) (ii)

The total time in service of the airframe, engine, propeller, and rotor.

The current status of life-limited parts of each airframe, engine, propeller, rotor, and appliance.

(iii)  The time since last overhaul of each item installed on the aircraft that are required to be overhauled on a specified time basis.

(iv)  The identification ofthe current inspection status of the aircraft, including the time since the last inspections required bythe inspection program under which the aircraft and its appliances are maintained.

(v)

The current status of applicable airworthiness directives, including the date and methods of compliance, and, if the airworthiness directive involves recurring action, the time and date when the next action is required.

(b)

(vi) A list of current major alterations and repairs to each airframe, engine, propeller, rotor, and appliance.

Each program manager must retain the records required to be kept bythis section for the following periods:

14 CFR91.1439(b) (enhanceddisplay)

page 203 of243

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.1439(b)(1)

(1)

(2) (3)

Except forthe records of the last complete overhaul of each airframe, engine, propeller, rotor, and appliance the records specified in paragraph (a)(1) of this section must be retained until the work is repeated or superseded by other work or for one year afterthe work is performed.

The records of the last complete overhaul of each airframe, engine, propeller, rotor, and appliance must be retained until the work is superseded by work of equivalent scope and detail.

The records specified in paragraph (a)(2) ofthis section must be retained as specified unless transferred with the aircraft at the time the aircraft is sold.

(c)

The program manager must make all maintenance records required to be kept bythis section available for inspection bythe Administrator or any representative of the National Transportation Safety Board.

§ 91.1441 CAMP: Transfer ofmaintenance records.

When a U.S.-registered fractional ownership program aircraft maintained under a CAMP is removed from the list of program aircraft in the management specifications, the program manager must transfer to the purchaser, at the time of the sale, the following records ofthat aircraft, in plain language form or in coded form that provides for the preservation and retrieval of information in a manner acceptable to the Administrator:

(a) (b)

The records specified in § 91.1439(a)(2).

The records specified in § 91.1439(a)(1) that are not included in the records covered by paragraph (a) of this section, except that the purchaser may allow the program manager to keep physical custody of such records. However, custody of records by the program manager does not relieve the purchaser of its responsibility under § 91.1439(c) to make the records available for inspection by the Administrator or any representative of the National Transportation Safety Board.

§ 91.1443 CAMP:Airworthiness release or aircraft maintenance log entry.

(a)  No program aircraft maintained under a CAMP may be operated after maintenance, preventive maintenance, or alterations are performed unless qualified, certificated personnel employed by the program manager prepare, or cause the person with whom the program manager arranges for the performance of the maintenance, preventive maintenance, or alterations, to prepare—

(1)   An airworthiness release; or

(2)   An appropriate entry in the aircraft maintenance log.

(b)

The airworthiness release or log entry required by paragraph (a) of this section must—

(1)

Be prepared in accordance with the procedure in the program manager's manual;

(2)   Include a certification that—

(i)   The workwas performed in accordance with the requirements of the program manager's manual;

(ii)   All items required to be inspected were inspected by an authorized person who determined that the work was satisfactorily completed;

(iii)  No known condition exists that would make the aircraft unairworthy;

(iv)  So far as the work performed is concerned, the aircraft is in condition for safe operation; and (3)  Be signed by an authorized certificated mechanic.

14 CFR91.1443(b)(3)(enhanceddisplay)

page 204of243

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.1443(c)

(c) (d)

Notwithstanding paragraph (b)(3) ofthis section, after maintenance, preventive maintenance, or alterations performed by a repair station certificated under the provisions of part 145 of this chapter, the approval for return to service or log entry required by paragraph (a) of this section may be signed by a person authorized by that repair station.

Instead of restating each ofthe conditions of the certification required by paragraph (b) of this section,

the program manager may state in its manual that the signature of an authorized certificated mechanic or repairman constitutes that certification.

Subpart L—ContinuedAirworthiness and Safety Improvements

Source:

Amdt. 91-297, 72 FR 63410, Nov. 8, 2007, unless otherwise noted.

§ 91.1501 Purpose and definition.

(a)   This subpart requires operators to support the continued airworthiness of each airplane. These requirements may include, but are not limited to, revising the inspection program, incorporating design changes, and incorporating revisions to Instructions for Continued Airworthiness.

(b)  [Reserved]

[Amdt. 91-297, 72 FR 63410, Nov. 8, 2007, as amendedbyDocketFAA-2018-0119,Amdt. 91-350, 83 FR 9171, Mar. 5, 2018]

§ 91.1503 [Reserved]

§ 91.1505 Repairs assessment for pressurized fuselages.

(a)  No person may operate an Airbus Model A300 (excluding the -600 series), British Aerospace Model BAC 1-11, Boeing Model 707, 720, 727, 737 or 747, McDonnell Douglas Model DC-8, DC-9/MD-80 or DC-10, Fokker Model F28, or Lockheed Model L-1011 airplane beyond applicable flight cycle implementation time specified below, or May 25, 2001, whichever occurs later, unless repair assessment guidelines applicable to the fuselage pressure boundary (fuselage skin, door skin, and bulkhead webs) are incorporated within its inspection program. The repair assessment guidelines must be approved by the responsible Aircraft Certification Service office for the type certificate for the affected airplane.

(1)   Forthe Airbus Model A300 (excluding the -600 series), the flight cycle implementation time is: (i)   Model B2: 36,000 flights.

(ii)  Model B4-100 (including Model B4-2C): 30,000 flights above the window line, and 36,000 flights below the window line.

(iii)  Model B4-200: 25,500 flights above the window line, and 34,000 flights below the window line.

(2)   For all models ofthe British Aerospace BAC 1-11, the flight cycle implementation time is 60,000 flights.

(3)   For all models ofthe Boeing 707, the flight cycle implementation time is 15,000 flights. (4)   For all models ofthe Boeing 720, the flight cycle implementation time is 23,000 flights. (5)   For all models ofthe Boeing 727, the flight cycle implementation time is 45,000 flights.

14 CFR91.1505(a)(5)(enhanced display)

page 205of243

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.1505(a)(6)

(6) (7) (8) (9)

For all models ofthe Boeing 737, the flight cycle implementation time is 60,000 flights. For all models ofthe Boeing 747, the flight cycle implementation time is 15,000 flights.

For all models ofthe McDonnell Douglas DC-8, the flight cycle implementation time is 30,000 flights.

For all models ofthe McDonnell Douglas DC-9/MD-80, the flight cycle implementation time is 60,000 flights.

(10)  For all models ofthe McDonnell Douglas DC-10, the flight cycle implementation time is 30,000 flights.

(11)  For all models ofthe Lockheed L-1011, the flight cycle implementation time is 27,000 flights.

(12) Forthe Fokker F-28 Mark 1000, 2000, 3000, and 4000, the flight cycle implementation time is 60,000 flights.

(b)  [Reserved]

[Docket 29104, 65 FR 24125, Apr. 25, 2000; 65 FR 35703, June 5, 2000; 65 FR 50744, Aug. 21, 2000, as amendedbyAmdt. 91-266, 66 FR 23130, May 7, 2001;Amdt. 91-277, 67 FR 72834, Dec. 9, 2002;Amdt. 91-283, 69 FR 45941, July 30, 2004. Redesignated and amended byAmdt. 91-297, 72 FR 63410, Nov. 8, 2007; Docket FAA-2018-0119, Amdt. 91-350, 83 FR 9171, Mar. 5, 2018]

§ 91.1507Fuel tanksysteminspection program.

(a)   Except as provided in paragraph (g) ofthis section, this section applies to transport category, turbine- powered airplanes with a type certificate issued after January 1, 1958, that, as a result of original type certification or later increase in capacity, have—

(1)   A maximum type-certificated passenger capacity of 30 or more, or (2)   A maximum payload capacity of 7,500 pounds or more.

(b)

(c) (d)

(e)

For each airplane on which an auxiliary fuel tank is installed under a field approval, before June 16, 2008, the operator must submit to the responsible Aircraft Certification Service Office proposed maintenance instructions forthe tank that meet the requirements of Special Federal Aviation Regulation No. 88 (SFAR 88) of this chapter.

After December 16, 2008, no operator may operate an airplane identified in paragraph (a) of this section unless the inspection program forthat airplane has been revised to include applicable inspections, procedures, and limitations for fuel tank systems.

The proposed fuel tank system inspection program revisions specified in paragraph (c) ofthis section must be based on fuel tank system Instructions for Continued Airworthiness (ICA) that have been developed in accordance with the applicable provisions of SFAR 88 ofthis chapter or § 25.1529 and part 25, Appendix H, of this chapter, in effect on June 6, 2001 (including those developed for auxiliary fuel tanks, if any, installed under supplemental type certificates or other design approval) and that have been approved by the responsible Aircraft Certification Service Office.

After December 16, 2008, before returning an airplane to service after any alterations for which fuel tank ICA are developed under SFAR 88, or under § 25.1529 in effect on June 6, 2001, the operator must include in the inspection program for the airplane inspections and procedures for the fuel tank system based on those ICA.

14 CFR91.1507(e) (enhanceddisplay)

page 206of243

(a)

(b) (c)

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.1507(f)

(f) (g)

The fuel tank system inspection program changes identified in paragraphs (d) and (e) of this section and any later fuel tank system revisions must be submitted to the Flight Standards office responsible for review and approval.

This section does not apply to the following airplane models: (1)  Bombardier CL-44

(2)  Concorde

(3)  deHavilland D.H. 106 Comet 4C

(4) VFW-Vereinigte Flugtechnische WerkVFW-614 (5)   Illyushin Aviation IL 96T

(6)   Bristol Aircraft Britannia 305 (7) Handley Page Herald Type 300

(8)

Avions Marcel Dassault—Breguet Aviation Mercure 100C

(9)  Airbus Caravelle (10) Lockheed L-300

[Amdt. 91-297, 72 FR 63410, Nov. 8, 2007, as amendedbyDocketFAA-2018-0119,Amdt. 91-350, 83 FR 9172, Mar. 5, 2018]

Subpart M—Special FederalAviation Regulations

§ 91.1603 Special Federal Aviation Regulation No. 112—ProhibitionAgainst CertainFlights inthe Territory and AirspaceofLibya.

Applicability. This Special Federal Aviation Regulation (SFAR) applies to the following persons:

(1) (2)

(3)

All U.S. air carriers and U.S. commercial operators;

All persons exercising the privileges of an airman certificate issued by the FAA, except when such persons are operating U.S.-registered aircraft for a foreign air carrier; and

All operators of U.S.-registered civil aircraft, except when the operator of such aircraft is a foreign air carrier.

Flight prohibition. Except as provided in paragraphs (c) and (d) of this section, no person described in paragraph (a) of this section may conduct flight operations in the territory and airspace of Libya.

Permitted operations. This section does not prohibit persons described in paragraph (a) of this section from conducting flight operations in the territory and airspace of Libya, provided that such flight operations occur under a contract, grant, or cooperative agreement with a department, agency, or instrumentality  ofthe U.S. Government (or under a subcontract between the prime contractor ofthe department, agency, or instrumentality and the person described in paragraph (a) ofthis section), with the approval of the FAA, or under an exemption issued bythe FAA. The FAA will consider requests for approval or exemption in a timely manner, with the order of preference being: First, for those operations in support of U.S. Government-sponsored activities; second, for those operations in support of government- sponsored activities of a foreign country with the support of a U.S. Government department, agency, or instrumentality; and third, for all other operations.

14 CFR91.1603(c)(enhanceddisplay)

page 207of243

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.1603(d)

(d)

(e)

Emergency situations. In an emergency that requires immediate decision and action for the safety of the flight, the pilot in command of an aircraft may deviate from this section to the extent required bythat emergency. Except for U.S. air carriers and commercial operators that are subject to the requirements of 14 CFR part 119, 121, 125, or 135, each person who deviates from this section must, within 10 days of the deviation, excluding Saturdays, Sundays, and Federal holidays, submit to the responsible Flight Standards Office a complete report ofthe operations ofthe aircraft involved in the deviation, including a description of the deviation and the reasons for it.

Expiration. This SFAR will remain in effect until March 20, 2028. The FAA may amend, rescind, or extend this SFAR, as necessary.

[Docket No FAA-2011-0246, Amdt. 91-321E, 85 FR 45091, July27, 2020, asamendedby Docket FAA-2011-0246, Amdt. 91-321F, 88 FR 16878, Mar. 21, 2023; FAA-2011-0246, Amdt. 91-321G, 90 FR 13076, Mar. 19, 2025]

§ 91.1605 Special FederalAviation Regulation No.77—ProhibitionAgainst CertainFlights inthe Baghdad FlightInformation Region (FIR) (ORBB).

(a)

(b) (c)

(d)

(e)

Applicability. This section applies to the following persons: (1)  All U.S. air carriers and U.S. commercial operators;

(2)  All persons exercising the privileges of an airman certificate issued by the FAA, except when such persons are operating U.S.-registered aircraft for a foreign air carrier; and

(3)   All operators of civil aircraft registered in the United States, except when the operator of such aircraft is a foreign air carrier.

Flight prohibition. Except as provided in paragraphs (c) and (d) of this section, no person described in paragraph (a) of this section may conduct flight operations in the Baghdad Flight Information Region (FIR) (ORBB) at altitudes below Flight Level (FL) 320.

Permitted operations. This section does not prohibit persons described in paragraph (a) of this section from conducting flight operations in the Baghdad FIR (ORBB) at altitudes below FL320, provided that such flight operations occur under a contract, grant, or cooperative agreement with a department, agency, or instrumentality  ofthe U.S. Government (or under a subcontract between the prime contractor ofthe department, agency, or instrumentality, and the person described in paragraph (a) of this section) with the approval of the FAA, or under an exemption issued bythe FAA. The FAA will consider requests for approval or exemption in a timely manner, with the order of preference being: first, for those operations in support of U.S. Government-sponsored activities; second, for those operations in support of government- sponsored activities of a foreign country with the support of a U.S. Government department, agency, or instrumentality; and third, for all other operations.

Emergency situations. In an emergency that requires immediate decision and action for the safety of the flight, the pilot in command of an aircraft may deviate from this section to the extent required bythat emergency. Except for U.S. air carriers and commercial operators that are subject to the requirements of part 119, 121, 125, or 135 of this chapter, each person who deviates from this section must, within 10

days of the deviation, excluding Saturdays, Sundays, and Federal holidays, submit to the responsible Flight Standards office a complete report ofthe operations ofthe aircraft involved in the deviation, including a description of the deviation and the reasons for it.

Expiration. This SFAR will remain in effect until October 26, 2027. The FAA may amend, rescind, or extend this SFAR, as necessary.

14 CFR91.1605(e)(enhanceddisplay)

page 208 of243

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR 91.1607

[Docket FAA-2018-0927, Amdt. 91-353A, 85 FR 65693, Oct. 16, 2020, as amended byAmdt. 91-353B, 87 FR 57390, Sept. 20, 2022; Amdt. 91-353C, 89 FR 83427, Oct. 16, 2024]

§ 91.1607 [Reserved]

§ 91.1609 Special Federal Aviation RegulationNo. 114—ProhibitionAgainst CertainFlights inthe Damascus Flight Information Region(FIR) (OSTT).

(a)

(b) (c)

(d)

(e)

Applicability. This section applies to the following persons: (1)  All U.S. air carriers and U.S. commercial operators;

(2)  All persons exercising the privileges of an airman certificate issued by the FAA, except when such persons are operating U.S.-registered aircraft for a foreign air carrier; and

(3)   All operators of U.S.-registered civil aircraft, except when the operator of such aircraft is a foreign air carrier.

Flight prohibition. Except as provided in paragraphs (c) and (d) of this section, no person described in paragraph (a) of this section may conduct flight operations in the Damascus Flight Information Region (FIR) (OSTT).

Permitted operations. This section does not prohibit persons described in paragraph (a) of this section from conducting flight operations in the Damascus Flight Information Region (FIR) (OSTT), provided that such flight operations are conducted under a contract, grant, or cooperative agreement with a department, agency, or instrumentality of the U.S. government (or under a subcontract between the prime contractor

of the department, agency, or instrumentality and the person described in paragraph (a) of this section) with the approval ofthe FAA, or under an exemption issued by the FAA. The FAA will consider requests for approval or exemption in a timely manner, with the order of preference being: First, for those operations in support of U.S. government-sponsored activities; second, for those operations in support of government- sponsored activities of a foreign country with the support of a U.S. government department, agency, or instrumentality; and third, for all other operations.

Emergency situations. In an emergency that requires immediate decision and action for the safety of the flight, the pilot in command of an aircraft may deviate from this section to the extent required bythat emergency. Except for U.S. air carriers and commercial operators that are subject to the requirements of 14 CFR part 119, 121, 125, or 135, each person who deviates from this section must, within 10 days of the deviation, excluding Saturdays, Sundays, and Federal holidays, submit to the responsible Flight Standards office a complete report of the operations of the aircraft involved in the deviation, including a description of the deviation and the reasons for it.

Expiration. This SFAR will remain in effect until December 30, 2028. The FAA may amend, rescind, or extend this SFAR, as necessary.

[Docket FAA-2017-0768, Amdt. 91-348, 82 FR 40949, Aug. 29, 2017;Amdt. 91-348A, 82 FR 42592, Sept. 11, 2017, as amendedby Amdt. 91-348B, 83 FR 63414, Dec. 10, 2018;Amdt. 91-348C, 85 FR 75845, Nov. 27, 2020;Amdt. 91-348D, 88 FR 89300, Dec. 27, 2023]

§ 91.1611 Special FederalAviationRegulation No.115—Prohibition AgainstCertain Flights in Specified Areas ofthe Sanaa FlightInformation Region(FIR) (OYSC).

(a)

Applicability. This Special Federal Aviation Regulation (SFAR) applies to the following persons:

14 CFR91.1611(a)(enhanceddisplay)

page 209 of243

(b) (c)

(d)

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.1611(a)(1)

(1) (2)

(3)

All U.S. air carriers and U.S. commercial operators;

All persons exercising the privileges of an airman certificate issued by the FAA, except when such persons are operating U.S.-registered aircraft for a foreign air carrier; and

All operators of U.S.-registered civil aircraft, except when the operator of such aircraft is a foreign air carrier.

Flight prohibition. Except as provided in paragraphs (c) and (d) of this section, no person described in paragraph (a) of this section may conduct flight operations in the portion ofthe Sanaa Flight Information Region (FIR) (OYSC) that is west of a line drawn direct from KAPET (163322N 0530614E) to NODMA (152603N 0533359E), northwest of a line drawn direct from NODMAto IMPAG (140638N 0503924E) then from IMPAG to TIMAD (115500N 0463500E), north of a line drawn direct from TIMAD to PARIM (123200N 0432720E), and east of a line drawn direct from PARIM to RIBOK (154700N 0415230E). Use of jet route UN303 is not authorized.

Permitted operations. This section does not prohibit persons described in paragraph (a) of this section from conducting flight operations in the Sanaa FIR (OYSC) under the following circumstances:

(1) (2)

Permitted operations that do notrequire an approval or exemption from the FAA. Flight operations may be conducted in the Sanaa FIR (OYSC) in that airspace east of a line drawn direct from KAPET (163322N 0530614E) to NODMA (152603N 0533359E), southeast of a line drawn direct from NODMA to IMPAG (140638N 0503924E) then from IMPAG to TIMAD (115500N 0463500E), south of

a line drawn direct from TIMAD to PARIM (123200N 0432720E), and west of a line drawn direct from PARIM to RIBOK (154700N 0415230E). Use ofjet routes UT702 and M999 are authorized. All flight operations conducted under this subparagraph must be conducted subject to the approval of, and in accordance with the conditions established by, the appropriate authorities of Yemen.

Operations permitted under an approvalorexemption issuedby the FAA. Flight operations may be conducted in the Sanaa FIR (OYSC) in that airspace west of a line drawn direct from KAPET (163322N 0530614E) to NODMA (152603N 0533359E), northwest of a line drawn direct from NODMA to IMPAG (140638N 0503924E) then from IMPAG to TIMAD (115500N 0463500E), north of a line drawn direct from TIMAD to PARIM (123200N 0432720E), and east of a line drawn direct from PARIM to RIBOK (154700N 0415230E) if such flight operations are conducted under a contract, grant, or cooperative agreement with a department, agency, or instrumentality of the U.S. Government (or under a subcontract between the prime contractor ofthe U.S. Government

department, agency, or instrumentality and the person subject to paragraph (a)), with the approval of the FAA, or under an exemption issued bythe FAA. The FAA will consider requests for approval or exemption in a timely manner, with the order of preference being: First, for those operations in support of U.S. Government-sponsored activities; second, for those operations in support of government-sponsored activities of a foreign country with the support of a U.S. government department, agency, or instrumentality; and third, for all other operations.

Emergency situations. In an emergency that requires immediate decision and action for the safety of the flight, the pilot in command of an aircraft may deviate from this section to the extent required bythat emergency. Except for U.S. air carriers and commercial operators that are subject to the requirements of 14 CFR part 119, 121, 125, or 135, each person who deviates from this section must, within 10 days of the deviation, excluding Saturdays, Sundays, and Federal holidays, submit to the responsible Flight Standards office a complete report of the operations of the aircraft involved in the deviation, including a description of the deviation and the reasons for it.

14 CFR91.1611(d) (enhanceddisplay)

page 210 of243

(a)

(b) (c)

(1) (2)

(3)

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.1611(e)

(e)

Expiration. This SFAR will remain in effect until January 7, 2028. The FAA may amend, rescind, or extend this SFAR, as necessary.

[Amdt. 91-340B, 84 FR 67665, Dec. 11, 2019, as amendedbyAmdt. 91-340C, 86 FR 69173, Dec. 7, 2021;Docket FAA-2015-8672; Amdt. 91-340D, 88 FR 65320, Sept. 22, 2023; Amdt. 91-340E, 89 FR 106307, Dec. 30, 2024]

§ 91.1613 Special FederalAviation Regulation No. 107—ProhibitionAgainst CertainFlights inthe Territory and AirspaceofSomalia.

Applicability. This Special Federal Aviation Regulation (SFAR) applies to the following persons:

(1) (2)

(3)

All U.S. air carriers and U.S. commercial operators;

All persons exercising the privileges of an airman certificate issued by the FAA, except when such persons are operating U.S.-registered aircraft for a foreign air carrier; and

All operators of U.S.-registered civil aircraft, except when the operator of such aircraft is a foreign air carrier.

Flight prohibition. Except as provided in paragraphs (c) and (d) of this section, no person described in paragraph (a) of this section may conduct flight operations in the territory and airspace of Somalia at altitudes below Flight Level (FL) 260.

Permitted operations. This section does not prohibit persons described in paragraph (a) of this section from conducting flight operations in the territory and airspace of Somalia under the following circumstances:

Overflights of Somalia may be conducted at altitudes at or above FL260 subject to the approval of, and in accordance with the conditions established by, the appropriate authorities of Somalia.

Aircraft departing from Djibouti Ambouli International Airport (International Civil Aviation Organization (ICAO) code: HDAM) may operate overwater in the territory and airspace of Somalia at altitudes below FL260 only to the extent necessary to permit a climb during takeoff ifthe operator of that aircraft:

(i)

Receives any necessary approval from the appropriate authorities of Djibouti;

(ii)   Conducts operations that comply with applicable conditions established by the appropriate authorities of Djibouti and air traffic control instructions; and

(iii)  Is either on a published instrument procedure or under the direction of air traffic control.

Aircraft descending into Djibouti Ambouli International Airport (HDAM) may operate overwater at altitudes below FL260 in the territory and airspace of Somalia only to the extent necessary to permit descent for landing at Djibouti Ambouli International Airport (HDAM), if the operator ofthat aircraft:

(i)

Receives any necessary approval from the appropriate authorities of Djibouti;

(ii)   Conducts operations that comply with applicable conditions established by the appropriate authorities of Djibouti and air traffic control instructions; and

(iii)  Is either on a published instrument procedure or under the direction of air traffic control.

14 CFR91.1613(c)(3)(iii)(enhanceddisplay)

page 211of243

(a)

(b) (c)

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.1613(c)(4)

(4)

Flight operations may be conducted in the territory and airspace of Somalia at altitudes below FL260 if such flight operations are conducted under a contract, grant, or cooperative agreement with a department, agency, or instrumentality of the U.S. Government (or under a subcontract between the prime contractor ofthe U.S. Government department, agency, or instrumentality and the person described in paragraph (a) of this section) with the approval of the FAA or under an exemption

issued bythe FAA. The FAA will consider requests for approval or exemption in a timely manner, with the order of preference being: First, for those operations in support of U.S. Government-sponsored activities; second, for those operations in support of government-sponsored activities of a foreign country with the support of a U.S. Government department, agency, or instrumentality; and third, for all other operations.

(d)

(e)

Emergency situations. In an emergency that requires immediate decision and action for the safety of the flight, the pilot in command of an aircraft may deviate from this section to the extent required bythat emergency. Except for U.S. air carriers and commercial operators that are subject to the requirements of 14 CFR part 119, 121, 125, or 135, each person who deviates from this section must, within 10 days of the deviation, excluding Saturdays, Sundays, and Federal holidays, submit to the responsible Flight Standards office a complete report of the operations of the aircraft involved in the deviation, including a description of the deviation and the reasons for it.

Expiration. This SFAR will remain in effect until January 7, 2027. The FAA may amend, rescind, or extend this SFAR, as necessary.

[Docket FAA-2007-27602, Amdt.91-339, 81 FR 726, Jan. 7, 2016, as amended byAmdt. 91-339A, 82 FR 58550, Dec. 13, 2017; Docket FAA-2018-0119, Amdt. 91-350, 83 FR 9172, Mar. 5, 2018;Amdt. 91-339B, 84 FR 67671, Dec. 11, 2019;Docket FAA-2007-27602, Amdt. 91-339C, 87 FR 79245, Dec. 27, 2022]

§ 91.1615 Special Federal Aviation Regulation No. 79—Prohibition Against Certain Flights in the Pyongyang FlightInformation Region (FIR) (ZKKP).

Applicability. This Special Federal Aviation Regulation (SFAR) applies to the following persons:

(1) (2)

(3)

All U.S. air carriers and U.S. commercial operators;

All persons exercising the privileges of an airman certificate issued by the FAA, except when such persons are operating U.S.-registered aircraft for a foreign air carrier; and

All operators of U.S.-registered civil aircraft, except when the operator of such aircraft is a foreign air carrier.

Flight prohibition. Except as provided in paragraphs (c) and (d) of this section, no person described in paragraph (a) of this section may conduct flight operations in the Pyongyang Flight Information Region (FIR) (ZKKP).

Permitted operations. This section does not prohibit persons described in paragraph (a) of this section from conducting flight operations in the Pyongyang Flight Information Region (FIR) (ZKKP), provided that such flight operations are conducted under a contract, grant, or cooperative agreement with a department, agency, or instrumentality of the U.S. government (or under a subcontract between the prime contractor

of the department, agency, or instrumentality and the person described in paragraph (a) of this section) with the approval ofthe FAA, or under an exemption issued by the FAA. The FAA will consider requests for approval or exemption in a timely manner, with the order of preference being: First, for those operations in

14 CFR91.1615(c)(enhanceddisplay)

page 212of243

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.1615(d)

(d)

(e)

support of U.S. government-sponsored activities; second, for those operations in support of government- sponsored activities of a foreign country with the support of a U.S. Government department, agency, or instrumentality; and third, for all other operations.

Emergency situations. In an emergency that requires immediate decision and action for the safety of the flight, the pilot in command of an aircraft may deviate from this section to the extent required bythat emergency. Except for U.S. air carriers and commercial operators that are subject to the requirements of 14 CFR part 119, 121, 125, or 135, each person who deviates from this section must, within 10 days of the deviation, excluding Saturdays, Sundays, and Federal holidays, submit to the responsible Flight Standards Office a complete report ofthe operations ofthe aircraft involved in the deviation, including a description of the deviation and the reasons for it.

Expiration. This SFAR will remain in effect until September 18, 2028. The FAA may amend, rescind, or extend this SFAR, as necessary.

[Docket FAA-2018-0838, Amdt. 91-352, 83 FR 47064, Sept. 18, 2018, as amendedbyAmdt. 91-352A, 85 FR 55377, Sept. 8, 2020; Docket FAA-2018-0838, Amdt. 91-352B, 88 FR 63525, Sept. 15, 2023]

§ 91.1617 SpecialFederal AviationRegulation No.117—ProhibitionAgainst CertainFlights inthe Tehran FlightInformation Region(FIR) (OIIX).

(a)

(b) (c)

(d)

Applicability. This Special Federal Aviation Regulation (SFAR) applies to the following persons: (1)  All U.S. air carriers and U.S. commercial operators;

(2)  All persons exercising the privileges of an airman certificate issued by the FAA, except when such persons are operating U.S.-registered aircraft for a foreign air carrier; and

(3)   All operators of U.S.-registered civil aircraft, except when the operator of such aircraft is a foreign air carrier.

Flight prohibition. Except as provided in paragraphs (c) and (d) of this section, no person described in paragraph (a) of this section may conduct flight operations in the Tehran Flight Information Region (FIR) (OIIX).

Permitted operations. This section does not prohibit persons described in paragraph (a) of this section from conducting flight operations in the Tehran FIR (OIIX), provided that such flight operations are conducted under a contract, grant, or cooperative agreement with a department, agency, or instrumentality  ofthe U.S. Government (or under a subcontract between the prime contractor ofthe department, agency, or instrumentality and the person described in paragraph (a) ofthis section) with the approval of the FAA, or under an exemption issued bythe FAA. The FAA will consider requests for

approval or exemption in a timely manner, with the order of preference being: First, for those operations in support of U.S. Government-sponsored activities; second, for those operations in support of government- sponsored activities of a foreign country with the support of a U.S. Government department, agency, or instrumentality; and third, for all other operations.

Emergency situations. In an emergency that requires immediate decision and action for the safety of the flight, the pilot in command of an aircraft may deviate from this section to the extent required bythat emergency. Except for U.S. air carriers and commercial operators that are subject to the requirements of 14 CFR parts 119, 121, 125, or 135, each person who deviates from this section must, within 10 days of

14 CFR91.1617(d) (enhanceddisplay)

page 213 of243

(a)

(b) (c)

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.1617(e)

(e)

the deviation, excluding Saturdays, Sundays, and Federal holidays, submit to the responsible Flight Standards Office a complete report ofthe operations ofthe aircraft involved in the deviation, including a description of the deviation and the reasons for it.

Expiration. This SFAR will remain in effect until October 31, 2027. The FAA may amend, rescind, or extend this SFAR as necessary.

[Docket FAA-2020-0874, Amdt. 91-359, 85 FR 68440, Oct. 29, 2020, as amended byAmdt. 91-359A, 87 FR 57384, Sept. 20, 2022; Amdt. 91-359B, 89 FR 80390, Oct. 3, 2024]

§ 91.1619 Special FederalAviation Regulation No.119—ProhibitionAgainst CertainFlights inthe Kabul Flight InformationRegion (FIR) (OAKX).

Applicability. This Special Federal Aviation Regulation (SFAR) applies to the following persons:

(1) (2)

(3)

All U.S. air carriers and U.S. commercial operators;

All persons exercising the privileges of an airman certificate issued by the FAA, except when such persons are operating U.S.-registered aircraft for a foreign air carrier; and

All operators of U.S.-registered civil aircraft, except when the operator of such aircraft is a foreign air carrier.

Flight prohibition. Except as provided in paragraphs (c) and (d) of this section, no person described in paragraph (a) of this section may conduct flight operations in the Kabul Flight Information Region (FIR) (OAKX).

Permitted operations. This section does not prohibit persons described in paragraph (a) of this section from conducting flight operations in the Kabul Flight Information Region (FIR) (OAKX) underthe following circumstances:

(1) (2)

Permitted operations that do notrequire an approval or exemption from the FAA.

(i)  Overflights of the Kabul Flight Information Region (FIR) (OAKX) may be conducted at altitudes at and above Flight Level (FL) 320, subject to the approval of, and in accordance with the conditions established by, the appropriate authorities of Afghanistan.

(ii)  Transiting overflights of the Kabul Flight Information Region (FIR) (OAKX) may be conducted on jet routes P500-G500 at altitudes at and above FL300, subject to the approval of, and in accordance with the conditions established by, the appropriate authorities of Afghanistan.

Operations permitted under an approvalorexemption issuedby the FAA. Flight operations may be conducted in the Kabul Flight Information Region (FIR) (OAKX) at altitudes below FL320, provided that such flight operations occur under a contract, grant, or cooperative agreement with a department, agency, or instrumentality of the U.S. Government (or under a subcontract between the prime contractor ofthe U.S. Government department, agency, or instrumentality and the person described in paragraph (a) of this section) with the approval of the FAA or under an exemption

issued bythe FAA. The FAA will consider requests for approval or exemption in a timely manner, with the order of preference being: first, for those operations in support of U.S. Government-sponsored activities; second, for those operations in support of government-sponsored activities of a foreign country with the support of a U.S. Government department, agency, or instrumentality; and third, for all other operations.

14 CFR91.1619(c)(2) (enhanceddisplay)

page 214of243

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.1619(d)

(d)

(e)

Emergency situations. In an emergency that requires immediate decision and action for the safety of the flight, the pilot in command of an aircraft may deviate from this section to the extent required bythat emergency. Except for U.S. air carriers and commercial operators that are subject to the requirements of 14 CFR part 119, 121, 125, or 135, each person who deviates from this section must, within 10 days of the deviation, excluding Saturdays, Sundays, and Federal holidays, submit to the responsible Flight Standards Office a complete report ofthe operations ofthe aircraft involved in the deviation, including a description of the deviation and the reasons for it.

Expiration. This SFAR will remain in effect until July 25, 2028. The FAA may amend, rescind, or extend this SFAR, as necessary.

[Docket FAA-2023-1415, Amdt. 91-369, 88 FR 47771, July25, 2023, as amended byAmdt. 91-369A, 89 FR 55507, July5, 2024; FAA-2023-1415, 90 FR 27987, July 1, 2025]

Subpart N—MitsubishiMU-2B Series SpecialTraining, Experience, and Operating Requirements

Source:

Docket FAA-2006-24981, Amdt. 91-344, 81 FR 61591, Sept. 7, 2016, unless otherwise noted.

§ 91.1701Applicability.

(a) On and after November 7, 2016, all training conducted in an MU-2B must follow an approved MU-2B training program that meets the standards ofthis subpart.

(b) (c)

This subpart applies to all persons who operate a Mitsubishi MU-2B series airplane, including those who act as pilot in command, act as second-in-command, or other persons who manipulate the controls while under the supervision of a pilot in command.

This subpart also applies to those persons who provide pilot training for a Mitsubishi MU-2B series airplane. The requirements in this subpart are in addition to the requirements of parts 61, 91, and 135 of this chapter.

§ 91.1703 Compliance and eligibility.

(a)   Except as provided in paragraph (b) ofthis section, no person may manipulate the controls, act as PIC, act as second-in-command, or provide pilot training for a Mitsubishi MU-2B series airplane unless that person meets the requirements of this subpart.

(b)

A person who does not meet the requirements of this subpart may manipulate the controls of a Mitsubishi MU-2B series airplane if a pilot in command who meets the requirements of this subpart is occupying a pilot station, no passengers or cargo are carried on board the airplane, and the flight is being conducted for one of the following reasons—

(1)   The pilot in command is providing pilot training to the manipulator of the controls;

(2)   The pilot in command is conducting a maintenance test flight with a second pilot or certificated mechanic; or

(3)   The pilot in command is conducting simulated instrument flight and is using a safety pilot otherthan the pilot in command who manipulates the controls for the purposes of § 91.109(b).

(c)   A person is required to complete Initial/transition training ifthat person has fewerthan— 14 CFR91.1703(c)(enhanceddisplay)

page 215of243

(d)

(e)

(f) (g)

(b)

(c) (d)

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.1703(c)(1)

(1) (2)

50 hours of documented flight time manipulating the controls while serving as pilot in command of a Mitsubishi MU-2B series airplane in the preceding 24 months; or

500 hours of documented flight time manipulating the controls while serving as pilot in command of a Mitsubishi MU-2B series airplane.

A person is eligible to receive Requalification training in lieu of Initial/transition training if that person has at least—

(1) (2)

50 hours of documented flight time manipulating the controls while serving as pilot in command of a Mitsubishi MU-2B series airplane in the preceding 24 months; or

500 hours of documented flight time manipulating the controls while serving as pilot in command of a Mitsubishi MU-2B series airplane.

A person is required to complete Recurrent training within the preceding 12 months. Successful completion of Initial/transition or Requalification training within the preceding 12 months satisfies the requirement of Recurrent training. A person must successfully complete Initial/transition training or Requalification training before being eligible to receive Recurrent training.

Successful completion of Initial/transition training or Requalification training is a one-time requirement. A person may elect to retake Initial/transition training or Requalification training in lieu of Recurrent training.

A person is required to complete Differences training in accordance with an FAA approved MU-2B training program if that person operates more than one MU-2B model as specified in § 91.1707(c).

§ 91.1705 Requiredpilot training.

(a)  Except as provided in § 91.1703(b), no person may manipulate the controls, act as pilot in command, or act as second-in-command of a Mitsubishi MU-2B series airplane for the purpose of flight unless—

(1)

(2)

The requirements for ground and flight training on Initial/transition, Requalification, Recurrent, and Differences training have been completed in accordance with an FAA approved MU-2B training program that meets the standards of this subpart; and

That person's logbook has been endorsed in accordance with paragraph (f) ofthis section.

Except as provided in § 91.1703(b), no person may manipulate the controls, act as pilot in command, or act as second-in-command, of a Mitsubishi MU-2B series airplane for the purpose of flight unless—

(1)

(2)

That person satisfactorily completes, if applicable, annual Recurrent pilot training on the Special Emphasis Items, and all items listed in the Training Course Final Phase Check in accordance with an FAA approved MU-2B training program that meets the standards of this subpart; and

That person's logbook has been endorsed in accordance with paragraph (f) ofthis section.

Satisfactory completion of the competency check required by § 135.293 of this chapter within the preceding 12 calendar months may not be substituted forthe Mitsubishi MU-2B series airplane annual recurrent flight training of this section.

Satisfactory completion of a Federal Aviation Administration sponsored pilot proficiency program, as described in § 61.56(e) of this chapter may not be substituted forthe Mitsubishi MU-2B series airplane annual recurrent flight training of this section.

14 CFR91.1705(d)(enhanced display)

page 216of243

(e) (f)

(g) (h)

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.1705(e)

If a person complies with the requirements of paragraph (a) or (b) of this section in the calendar month before or the calendar month afterthe month in which compliance with these paragraphs are required, that person is considered to have accomplished the training requirement in the month the training is due.

The endorsement required under paragraph (a) and (b) of this section must be made by—

(1) (2)

A certificated flight instructor or a simulator instructor authorized by a Training Center certificated under part 142 of this chapter and meeting the qualifications of § 91.1713; or

For persons operating the Mitsubishi MU-2B series airplane for a 14 CFR part 119certificate holder within the last 12 calendar months, the part 119 certificate holder's flight instructor if authorized by the FAA and ifthat flight instructor meets the requirements of § 91.1713.

All training conducted for a Mitsubishi MU-2B series airplane must be completed in accordance with an MU-2B series airplane checklist that has been accepted by the Federal Aviation Administration's MU-2B Flight Standardization Board or the applicable MU-2B series checklist (incorporated by reference, see § 91.1721).

MU-2B training programs must contain ground training and flight training sufficient to ensure pilot proficiency for the safe operation of MU-2B aircraft, including:

(1) (2)

A ground training curriculum sufficient to ensure pilot knowledge of MU-2B aircraft, aircraft systems, and procedures, necessary for safe operation; and

Flight training curriculum including flight training maneuver profiles sufficient in number and detail to ensure pilot proficiency in all MU-2B operations for each MU-2B model in correlation with MU-2B limitations, procedures, aircraft performance, and MU-2B Cockpit Checklist procedures applicable to the MU-2B model being trained. A MU-2B training program must contain, at a minimum, the following flight training maneuver profiles applicable to the MU-2B model being trained:

(i) (ii)

(iii) (iv)

Normal takeoff with 5- and 20- degrees flaps; Takeoff engine failure with 5- and 20- degrees flaps; Takeoff engine failure on runway or rejected takeoff;

Takeoff engine failure after liftoff—unable to climb (may be completed in classroom or flight training device only);

(v)   Steep turns;

(vi)  Slow flight maneuvers;

(vii)  One engine inoperative maneuvering with loss of directional control; (viii)  Approach to stall in clean configuration and with wings level;

(ix) (x)

Approach to stall in takeoff configuration with 15- to 30- degrees bank;

Approach to stall in landing configuration with gear down and 40-degrees of flaps;

(xi)  Accelerated stall with no flaps; (xii) Emergency descent at low speed;

(xiii) Emergency descent at high speed;

(xiv) Unusual attitude recovery with the nose high; 14 CFR91.1705(h)(2)(xiv) (enhanceddisplay)

page 217of243

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

(xv)  Unusual attitude recovery with the nose low; (xvi) Normal landing with 20- and 40- degrees flaps;

(xvii) Go around and rejected landing; (xviii) No flap or 5- degrees flaps landing;

(xix) One engine inoperative landing with 5- and 20- degrees flaps; (xx) Crosswind landing;

(xxi) Instrument landing system (ILS) and missed approach ; (xxii) Two engine missed approach;

(xxiii) One engine inoperative ILS and missed approach; (xxiv) One engine inoperative missed approach;

(xxv) Non-precision and missed approach;

(xxvi) Non-precision continuous descent final approach and missed approach; (xxvii) One engine inoperative non-precision and missed approach;

(xxviii) One engine inoperative non-precision CDFA and missed approach; (xxix) Circling approach at weather minimums;

(xxx) One engine inoperative circling approach at weather minimums.

14 CFR91.1705(h)(2)(xv)

(3) (4)

(5) (6)

Flight training must include a final phase check sufficient to document pilot proficiency in the flight training maneuver profiles at the completion of training; and

Differences training for applicable MU-2B model variants sufficient to ensure pilot proficiency in each model operated. Current MU-2B differences requirements are specified in § 91.1707(c). A person must complete Differences training if a person operates more than one MU-2B model as specified in § 91.1707(c). Differences training between the factory type design K and M models of the MU-2B airplane, and the factory type design J and L models of the MU-2B airplane, may be accomplished with Level Atraining. All other factory type design differences training must be accomplished with Level B training unless otherwise specified in § 91.1707(c) . A Level A or B differences training is not a recurring annual requirement. Once a person has completed Initial Level A or B Differences training between the applicable different models, no additional differences training between those models is required.

Icing training sufficient to ensure pilot knowledge and safe operation ofthe MU-2B aircraft in icing conditions as established by the FAA;

Ground and flight training programs must include training hours identified by § 91.1707(a) for ground instruction, § 91.1707(b) for flight instruction, and § 91.1707(c) for differences training.

(i)

(ii)

No training credit is given for second-in-command training and no credit is given for right seat time underthis program. Only the sole manipulator of the controls of the MU-2B airplane, flight training device, or Level C or D simulator can receive training credit under this program;

An MU-2B airplane must be operated in accordance with an FAA approved MU-2B training program that meets the standards of this subpart and the training hours in § 91.1707.

14 CFR91.1705(h)(6)(ii) (enhanceddisplay)

page 218 of243

(b) (c) (d)

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.1705(h)(7)

(7)

Endorsements given for compliance with paragraph (f) of this section must be appropriate to the content ofthat  specific MU-2B training program's compliance with standards of this subpart.

§ 91.1707 Training programhours.

(a)  Ground instruction hours are listed in the following table:

Initial/transition                                  Requalification                             Recurrent 20 hours                                                 12 hours                                             8 hours.

Flight instruction hours are listed in the following table:

Initial/transition                          Requalification                          Recurrent

12 hours with a minimum of 6 hours at      8 hours level C or         4 hours at level E, or 6 hours at level E                                                         level E                          level C.

Differences training hours are listed in the following table:

2 factory type design models concurrently

More than 2 factory type design models concurrently

Each additional factory type design model added separately

Definitions of levels of training as used in this subpart:

1.5 hours required at level B. 3 hours at level B.

1.5 hours at level B.

(1) (2)

(3)

(4)

LEVEL A Training—Training that is conducted through self-instruction by the pilot.

LEVEL B Training—Training that is conducted in the classroom environment with the aid of a qualified instructor who meets the requirements ofthis subpart.

LEVEL C Training—Training that is accomplished in an FAA-approved Level 5 or 6 flight training device. In addition to the basic FTD requirements, the FTD must be representative of the MU-2B cockpit controls and be specifically approved bythe FAA for the MU-2B airplane.

Level E Training—Training that must be accomplished in the MU-2B airplane, Level C simulator, or Level D simulator.

§ 91.1709 Training program approval.

To obtain approval for an MU-2B training program, training providers must submit a proposed training program to the Administrator.

14 CFR91.1709(enhanceddisplay)

page 219 of243

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.1709(a)

(a) (b)

(c) (d)

Only training programs approved bythe Administrator may be used to satisfy the standards of this subpart.

For part 91 training providers, training programs will be approved for 24 months, unless sooner superseded or rescinded.

The Administrator may require revision of an approved MU-2B training program at anytime.

A training provider must present its approved training program and FAA approval documentation to any representative of the Administrator, upon request.

§ 91.1711Aeronautical experience.

No person may act as a pilot in command of a Mitsubishi MU-2B series airplane for the purpose offlight unless that person holds an airplane category and multi-engine land class rating, and has logged a minimum of 100 flight hours of PIC time in multi-engine airplanes.

§ 91.1713 Instruction, checking, andevaluation.

(a)   Flight Instructor (Airplane). No flight instructor may provide instruction or conduct a flight review in a Mitsubishi MU-2B series airplane unless that flight instructor

(1)

(2) (3)

Meets the pilot training and documentation requirements of § 91.1705 before giving flight instruction in the Mitsubishi MU-2B series airplane;

Meets the currency requirements of §§ 91.1715(a) and 91.1715(c)

Has a minimum total pilot time of 2,000 pilot-in-command hours and 800 pilot-in-command hours in multiengine airplanes; and

(4)  Has: (i)

300 pilot-in-command hours in the Mitsubishi MU-2B series airplane, 50 hours of which must have been within the preceding 12 months; or

(b)

(ii)   100 pilot-in-command hours in the Mitsubishi MU-2B series airplane, 25 hours of which must have been within the preceding 12 months, and 300 hours providing instruction in a FAA- approved Mitsubishi MU-2B simulator or FAA-approved Mitsubishi MU-2B flight training device, 25 hours of which must have been within the preceding 12 months.

Flight Instructor (Simulator/Flight Training Device). No flight instructor may provide instruction for the Mitsubishi MU-2B series airplane unless that instructor meets the requirements of this paragraph—

(1) (2)

(3)

Each flight instructor who provides flight training forthe Mitsubishi MU-2B series airplane must meet the pilot training and documentation requirements of § 91.1705 before giving flight instruction for the Mitsubishi MU-2B series airplane;

Each flight instructor who provides flight training forthe Mitsubishi MU-2B series airplane must meet the currency requirements of § 91.1715(c) before giving flight instruction for the Mitsubishi MU-2B series airplane;

Each flight instructor who provides flight training forthe Mitsubishi MU-2B series airplane must have:

(i)

A minimum total pilot time of 2000 pilot-in-command hours and 800 pilot-in-command hours in multiengine airplanes; and

14 CFR91.1713(b)(3)(i) (enhanceddisplay)

page 220 of243

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.1713(b)(3)(ii)

(c)

(ii)   Within the preceding 12 months, either 50 hours of Mitsubishi MU-2B series airplane pilot-in- command experience or 50 hours providing simulator or flight training device instruction for the Mitsubishi MU-2B.

Checking andevaluation. No person may provide checking or evaluation for the Mitsubishi MU-2B series airplane unless that person meets the requirements of this paragraph—

(1)   Forthe purpose of checking, designated pilot examiners, training center evaluators, and check airmen must have completed the appropriate training in the Mitsubishi MU-2B series airplane in accordance with § 91.1705;

(2)  For checking conducted in the Mitsubishi MU-2B series airplane, each designated pilot examiner and check airman must have 100 hours pilot-in-command flight time in the Mitsubishi MU-2B series airplane and maintain currency in accordance with § 91.1715.

§ 91.1715 Currencyrequirements and flightreview.

(a)   The takeoff and landing currency requirements of § 61.57 of this chapter must be maintained in the Mitsubishi MU-2B series airplane. Takeoff and landings in other multiengine airplanes do not meet the takeoff and landing currency requirements forthe Mitsubishi MU-2B series plane. Takeoff and landings in either the short-body or long-body Mitsubishi MU-2B model airplane may be credited toward takeoff and landing currency for both Mitsubishi MU-2B model groups.

(b) (c)

(d)

Instrument experience obtained in other category and class of aircraft may be used to satisfy the instrument currency requirements of § 61.57 of this chapter for the Mitsubishi MU-2B series airplane.

Satisfactory completion of a flight review to satisfy the requirements of § 61.56 of this chapter is valid for operation of a Mitsubishi MU-2B series airplane only if that flight review is conducted in a Mitsubishi MU-2B series airplane or an MU-2B Simulator approved for landings with an approved course conducted under part 142 of this chapter. The flight review for Mitsubishi MU-2B series airplanes must include the Special Emphasis Items, and all items listed in the Training Course Final Phase Check in accordance with an approved MU-2B Training Program.

A person who successfully completes the Initial/transition, Requalification, or Recurrent training requirements under § 91.1705 of this chapter also meet the requirements of § 61.56 of this chapter and need not accomplish a separate flight review provided that at least 1 hour of the flight training was conducted in the Mitsubishi MU-2B series airplane or an MU-2B Simulator approved for landings with an approved course conducted under part 142 of this chapter.

[Docket FAA-2006-24981, Amdt. 91-344, 81 FR 61591, Sept. 7, 2016;Amdt. 91-344A, 82 FR 21472, May 9, 2017]

§ 91.1717 Operating requirements.

(a)   Except as provided in paragraph (b) ofthis section, no person may operate a Mitsubishi MU-2B airplane in single pilot operations unless that airplane has a functional autopilot.

(b)

A person may operate a Mitsubishi MU-2B airplane in single pilot operations without a functional autopilot when—

(1)  Operating under day visual flight rule requirements; or

(2)  Authorized under a FAA approved minimum equipment list for that airplane, operating under instrument flight rule requirements in daytime visual meteorological conditions.

14 CFR91.1717(b)(2)(enhanceddisplay)

page 221of243

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.1717(c)

(c) (d)

(e) (f)

No person may operate a Mitsubishi MU-2B series airplane unless a copy of the appropriate Mitsubishi Heavy Industries MU-2B Airplane Flight Manual is carried on board the airplane and is accessible during each flight at the pilot station.

No person may operate a Mitsubishi MU-2B series airplane unless an MU-2B series airplane checklist, appropriate for the model being operated and accepted by the Federal Aviation Administration MU-2B Flight Standardization Board, is accessible for each flight at the pilot station and is used by the flight crewmembers when operating the airplane.

No person may operate a Mitsubishi MU-2B series airplane contrary to the standards of this subpart.

If there are any differences between the training and operating requirements of this subpart and the MU-2B Airplane Flight Manual's procedures sections (Normal, Abnormal, and Emergency) and the MU-2B airplane series checklist incorporated by reference in § 91.1721, the person operating the airplane must operate the airplane in accordance with the training specified in this subpart.

§ 91.1719 Credit for priortraining.

Initial/transition, requalification, recurrent or Level B differences training conducted prior to November 7, 2016, compliant with SFAR No. 108, Section 3 ofthis part, is considered to be compliant with this subpart, ifthe student met the eligibility requirements forthe applicable category of training and the student's instructor met the experience requirements ofthis subpart.

§ 91.1721 Incorporation by reference.

(a)  The Mitsubishi Heavy Industries MU-2B Cockpit Checklists are incorporated by reference into this part. The Director of the Federal Register approved this incorporation by reference in accordance with 5 U.S.C. 552(a) and 1 CFR part 51. All approved material is available for inspection at U.S. Department of Transportation, Docket Management Facility, Room W 12-140, West Building Ground Floor, 1200 New Jersey Ave. SE., Washington, DC 20590-0001, or at the National Archives and Records Administration, call 202-741-6030, or go to: http://www.archives.gov/federal_register/code_of_federal_regulations/ ibr_locations.html.

(b)

Mitsubishi Heavy Industries America, Inc., 4951 Airport Parkway, Suite 530, Addison, TX 75001.

(1)  Mitsubishi Heavy Industries MU-2B Checklists:

(i)

Cockpit Checklist, Model MU-2B-60, Type Certificate A10SW, MHI Document No. YET06220C, accepted by FSB on February 12, 2007.

(ii)  Cockpit Checklist, Model MU-2B-40, Type Certificate A10SW, MHI Document No. YET06256A, accepted by FSB on February 12, 2007.

(iii)  Cockpit Checklist, Model MU-2B-36A, Type Certificate A10SW, MHI Document No. YET06257B, accepted by FSB on February 12, 2007.

(iv) Cockpit Checklist, Model MU-2B-36, Type Certificate A2PC, MHI Document No. YET06252B, accepted by FSB on February 12, 2007.

(v)  Cockpit Checklist, Model MU-2B-35, Type Certificate A2PC, MHI Document No. YET06251B, accepted by FSB on February 12, 2007.

(vi) Cockpit Checklist, Model MU-2B-30, Type Certificate A2PC, MHI Document No. YET06250A, accepted by FSB on March 2, 2007.

14 CFR91.1721(b)(1)(vi)(enhanceddisplay)

page 222of243

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFR91.1721(b)(1)(vii)

(vii) Cockpit Checklist, Model MU-2B-26A, Type Certificate A10SW, MHI Document No. YET06255A, accepted by FSB on February 12, 2007.

(viii) Cockpit Checklist, Model MU-2B-26, Type Certificate A2PC, MHI Document No. YET06249A, accepted by FSB on March 2, 2007.

(ix) Cockpit Checklist, Model MU-2B-26, Type Certificate A10SW, MHI Document No. YET06254A, accepted by FSB on March 2, 2007.

(x)

Cockpit Checklist, Model MU-2B-25, Type Certificate A10SW, MHI Document No. YET06253A, accepted by FSB on March 2, 2007.

(xi) Cockpit Checklist, Model MU-2B-25, Type Certificate A2PC, MHI Document No. YET06248A, accepted by FSB on March 2, 2007.

(xii) Cockpit Checklist, Model MU-2B-20, Type Certificate A2PC, MHI Document No. YET06247A, accepted by FSB on February 12, 2007.

(xiii)-(xiv) [Reserved]

(xv) Cockpit Checklist, Model MU-2B-15, Type Certificate A2PC, MHI Document No. YET06246A, accepted by FSB on March 2, 2007.

(xvi) Cockpit Checklist, Model MU-2B-10, Type Certificate A2PC, MHI Document No. YET06245A, accepted by FSB on March 2, 2007.

(xvii) Cockpit Checklist, Model MU-2B, Type Certificate A2PC, MHI Document No. YET06244A, accepted by FSB on March 2, 2007.

(2)  [Reserved]

[Docket FAA-2006-24981, Amdt. 91-344, 81 FR 61591, Sept. 7, 2016;Amdt. 91-344A, 82 FR 21472, May 9, 2017]

Appendix Ato Part 91—Category II Operations: Manual, Instruments, Equipment, and Maintenance

1. Category II Manual

(a)   Application forapproval. An applicant for approval of a Category II manual or an amendment to an approved Category II manual must submit the proposed manual or amendment to the responsible Flight Standards office. If the application requests an evaluation program, it must include the following:

(1) (2)

The location ofthe aircraft and the place where the demonstrations are to be conducted; and The date the demonstrations are to commence (at least 10 days after filing the application).

(b)

Contents. Each Category II manual must contain:

(1) (2)

The registration number, make, and model of the aircraft to which it applies; A maintenance program as specified in section 4 of this appendix; and

14 CFRAppendix-A-to-Part-911.(b)(2) (enhanceddisplay)

page 223 of243

(a)

(b)

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFRAppendix-A-to-Part-911.(b)(3)

(3)

The procedures and instructions related to recognition of decision height, use of runway visual range information, approach monitoring, the decision region (the region between the middle marker and the decision height), the maximum permissible deviations ofthe basic ILS indicator within the decision region, a missed approach, use of airborne low approach equipment, minimum altitude for the use of the autopilot, instrument and equipment failure warning systems, instrument failure, and other procedures, instructions, and limitations that may be found necessary by the Administrator.

2. Required Instruments and Equipment

The instruments and equipment listed in this section must be installed in each aircraft operated in a Category II operation. This section does not require duplication of instruments and equipment required by § 91.205 or any other provisions of this chapter.

Group I. (1) Two localizer and glide slope receiving systems. Each system must provide a basic ILS display and each side of the instrument panel must have a basic ILS display. However, a single localizer antenna and a single glide slope antenna may be used.

(2) (3)

(4)

A communications system that does not affect the operation of at least one of the ILS systems.

A marker beacon receiver that provides distinctive aural and visual indications ofthe outer and the middle markers.

Two gyroscopic pitch and bank indicating systems.

(5)   Two gyroscopic direction indicating systems. (6)   Two airspeed indicators.

(7)

Two sensitive altimeters adjustable for barometric pressure, each having a placarded correction for altimeter scale error and for the wheel height of the aircraft. After June 26, 1979, two sensitive altimeters adjustable for barometric pressure, having markings at 20-foot intervals and each having a placarded correction for altimeter scale error and forthe wheel height of the aircraft.

(8)   Two vertical speed indicators.

(9)

A flight control guidance system that consists of either an automatic approach coupler or a flight director system. A flight director system must display computed information as steering command in relation to an ILS localizer and, on the same instrument, either computed information as pitch command in relation to an ILS glide slope or basic ILS glide slope information. An automatic approach coupler must provide at least automatic steering in relation to an ILS localizer. The flight control guidance system may be operated from one of the receiving systems required by subparagraph (1) of this paragraph.

(10) For Category II operations with decision heights below 150 feet either a marker beacon receiver providing aural and visual indications of the inner marker or a radio altimeter.

Group II. (1) Warning systems for immediate detection by the pilot of system faults in items (1), (4), (5), and (9) of Group I and, if installed for use in Category III operations, the radio altimeter and autothrottle system.

(2)  Dual controls.

14 CFRAppendix-A-to-Part-912.(b)(2) (enhanceddisplay)

page 224of243

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFRAppendix-A-to-Part-912.(b)(3)

(3) (4)

(5)

An externally vented static pressure system with an alternate static pressure source.

A windshield wiper or equivalent means of providing adequate cockpit visibility for a safe visual transition by either pilot to touchdown and rollout.

A heat source for each airspeed system pitot tube installed or an equivalent means of preventing malfunctioning due to icing of the pitot system.

3. Instruments and Equipment Approval

(a)  General. The instruments and equipment required by section 2 ofthis appendix must be approved as provided in this section before being used in Category II operations. Before presenting an aircraft for approval of the instruments and equipment, it must be shown that since the beginning ofthe 12th calendar month before the date of submission—

(1)

(2) (3)

The ILS localizer and glide slope equipment were bench checked according to the manufacturer's instructions and found to meet those standards specified in RTCA Paper 23-63/ DO-117 dated March 14, 1963, “Standard Adjustment Criteria for Airborne Localizer and Glide Slope Receivers,” which may be obtained from the RTCA Secretariat, 1425 K St., NW., Washington, DC 20005.

The altimeters and the static pressure systems were tested and inspected in accordance with appendix E to part 43 of this chapter; and

All other instruments and items of equipment specified in section 2(a) of this appendixthat are listed in the proposed maintenance program were bench checked and found to meet the manufacturer's specifications.

(b)

(c)

Flight controlguidance system. All components ofthe flight control guidance system must be approved as installed by the evaluation program specified in paragraph (e) of this section if they have not been approved for Category III operations under applicable type or supplemental type certification procedures. In addition, subsequent changes to make, model, or design of the components must be approved under this paragraph. Related systems or devices, such as the autothrottle and computed missed approach guidance system, must be approved in the same manner if they are to be used for Category II operations.

Radio altimeter. A radio altimeter must meet the performance criteria of this paragraph for original approval and after each subsequent alteration.

(1) (2)

It must display to the flight crew clearly and positively the wheel height of the main landing gear above the terrain.

It must display wheel height above the terrain to an accuracy of plus or minus 5 feet or 5 percent, whichever is greater, underthe following conditions:

(i)   Pitch angles of zero to plus or minus 5 degrees about the mean approach attitude. (ii)  Roll angles of zero to 20 degrees in either direction.

(iii)  Forward velocities from minimum approach speed up to 200 knots.

(iv)  Sink rates from zero to 15 feet per second at altitudes from 100 to 200 feet.

14 CFRAppendix-A-to-Part-913.(c)(2)(iv) (enhanceddisplay)

page 225of243

(1) (2)

(3)

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFRAppendix-A-to-Part-913.(c)(3)

(3) (4)

(5) (6)

Over level ground, it must track the actual altitude ofthe aircraft without significant lag or oscillation.

With the aircraft at an altitude of 200 feet or less, any abrupt change in terrain representing no more than 10 percent ofthe aircraft's altitude must not cause the altimeter to unlock, and indicator response to such changes must not exceed 0.1 seconds and, in addition, ifthe system unlocks for greater changes, it must reacquire the signal in less than 1 second.

Systems that contain a push-to-test feature must test the entire system (with or without an antenna) at a simulated altitude of less than 500 feet.

The system must provide to the flight crew a positive failure warning display any time there is a loss of power or an absence of ground return signals within the designed range of operating altitudes.

(d)

Other instruments andequipment. All other instruments and items of equipment required by § 2 of this appendix must be capable of performing as necessary for Category II operations. Approval is also required after each subsequent alteration to these instruments and items of equipment.

(e) Evaluation program —

Application. Approval by evaluation is requested as a part ofthe application for approval of the Category II manual.

Demonstrations. Unless otherwise authorized by the Administrator, the evaluation program for each aircraft requires the demonstrations specified in this paragraph. At least 50 ILS approaches must be flown with at least five approaches on each of three different ILS facilities and no more than one half of the total approaches on any one ILS facility. All approaches shall be flown under simulated instrument conditions to a 100-foot decision height and 90 percent of the total approaches made must be successful. A successful approach is one in which—

(i)

(ii) (iii) (iv)

(v)

At the 100-foot decision height, the indicated airspeed and heading are satisfactory for a normal flare and landing (speed must be plus or minus 5 knots of programmed airspeed, but may not be less than computed threshold speed if autothrottles are used);

The aircraft at the 100-foot decision height, is positioned so that the cockpit is within, and tracking so as to remain within, the lateral confines of the runway extended;

Deviation from glide slope after leaving the outer marker does not exceed 50 percent of full-scale deflection as displayed on the ILS indicator;

No unusual roughness or excessive attitude changes occur after leaving the middle marker; and

In the case of an aircraft equipped with an approach coupler, the aircraft is sufficiently in trim when the approach coupler is disconnected at the decision height to allow for the continuation of a normal approach and landing.

Records. During the evaluation program the following information must be maintained bythe applicant for the aircraft with respect to each approach and made available to the Adninistrator upon request:

(i)

Each deficiency in airborne instruments and equipment that prevented the initiation of an approach.

14 CFRAppendix-A-to-Part-913.(e)(3)(i)(enhanced display)

page 226of243

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFRAppendix-A-to-Part-913.(e)(3)(ii)

(ii)

The reasons for discontinuing an approach, including the altitude above the runway at which it was discontinued.

(iii)  Speed control at the 100-foot decision height if auto throttles are used.

(iv)  Trim condition of the aircraft upon disconnecting the auto coupler with respect to continuation to flare and landing.

(v)

Position of the aircraft at the middle marker and at the decision height indicated both on a diagram of the basic ILS display and a diagram ofthe runway extended to the middle marker. Estimated touchdown point must be indicated on the runway diagram.

(4)

(vi)   Compatibility of flight director with the auto coupler, if applicable. (vii)  Quality of overall system performance.

Evaluation. A final evaluation ofthe flight control guidance system is made upon successful completion of the demonstrations. If no hazardous tendencies have been displayed or are otherwise known to exist, the system is approved as installed.

4. Maintenance program

(a)  Each maintenance program must contain the following:

(1)   A list of each instrument and item of equipment specified in § 2 of this appendix that is installed in the aircraft and approved for Category II operations, including the make and model of those specified in § 2(a).

(2)

(3) (4) (5)

(6) (7)

A schedule that provides for the performance of inspections under subparagraph (5) of this paragraph within 3 calendar months afterthe date ofthe previous inspection. The inspection must be performed by a person authorized by part 43 of this chapter, except that each alternate inspection may be replaced by a functional flight check. This functional flight check must be performed by a pilot holding a Category II pilot authorization forthe type aircraft checked.

A schedule that provides for the performance of bench checks for each listed instrument and item of equipment that is specified in section 2(a) within 12 calendar months after the date of the previous bench check.

A schedule that provides for the performance of a test and inspection of each static pressure system in accordance with appendix E to part 43 of this chapter within 12 calendar months after the date of the previous test and inspection.

The procedures for the performance of the periodic inspections and functional flight checks to determine the ability of each listed instrument and item of equipment specified in section 2(a) of this appendix to perform as approved for Category II operations including a procedure for recording functional flight checks.

A procedure for assuring that the pilot is informed of all defects in listed instruments and items of equipment.

A procedure for assuring that the condition of each listed instrument and item of equipment upon which maintenance is performed is at least equal to its Category II approval condition before it is returned to service for Category II operations.

14 CFRAppendix-A-to-Part-914.(a)(7) (enhanceddisplay)

page 227of243

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFRAppendix-A-to-Part-914.(a)(8)

(8)

A procedure for an entry in the maintenance records required by § 43.9 of this chapter that shows the date, airport, and reasons for each discontinued Category II operation because of a malfunction of a listed instrument or item of equipment.

(b)  Bench check. A bench check required bythis section must comply with this paragraph.

(1) (2)

It must be performed by a certificated repair station holding one ofthe following ratings as appropriate to the equipment checked:

(i)   An instrument rating. (ii)   A radio rating.

It must consist of removal of an instrument or item of equipment and performance of the following:

(i)   A visual inspection for cleanliness, impending failure, and the need for lubrication, repair, or replacement of parts;

(ii)   Correction of items found by that visual inspection; and

(iii)  Calibration to at least the manufacturer's specifications unless otherwise specified in the approved Category II manual for the aircraft in which the instrument or item of equipment is installed.

(c)  Extensions. After the completion of one maintenance cycle of 12 calendar months, a request to extend the period for checks, tests, and inspections is approved if it is shown that the performance of particular equipment justifies the requested extension.

[Docket 18334, 54 FR 34325, Aug. 18, 1989, as amendedbyAmdt. 91-269, 66 FR 41116, Aug. 6, 2001; DocketFAA-2018-0119, Amdt. 91-350, 83 FR 9172, Mar. 5, 2018]

Appendixes B—C [Reserved]

Appendix Dto Part 91—Airports/Locations: Special Operating Restrictions

Section 1. Locations at which the requirements of § 91.215(b)(2) and § 91.225(d)(2) apply. The requirements of §§ 91.215(b)(2) and 91.225(d)(2) apply below 10,000 feet MSL within a 30-nautical-mile radius of each location in the following list.

Atlanta, GA (Hartsfield-Jackson Atlanta International Airport)

Baltimore, MD (Baltimore/Washington International Thurgood Marshall Airport) Boston, MA (General Edward Lawrence Logan International Airport)

Camp Springs, MD (Joint Base Andrews)

Chantilly, VA (Washington Dulles International Airport) Charlotte, NC (Charlotte/Douglas International Airport)

Chicago, IL (Chicago-O'Hare International Airport)

14 CFRAppendix-D-to-Part-91Section1.(enhanceddisplay)

page 228 of243

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

Cleveland, OH (Cleveland-Hopkins International Airport) Covington, KY (Cincinnati/Northern Kentucky International Airport) Dallas, TX (Dallas/Fort Worth International Airport)

Denver, CO (Denver International Airport)

Detroit, MI (Detroit Metropolitan Wayne County Airport) Honolulu, HI (Honolulu International Airport)

Houston, TX (George Bush Intercontinental/Houston Airport) Houston, TX (William P. Hobby Airport)

Kansas City, MO (Kansas City International Airport) Las Vegas, NV (McCarran International Airport)

Los Angeles, CA (Los Angeles International Airport) Memphis, TN (Memphis International Airport) Miami, FL (Miami International Airport)

14 CFRAppendix-D-to-Part-91Section1.

Minneapolis, MN (Minneapolis-St. Paul International/Wold-Chamberlain Airport) Newark, NJ (Newark Liberty International Airport)

New Orleans, LA (Louis Armstrong New Orleans International Airport) New York, NY (John F. Kennedy International Airport)

New York, NY (LaGuardia Airport)

Orlando, FL (Orlando International Airport) Philadelphia, PA (Philadelphia International Airport) Phoenix, AZ (Phoenix Sky Harbor International Airport) Pittsburgh, PA (Pittsburgh International Airport)

St. Louis, MO (Lambert-St. Louis International Airport)

14 CFRAppendix-D-to-Part-91Section1.(enhanceddisplay)

page 229 of243

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

Salt Lake City, UT (Salt Lake City International Airport) San Diego, CA (Miramar Marine Corps Air Station) San Diego, CA (San Diego International Airport)

San Francisco, CA (San Francisco International Airport) Seattle, WA (Seattle-Tacoma International Airport) Tampa, FL (Tampa International Airport)

14 CFRAppendix-D-to-Part-91Section1.“Section2”

Washington, DC (Ronald Reagan Washington National Airport)

Section 2. Airports at which the requirements of § 91.215(b)(5)(ii) apply. [Reserved] Section 3. Locations at which fixed-wing Special VFR operations are prohibited.

The Special VFR weather minimums of § 91.157 do not apply to the following airports: Atlanta, GA (Hartsfield-Jackson Atlanta International Airport)

Baltimore, MD (Baltimore/Washington International Thurgood Marshall Airport) Boston, MA (General Edward Lawrence Logan International Airport)

Buffalo, NY (Greater Buffalo International Airport) Camp Springs, MD (Joint Base Andrews)

Chicago, IL (Chicago-O'Hare International Airport) Cleveland, OH (Cleveland-Hopkins International Airport) Columbus, OH (Port Columbus International Airport)

Covington, KY (Cincinnati/Northern Kentucky International Airport) Dallas, TX (Dallas/Fort Worth International Airport)

Dallas, TX (Dallas Love Field Airport) Denver, CO (Denver International Airport)

Detroit, MI (Detroit Metropolitan Wayne County Airport) Honolulu, HI (Honolulu International Airport)

Houston, TX (George Bush Intercontinental/Houston Airport)

14 CFRAppendix-D-to-Part-91Section1.“Section3”(enhanceddisplay)

page 230 of243

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

Indianapolis, IN (Indianapolis International Airport) Los Angeles, CA (Los Angeles International Airport)

14 CFRAppendix-D-to-Part-91Section1.“Section4”

Louisville, KY (Louisville International Airport-Standiford Field) Memphis, TN (Memphis International Airport)

Miami, FL (Miami International Airport)

Minneapolis, MN (Minneapolis-St. Paul International/Wold-Chamberlain Airport) Newark, NJ (Newark Liberty International Airport)

New York, NY (John F. Kennedy International Airport) New York, NY (LaGuardia Airport)

New Orleans, LA (Louis Armstrong New Orleans International Airport) Philadelphia, PA (Philadelphia International Airport)

Pittsburgh, PA (Pittsburgh International Airport) Portland, OR (Portland International Airport)

San Francisco, CA (San Francisco International Airport) Seattle, WA (Seattle-Tacoma International Airport)

St. Louis, MO (Lambert-St. Louis International Airport) Tampa, FL (Tampa International Airport)

Washington, DC (Ronald Reagan Washington National Airport)

Section 4. Locations at which solo student, sport, and recreational pilot activity is not permitted.

Pursuant to § 91.131(b)(2), solo student, sport, and recreational pilot operations are not permitted at any ofthe following airports.

Atlanta, GA (Hartsfield-Jackson Atlanta International Airport) Boston, MA (General Edward Lawrence Logan International Airport) Camp Springs, MD (Joint Base Andrews)

Chicago, IL (Chicago-O'Hare International Airport)

14 CFRAppendix-D-to-Part-91Section1.“Section4”(enhanceddisplay)

page 231of243

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

Dallas, TX (Dallas/Fort Worth International Airport) Los Angeles, CA (Los Angeles International Airport) Miami, FL (Miami International Airport)

Newark, NJ (Newark Liberty International Airport) New York, NY (John F. Kennedy International Airport) New York, NY (LaGuardia Airport)

San Francisco, CA (San Francisco International Airport) Washington, DC (Ronald Reagan Washington National Airport)

[Amdt. 91-227, 56 FR 65661, Dec. 17, 1991]

14 CFRAppendix-D-to-Part-91Section1.“Section4”

Editorial Note: For FEDERAL REGISTER citations affecting appendix D to part 91, see the List of CFR Sections Affected, which appears in the Finding Aids section of the printed volume and at www.govinfo.gov.

Effective Date Note: ByAmdt. 91-236, 59 FR 2918, Jan. 19, 1994, as corrected byAmdt. 91-237, 59 FR 6547, Feb. 11, 1994, appendix D to part 91 was amended in sections 1 and 3 in the Denver, CO, entry by revising “Stapleton” to read “Denver” effective Mar. 9, 1994. By Amdt. 91-238, 59 FR 10958, Mar. 9, 1994, the effective date was delayed to May 15, 1994. By Amdt. 91-241, 59 FR 24916, May 13, 1994, the effective date was suspended indefinitely.

14 CFRAppendix-D-to-Part-91Section1.“Section4”(enhanceddisplay)

page 232of243

1

4 (or 1 per second where peaks, ref. to 1g are recorded)

2

Installed system 1 minimum accuracy (to recovered data)

±0.125% per hour

±5% or ±10 kts., whichever is greater.

Resolution 2 kts. below 175 KIAS

±100 to ±700 ft. (see Table 1, TSO C51-a)

±5°

±0.2g in addition to ±0.3g maximum datum

Resolution 4 read out

1 sec. 1% 3

25 to 150 ft.

1° 0.03g.

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFRAppendix-D-to-Part-91Section1.“Section4”

Appendix E to Part91—Airplane Flight Recorder Specifications

Parameters                    Range

Sampling interval (per

second)

Relative Time (From Recorded on Priorto Takeoff)

Indicated Airspeed

Altitude

Magnetic Heading Vertical Acceleration

8 hr minimum

Vso to VD (KIAS)

−1,000 ft. to max cert. alt. of

A/C 360°

−3g to + 6g

1 1

1

Longitudinal                       ±1.0g                   ±1.5% max. range                                          0.01g. Acceleration                                                    excluding datum error

of ±5%

Pitch Attitude                     100% of usable   ±2°                                  1                             0.8°

Roll Attitude                        ±60° or 100%      ±2°                                  1                             0.8° of usable

range, whichever is greater

Stabilizer Trim Position,     Full Range           ±3% unless higher          1                             1% 3 or                                                                    uniquely required

Pitch Control Position 5

1 When data sources are aircraftinstruments  (exceptaltimeters)  ofacceptable qualityto flythe aircraft the recording system excluding these sensors (but including all other characteristics ofthe recording system) shall contribute no more than halfofthe values in this column.

2 Ifdata from the altitude encoding altimeter (100 ft. resolution) is used, then eitherone ofthese parameters should also be recorded. Ifhowever, altitude is recordedat a minimum resolution of25 feet, then these twoparameters can be omitted.

3 Percent offull range.

4 This column applies to aircraftmanufactured after October 11, 1991.

5 ForPitch ControlPosition only,  forallaircraftmanufactured  on  orafterApril 6, 2012, the sampling interval (persecond) is 8. Each input mustbe recorded atthis rate. Alternatelysampling inputs (interleaving) to meet this sampling intervalisprohibited.

14 CFRAppendix-D-to-Part-91Section1.“Section4”(enhanceddisplay)

page 233 of243

Engine Power, Each Engine:

Fan or N 1 Speed or EPR or Cockpit indications Used for Aircraft Certification OR

Prop. speed and Torque (Sample Once/Sec as Close together as Practicable)

Altitude Rate 2 (need depends on altitude resolution)

Angle of Attack 2 (need depends on altitude resolution)

Radio Transmitter Keying (Discrete)

TE Flaps (Discrete or Analog)

Installed system 1 minimum accuracy (to recovered data)

±3% unless higher uniquely required

±5%

±10%. Resolution 250 fpm below 12,000 ft.

indicated ±2°

Resolution 4 read out

1% 3 1% 3

1%3 1% 3

250 fpm. below 12,000 0.8%3

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

Parameters                    Range

14 CFRAppendix-D-to-Part-91Section1.“Section4”

Sampling interval (per

second)

Full Range

Maximum Range

±8,000 fpm

−20° to 40° or 100% of usable range

On/Off

1 1

1 (prop Speed) 1 (torque)

1 1

1

Each discrete                                             1 position (U, D,

T/O, AAP) OR

LE Flaps (Discrete or          Analog 0-100%   ±3%                                1                             1%3 Analog)                              range

Each discrete                                             1 position (U, D,

T/O, AAP) OR

Thrust Reverser, Each       Analog 0-100%   ±3°                                 1                            1%3 Engine (Discrete)               range

Stowed or full reverse

1 When data sources are aircraftinstruments  (exceptaltimeters)  ofacceptable qualityto flythe aircraft the recording system excluding these sensors (but including all other characteristics ofthe recording system) shall contribute no more than halfofthe values in this column.

2 Ifdata from the altitude encoding altimeter (100 ft. resolution) is used, then eitherone ofthese parameters should also be recorded. Ifhowever, altitude is recordedat a minimum resolution of25 feet, then these twoparameters can be omitted.

3 Percent offull range.

4 This column applies to aircraftmanufactured after October 11, 1991.

5 ForPitch ControlPosition only,  forallaircraftmanufactured  on  orafterApril 6, 2012, the sampling interval (persecond) is 8. Each input mustbe recorded atthis rate. Alternatelysampling inputs (interleaving) to meet this sampling intervalisprohibited.

14 CFRAppendix-D-to-Part-91Section1.“Section4”(enhanceddisplay)

page 234of243

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

Parameters                    Range

14 CFRAppendix-D-to-Part-91Section1.“Section4”

Installed system 1 minimum accuracy (to recovered data)

Sampling interval (per

second)

Resolution 4 read out

Spoiler/Speedbrake           Stowed or out                                             1 (Discrete)

Autopilot Engaged             Engaged or                                                 1 (Discrete)                           Disengaged

1 When data sources are aircraftinstruments  (exceptaltimeters)  ofacceptable qualityto flythe aircraft the recording system excluding these sensors (but including all other characteristics ofthe recording system) shall contribute no more than halfofthe values in this column.

2 Ifdata from the altitude encoding altimeter (100 ft. resolution) is used, then eitherone ofthese parameters should also be recorded. Ifhowever, altitude is recordedat a minimum resolution of25 feet, then these twoparameters can be omitted.

3 Percent offull range.

4 This column applies to aircraftmanufactured after October 11, 1991.

5 ForPitch ControlPosition only,  forallaircraftmanufactured  on  orafterApril 6, 2012, the sampling interval (persecond) is 8. Each input mustbe recorded atthis rate. Alternatelysampling inputs (interleaving) to meet this sampling intervalisprohibited.

[Docket 18334, 54 FR 34327, Aug. 18, 1989, as amendedbyAmdt. 91-300, 73 FR 12565, Mar. 7, 2008; 73 FR 15280, Mar. 21, 2008; Amdt. 91-313, 75 FR 17046, Apr. 5, 2010;Amdt. 91-329, 78 FR 39971, July 3, 2013]

Appendix F toPart 91—Helicopter Flight Recorder Specifications

Parameters                       Range

Installed system 1 minimum accuracy (to recovered data)

Sampling interval (per

second)

Resolution 3 read out

Relative Time      4 hr minimum                           ±0.125% per hour    1 (From

Recorded on Prior to Takeoff)

Indicated             VM in to VD (KIAS)                                                   1 1 When data sources are aircraftinstruments

aircraft the recording system excluding these

recording system) shall contribute no more than halfofthe values in this column. 2 Percent offull range.

3 This column applies to aircraftmanufactured after October 11, 1991.

4 Forall aircraft manufacturedon  orafterApril6, 2012, the sampling intervalpersecond is 4.

1 sec.

±5% or ±10 kts.,                                      1 kt. (except altimeters) ofacceptable qualityto flythe sensors (but including all other characteristics ofthe

14 CFRAppendix-D-to-Part-91Section1.“Section4”(enhanceddisplay)

page 235of243

Installed system 1

minimum accuracy (to recovered data)

whichever is greater

±100 to ±700 ft. (see Table 1, TSO C51-a)

±5°

±0.2g in addition to ±0.3g maximum datum

±1.5% max. range excluding datum error of ±5%

±2° ±2°

±10% Resolution 250 fpm below 12,000 ft. indicated

Resolution 3 read out

25 to 150 ft.

1° 0.05g.

0.03g.

0.8° 0.8°

250 fpm below 12,000.

1

1

4 (or 1 per second where peaks, ref. to 1g are recorded)

2

1 1

1

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

Parameters                       Range

14 CFRAppendix-D-to-Part-91Section1.“Section4”

Sampling interval (per

second)

Airspeed Altitude

(minimum airspeed signal attainable with installed pilot-static system) −1,000 ft. to 20,000 ft. pressure altitude

Magnetic             360° Heading

Vertical                −3g to + 6g Acceleration

Longitudinal         ±1.0g Acceleration

Pitch Attitude Roll Attitude

Altitude Rate

100% of usable range

±60 or 100% of usable range, whichever is greater

±8,000 fpm

Engine Power, Each Engine

Main Rotor           Maximum Range                      ±5%                         1 Speed

Free or Power      Maximum Range                      ±5%                         1 Turbine

Engine Torque     Maximum Range                      ±5%                         1 Flight Control

Hydraulic Pressure

Primary                High/Low

1 When data sources are aircraftinstruments aircraft the recording system excluding these

recording system) shall contribute no more than halfofthe values in this column. 2 Percent offull range.

3 This column applies to aircraftmanufactured after October 11, 1991.

4 Forall aircraft manufacturedon  orafterApril6, 2012, the sampling intervalpersecond is 4. 1%2.

1%2. 1%2.

1

(except altimeters) ofacceptable qualityto flythe sensors (but including all other characteristics ofthe

14 CFRAppendix-D-to-Part-91Section1.“Section4”(enhanceddisplay)

page 236of243

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

Parameters                       Range

(Discrete)

Secondary—if      High/Low applicable

(Discrete)

Radio                   On/Off Transmitter

Keying (Discrete)

Autopilot              Engaged or Disengaged Engaged

(Discrete)

SAS Status-        Engaged or Disengaged Engaged

(Discrete)

SAS Fault            Fault/OK Status

(Discrete) Flight Controls

Collective 4          Full range

Pedal Position     Full range 4

Resolution 3 read out

1%2. 1%2.

14 CFRAppendix-D-to-Part-91Section1.“Section4”

Installed system 1

minimum accuracy (to recovered data)

1 1

1 1 1

Sampling interval (per

second)

±3% ±3%

2 2

Lat. Cyclic 4         Full range                                  ±3%                         2 Long. Cyclic 4      Full range                                  ±3%                         2 Controllable         Full range                                  ±3%                         2 Stabilator

Position 4

1 When data sources are aircraftinstruments aircraft the recording system excluding these

recording system) shall contribute no more than halfofthe values in this column. 2 Percent offull range.

3 This column applies to aircraftmanufactured after October 11, 1991.

4 Forall aircraft manufacturedon  orafterApril6, 2012, the sampling intervalpersecond is 4. 1%2.

1%2. 1%2.

(except altimeters) ofacceptable qualityto flythe sensors (but including all other characteristics ofthe

[Docket 18334, 54 FR 34328, Aug. 18, 1989; 54 FR 41211, Oct. 5, 1989; 54 FR 53036, Dec. 26, 1989;Amdt. 91-300, 73 FR 12565, Mar. 7, 2008; 73 FR 15280, Mar. 21, 2008;Amdt. 91-313, 75 FR 17046, Apr. 5, 2010]

Appendix Gto Part 91—Operations in Reduced Vertical Separation Minimum (RVSM) Airspace

Section 1. Definitions

14 CFRAppendix-D-to-Part-91Section1.“Section4”(enhanceddisplay)

page 237of243

(a)

(b)

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFRAppendix-D-to-Part-91Section1.“Section4”

Reduced Vertical Separation Minimum (RVSM) Airspace. Within RVSM airspace, air traffic control (ATC) separates aircraft by a minimum of 1,000 feet vertically between FL 290 and FL 410 inclusive. Air-traffic control notifies operators of RVSM airspace by providing route planning information.

RVSM GroupAircraft. Aircraft within a group of aircraft, approved as a group bythe Administrator, in which each of the aircraft satisfy each ofthe following:

(a) (b)

(c)

The aircraft have been manufactured to the same design, and have been approved under the same type certificate, amended type certificate, or supplemental type certificate.

The static system of each aircraft is installed in a manner and position that is the same as those of the other aircraft in the group. The same static source error correction is incorporated in each aircraft of the group.

The avionics units installed in each aircraft to meet the minimum RVSM equipment requirements of this appendix are:

(1) (2)

Manufactured to the same manufacturer specification and have the same part number; or

Of a different manufacturer or part number, if the applicant demonstrates that the equipment provides equivalent system performance.

RVSM Nongroup Aircraft. An aircraft that is approved for RVSM operations as an individual aircraft.

RVSM Flight envelope. An RVSM flight envelope includes the range of Mach number, weight divided by atmospheric pressure ratio, and altitudes over which an aircraft is approved to be operated in cruising flight within RVSM airspace. RVSM flight envelopes are defined as follows:

The full RVSM flight envelope is bounded as follows:

(1)

The altitude flight envelope extends from FL 290 upward to the lowest altitude of the following: (i)   FL 410 (the RVSM altitude limit);

(ii)   The maximum certificated altitude for the aircraft; or

(iii)  The altitude limited by cruise thrust, buffet, or other flight limitations.

(2)  The airspeed flight envelope extends:

(i)

From the airspeed of the slats/flaps-up maximum endurance (holding) airspeed, or the maneuvering airspeed, whichever is lower;

(3)

(ii)  To the maximum operating airspeed (Vmo/Mmo), or airspeed limited by cruise thrust buffet, or other flight limitations, whichever is lower.

All permissible gross weights within the flight envelopes defined in paragraphs (1) and (2) of this definition.

The basic RVSM flight envelope is the same as the full RVSM flight envelope except that the airspeed flight envelope extends:

(1)

From the airspeed of the slats/flaps-up maximum endurance (holding) airspeed, or the maneuver airspeed, whichever is lower;

14 CFRAppendix-D-to-Part-91Section1.“Section4”(enhanceddisplay)

page 238 of243

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFRAppendix-D-to-Part-91Section1.“Section4”

(2)  To the upper Mach/airspeed boundary defined for the full RVSM flight envelope, or a specified lower value not less than the long-range cruise Mach number plus .04 Mach, unless further limited by available cruise thrust, buffet, or other flight limitations.

Section 2. Aircraft Approval

(a) (b)

(c)

(d)

Except as specified in Section 9 of this appendix, an operator may be authorized to conduct RVSM operations if the Administrator finds that its aircraft comply with this section.

The applicant for authorization shall submit the appropriate data package for aircraft approval. The package must consist of at least the following:

(1)   An identification ofthe RVSM aircraft group or the nongroup aircraft;

(2)   A definition ofthe RVSM flight envelopes applicable to the subject aircraft;

(3)   Documentation that establishes compliance with the applicable RVSM aircraft requirements of this section; and

(4)   The conformity tests used to ensure that aircraft approved with the data package meet the RVSM aircraft requirements.

Altitude-keeping equipment:All aircraft. To approve an aircraft group or a nongroup aircraft, the Administrator must find that the aircraft meets the following requirements:

(1)   The aircraft must be equipped with two operational independent altitude measurement systems.

(2)   The aircraft must be equipped with at least one automatic altitude control system that controls the aircraft altitude—

(i)   Within a tolerance band of ±65 feet about an acquired altitude when the aircraft is operated in straight and level flight under nonturbulent, nongust conditions; or

(ii)   Within a tolerance band of ±130 feet under nonturbulent, nongust conditions for aircraft for which application for type certification occurred on or before April 9, 1997 that are equipped with an automatic altitude control system with flight management/performance system inputs.

(3)   The aircraft must be equipped with an altitude alert system that signals an alert when the altitude displayed to the flight crew deviates from the selected altitude by more than:

(i)   ±300 feet for aircraft for which application for type certification was made on or before April 9, 1997; or

(ii)   ±200 feet for aircraft for which application for type certification is made after April 9, 1997.

Altimetry system errorcontainment: Group aircraft for which application for type certification was made on orbefore April 9, 1997. To approve group aircraft for which application for type certification was made on or before April 9, 1997, the Administrator must find that the altimetry system error (ASE) is contained as follows:

(1)  At the point in the basic RVSM flight envelope where mean ASE reaches its largest absolute value, the absolute value may not exceed 80 feet.

14 CFRAppendix-D-to-Part-91Section1.“Section4”(enhanceddisplay)

page 239 of243

(e)

(f)

(g) (h)

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFRAppendix-D-to-Part-91Section1.“Section4”

(2) (3) (4) (5)

At the point in the basic RVSM flight envelope where mean ASE plus three standard deviations reaches its largest absolute value, the absolute value may not exceed 200 feet.

At the point in the full RVSM flight envelope where mean ASE reaches its largest absolute value, the absolute value may not exceed 120 feet.

At the point in the full RVSM flight envelope where mean ASE plus three standard deviations reaches its largest absolute value, the absolute value may not exceed 245 feet.

Necessary operating restrictions. If the applicant demonstrates that its aircraft otherwise comply with the ASE containment requirements, the Administrator may establish an operating restriction on that applicant's aircraft to restrict the aircraft from operating in areas of the basic RVSM flight envelope where the absolute value of mean ASE exceeds 80 feet, and/or the absolute value of mean ASE plus three standard deviations exceeds 200 feet; or from operating in areas ofthe full RVSM flight envelope where the absolute value of the mean ASE exceeds 120 feet and/or the absolute value of the mean ASE plus three standard deviations exceeds

245 feet.

Altimetry system errorcontainment: Group aircraft for which application for type certification is made after April 9, 1997. To approve group aircraft for which application for type certification is made after April 9, 1997, the Administrator must find that the altimetry system error (ASE) is contained as follows:

(1) (2)

At the point in the full RVSM flight envelope where mean ASE reaches its largest absolute value, the absolute value may not exceed 80 feet.

At the point in the full RVSM flight envelope where mean ASE plus three standard deviations reaches its largest absolute value, the absolute value may not exceed 200 feet.

Altimetry system errorcontainment: Nongroup aircraft. To approve a nongroup aircraft, the Administrator must find that the altimetry system error (ASE) is contained as follows:

(1) (2)

For each condition in the basic RVSM flight envelope, the largest combined absolute value for residual static source error plus the avionics error may not exceed 160 feet.

For each condition in the full RVSM flight envelope, the largest combined absolute value for residual static source error plus the avionics error may not exceed 200 feet.

Traffic Alert and Collision Avoidance System (TCAS) Compatibility With RVSM Operations: All aircraft. After March 31, 2002, unless otherwise authorized by the Administrator, ifyou operate an aircraft that is equipped with TCAS II in RVSM airspace, it must be a TCAS II that meets TSO C-119b (Version 7.0), or a later version.

If the Administrator finds that the applicant's aircraft comply with this section, the Administrator notifies the applicant in writing.

Section 3. Operator Authorization

14 CFRAppendix-D-to-Part-91Section1.“Section4”(enhanceddisplay)

page 240 of243

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFRAppendix-D-to-Part-91Section1.“Section4”

(a)

Except as specified in Section 9 of this appendix, authority for an operatorto conduct flight in airspace where RVSM is applied is issued in operations specifications, a Letter ofAuthorization, or management specifications issued under subpart K of this part, as appropriate. To issue an RVSM authorization under this section, the Administrator must find that the operator's aircraft have been approved in accordance with Section 2 ofthis appendix and the operator complies with this section.

(b)

(c)

Except as specified in Section 9 of this appendix, an applicant seeking authorization to operate within RVSM airspace must apply in a form and manner prescribed by the Administrator. The application must include the following:

(1)  [Reserved]

(2)   For an applicant who operates under part 121 or 135 of this chapter or under subpart K of this part, initial and recurring pilot training requirements.

(3)   Policies and procedures: An applicant who operates under part 121 or 135 of this chapter or under subpart K of this part must submit RVSM policies and procedures that will enable it to conduct RVSM operations safely.

In a manner prescribed by the Administrator, an operator seeking authorization under this section must provide evidence that:

(1)   It is capable to operate and maintain each aircraft or aircraft group for which it applies for approval to operate in RVSM airspace; and

(2)  Each pilot has knowledge of RVSM requirements, policies, and procedures sufficient for the conduct of operations in RVSM airspace.

Section 4. RVSM Operations

(a) (b)

Each person requesting a clearance to operate within RVSM airspace shall correctly annotate the flight plan filed with air traffic control with the status ofthe operator and aircraft with regard to RVSM approval. Each operator shall verify RVSM applicability for the flight planned route through the appropriate flight planning information sources.

No person may show, on the flight plan filed with air traffic control, an operator or aircraft as approved for RVSM operations, or operate on a route or in an area where RVSM approval is required, unless:

(1)   The operator is authorized by the Administrator to perform such operations in accordance with Section 3 or Section 9 ofthis appendix, as applicable.

(2)  The aircraft—

(i)   Has been approved and complies with Section 2 this appendix; or (ii)   Complies with Section 9 of this appendix.

(3)  Each pilot has knowledge of RVSM requirements, policies, and procedures sufficient for the conduct of operations in RVSM airspace.

14 CFRAppendix-D-to-Part-91Section1.“Section4”(enhanceddisplay)

page 241of243

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

Section 5. Deviation Authority Approval

14 CFRAppendix-D-to-Part-91Section1.“Section4”

The Administrator may authorize an aircraft operatorto deviate from the requirements of §§ 91.180 or 91.706 for a specific flight in RVSM airspace if—

(a) (b)

The operator submits a request in a time and manner acceptable to the Administrator; and

At the time of filing the flight plan for that flight, ATC determines that the aircraft may be provided appropriate separation and that the flight will not interfere with, or impose a burden on, RVSM operations.

Section 6. Reporting Altitude-Keeping Errors

Each operator shall report to the Administrator each event in which the operator's aircraft has exhibited the following altitude-keeping performance:

(a)

Total vertical error of 300 feet or more;

(b) (c)

Altimetry system error of 245 feet or more; or Assigned altitude deviation of 300 feet or more.

Section 7. Removal or Amendment of Authority

The Administrator may prohibit or restrict an operator from conducting operations in RVSM airspace, if the Administrator determines that the operator is not complying, or is unable to comply, with this appendix or subpart H of this part. Examples of reasons for amendment, revocation, or restriction include, but are not limited to, an operator's:

(a)

Committing one or more altitude-keeping errors in RVSM airspace;

(b) (c)

Failing to make an effective and timely response to identify and correct an altitude-keeping error; or Failing to report an altitude-keeping error.

Section 8. Airspace Designation

RVSM may be applied in all ICAO Flight Information Regions (FIRs).

Section 9. Aircraft Equipped With Automatic Dependent Surveillance—Broadcast Out

An operator is authorized to conduct flight in airspace in which RVSM is applied provided:

(a)

The aircraft is equipped with the following:

(1)   Two operational independent altitude measurement systems.

14 CFRAppendix-D-to-Part-91Section1.“Section4”(enhanceddisplay)

page 242of243

(2)

(3)

(4) (5)

14 CFRPart91(uptodateas  of1/07/2026) General OperatingandFlightRules

14 CFRAppendix-D-to-Part-91Section1.“Section4”

At least one automatic altitude control system that controls the aircraft altitude—

(i) (ii)

Within a tolerance band of ±65 feet about an acquired altitude when the aircraft is operated in straight and level flight under nonturbulent, nongust conditions; or

Within a tolerance band of ±130 feet under nonturbulent, nongust conditions for aircraft for which application for type certification occurred on or before April 9, 1997, that are equipped with an automatic altitude control system with flight management/performance system inputs.

An altitude alert system that signals an alert when the altitude displayed to the flightcrew deviates from the selected altitude by more than—

(i)

(ii)

±300 feet for aircraft for which application for type certification was made on or before April 9, 1997; or

±200 feet for aircraft for which application for type certification is made after April 9, 1997.

A TCAS II that meets TSO C-119b (Version 7.0), or a laterversion, if equipped with TCAS II, unless otherwise authorized by the Administrator.

Unless authorized by ATC or the foreign country where the aircraft is operated, an ADS-B Out system that meets the equipment performance requirements of § 91.227 of this part. The aircraft must have its height-keeping performance monitored in a form and manner acceptable to the Administrator.

(b)  The altimetry system error (ASE) of the aircraft does not exceed 200 feet when operating in RVSM airspace.

[Docket 28870, 62 FR 17487, Apr. 9, 1997, as amendedbyAmdt. 91-261, 65 FR 5942, Feb. 7, 2000;Amdt. 91-271, 66 FR 63895, Dec. 10, 2001; Amdt. 91-274, 68 FR 54584, Sept. 17, 2003;Amdt. 91-276, 68 FR 70133, Dec. 17, 2003; Docket FAA-2015-1746, Amdt. 91-342, 81 FR 47017, July20, 2016;Docket FAA-2016-9154, Amdt. 91-348, 82 FR 39664, Aug. 22, 2017; FAA-2017-0782, Amdt. 91-354, 83 FR 65492, Dec. 21, 2018]

14 CFRAppendix-D-to-Part-91Section1.“Section4”(enhanceddisplay)

page 243 of243